3GPP TSG RAN meeting #97-e									RP-222159

Electronic Meeting, September 12 - 16, 2022

## Status Report to TSG

Agenda item:			9.3.3.3

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_mobile_IAB |  |  |  |  |
| Unique ID | 941009 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-221815 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 12/2023 | Performance part: 06/2024 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 5% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN3 |
| --- | --- | --- |
| Rapporteur | Name | Georg Hampel |
|  | Company | Qualcomm Incorporated |
|  | Email | ghampel@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

Agreements on basic aspects:

- The method of not broadcasting “iab-Support” indication, is sufficient to prevent other IAB-node from accessing mobile IAB (without further spec impact).

- R2 assumes RACH-less procedure may be considered for on-board RRC_CONNECTED UEs, which are to be handed over together with the mobile IAB-node (would depend also on the assumptions for UL synch).

Agreements on group mobility

- R2 assumes that CHO or delayed RRC config could be the baseline for group mobility (FFS if could be applicable for mobility of IAB MT), i.e. with a preparation in advance (not immediately) of the execution.

Agreements on multihop

- R2 assumes that Mobile IAB connecting to a stationary (intermediate) IAB node is/can be supported. R2 assumes this can be supported with no (or limited) impact.

Endorsements for discussion in next meeting:

- The following Points are Endorsed, i.e. for the plan for next meeting (after one round of discussion at R2 119-e):

  - P1: RAN2 to discuss scenarios, if and where enhancements to cell (re-)selection to/from the mobile IAB-node apply, e.g. based on mobile IAB-node broadcast parameter (this point doesn’t preclude other potential usage of Bcast info).

  - P2: Can discuss whether The mobile IAB-MT need to send a mobile-IAB indication (capability or mobility) to the IAB-donor-CU,

  - P3: For “dual-DU-way” of doing full migration, RAN2 may discuss whether the legacy UE should see the two logical cells/DUs as separate or same physical cell(s), and what procedure(s) the legacy UE needs to perform in either case.

  - P4: RAN2 may discuss whether there are issues with PCI partitioning that needs to/can be addressed (to be used in applicable scenario), if any found within R2 scope. May discuss need for and feasibility from R2 point of view of a dynamic PCI change mechanism. May also discuss whether enhancements to/vs current UE/MT reporting are useful/necessary to improve PCI collision detection.

  - P5: RAN2 may discuss whether there is a problem of RACH configuration collision between mobile IAB and stationary network from RAN2 perspective and/or whether RAN2 should ask RAN1 to consider RAN1-related aspects.

#### 2.2.2	Remaining Open issues

7% of the items defined in the RAN2 WID objectives have been accomplished. All objectives from WID on enhancements to IAB for NR require further work.

## 2.3	RAN3

#### 2.3.1	Agreements

Agreements on general topics:

- Rel-16/17 IAB can support mobile IAB-node connectivity to a stationary intermediate node.

- Enhancements/optimizations that are specific to the scenario where the mobile IAB-node connects to a stationary (intermediate) IAB-node are deprioritized.

- Optimizations/enhancements that are specific to the scenario where the mobile IAB-node connects directly to a donor-DU are deprioritized.

- RAN3 will not discuss separate authorization for mobile IAB without request by SA2.

- Mobility of dual-connected IAB-nodes is down prioritized in Rel18.

Agreements on support for IAB-node mobility:

- As already supported in Rel17, a mobile IAB-MT and its co-located mobile IAB-DU may be served by different donor CUs.

- The mobile IAB donor that the co-located IAB-DU connects to may remain unchanged after the IAB-MT HO.

- (1-3) RAN3 to discuss whether a mobile IAB-DU can execute inter-donor migration, while the co-located mobile IAB-MT stays connected to the same donor before and after the mobile IAB-DU migration.

- RAN3 to discuss whether a mobile IAB-DU can execute inter-donor migration, while the co-located mobile IAB-MT executes inter-donor migration.

- When IP connectivity between target IAB-donor DU and source IAB-donor CU is available, and when Xn connectivity between source and target donor CU is available, the Rel-17 partial migration is used as baseline for supporting the F1 transport migration and inter-donor routing when an mobile IAB-DU and its co-located mobile IAB-MT are connected to different donor CUs.

- The mobile IAB-node may perform multiple consecutive partial migrations without inter-donor migration of its mobile IAB-DU.

- RAN3 to discuss how inter-donor topology adaptation can be supported for mobile IAB in absence of Xn and/or inter-donor IP routability.

- Rel17 mechanisms support intra donor CU migration of mobile IAB.

- For DU migration cases, to execute the handover of the served UEs, the mobile IAB-node concurrently supports two logical mobile IAB-DUs, which have F1AP associations with the source CU and the target CU, respectively.

- The UEs connected to the mobile IAB-node are handed over from the cell of the logical mobile IAB-DU (i.e., the source logical mobile IAB-DU) that has an F1AP association with the source CU to the cell of the logical mobile IAB-DU (i.e., the target logical mobile IAB-DU) that has an F1AP association with the target CU.

- RAN3 to discuss whether a mobile IAB node may be configured with multiple configurations, each corresponding to a different target donor, that can be activated upon fulfillment of certain condition(s). The details of the configurations are FFS.

Agreements on mobility enhancements:

- For group mobility enhancement, RAN3 to discuss the benefit and whether to support signaling of information related to multiple UE contexts in a single message, during e.g. the handover preparation, path switch, and context release procedures.

- The donor CU should know that the IAB node is “mobile”.

- RAN3 to discuss whether the target IAB-donor should know the migrating IAB-node is “mobile IAB-node” from the source IAB-donor.

- RAN3 to discuss whether to support means to identify onboard UEs.

Agreements on mitigation of interference:

- PCI space partitioning via OAM configuration can be used in some cases for avoidance of PCI collisions.

- From RAN3 perspective, existing mechanism can be used for PCI collision detection in mobile IAB scenario. Further enhancement is FFS.

- RAN3 to discuss whether mobile IAB needs any enhancements to the existing mechanisms for PCI collision avoidance and/or optimization.

- From RAN3 perspective, no enhancements are needed for RACH collision avoidance unless requested by other WGs.

#### 2.3.2	Remaining Open issues

7% of the items defined in the RAN3 WID objectives have been accomplished. All objectives from WID on enhancements to IAB for NR require further work.

## 2.4	RAN4

#### 2.4.1	Agreements

None. Work has not started.

#### 2.4.2	Remaining Open issues

0% of items defined in the RAN4 WID objectives have been accomplished. All objectives from WID on enhancements to IAB for NR require further work.

.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN2#119e:

| Tdoc | Title | Source |
| --- | --- | --- |
| R2-2207121 | Mobility Enhancement of mobile IAB-node and served UEs | Intel Corporation |
| R2-2207122 | Discussion on Migration and PCI handling of mobile IAB-node | Intel Corporation |
| R2-2207124 | Discussion on multi-hop scenario for mobile IAB-node | Intel Corporation, Qualcomm, Huawei, Ericsson, Nokia, InterDigital |
| R2-2207128 | Mobile IAB mobility enhancement | Huawei, HiSilicon |
| R2-2207129 | Full migration and interference mitigation | Huawei, HiSilicon |
| R2-2207185 | Discussion on topology adaptation in mobile IAB scenario | ZTE, Sanechips |
| R2-2207186 | Discussion on group mobility of UEs served by mobile IAB | ZTE, Sanechips |
| R2-2207282 | Workplan for Rel-18 mobile IAB | Qualcomm Inc. (Rapporteur) |
| R2-2207283 | Enhancements for IAB-node mobility | Qualcomm Inc. |
| R2-2207284 | Other enhancements for mobile IAB | Qualcomm Inc. |
| R2-2207421 | Discussion on mobility enhancement in mobile IAB | Apple |
| R2-2207422 | Discussion on RAN2 aspects of inter-donor full migration and mitigation of interference in mobile IAB | Apple |
| R2-2207627 | mIAB - other key issues | Samsung R&D Institute UK |
| R2-2207708 | Mobility enhancements for mobile IAB-node and its served UE | Lenovo |
| R2-2207709 | Discussion on inter-donor full migration of mobile IAB | Lenovo |
| R2-2207816 | Discussion on the enhancement of IAB node mobility | Samsung R&D Institute UK |
| R2-2207826 | Mobility enhancement for mobile IAB | Sony |
| R2-2207827 | PCI collision in mobile IAB | Sony |
| R2-2208103 | Mobility enhancements for mIAB node | Ericsson |
| R2-2208104 | On Migration and Interference mitigation | Ericsson |
| R2-2208242 | IAB mobility | Nokia, Nokia Shanghai Bell |
| R2-2208251 | Consideration on PCI collisions for Mobile IAB | Sharp |
| R2-2208267 | Mobility state of an IAB cell | InterDigital, Inc. |
| R2-2208268 | Group mobility in mobile IAB | InterDigital, Inc. |
| R2-2208291 | Scenarios on mobile IAB topology | Kyocera |
| R2-2208292 | UE handover aspects for mobile IAB | Kyocera |
| R2-2208459 | Discussion on mobile IAB | vivo |
| R2-2208514 | Consideration on potential complexity of a scenario | LG Electronics Inc. |
| R2-2208523 | Concurrent UE handovers resulting from IAB node full migration | LG Electronics |

RAN3#117e:

| TDoc | Title | Source |
| --- | --- | --- |
| R3-224233 | Handling legacy UE in UAM using Mobile-IAB | KT Corp., LG Uplus, SK Telecom, ETRI |
| R3-224237 | Workplan for Rel-18 mobile IAB | Qualcomm Inc. (Rapporteur) |
| R3-224238 | Topology adaptation for mobile IAB | Qualcomm Inc. |
| R3-224239 | Enhancements for mobility of IAB-node and its served UEs | Qualcomm Inc. |
| R3-224240 | PCI collision avoidance for mobile IAB | Qualcomm Inc. |
| R3-224353 | Discussion on the full migration for mobile IAB | Huawei |
| R3-224354 | Discussion on the inter-donor transport for full migration of mobile IAB | Huawei |
| R3-224355 | Discussion on group mobility for mobile IAB and Ues | Huawei |
| R3-224356 | Discussion on RACH optimization for connected UE of mobile IAB | Huawei |
| R3-224357 | Discussion on the interference mitigation | Huawei |
| R3-224375 | Discussion on general mobile IAB aspects | Nokia, Nokia Shanghai Bell |
| R3-224376 | IAB mobility | Nokia, Nokia Shanghai Bell |
| R3-224377 | Discussion on mobile IAB aspects based on dual-DU | Nokia, Nokia Shanghai Bell |
| R3-224378 | Discussion on mobility enhancements | Nokia, Nokia Shanghai Bell |
| R3-224379 | Interference mitigation for mobile IAB | Nokia, Nokia Shanghai Bell |
| R3-224428 | Discussion on multi-hop backhauling for mobile IAB | Lenovo |
| R3-224429 | Inter-donor full migration procedure of mobile IAB | Lenovo |
| R3-224430 | Mobility enhancements for mobile IAB-node and its served UE | Lenovo |
| R3-224431 | (TP to TS 38.401) Support for group mobility of mobile IAB-node. | Lenovo |
| R3-224432 | Interference mitigation of mobile IAB-node mobility | Lenovo |
| R3-224495 | Clarifications of Rel-18 Mobile IAB Work Item Scope | Ericsson |
| R3-224496 | The Migration Procedure for Mobile IAB-Nodes | Ericsson |
| R3-224497 | Enhancements for IAB-Node Mobility IAB | Ericsson |
| R3-224498 | paper | Ericsson |
| R3-224499 | Mitigation of Interference and Reference Signal Collisions for Mobile IAB-Nodes | Ericsson |
| R3-224502 | Workplan for Rel-18 mobile IAB | Qualcomm Inc. (Rapporteur) |
| R3-224504 | Topology adaptation for mobile IAB | Qualcomm Inc. |
| R3-224505 | Enhancements for mobility of IAB-node and its served UEs | Qualcomm Inc. |
| R3-224506 | PCI collision avoidance for mobile IAB | Qualcomm Inc. |
| R3-224704 | Support of intra-m-CU mobility | Fujitsu |
| R3-224705 | Discussion on IAB full migration | Fujitsu |
| R3-224706 | Update of location information for cells of mobile IAB | Fujitsu |
| R3-224707 | Avoidance of resource collisions due to IAB-node mobility | Fujitsu |
| R3-224710 | Discussion on inter-donor full migration in mobile IAB scenario | ZTE |
| R3-224711 | Discussion on migration sequence of full migration procedure | ZTE |
| R3-224712 | Reduction of UE migration in mobile IAB scenario | ZTE |
| R3-224713 | Discussion on location update of UEs served by mobile IAB | ZTE |
| R3-224714 | Discussion on mitigation of interference in mobile IAB scenario | ZTE |
| R3-224767 | Discussion on IAB full migration | Xiaomi |
| R3-224768 | Discussion on group mobility | Xiaomi |
| R3-224769 | Discussion on mitigation of interference | Xiaomi |
| R3-224777 | Discussion on Full Migration of mobile IAB-node | Intel Corporation |
| R3-224786 | Discussion on multi-hop scenario for mobile IAB-node | Intel Corporation, Qualcomm, Huawei, Ericsson, Nokia, InterDigital |
| R3-224825 | Discussion on potential complexity of single-hop and multi-hop scenarios | samsung |
| R3-224826 | Discussion on full migration procedure | samsung |
| R3-224827 | Discussion on mobility enhancements | samsung |
| R3-224828 | Discussion on mitigation of interference | samsung |

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template