3GPP TSG-RAN Meeting #97-e 																			RP-222595

Electronic Meeting, September 12 – 16, 2022

Agenda item:			9.1.5, 10.1.5

Source:	Moderator (RAN4 Chair)

Title:	Email discussion summary for [97e-35-CompanyCR-Band-n77]

Document for:	Information

Introduction

In this email thread we will discussion the CRs for n77 in US and Canada.

The following contributions will be covered.

| TDoc | Title | Source | Type | AI |
| --- | --- | --- | --- | --- |
| RP-222344 | Operation in the n77 frequency range in US and Canada | Nokia, Nokia Shanghai Bell | Other | 9.12 |
| RP-222350 | Extension of operation in the n77 frequency range in US (Cat-C Rel-16) | Nokia, Nokia Shanghai Bell | CR | 9.12 |
| RP-222352 | Extension of operation in the n77 frequency range in US (Cat-A Rel-17) | Nokia, Nokia Shanghai Bell | CR | 9.12 |
| RP-222353 | Extension of operation in the n77 frequency range in Canada | Nokia, Nokia Shanghai Bell | CR | 9.12 |

In this document, we capture comments and conclusions for this email thread.

Topic #1: n77 requirements in US and Canada

Companies’ contributions summary

The background information is provided in RP-222344.

Handling of NS_55 for US and NS_57 for Canada has been discussed for several meetings in RAN4 as well as RAN2 and requirements for non-CA case was already addressed. Although RAN4#104-e addressed CA case, CRs were not agreed. One of the reasons was RAN2 addressed both an n77 specific issue as well as a non-CA/CA NS mapping issue common to all bands and some companies preferred to wait for the RAN2 conclusion before RAN4 agrees relevant CRs.

On the other hand, RAN2#119-e agreed following CRs [1, 2] specific to the n77 CA issue, though the conclusion of the NS mapping issue was postponed. Hence, this contribution provides background of our companion CRs of [3, 4] which  address both US and Canada n77 issues without NC_CA_NS and CA_NS, respectively to complete the issues independently from NS mapping issue based on the agreed RAN2 CRs [1, 2].

Initial round

Comments & responses for Rel-16/17 38.101-1 CRs (RP-222350/352) for n77 in US

Sub-topic 1-1: Comments for 38.101-1 CR RP-222350 for n77 in US

------------------------------------------ Changes ------------------------------------------------------------------------

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 (NOTE 7) | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | Table 5.3.2-1 | N/A |
| … | … | … | … | … | … |
| NS_100 | 6.5.2.4.2 | n1, n2, n3, n5, n8, n18, n25, n26, n65, n66, n80, n81, n84, n86, n89 (NOTE 1) |  |  | Table 6.2.3.1-2 |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: No A-MPR is applied for 5 MHz BWChannel where the lower channel edge is ≥ 1930 MHz,10 MHz BWChannel where the lower channel edge is ≥ 1950 MHz and 15 MHz BWChannel where the lower channel edge is ≥ 1955 MHz. NOTE 3: Applicable when the NR carrier is within 1447.9 – 1462.9 MHz. NOTE 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980 MHz. NOTE 5: Applicable when the NR carrier is within 2545 – 2575 MHz. NOTE 6: This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional emission requirements. NOTE 7: The NS_01 label with the field additionalPmax [7] absent is default for all NR bands. |  |  |  |  |  |

------------------------------------------ Changes ------------------------------------------------------------------------

------------------------------------------ Changes ------------------------------------------------------------------------

6.2A.3.1.2.0	General

Table 6.2A.3.1.2-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable CA band(s) for each CA_NC_NS value. The mapping of NR CA band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2A.3.1.2-2.

Table 6.2A.3.1.2-1: Additional Maximum Power Reduction (A-MPR) for intra-band non-contiguous CA

| CA Network Signalling value | Requirements (clause) | Uplink CA Configuration | A-MPR for sub-blocks in order of increasing uplink carrier frequency |
| --- | --- | --- | --- |
|  |  |  | A-MPR [dB] (clause) |
| CA_NC_NS_01 |  | All applicaple NR CA configurations | N/A |
| CA_NC_NS_04 | 6.5A.2.3.2.1 6.5A.3.3.2.1 | CA_n41(2A) | 6.2A.3.1.2.1 |

When UEs are configured with intra-band non-contiguous CA in n77 with NS_01 for an uplink component carrier in the range 3700-3980 MHz and NS_55 for an uplink component carrier in the range 3450-3550 MHz in FrequencyInfoUL-SIB, A-MPR does not apply to the UEs regardless of which value of additionalSpectrumEmission in FrequencyInfoUL is used for the carrier in the range of 3450-3550 MHz.

------------------------------------------ Changes ------------------------------------------------------------------------

Companies are invited to provide the general comments, including comments on justification part, whether the WI is needed, how to handle the work, in the follow table.

| Company | Comments |
| --- | --- |
| Qualcomm | We recognize there is misalignment between RAN2 agreed CR’s and the lack of agreement of RAN4 CR’s. Nonetheless, we do not agree to these CR’s presented at RAN when further discussion is needed in RAN4. In our view, it would be better to wait for the discussion and common understanding in RAN4 and then propose and agree RAN4 CR’s to the RAN4 TS, rather than a company CR at RAN now. We also understand that there is no immediate urgency from the interested operators to agree to CR’s at this meeting. |
| Apple | 1. For NOTE 6 in Table 6.2.3.1-1, is it necessary to remove “spurious” and “A-MPR”? The “spurious” has been used in 6.5.5.3 clause title for NS values associated additional spurious emissions requirements. A-MPR requirements are also associated with NS values. 2. We do not have strong view on the added paragraph in 6.2A.3.1.2.0. We understand that it is meant to align with RAN2 specifications change in additionalSpectrumEmission in FrequencyInfoUL field descriptions. On the other hand, with RAN2 specifications change and NOTE 6 in Table 6.2.3.1-1, we think the expected UE behavior should be clear enough without the added paragraph. |
| T-Mobile USA | We agree with Qualcomm that this should be discussed in RAN4, not as company CRs at RAN Plenary. |
| ZTE | For the first change on NOTE 6, we are fine since it is aligned with NOTE 2 in Table 6.2.3.1-1A, where the term “additional emission requirements” is used. For the second change, we don’t have any strong concern since it is helpful to have the specs to reflect the latest agreements. However, we understand this is an intermediate version, and the added texts will be updated and moved to Table 6.2A.3.1.2-1 according to the corresponding conclusion. |
| AT&T | We don’t agree with QC that there is no immediate urgency from interested operators to agree CR’s at this meeting. This interested operator would like to see the corresponding core requirements clearly identified in the RAN4 specification based on the RAN2 agreed CRs. We do not see the need to update NOTE 6 although the word “spurious” could be removed but we don’t have a strong opinion on that part. NOTE 6 was agreed by RAN4 to convey that additional RF conformance tests were not required since this NS value is used for barring purposes only. As such, we prefer not to remove the text “and maximum output power reduction” from NOTE 6. Even though “N/A” is shown in the A-MPR column, the table note text adds value to make it clear to RAN5 that no additional RF conformance tests are required as in the case of NS_06. Maybe we can modify the table note as follows. “NOTE 6: This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements as this NS value is used for barring purposes only.” For clause 6.2A.3.1.2.0, we support having a clear core requirement identified in the RAN4 core specification at this RAN meeting given that the RAN2 core specification has been updated to account for the UL CA configuration issue. We also agree with ZTE that it is helpful for the specs to reflect the latest agreements on n77 UL CA configuration, and that no relaxation would apply. It would also be good to have similar text added to the clause as above to make it very clear that additional RF conformance tests are not required for these cases when NS values have been used for barring purposes only. The core requirement text in this intermediate version can be updated later once the general NS mapping issue is resolved in RAN4. |
| Huawei | For note 6, we don’t think it is necessary to remove “spurious” and “max output power reduction” as it would not cause ambiguity with these wording, but no strong view. For the changes under clause 6.2A.3.1.2.0, in general we are fine with the proposed wording as it is aligned with previous agreement. |
| Samsung | For note 6, share similar view as Apple and Huawei. For 2nd change, we understand there is ongoing discussion in RAN4 on how to mapping NS value from RAN2 to RAN4 with CA cases. If there is a urgent demand from operators, we can accept to approve this “intermediate” version of CR with a Note under meeting report: further update maybe required once RAN4 has draw conclusion on the generic issue of NS value mapping from RAN2 to RAN4 under CA cases. |
| MediaTek | For note 6, we think the original wordings are clear. Probably, to add “as this NS value is used for barring purposes only” can make note 6 clearest. To remove wordings of “spurious” and “max output power reduction” is unnecessary. For the 2nd change, we share similar view as Samsung. |
| OPPO | We understand there is desire to align between RAN2 and RAN4, but we prefer to do it in RAN4 instead of RAN plenary since this need technical discussion anyway. |
| Nokia | The reason for the CR was to finish n77 issue since it’s been discussed for several meetings and a corresponding RAN2 CRs were agreed already. We understand that NS mapping discussion is on-going, but it’s unfortunate that we cannot close n77 issue since NS mapping issue itself is a common issue for all the bands and band configuration. For NOTE 6, We are ok to keep the original wordings. We just took into account the proposal from R4-2212769. Regarding the proposal by AT&T, we think that it would be more suitable to put that behaviour into RAN2 spec and actually, it’s already in 38.331 as generic UE behaviour meaning that if a UE cannot understand an NS, the cell is considered as barred, though it depends on how others think. To Qualcomm and others. We understand the comments. But NS mapping issue itself is independent from n77. If we follow the direction that QC wants to follow, unless we solve the NS mapping issue, n77 issue cannot be solved. It is very unfortunate situation. To Apple, We also thought not changing 38.101-1 is one of the options. RAN2 spec says UE shall be able to deal with whichever value of 0 (or absent) or 1 is signalled, but it doesn’t say what the requirement(s) is. That’s why we thought it would be better to clarify it. |
| Ericsson | Nokia company CR is not technically correct on the A-MPR. The UE does no read the SI of the SCell (FrequencyInfoUL-SIB), which contains NS_55. But the UE is configured with the NS value for SCell in dedicated signaling upon CA configuration. The "regardless of which value is signalled" is also ambiguous, only NS_01 or NS_55 can be signalled. We therefore suggest to modify the wording in the Nokia CR as follows: “For UEs configured with intra-band non-contiguous CA in n77 and with NS_01 indicated or configured for an uplink component carrier the range 3700-3980 MHz and NS_55 for another uplink component carrier in the range 3450-3550 MHz, the allowed A-MPR is according to CA_NC_NS_01.” The above updated wording will cover the case, when: NS_01 is on one carrier and NS_55 is on another (this exception is now covered in 38.331). The alternative with NS_01 configured on both carriers (with a different NS in the SIB1 for one of the carriers) is already covered by existing specification. |
| Intel | Regarding Note 6 we do not have a strong opinion. In our view the original wording is OK, but also the revised wording is acceptable. Regarding the new paragraph in 6.2A.3.1.2.0 we can accept the changes in this meeting as an intermediate step, noting the possibility that it may need some revision at next RAN4 when the more general mapping issue is resolved. For the wording, we think that Ericsson's suggested text is clearer. Specifically it seems better to refer to the already defined CA_NC_NS_01 instead of introducing new text to say the same thing. |

Comments & responses for Rel-17 38.101-1 CR (RP-222353) for n77 in Canada

Sub-topic 1-2: Comments for 38.101-1 CR RP-222353 for n77 in Canada

------------------------------------------ Changes ------------------------------------------------------------------------

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 (NOTE 8) | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | Table 5.3.2-1 | N/A |
| … | … | … | … | … | … |
| NS_100 | 6.5.2.4.2 | n1, n2, n3, n5, n8, n18, n25, n26, n65, n66, n80, n81, n84, n86, n89 (NOTE 1) |  |  | Table 6.2.3.1-2 |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: No A-MPR is applied for 5 MHz BWChannel where the lower channel edge is ≥ 1930 MHz,10 MHz BWChannel where the lower channel edge is ≥ 1950 MHz and 15 MHz BWChannel where the lower channel edge is ≥ 1955 MHz. NOTE 3: Applicable when the NR carrier is within 1447.9 – 1462.9 MHz. NOTE 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980 MHz. NOTE 5: Applicable when the NR carrier is within 2545 – 2575 MHz. NOTE 6: This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements. NOTE 7: The 1Tx architecture is assumed. For power class 2 UE indicating txDiversity-r16 [TS 38.306], the additional relaxation of [2] dB is applicable. NOTE 8: The NS_01 label with the field additionalPmax [7] absent is default for all NR bands. NOTE 9: 5 MHz only applies to n90, not n41 NOTE 10: This NS value is applicable for cells in the range 3650-3980 MHz for operations in Canada. This NS value does not indicate any additional emission requirements. |  |  |  |  |  |

------------------------------------------ Changes ------------------------------------------------------------------------

------------------------------------------ Changes ------------------------------------------------------------------------

6.2A.3.1.1	 UE additional maximum output power reduction for Intra-band contiguous CA

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [7]. Relation between NR CA band and NR frequency band is specified in Table 5.2A.1-1.

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2A.1.5-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2A.2.4. In absense of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2A.3.1.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable CA band(s) for each CA_NS value. The mapping of NR CA band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.1.1-2.

Table 6.2A.3.1.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR CA Band | Aggregated channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| CA_NS_01 |  | Table 5.2A.1-1 | All applicaple NR CA bands | All applicaple NR CA configurations | N/A |
| CA_NS_04 | 6.5A.2.3.1.1 6.5A.3.3.1.1 | CA_n41 | Table 5.5A.1-1 | 6.2A.3.1.1.1 | 6.2A.3.1.1.1 |
| CA_NS_27 | 6.5A.2.3.1.2 6.5A.3.3.1.2 | CA_n48 | Table 5.5A.1-1 | 6.2A.3.1.1.2 | 6.2A.3.1.1.2 |
| CA_NS_46 | 6.5A.3.3.1.3 | CA_n7 | Table 5.5A.1-1 | 6.2A.3.1.1.3 | 6.2A.3.1.1.3 |

[The CA_NS_01 label with the field additionalPmax [7] absent is default for all NR bands.]

When UEs are configured with intra-band contiguous CA in n77 with NS_01 for an uplink component carrier in the range 3450-3650 MHz and NS_57 for an uplink component carrier in the range 3650-3980 MHz in FrequencyInfoUL-SIB, A-MPR does not apply to the UEs regardless of which value of additionalSpectrumEmission in FrequencyInfoUL is used for the carrier in the range of 3650-3980 MHz

------------------------------------------ Changes ------------------------------------------------------------------------

Companies are invited to provide the general comments, including comments on justification part, whether the WI is needed, how to handle the work, in the follow table.

| Company | Comments |
| --- | --- |
| Qualcomm | We recognize there is misalignment between RAN2 agreed CR’s and the lack of agreement of RAN4 CR’s. Nonetheless, we do not agree to these CR’s presented at RAN when further discussion is needed in RAN4. In our view, it would be better to wait for the discussion and common understanding in RAN4 and then propose and agree RAN4 CR’s to the RAN4 TS, rather than a company CR at RAN now. We also understand that there is no immediate urgency from the interested operators to agree to CR’s at this meeting. |
| Apple | 1. For NOTE 10 in Table 6.2.3.1-1, is it necessary to remove “spurious” and “A-MPR”? The “spurious” has been used in 6.5.5.3 clause title for NS values associated additional spurious emissions requirements. A-MPR requirements are also associated with NS values. 2. We do not have strong view on the added paragraph in 6.2A.3.1.1. We understand that it is meant to align with RAN2 specifications change in additionalSpectrumEmission in FrequencyInfoUL field descriptions. On the other hand, with RAN2 specifications change and NOTE 10 in Table 6.2.3.1-1, we think the expected UE behavior should be clear enough without the added paragraph. 3. If the added paragraph is indeed needed, we think it also needs to be added in clause 6.2A.3.1.2.0 for intra-band non-contiguous UL CA as UL CA in Canada Band n77 may not always be contiguous. |
| T-Mobile USA | We agree with Qualcomm that this should be discussed in RAN4, not as company CRs at RAN Plenary. One question we have is why the new text is only being added for contiguous CA and not non-contiguous UL CA? The header levels are incorrect for the sub-clauses in 6.2A.3, and they should also be fixed. |
| ZTE | Similar comments as in previous CR. |
| AT&T | Similar comments as with the n77 USA CR. Any final agreed text should be aligned. We also agree with Apple that the intra-band non-contiguous UL CA case should also be accounted for from a core requirement perspective but perhaps the Canadian operator(s) could comment if the intent is to always use intra-band contiguous UL CA. |
| Huawei | Similar comments in 1.2.1. |
| Samsung | Similar comments in 1.2.1. |
| MediaTek | Similar comments as in previous CR. |
| OPPO | Similar comments in 1.2.1. |
| Nokia | To Apple and AT&T, thank you for the comment on the case intra band non-contiguous UL CA for n77 in Canada. We agree that n77(2A) is used in Canada, NS_57 and accompanied texts are needed to be written in NC_CA as well. |
| Ericsson | Same comments as in 1.2.1. |

Summary

Sub-topic 1-1: Comments for 38.101-1 CR RP-222350 for n77 in US

12 companies made comments. Among them 3 companies would like to discuss the technique details in RAN4 meeting rather than in RAN. The other companies could accept the CR but had comments on the changes in the CRs.

Basically, there would be two options to move forward. Based on most companies’ views, the moderator would like to propose

- Proposal 1-1: for 38.101-1 CR RP-222350 for n77 in US

  - Option 1:

    - For Change #1 for section 6.2.3, keep original NOTE 6 unchanged and add clarification for NS values

      - NOTE 6:	This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements as this NS value is used for barring purposes only.

    - For Change #2 for section 6.2A.3.1.2, add clarification for NS values on top of current changes

      - When UEs are configured with intra-band non-contiguous CA in n77 with NS_01 for an uplink component carrier in the range 3700-3980 MHz and NS_55 for an uplink component carrier in the range 3450-3550 MHz in FrequencyInfoUL-SIB, A-MPR does not apply to the UEs regardless of which value of additionalSpectrumEmission in FrequencyInfoUL is used for the carrier in the range of 3450-3550 MHz. The additional RF conformance tests are not required for these cases when NS values have been used for barring purposes only.

      - Further update is needed for Change #2 in future WG meetings.

  - Option 2:

    - For Change #1 for section 6.2.3, keep original NOTE 6 unchanged and add clarification for NS values

      - NOTE 6:	This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements as this NS value is used for barring purposes only.

    - For Change #2 for section 6.2A.3.1.2, replace the Change # 2 by the following alternative

      - For UEs configured with intra-band non-contiguous CA in n77 and with NS_01 indicated or configured for an uplink component carrier the range 3700-3980 MHz and NS_55 for another uplink component carrier in the range 3450-3550 MHz, the allowed A-MPR is according to CA_NC_NS_01.

Sub-topic 1-2: Comments for 38.101-1 CR RP-222353 for n77 in Canada

Companies’ views are similar to sub-topic 1-1.

In addition, Apple commented if the added paragraph is indeed needed, we think it also needs to be added in clause 6.2A.3.1.2.0 for intra-band non-contiguous UL CA as UL CA in Canada Band n77 may not always be contiguous. T-Mobile USA and AT&T had similar comments. According to response from proponent, the similar change should also be done for NC CA.

- Proposal 1-2: for 38.101-1 CR RP-222353 for n77 in Canada

  - The similar wording agreed for sub-topic 1-1 can be reused

  - The similar changes in Section 6.2A.3.1.1 should also be made for NC CA.

Intermediate round

Comments & responses

Sub-topic 1-1: Comments for 38.101-1 CR RP-222350 for n77 in US

In the intermediate round, please comment on Proposal 1-1. The moderator wonder if companies still have strong view to go back to WG meeting to address this issue.

| Company | Comments |
| --- | --- |
| XXX |  |
| Qualcomm | The agreement in RAN4 (R4-2214409) was to treat both a general solution and the n77 solution at RAN4 #104-bis-e. If there truly is an “urgent” need by an operator, we would not object to inserting an intermediate solution from RAN, but otherwise, we still prefer to have the technical solution in the WG. |
| AT&T | We think that not having clear UE performance requirements in the RAN4 core spec at this time is urgent for the band n77 UL CA configuration issue given that RAN2 has concluded on the UL CA issue and the RAN4 specification does not indicate the expected performance level for the UE. We are OK with the suggestions from other companies in the first round to agree on a CR at this meeting and include a note in the meeting report that further updates maybe required once RAN4 has concluded on the generic issue of NS value mapping from RAN2 to RAN4 under CA cases. We think that this note is adequately captured in either option presented by the moderator concerning change #2 but this text could also be copied into the official meeting report. We support either Option 1 or Option 2 with a slight preference to Option 2. However, Option 2 Change #2 should also include the same clarification statement that no additional RF conformance tests are required. |
| Nokia | For NOTE 6, we are ok to add “as this NS value is used for barring purposes only” to the tail of the NOTE 6 if other companies are OK. We, however, suggest that keep the NOTE 6 as it is in this TSG RAN to avoid further discussion on this since this is not the main point of the CR. For the text to be inserted in section 6.2A.3.1.2, we still prefer our original proposal. We understand using CA_NS_01 as proposed by Ericsson. But now NS mapping discussion is on-going. Thus, we intentionally aimed at avoiding CA_NS and CA_NC_NS in this CR. Also, if we just use additionalSpectrumEmission, it’s not clear which additionalSpectrumEmission in SIB or dedicated signalling. We are open to discuss if further clarification is needed or not in the future RAN4 meetings. With all above reasons, we provided the text in the CR. Hence, we would like to keep it in this TSG RAN. To AT&T: we may misunderstand the comment, but our understanding is that the proposed text in section 6.2A.3.1.2 has no relation with cell barring. The text is just what UE needs to meet in case UL CA is configured. Overall, our suggestion in this meeting is as follows. - For Change #1 for section 6.2.3, keep original NOTE 6 unchanged and add clarification for NS values if everyone is OK with it. If not, we just keep NOTE 6 as it is. - NOTE 6: This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements as this NS value is used for barring purposes only. - For Change #2 for section 6.2A.3.1.2, add clarification for NS values on top of current changes - When UEs are configured with intra-band non-contiguous CA in n77 with NS_01 for an uplink component carrier in the range 3700-3980 MHz and NS_55 for an uplink component carrier in the range 3450-3550 MHz in FrequencyInfoUL-SIB, A-MPR does not apply to the UEs regardless of which value of additionalSpectrumEmission in FrequencyInfoUL is used for the carrier in the range of 3450-3550 MHz. The additional RF conformance tests are not required for these cases when NS values have been used for barring purposes only. - Note that according to the outcome of NS mapping discussion, the above change(s) can be further polished in the future RAN4 meetings. |
| Apple | We are fine with Nokia’s proposal above. |
| Samsung | Either option 1 and option 2 fine for us with the condition that further updated in future RAN4 meetings not precluded |
| MediaTek | We are fine with option 1 and slightly prefer option 2. We share similar view as Samsung that further updated in future RAN4 meetings is not precluded |
| OPPO | As for the change in Note6, we are fine with the additional clarification. But we think the wording need be improved since the cell is barred for UE not supporting new NS value but not barred for UE supporting new NS value i.e. the new NS value is for cell access control but not just to bar a cell. So we’d better say “as this NS value is used for barring cell access purposes only” instead of “as this NS value is used for barring purposes only” As for the inserted text in section 6.2A.3.1.2, the sprit is already clear in RAN2’s CR and the text itself is kind of intermediate procedure since there could be further clarification when general solution is concluded in future. Considering there is also debate between two versions of text, we think it should be better not to figure out the detail wording in plenary but in RAN4. |
| ZTE | Nokia’s proposal is fine to us, though both options can work, however, if Plenary eventually chooses one of the options in this week, RAN4 could still further polish the texts since the new proposals are not sufficiently discussed here. |
| vivo (Xiaodong) | Actually, we prefer leaving this CR discussion in RAN4 group, i.e., how is the wording difference for “A-MPR does not apply to the UEs regardless of which value of additionalSpectrumEmission” and “the allowed A-MPR is according to CA_NC_NS_01”. However, we are also ok to agree this CR in RAN and update it in RAN4 based no operator requirement. |
| Ericsson | We support Option 2, change #2. The change #1 of Option 2 is not needed, it suffices to specify that the NS_55 value does not indicate any additional spurious (or even better 'unwanted') emission requirements. The general requirements (such as MPR) still apply when NS_55 is indicated in a serving cell, the addition "used for barring purposes only" could suggest something else. Option 1 is not technically correct, the UE does not obtain the NS value for the SCell from SI (FrequencyInfoUL-SIB), it is configured with the NS value for the SCell by dedicated signaling of common parameters. Moreover, the formulation "regardless of which value is used" could suggest that values other than NS_01 and NS_55 could be used. While we are fine to resolve it at the RAN. But given the views it might be difficult to converge in RAN. Therefore, it is better to task RAN4 to resolve this and finalize the 38.101-1 CR in Q4. |

Sub-topic 1-2: Comments for 38.101-1 CR RP-222353 for n77 in Canada

In the intermediate round, please comment on Proposal 1-2.

| Company | Comments |
| --- | --- |
| XXX |  |
| Qualcomm | Same as above |
| AT&T | Similar comments to above. |
| Apple | Same as our comment on Proposal 1-1. |
| Samsung | Proposal 1-2 fine for us. |
| MediaTek | Similar comments as above. |
| ZTE | Similar comments to above. |
| vivo | Same as above |
| Ericsson | Similar change for the Canadian case (Option 2, change #2) but also covering the contiguous case across 3650 MHz. |

Summary

10 companies made the comments. 1 company proposed to leave it to RAN4 discussion. Considering the input from operators, other companies were OK to approve the CRs with intermediate solution and add a note in meeting minutes to allow RAN4 further work on the wording.

Companies’ views on Option 1 or Option 2 are diverse. 2 companies prefer Option 1 while 3 companies prefer Option 2. In the final round, we need try to harmonize those two options.

Based on the companies’ comment, the moderator modifies the options a bit:

- Proposal 1-1: for 38.101-1 CR RP-222350 for n77 in US

  - Option 1:

    - For Change #1 for section 6.2.3, keep original NOTE 6 unchanged and add clarification for NS values

      - NOTE 6:	This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements as this NS value is used for barring cell access purposes only.

    - For Change #2 for section 6.2A.3.1.2, add clarification for NS values on top of current changes

      - When UEs are configured with intra-band non-contiguous CA in n77 with NS_01 for an uplink component carrier in the range 3700-3980 MHz and NS_55 for an uplink component carrier in the range 3450-3550 MHz in FrequencyInfoUL-SIB, A-MPR does not apply to the UEs regardless of which value of additionalSpectrumEmission in FrequencyInfoUL is used for the carrier in the range of 3450-3550 MHz. The additional RF conformance tests are not required for these cases when NS values have been used for barring purposes only.

      - Further update is needed for Change #2 in future WG meetings. Note that according to the outcome of NS mapping discussion, the above change(s) can be further polished in the future RAN4 meetings.

  - Option 2:

    - For Change #1 for section 6.2.3, keep original NOTE 6 unchanged and add clarification for NS values

      - NOTE 6:	This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements as this NS value is used for barring cell access purposes only.

    - For Change #2 for section 6.2A.3.1.2, replace the Change # 2 by the following alternative

      - For UEs configured with intra-band non-contiguous CA in n77 and with NS_01 indicated or configured for an uplink component carrier the range 3700-3980 MHz and NS_55 for another uplink component carrier in the range 3450-3550 MHz, the allowed A-MPR is according to CA_NC_NS_01. The additional RF conformance tests are not required.

      - Note that according to the outcome of NS mapping discussion, the above change(s) can be further polished in the future RAN4 meetings.

Proposal 1-2 seems acceptable.

- Proposal 1-2: for 38.101-1 CR RP-222353 for n77 in Canada

  - The similar wording agreed for sub-topic 1-1 can be reused

  - The similar changes in Section 6.2A.3.1.1 should also be made for NC CA.

Final round

Comments & responses

Companies are invited to provide comments and responses about how to harmonize Option 1 and Option 2 in Proposal 1-1 in Section 1.3.2 in the following table.

| Company | Comments |
| --- | --- |
| T-Mobile USA | For Proposal 1-1: We don’t have a strong view on Option 1 vs. Option 2, but for both options we’re concerned about the wording change from “barring” to “cell access.” Barring is spelled out in 38.331 if the UE does not support any NS values (additionalSpectrumEmission) as follows: 2> else: 3> consider the cell as barred in accordance with TS 38.304 [20]; and 3> perform barring as if intraFreqReselection is set to notAllowed; Where would “cell access” be defined? The purpose of NS_55 is literally to bar UEs that do not report extendedBand-n77-r16 from accessing 3450-3550 MHz in the USA. There are “cell access related” information elements in 38.331, but additionalSpectrumEmission is not one of them. Therefore, we think that saying the NS value is for “cell access” would be confusing. We don’t think the text at the end of Note 6 is necessary, but if it is added we think that “barring” is better than “cell access.” Editorial comments: A minor editorial suggestion: the changed text in both Option 1 and Option 2 says “The additional RF conformance tests are not required.” This should probably say “Additional RF conformance tests are not required.” The changes in Table 6.2.3.1-1 say “This NS value does not indicate any additional spurious emission and maximum output power reduction requirements” but the changes in 6.2A.3 only talk about no A-MPR and don’t mention emission requirements. These should be consistent, and the fact that there are not additional emissions requirements is the key piece of information. There is no A-MPR because there are no additional emissions requirements. “in FrequencyInfoUL-SIB” should probably be removed as it is unnecessary in 38.101-1, and as Ericsson said in the intermediate round, for the S-Scell the NS value comes from RRC signalling, not the SIB. Also, as we commented in the initial round, the subclause numbers in 6.2A.3 have a lot of problems. 6.2A.3.1.1.1 follows 6.2A.3 with not 6.2A.3.1. Also, the header formatting levels are incorrect on many of the headers in 6.2A.3. These errors should be fixed next quarter as editorial changes. For Proposal 1-1: The most of the same comments above apply, except for the frequency ranges and NS values applicable to Canada. |
| AT&T | For Proposal 1-1: We also do not have a strong view on Option 1 vs. Option 2 as we think that either option makes it clear that A-MPR does not apply for the n77 UL CA case. We would like to see one of the options accepted at this meeting so that the requirement is clear in the RAN4 spec until the general mapping issue is resolved. At which stage, the text will be updated accordingly. Concerning the T-Mobile USA comment on “cell access” versus “barring”, we also agree that it would be better to use the original proposed text from AT&T. The NS value was introduced for the sole purpose of “barring” legacy roaming UEs and UEs not reporting extendedBand-n77-r16. Perhaps, we can modify it to use “cell barring” which would also be consistent with RAN2 terminology. The term “barring’ seemed to be acceptable to all companies except for one in the intermediate round. We also agree with the editorial suggestion to use “Additional RF conformance tests are not required.” For Proposal 1-2: Similar comments apply for the Canada situation. |
| MediaTek | For Proposal 1-1, we prefer to keep “barring” for both options. We are okay with “barring” or “cell barring”. The same comment as in intermediate round, we are fine with option 1 and slightly prefer option 2. |
| OPPO | There is difference between wording “barring” and “cell access” for the 1st change. I understand operator emphasize the new NS value i.e. NS_55 for USA case is to prevent legacy/roaming UEs to access the new part of the n77. But I guess operators also want new UE supporting this NS value to access the cell, right? To put “barring” here only shows the one side of the coin i.e. technically this new value is to allow or bar UEs to access the new part of the n77 band. Anyway if we are the only company like “cell access” we can accept not to change it. For Note 6 , if people really want to fix it in plenary either option is fine but slightly prefer option2 |
| Nokia | For the text to be inserted in section 6.2A.3.1.2, thanks Ericsson for the comment, perhaps, the first text may bring different understandings. Again, at this stage, we don’t know if it’s appropriate to keep using CA_NS and/or CA_NC_NS, we avoided using it for CA while what we clearly know is what network refers to is additionalSpectrumEmission is absent or present with value 0 or 1 in FrequencyInfoUL. If NS mapping discussion derives a clear conclusion, the text may be modified with simpler expression if necessary, we are open to discuss it. When UEs are configured with intra-band non-contiguous CA in n77 with under cells using NS_01 for an uplink component carrier in the range 3700-3980 MHz and NS_55 for an uplink component carrier in the range 3450-3550 MHz in FrequencyInfoUL-SIB, and if the UEs are configured with intra-band non-contiguous CA in n77, A-MPR does not apply to the UEs regardless of whether which value of additionalSpectrumEmission is absent or present with value 0 or 1 in FrequencyInfoUL is used for the carrier in the range of 3450-3550 MHz. The additional RF conformance tests are not required |
| Ericsson | Our technical concerns with the Option 1 change #2 remain: the specification text should not suggest that the UE obtains the NS value for the SCell from SI. For Option 1 change #1, it suffices to specify that the NS_55 value does not indicate any additional spurious (or even better 'unwanted') emission requirements. The general requirements still apply when NS_55 is indicated in a serving cell, the addition "used for barring purposes only" or similar could suggest something else and lead to different interpretations. The NS_55 is related to the “extended n77” capability as specified in 38.306. Moreover, in the updated proposal, there is no need to make references to conformance testing, this is RAN5 responsibility. The core requirements also apply with NS_55 indicated/configured (but no additional unwanted emissions requirements). To Nokia: the CA_NS and CA_NC_NS are internal notions mapping the indicated/configured NS on each UL serving cell to additional requirements and an A-MPR if allowed. These should not be confused with the NS indicated in RRC (i.e. an addtionalSpectrumEmission associated with the FBI). Hence for the non-contiguous case the specification should make clear 1. NS_01 indicated/configured on both serving cells, then CA_NS_NC_01 for A-MPR applies as per the existing specification (no change needed) 2. NS_01 indicated/configured on one serving cell and NS_55 on another then CA_NC_NS_01 for A-MPR also applies (Option 2 change 2). There is no need to mention default RRC signaling, absence of an indicated/configured value in the RRC field implies NS_01. |

Summary

Based on the companies’ comments, the moderator updates the proposals below. According to feedback of the final round,

- Option 1: 1 support

  - Support: T-Mobile USA, AT&T, Nokia

- Option 2: 3 support

  - Support: MediaTek (also OK with Option 1 but slightly prefer Option 2), OPPO  (also OK with Option 1 but slightly prefer Option 2), Ericsson

- Either Option 1 or Option 2: 2 companies

  - Support: T-Mobile USA, AT&T

In the moderator’s view, the text is not stable and there is no consensus in the final round. Either making the decision in GTW on final day or task RAN4 to further work on the CR based on Proposal 1-1 and Proposal 1-2 could be the way forward.

-------------------------------------------- Updated proposals ---------------------------------------------------------------

- Proposal 1-1: for 38.101-1 CR RP-222350 for n77 in US

  - Option 1:

    - For Change #1 for section 6.2.3, keep original NOTE 6 unchanged and add clarification for NS values

      - NOTE 6:	This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements as this NS value is used for barring cell access cell barring purposes only.

    - For Change #2 for section 6.2A.3.1.2, add clarification for NS values on top of current changes

      - When UEs are configured with intra-band non-contiguous CA in n77 with under cells using NS_01 for an uplink component carrier in the range 3700-3980 MHz and NS_55 for an uplink component carrier in the range 3450-3550 MHz in FrequencyInfoUL-SIB, and if the UEs are configured with intra-band non-contiguous CA in n77, A-MPR additional spurious emission and maximum output power reduction requirements does not apply to the UEs regardless of whether which value of additionalSpectrumEmission is absent or present with value 0 or 1 in FrequencyInfoUL is used for the carrier in the range of 3450-3550 MHz. The Additional RF conformance tests are not required for these cases when NS values have been used for barring purposes only.

      - Further update is needed for Change #2 in future WG meetings. Note that according to the outcome of NS mapping discussion, the above change(s) can be further polished in the future RAN4 meetings.

  - Option 2:

    - For Change #1 for section 6.2.3, keep original NOTE 6 unchanged and add clarification for NS values

      - NOTE 6:	This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements as this NS value is used for barring cell access cell barring purposes only.

    - For Change #2 for section 6.2A.3.1.2, replace the Change # 2 by the following alternative

      - For UEs configured with intra-band non-contiguous CA in n77 and with NS_01 indicated or configured for an uplink component carrier the range 3700-3980 MHz and NS_55 for another uplink component carrier in the range 3450-3550 MHz, the allowed A-MPR additional spurious emission and maximum output power reduction requirements is are according to CA_NC_NS_01. The additional RF conformance tests are not required.

      - Note that according to the outcome of NS mapping discussion, the above change(s) can be further polished in the future RAN4 meetings.

- Proposal 1-2: for 38.101-1 CR RP-222353 for n77 in Canada

  - The similar wording agreed for sub-topic 1-1 can be reused

  - The similar changes in Section 6.2A.3.1.1 should also be made for NC CA.