3GPP TSG-RAN WG5 Meeting #97	R5-227703

Toulouse, France, Nov 14th - 18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2602 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to IE Table 4.6.3-62A - HighSpeedConfig |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR1_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Rel-17 HighSpeedConfig IE has been missing in Table 4.6.3-62A and needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rel-17 HighSpeedConfig IE has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE conformance test for Rel-17 HST enh can not be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

4.6.3	Radio resource control information elements

<<<UNCHANGED PARTS SKIPPED >>>

–	HighSpeedConfig

Editor's note:	The IEs in Table 4.6.3-62B are kept for R17 enh HST RRM and Demod testing.

Table 4.6.3-62A: HighSpeedConfig-r16

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| HighSpeedConfig-r16 ::= SEQUENCE { |  |  |  |
| highSpeedMeasFlag-r16 | Not present |  |  |
| highSpeedDemodFlag-r16 | Not present |  |  |
| } |  |  |  |

Table 4.6.3-62B: HighSpeedConfig-v1700

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| HighSpeedConfig-v1700 ::= SEQUENCE { |  |  |  |
| highSpeedMeasCA-Scell-r17 | Not present |  |  |
| highSpeedMeasInterFreq-r17 | Not present |  |  |
| highSpeedDemodCA-Scell-r17 | Not present |  |  |
| } |  |  |  |

<< END OF CHANGES >>