3GPP TSG RAN Meeting #98-e		RP-223282

Electronic Meeting, 12th – 16th December, 2022

Agenda Item:	9.3.2.1

Source: 	Ericsson

Title:  	Discussion on Rel-18 Further NR Mobility Enhancements WID Revision

Document for:	Discussion and decision

# Introduction

The objective 7 in the Further NR mobility enhancements had a study phase with a checkpoint at RAN#98e. RAN4 made an agreement on the feasible future work  in [1] and additionally sent LS to RAN in R4-2220734 to report the identified feasible solution [2]. Considering that further work is needed in RAN WG meetings, the WID may need to be revised at RAN#98-e.

# Discussion

## Revision of Objective 7

The original objective 7 listed below:

7. To study the following, with completion targeted by RAN#98 meeting [RAN4]: - The impact of FR2 RRM mobility measurement acquisition and reporting on FR2 SCell/SCG setup/resume delay for a UE connecting from idle/inactive mode. - The level of feasible improvement in FR2 SCell/SCG setup delay from defining new UE measurement procedures and RRM core requirements, and whether additional information from the network would help the UE to perform those measurements effectively. The following sequence of events should be assumed. - The UE initiates and performs improved measurements when it requests RRC connection setup/resume. - After acquiring those improved measurements, the UE subsequently reports those measurements to the network to support SCell/SCG setup.

RAN4 is tasked to study a feasible solution to improve FR2 SCell/SCG setup delay when the UE moves to RRC connected state from idle/inactive mode. After discussing several possible enhancements and solution directions, RAN4 agreed the following in RAN4#105 meeting and conveyed [2] the same as RAN4 study conclusion to RAN.

RAN4 #105 identified at least the following potential feasible direction: Agreement RAN4 #105: - Enhancements on UE behaviour in IDLE/INACTIVE mode are not in the scope of the WI - The following aspects can be further studied - Using measurement results obtained during IDLE/INACTIVE mode for measurements during RRC connection procedure Note: enhancements on IDLE/INACTIVE mode measurements are not in the scope

Since other possible enhancements are not agreed and at least some feasible solution directions are identified, we think the rest of the work of studying and specifying detailed solutions can be done in WI phase. Based on this, we believe the Objective#7 in the current WID should be modified accordingly. In the following, we provide our views on how to modify the objective.

- Using the measurement results obtained in IDLE/INACTIVE mode: Current Idle/Inactive mode measurement are not beneficial for network to setup CA/DC especially from both delay and reliability perspective. RAN4 consensus after RAN4 105 meeting is to only focus on how to enhance the reliability of the measurement results obtained during Idle/Inactive mode.

- Involvement of RAN2: There may be some RAN2 specific work based on the solution agreed in RAN4. Hence, it is important to list RAN2 as another responsible WG for Objective #7.

Based on above discussions, we propose the modified Objective #7 as following

- Study and specify the enhancements to IDLE/INACTIVE mode measurements by considering at least the following. - Study and specify validity of IDLE/INACTIVE mode measurement results which are to be reported during RRC connection setup/resume [RAN4]: - Using measurement results obtained during IDLE/INACTIVE mode and measurements during RRC connection procedure to obtain valid and reliable measurement report. - Note: The UE initiates and performs improved measurements when it requests RRC connection setup/resume. - Note: Enhancements on IDLE/INACTIVE mode measurements are not in the scope - Introduce the corresponding signalling, reporting mechanism based on RAN4 agreed solution [RAN2]

Proposal 1:	Revise Objective 7 to

- Study and specify the enhancements to IDLE/INACTIVE mode measurements by considering at least the following.

  - Study and specify validity of IDLE/INACTIVE mode measurement results which are to be reported during RRC connection setup/resume [RAN4]:

  - Using measurement results obtained during IDLE/INACTIVE mode and measurements during RRC connection procedure to obtain valid and reliable measurement report.

    - Note: The UE initiates and performs improved measurements when it requests RRC connection setup/resume.

    - Note: Enhancements on IDLE/INACTIVE mode measurements are not in the scope

  - Introduce the corresponding signalling, reporting mechanism based on RAN4 agreed solution [RAN2]

# Conclusion

It is proposed to discuss and decide on the following proposals:

Proposal 1:	Revise Objective 7 to

- Study and specify the enhancements to IDLE/INACTIVE mode measurements by considering at least the following.

  - Study and specify validity of IDLE/INACTIVE mode measurement results which are to be reported during RRC connection setup/resume [RAN4]:

  - Using measurement results obtained during IDLE/INACTIVE mode and measurements during RRC connection procedure to obtain valid and reliable measurement report.

    - Note: The UE initiates and performs improved measurements when it requests RRC connection setup/resume.

    - Note: Enhancements on IDLE/INACTIVE mode measurements are not in the scope

  - Introduce the corresponding signalling, reporting mechanism based on RAN4 agreed solution [RAN2]

# Reference

- R4-2220415 WF on NR Mobility Enhancements RRM requirements – Part 2, Apple, MediaTek, November 2022

- R4-2220734 LS on improvement in FR2 SCell/SCG setup/resume delay, RAN WG4, November 2022