3GPP TSG RAN WG1 #112	        		              R1-2302242

Athens, Greece, February 27th – March 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0462 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 editorial corrections for TS 38.213 (mirrored to Rel-17) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC-Core |  |  |  |  |  | Date: |  |  | 2023-03-08 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 16) Rel-16 (Release 17) Rel-17 (Release 18) Rel-18 (Release 19) |  |
| Reason for change: |  | For UCI multiplexing on a PUSCH that is scheduled by DCI format 0_1, beta offsets are provided by uci-OnPUSCH in Rel-15 and, for UCI of different priorities, by uci-OnPUSCH-ListDCI-0-1. Both parameters are provided by UCI-OnPUSCH. The current parameter ’uci-OnPUSCH’ in Clause 9.3 does not cover the case when the UE is configured to provide HARQ-ACK codebooks of different priorities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Beta_offsets for DCI format 0_1 and DCI format 0_2 are separately configured in UCI-OnPUSCH and UCI-OnPUSCH-DCI-0-2 that include betaOffsets and betaOffsetsDCI-0-2. For UCI multiplexing on a PUSCH that is scheduled by DCI format 0_2, UCI-OnPUSCH-DCI-0-2 is used in Clause 9.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect RRC parameters would be used for UCI multiplexing in a PUSCH in case of multiple priorities for the UCI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *** Unchanged text is omitted ***

## 9.3	UCI reporting in physical uplink shared channel

Offset values are defined for a UE to determine a number of resources for multiplexing HARQ-ACK information and for multiplexing CSI reports in a PUSCH. Offset values are also defined for multiplexing CG-UCI [5, TS 38.212] in a CG-PUSCH. The offset values are signalled to a UE either by a DCI format scheduling the PUSCH transmission or by higher layers.

If a DCI format that does not include a beta_offset indicator field schedules the PUSCH transmission from the UE and the UE is provided betaOffsets = 'semiStatic' or betaOffsetsDCI-0-2 = 'semiStaticDCI-0-2', the UE applies the βoffsetHARQ-ACK, βoffsetCSI-1, and βoffsetCSI-2 values that are provided by betaOffsets = 'semiStatic' for DCI formats 0_0/0_1 or by betaOffsetsDCI-0-2 = 'semiStaticDCI-0-2' for DCI format 0_2 for the corresponding HARQ-ACK information, Part 1 CSI reports and Part 2 CSI reports. If the PUSCH transmission has priority 0 or priority 1 and the UE is configured by uci-MuxWithDiffPrio to multiplex HARQ-ACK information of priority 1 or priority 0, respectively, and if the UE multiplexes HARQ-ACK information of priority 1 or priority 0, the UE applies corresponding βoffsetHARQ-ACK,1 or βoffsetHARQ-ACK,0 provided by betaOffsetCrossPri1 = 'semiStatic' for DCI formats 0_0/0_1 and by betaOffsetsCrossPri1DCI-0-2= 'semiStatic' for DCI format 0_2, or by betaOffsetCrossPri0 = 'semiStatic' for DCI format 0-1 and by betaOffsetsCrossPri0DCI-0-2= 'semiStatic' for DCI format 0_2, respectively.

If the PUSCH transmission is with a configured grant and the UE is provided CG-UCI-OnPUSCH= 'semiStatic', the UE applies the βoffsetHARQ-ACK, βoffsetCSI-1, and βoffsetCSI-2 values that are provided by CG-UCI-OnPUSCH = 'semiStatic' for the corresponding HARQ-ACK information, Part 1 CSI reports and Part 2 CSI reports. If the PUSCH transmission has priority 0 or priority 1 and the UE is configured by uci-MuxWithDiffPrio to multiplex HARQ-ACK information of priority 1 or priority 0, respectively, and if the UE multiplexes HARQ-ACK information of priority 1 or priority 0, the UE applies corresponding βoffsetHARQ-ACK,1 or βoffsetHARQ-ACK,0 provided by cg-betaOffsetsCrossPri1 = 'semiStatic' or cg-betaOffsetsCrossPri0 = 'semiStatic', respectively.

If the PUSCH transmission is scheduled by DCI format 0_0 and the UE is provided betaOffsets = 'dynamic', the UE applies the βoffsetHARQ-ACK, βoffsetCSI-1, and βoffsetCSI-2 values that are determined from the first value of betaOffsets = 'dynamic'. If the UE is configured by uci-MuxWithDiffPrio to multiplex HARQ-ACK information of priority 1, the UE applies corresponding βoffsetHARQ-ACK,1 provided by the first value of betaOffsetCrossPri1 = 'dynamic'.

If the PUSCH transmission is a configured grant Type 2 PUSCH and the UE is provided CG-UCI-OnPUSCH ='dynamic', the UE applies the βoffsetHARQ-ACK, βoffsetCSI-1, and βoffsetCSI-2 values that are determined from the first value of CG-UCI-OnPUSCH = 'dynamic'. If the PUSCH transmission has priority 0 or priority 1 and the UE is configured by uci-MuxWithDiffPrio to multiplex HARQ-ACK information of priority 1 or priority 0, respectively, and if the UE multiplexes HARQ-ACK information of priority 1 or priority 0, the UE applies corresponding βoffsetHARQ-ACK,1 or βoffsetHARQ-ACK,0 provided by the first value of cg-betaOffsetsCrossPri1 = 'dynamic' or cg-betaOffsetsCrossPri0 = 'dynamic', respectively.

HARQ-ACK information offsets βoffsetHARQ-ACK are configured to values according to Table 9.3-1. The betaOffsetACK-Index1, betaOffsetACK-Index2, and betaOffsetACK-Index3 respectively provide indexes Ioffset,0HARQ-ACK, Ioffset,1HARQ-ACK, and Ioffset,2HARQ-ACK for the UE to use if the UE multiplexes up to 2 HARQ-ACK information bits, more than 2 and up to 11 HARQ-ACK information bits, and more than 11 bits in the PUSCH, respectively.

Offsets βoffsetHARQ-ACK,0 for multiplexing HARQ-ACK information with priority 0 in a PUSCH transmission with priority 1 are configured to values according to Table 9.3-1. The first, second and third values provided by any of BetaOffsetsCrossPri0, betaOffsetsCrossPri0DCI-0-2, or cg-betaOffsetsCrossPri0 respectively provide indexes Ioffset,0HARQ-ACK,0, Ioffset,1HARQ-ACK,0, and Ioffset,2HARQ-ACK,0 for the UE to use if the UE multiplexes up to 2 bits, more than 2 and up to 11 bits, and more than 11 bits of HARQ-ACK information with priority 0 in the PUSCH transmission with priority 1, respectively.

Offsets βoffsetHARQ-ACK,1 for multiplexing HARQ-ACK information with priority 1 in a PUSCH transmission with priority 0 are configured to values according to Table 9.3-1. The first, second and third values provided by any of BetaOffsetsCrossPri1, betaOffsetsCrossPri1DCI-0-2, or cg-betaOffsetsCrossPri1 respectively provide indexes Ioffset,0HARQ-ACK,1, Ioffset,1HARQ-ACK,1, and Ioffset,2HARQ-ACK,1 for the UE to use if the UE multiplexes up to 2 bits, more than 2 and up to 11 bits, and more than 11 bits of HARQ-ACK information with priority 1 in the PUSCH transmission with priority 0, respectively.

Part 1 CSI report and Part 2 CSI report offsets βoffsetCSI-1 and βoffsetCSI-2, respectively, are configured to values according to Table 9.3-2. The betaOffsetCSI-Part1-Index1 and betaOffsetCSI-Part2-Index1 respectively provide indexes Ioffset,0CSI-1 and Ioffset,0CSI-2 for the UE to use if the UE multiplexes up to 11 bits for Part 1 CSI reports or Part 2 CSI reports in the PUSCH. The betaOffsetCSI-Part1-Index2 and betaOffsetCSI-Part2-Index2 respectively provide indexes Ioffset,1CSI-1 or Ioffset,1CSI-2 for the UE to use if the UE multiplexes more than 11 bits for Part 1 CSI reports or Part 2 CSI reports in the PUSCH.

If a DCI format that includes a beta_offset indicator field with one bit or two bits, as configured by UCI-OnPUSCH for DCI format 0_1 or by UCI-OnPUSCH-DCI-0-2 for DCI format 0_2, schedules the PUSCH transmission from the UE, the UE is provided by each of {betaOffsetACK-Index1, betaOffsetACK-Index2, betaOffsetACK-Index3}, the {first, second, third} values provided by BetaOffsetsCrossPri0, or betaOffsetsCrossPri0DCI-0-2, and the {first, second, third} values provided by BetaOffsetsCrossPri1, or betaOffsetsCrossPri1DCI-0-2 a set of two or four IoffsetHARQ-ACK, IoffsetHARQ-ACK,0, and IoffsetHARQ-ACK,1  indexes from Table 9.3-1 for multiplexing HARQ-ACK information in the PUSCH transmission and by each of {betaOffsetCSI-Part1-Index1, betaOffsetCSI-Part1-Index2} a set of two or four IoffsetCSI-1 indexes, and by each of {betaOffsetCSI-Part2-Index1, betaOffsetCSI-Part2-Index2} a set of two or four IoffsetCSI-2 indexes from Table  9.3-2, respectively, for multiplexing Part 1 CSI reports and Part 2 CSI reports, respectively, in the PUSCH transmission. The beta_offset indicator field indicates a IoffsetHARQ-ACK value and/or a IoffsetHARQ-ACK,0 value, and/or a IoffsetHARQ-ACK,1 value, a IoffsetCSI-1 value and a IoffsetCSI-2 value from the respective sets of values, with the mapping defined in Table 9.3-3 and in Table 9.3-3A. If the PUSCH transmission has priority 0 or priority 1, and the UE is provided uci-MuxWithDiffPrio, and the UE multiplexes HARQ-ACK information of priority 1 or priority 0 in the PUSCH, the UE applies the {first, second, third} values provided by betaOffsetCrossPri1 = 'dynamic' for DCI format 0_1, betaOffsetsCrossPri1DCI-0-2= 'dynamic' for DCI format 0_2, or applies the {first, second, third} values provided by betaOffsetCrossPri0 = 'dynamic' for DCI format 0_1, betaOffsetsCrossPri0DCI-0-2= 'dynamic' for DCI format 0_2.

## *** Unchanged text is omitted ***