3GPP TSG RAN WG1 Meeting #100-e	R1-2000199

February 24-March 6, 2020

Agenda Item:	7.2.2.2.3

Source:	Huawei, HiSilicon

Title:	Corrections on HARQ-ACK feedback

Document for:	Discussion and Decision

Introduction

In this contribution, we propose to fix the remaining issues of HARQ-ACK feedback for Rel-16 NR-U work item, including the issues for Type-2 HARQ-ACK codebook, Type-3 HARQ-ACK codebook, non-numerical value of K1, and multi-TTI scheduling.

Corrections on the Type-2 HARQ-ACK codebook

Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission

According to the RAN1#98bis meeting agreements below, the indication of NFI and total DAI for the non-scheduled group can be configured. It means that two total DAI could be indicated in a same DCI, so the VDAIg+1mod2 should be set to the value of total DAI field for the non-scheduled group (g+1) mod2. It should be distinguished from the total DAI for group g.

Agreement:

- Indication of NFI and T-DAI in non-fallback DCI for the non-scheduled group with single or multiple configured DL cells can be configured by RRC.

Reason for change: T-DAI value for a non-scheduled PDSCH group may be set to a wrong value in the determination of the enhanced dynamic HARQ codebook because the specification is unclear about which total DAI field to consider when two T-DAI fields are signaled.

Summary of change: clarify that it is the second total DAI field that should be used to set the T-DAI value for a non-scheduled PDSCH group.

Consequences if not approved: T-DAI value for a non-scheduled PDSCH group may be set to a wrong value.

TP#1 for TS 38.213 Clause 9.1.3.3

============================= Unchanged part omitted ============================

Set VDAIg+1mod2 to the value of a second total DAI field, if any, in a DCI format providing a value of g

============================= Unchanged part omitted ============================

Alignment with TS38.212

The fields including PDSCH group index，PDSCH-to-HARQ_feedback timing indicator and Number of requested PDSCH group(s) should be aligned with specification TS38.212 V16.0.0.

TP#2 for TS 38.213 Clause 9.1.3

============================= Unchanged part omitted ===========================

If a UE receives a first PDSCH scheduled by a first DCI format that the UE detects in a first PDCCH monitoring occasion and includes a PDSCH-to-HARQ_feedback timing indicator field providing an inapplicable value from dl-DataToUL-ACK, the UE does not multiplex corresponding HARQ-ACK information in a PUCCH or PUSCH transmission. The UE multiplexes the HARQ-ACK information in a PUCCH or PUSCH transmission in a slot that is indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a second DCI format the UE detects in any PDCCH monitoring occasion after the first one

-	if the UE is provided pdsch-HARQ-ACK-Codebook = enhancedDynamic-r16, the first DCI format and the second DCI format include a PDSCH group indexfield with a same value as described in Clause 9.1.3.3.

============================= Unchanged part omitted =========================

If a UE is provided pdsch-HARQ-ACK-Codebook = enhancedDynamic-r16, the UE determines HARQ-ACK information for multiplexing in a PUCCH transmission occasion according to the following procedure.

Set g to the value of a PDSCH group index field in a DCI format. If the DCI format schedules PDSCH reception and does not include a PDSCH group index field, set g=0.

Set i(g) to denote a PUCCH transmission occasion for multiplexing HARQ-ACK information

Set k to the value of a PDSCH-to-HARQ_feedback timing indicator field, if any, in a DCI format providing a value of g. If the DCI format does not include a PDSCH-to-HARQ_feedback timing indicator field, set k to the value provided by dl-DataToUL-ACK

Set h(g) to the value of a first New_Feedback indicator field, if any, in a DCI format providing a value of g

Set hg+1mod2(g) to a value of a second New_Feedback indicator field, if any, in a DCI format providing a value of g

Set VDAIg+1mod2 to the value of a total DAI field, if any, in a DCI format providing a value of g

Set q to the value of a Number of requested PDSCH group(s), if any

============================== Unchanged part omitted ===========================

Corrections on the type-3 HARQ-ACK codebook

For type-3 HARQ-ACK codebook, if spatial bundling is configured, UE should report HARQ-ACK information corresponding to the two TBs, but in TS38.213V16.0.0, NTB,cDL=1 if harq-ACK-SpatialBundlingPUCCH is provided, where HARQ-ACK information for only one TB is generated.

TP#3 for TS 38.213 Clause 9.1.4

============================== Unchanged part omitted ===========================

If a UE is provided pdsch-HARQ-ACK-OneShotFeedback-r16, the UE determines a Type-3 HARQ-ACK codebook according to the following procedure.

Set NcellsDL to the number of serving cells

Set NHARQ,cDL to the value of nrofHARQ-ProcessesForPDSCH for serving cell c, if provided; else, set NHARQ,cDL=8

Set NTB,cDL to the value of maxNrofCodeWordsScheduledByDCI for serving cell c NTB,cDL=1

Set NHARQ-ACK,cCBG/TB,max to the number of HARQ-ACK information bits per TB for PDSCH receptions on serving cell c as described in Clause 9.1.1 if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c and pdsch-HARQ-ACK-OneShotFeedbackCBG-r16 is provided; else, set NHARQ-ACK,cCBG/TB,max=0

============================== Unchanged part omitted ===========================

Open issue on one-shot HARQ-ACK codebook

In last meeting, the following working assumption was made:

Working assumption: One value of the frequency domain resource assignment field indicates that this DCI does not schedule a PDSCH

Like the following special fields for DL SPS or UL grant Type 2 scheduling release PDCCH validation (see table below), setting frequency domain resource assignment field to all ‘1’s and One-shot HARQ-ACK request field to ‘1’ could jointly indicate that the DCI does not schedule a PDSCH.

Special fields for single DL SPS or single UL grant Type 2 scheduling release PDCCH validation

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_1/1_2 |
| --- | --- | --- |
| HARQ process number | set to all '0's/0_1/0_2 | set to all '0's |
| Redundancy version | set to all '0's | set to all '0's |
| Modulation and coding scheme | set to all '1's | set to all '1's |
| Frequency domain resource assignment | set to all '1's | set to all '0's for FDRA Type 0 set to all '1's for FDRA Type 1 |

Proposal 1: To indicate that a DCI 1_1 does not schedule a PDSCH, the frequency domain resource assignment field should be set to all ‘1’s and One-shot HARQ-ACK request field should be set to ‘1’.

Inapplicable (non-numerical) value for K1

According to agreements, upon receiving an inapplicable value of K1, a UE should wait for a second DCI scheduling PDSCH with an applicable K1 value. The “second DCI” could be a non-fallback DCI or a fallback DCI, however, only the case where the second DCI is a non-fallback DCI is included in TS38.213 v16.0.0. RAN1 has agreed that the PDSCHs scheduled by the fallback DCI belong to group #0, so when the second DCI is a fallback DCI 1_0 the UE should multiplex the HARQ-ACK information for PDSCH group#0 in the slot which is indicated by the second DCI even though that DCI 1_0 does not include a PDSCH_group indicator field.

Reason for change: the specifications don’t allow HARQ-ACK feedback timing to be provided by a fallback DL DCI 1_0 for a PDSCH scheduled earlier in group 0 with an inapplicable value from dl-DataToUL-ACK.

Summary of change: to allow HARQ-ACK feedback timing to be provided by a fallback DL DCI 1_0 for a PDSCH scheduled earlier in group 0 with an inapplicable value from dl-DataToUL-ACK.

Consequences if not approved: the gNB would not be able to use a DL DCI 1_0 for providing HARQ-ACK feedback timing for a PDSCH scheduled earlier in group 0 with an inapplicable value from dl-DataToUL-ACK, resulting in either the inability to scheduling using a fallback DL DCI when needed unless it comes at the expense of a loss of HARQ-ACK feedback.

TP#4 for TS 38.213 Clause 9.1.3

============================== Unchanged part omitted ===========================

If a UE receives a first PDSCH scheduled by a first DCI format that the UE detects in a first PDCCH monitoring occasion and includes a PDSCH-to-HARQ_feedback timing indicator field providing an inapplicable value from dl-DataToUL-ACK, the UE does not multiplex corresponding HARQ-ACK information in a PUCCH or PUSCH transmission. The UE multiplexes the HARQ-ACK information in a PUCCH or PUSCH transmission in a slot that is indicated by a value of a PDSCH-to-HARQ_feedback indicator timing field in a second DCI format the UE detects in any PDCCH monitoring occasion after the first one

-	if the UE is provided pdsch-HARQ-ACK-Codebook = enhancedDynamic-r16, the first DCI format and the second DCI format include a PDSCH group index field with a same value as described in Clause 9.1.3.3. or the first DCI format include a PDSCH group index field = 0 and the second DCI format does not include a PDSCH group index field.

============================== Unchanged part omitted ===========================

Open issues on multi-TTI scheduling

In RAN1#99 meeting, the following open issue remains on multi-TTI scheduling:

Agreement:

The single bit for RV values chooses from RV {0, [2 or 3]}

To save the payload size, only single RV bit is indicated when multiple PUSCHs are scheduled. Since the 4 redundancy versions correspond to different starting points for the extraction of coded bits from the circular buffer, and only RV 0 and RV 3 are self-decodable, the value of single RV bit for multi-TTI scheduling should be chosen from {0,3}.

Proposal 2: The value of single RV bit for multi-TTI scheduling should be chosen from {0, 3}.

Reason for change: RV corresponding to field value 1 is unspecified for scheduling multiple PUSCHs with a single DCI on a NR-U cell.

Summary of change: RV 3 corresponds to field value 1 for scheduling multiple PUSCHs with a single DCI on a NR-U cell.

Consequences if not approved: the network is unable to signal a RV when scheduling multiple PUSCHs with a single DCI on a NR-U cell.

TP#5 for TS 38.212 Clause 7.3.1.1.2

============================== Unchanged part omitted ===========================

Table 7.3.1.1.2-34: Redundancy version

| Value of the Redundancy version field | Value of to be applied |
| --- | --- |
| 0 | 0 |
| 1 | 3 |

============================== Unchanged part omitted ===========================

Conclusions

Based on the discussion, we have the following proposals.

Proposal 1: To indicate that a DCI 1_1 does not schedule a PDSCH, the frequency domain resource assignment field should be set to all ‘1’s and One-shot HARQ-ACK request field should be set to ‘1’.

Proposal 2: The value of single RV bit for multi-TTI scheduling should be chosen from {0, 3}.

References

- [1] RAN1 Chairman’s Notes, 3GPP TSG RAN1 Meeting#99, Reno, USA, Nov 2019

- [2] 3GPP TS38.213 v 16.0.0

- [3] 3GPP TS38.212 v 16.0.0