3GPP TSG RAN WG1#100		R1-2000311

e-Meeting, February 24th – March 6nd, 2020

Source:	vivo

Title:	Remaining issues on HARQ operation for NR-U

Agenda Item:	7.2.2.2.3

Document for:	Discussion and Decision

- Introduction & Background

In RAN#82 meeting, the WID of the NR access to the unlicensed spectrum has been approved [1]. The following physical layer procedures have been identified as the objectives of the work item.

-	Physical layer procedure(s) including [RAN1, RAN2]:

- 	HARQ operation: NR HARQ feedback mechanisms are the baseline for NR-U operation with extensions in line with agreements during the study phase (NR-U TR section 7.2.1.3.3), including immediate transmission of HARQ A/N for the corresponding data in the same shared COT as well as transmission of HARQ A/N in a subsequent COT. Potentially support mechanisms to provide multiple and/or supplemental time and/or frequency domain transmission opportunities. (RAN1)

-	Scheduling multiple TTIs for PUSCH in-line with agreements from the study phase (TR 38.889, Section 7.2.1.3.3). (RAN1)

…

In RAN#82 meeting, the work item has been completed with some remaining issues. In this contribution we discuss the remaining issues on HARQ operation for NR-U Rel-16.

- Discussion

  - HARQ-ACK feedback

In NR-U Rel-16, enhancements for HARQ-ACK feedback have been introduced, mainly including enhancements for dynamic codebook, and introduction of a totally new codebook, i.e. one-shot feedback. In the following sections, we will discuss the remaining issues for these two items one by one.

- 2.1.1 Enhanced dynamic codebook

- Transmission of SPS PDSCH HARQ-ACK

In NR Rel-15, the network ensures SPS-Config is configured for at most one cell in a cell group. In NR-U Rel-16 this keeps valid when no enhancement about DL SPS is considered.

In the NR-U Rel-16 CR for TS38.213 [6], there are some descriptions as follows:

9.1.3.3	Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission

……

Generate first HARQ-ACK information for PUCCH transmission occasion i(g) in a slot, as described in Subclause 9.1.3.1, where

……

If hg+1mod2g=∅ or hg+1mod2g=h(g+1mod2), generate second HARQ-ACK information for PUCCH transmission occasion i(g+1mod2) in a slot, as described in Subclause 9.1.3.1, where

……

If hg+1mod2g≠∅ and hg+1mod2g≠h(g+1mod2), generate second HARQ-ACK information, as described in Subclause 9.1.3.1, by setting VC-DAI,c,mDL=0 for all c and all m and, after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Subclause 9.1.3.1, setting Vtemp2=VDAIg+1mod2.

……

In TS38.213(V15.8.0) subclause 9.1.3.1, there are some descriptions about SPS PDSCH HARQ-ACK as follows:

9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

……

If the UE transmits HARQ-ACK information in a PUCCH in slot  and for any PUCCH format, the UE determines the , for a total number of  HARQ-ACK information bits, according to the following pseudo-code:

……

Set

while

if SPS PDSCH reception is activated for a UE and the UE is configured to receive SPS PDSCH in a slot  for serving cell , where  is the PDSCH-to-HARQ-feedback timing value for SPS PDSCH on serving cell

= HARQ-ACK information bit associated with the SPS PDSCH reception

end if

;

end while

……

From the aforementioned descriptions it can be seen that none, one or both of the first HARQ-ACK information and the second HARQ-ACK information can contain a HARQ-ACK bit for SPS PDSCH reception, depending on the relative positions between PUCCH transmission occasion i(g) or i(g+1mod2) and SPS PDSCH receptions. A typical case when both of the first HARQ-ACK information and the second HARQ-ACK information contain a HARQ-ACK bit for respective SPS PDSCH reception is shown in Figure 1(a), in which HARQ-ACK feedback for some SPS PDSCHs could also have retransmission opportunities, e.g. U1 in Figure 1(a). When PUCCH transmission occasion i(g) and i(g+1mod2) are just in the same slot, and the transmitted HARQ-ACK codebook contains both the first HARQ-ACK information and the second HARQ-ACK information, HARQ-ACK corresponding to SPS PDSCH in a slot ig-K1,c or i(g+1mod2)-K1,c will be contained in the first HARQ-ACK information and the second HARQ-ACK information at the same time, according to the pseudo-code listed above, resulting in redundant feedback or information confusion, which is illustrated in Figure 1(b).

Figure 1 Transmission of SPS PDSCH HARQ-ACK both in first and second HARQ-ACK information

To avoid information redundancy or confusion for SPS PDSCH HARQ-ACK, a straightforward way is that HARQ-ACK for SPS PDSCH, if any, is always placed at the end of transmitted HARQ-ACK codebook, no matter whether the transmitted HARQ-ACK codebook contains only the first HARQ-ACK information, or both the first HARQ-ACK information and the second HARQ-ACK information. When it is assumed that SPS-Config is configured for at most one cell in a cell group, at most a HARQ-ACK bit for SPS PDSCH is appended at the end of the transmitted HARQ-ACK codebook.

Proposal 1: HARQ-ACK for SPS PDSCH, if any, is always placed at the end of enhanced HARQ-ACK codebook to be transmitted, no matter whether the transmitted HARQ-ACK codebook contains only the first HARQ-ACK information, or both the first HARQ-ACK information and the second HARQ-ACK information.

Regarding number of PDSCH groups involved in the transmitted HARQ-ACK codebook, there are some corresponding descriptions in the NR-U Rel-16 CR for TS38.213 [6] as follows:

9.1.3.3	Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission

……

Set q to the value of a HARQ-ACK information request field, if any

……

If q=0, the UE

includes only the first HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g)

elseif q=1

if g = 1

appends the first HARQ-ACK information to the second HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g)

else

append the second HARQ-ACK information to the first HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g)

end if

end if

……

It can be seen that whether the second HARQ-ACK information is triggered to be transmitted along with the first HARQ-ACK information is only based on q, i.e. the value of a HARQ-ACK information request field in a DCI format. But there may be a case that the PUCCH transmission occasion i(g+1mod2) for the second HARQ-ACK information and the PUCCH transmission occasion i(g) for the first HARQ-ACK information are just in the same slot, which leads to the first HARQ-ACK information and the second HARQ-ACK information being multiplexed in a same PUCCH transmission occasion irrespective of the value of q, as illustrated in Figure 2. In this case, the first HARQ-ACK information and the second HARQ-ACK information should also be multiplexed in an ascending order of group index. So it should be captured in the NR-U Rel-16 CR for TS38.213 that in any case when the first HARQ-ACK information and the second HARQ-ACK information are multiplexed in a same PUCCH transmission occasion, they should be placed in an ascending order of group index.

Figure 2 HARQ-ACK information for both groups reported in a same PUCCH transmission

Proposal 2: It should be captured in the NR-U Rel-16 CR for TS38.213 that in any case when the first HARQ-ACK information and the second HARQ-ACK information are multiplexed in a same PUCCH transmission occasion, they should be placed in an ascending order of group index.

- T-DAI related issues

In RAN1#98bis meeting [2], following agreement about indication of NFI and T-DAI in non-fallback DCI for the non-scheduled group has been achieved:

Agreement:

- For enhanced dynamic HARQ codebook with single or multiple configured DL cells,

  - NFI is indicated in non-fallback DCI scheduling PDSCH for the scheduled group

  - T-DAI is indicated in non-fallback DCI scheduling PDSCH for the scheduled group if more than one DL cell is configured

- Indication of NFI and T-DAI in non-fallback DCI for the non-scheduled group with single or multiple configured DL cells can be configured by RRC.

In the NR-U Rel-16 CR for TS38.213 [6], there are some descriptions as follows:

9.1.3.3	Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission

……

Set VDAIg+1mod2 to the value of a total DAI field, if any, in a DCI format providing a value of g

……

If hg+1mod2g=∅ or hg+1mod2g=h(g+1mod2), generate second HARQ-ACK information for PUCCH transmission occasion i(g+1mod2) in a slot, as described in Subclause 9.1.3.1, where

……

-	if VDAIg+1mod2≠∅, after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Subclause 9.1.3.1, set Vtemp2=VDAIg+1mod2 for both sub-codebooks, if any.

……

In a DCI format providing a value of g there may be a total DAI field for PDSCH group g if more than one DL cell is configured, and a total DAI field for PDSCH group g+1mod2 if RRC parameter NFI-TotalDAI-Included-r16 = enable. So it should be clarified in the NR-U Rel-16 CR for TS38.213 that VDAIg+1mod2 should be set to the value of a total DAI field for PDSCH group g+1mod2 to avoid ambiguity.

Proposal 3: It should be clarified in the NR-U Rel-16 CR for TS38.213 that VDAIg+1mod2 should be set to the value of a total DAI field for PDSCH group g+1mod2 to avoid ambiguity.

When VDAIg+1mod2≠∅, if the second HARQ-ACK codebook does not contain two sub-codebooks, i.e., only TB-based HARQ-ACK should be fed back, after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Subclause 9.1.3.1, set Vtemp2=VDAIg+1mod2 for this single codebook. Otherwise, if the second HARQ-ACK codebook contains two sub-codebooks, i.e., both TB-based HARQ-ACK and CBG-based HARQ-ACK should be fed back at the same time, set Vtemp2=VDAIg+1mod2 for both sub-codebooks after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Subclause 9.1.3.1. However, it will bring severe limitation for PDSCH scheduling for each sub-codebook, since these two sub-codebooks should match a same total DAI. In addition, there has been no agreement achieved regarding this issue during NR-U Rel-16 WI. A straightforward way to avoid this limitation is that when RRC parameter NFI-TotalDAI-Included-r16 = enable and two sub-codebooks may be applied, i.e., PDSCH-CodeBlockGroupTransmission is provided at least for a serving cell, two separate total DAI fields for PDSCH group g+1mod2 will be indicated in non-fallback DCI, one for the TB-based sub-codebook and the other for the CBG-based sub-codebook, when applicable. Another alternative way is that when two sub-codebooks may be applied, RRC parameter NFI-TotalDAI-Included-r16 is always not configured to avoid above scheduling limitation or indication overhead.

Proposal 4: It should be determined how to indicate and apply VDAIg+1mod2 when the second HARQ-ACK codebook contains two sub-codebooks.

- NFI related issues

In the NR-U Rel-16 CR for TS38.213 [6], there are some descriptions as follows:

9.1.3.3	Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission

……

If hg+1mod2g≠∅ and hg+1mod2g≠h(g+1mod2), generate second HARQ-ACK information, as described in Subclause 9.1.3.1, by setting VC-DAI,c,mDL=0 for all c and all m and, after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Subclause 9.1.3.1, setting Vtemp2=VDAIg+1mod2.

……

It could be understood that when a DCI format scheduling PDSCH group g indicates an NFI value NFIx for PDSCH group g+1mod2 other than an NFI value NFIy indicated by a latest DCI format scheduling PDSCH group g+1mod2 for exactly the scheduled group, it is mostly probable that after the latest DCI format scheduling PDSCH group g+1mod2, all DCI formats scheduling PDSCH group g+1mod2 with an NFI value NFIx have been miss-detected by UE. So in the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Subclause 9.1.3.1, no VC-DAI,c,mDL needs to be actually used, since there is no PDCCH detected for any PDCCH monitoring occasion m and any serving cell c. Considering that no agreement during NR-U Rel-16 WI has been achieved to deal with this case, it should be clarified that related operation captured in above descriptions is reasonable with common understanding.

Proposal 5: The corresponding operation when a DCI format scheduling PDSCH group g indicates an NFI value for PDSCH group g+1mod2 other than an NFI value indicated by a latest DCI format scheduling PDSCH group g+1mod2 for exactly the scheduled group should be clarified.

- Application of UL DAI

In RAN1#99 meeting [3], following agreement about single UL DAI for only one group has been achieved:

Agreement:

- If UL DAI is configured to be present in the non-fallback DCI for only one group, then for enhanced dynamic HARQ-ACK codebook feedback in PUSCH:

  - If feedback is requested for just one group, then this UL DAI applies to the reported group

  - Working assumption: If feedback is requested for both groups, the UL DAI applies to group #0 and the UE derives the DAI for group #1 by the latest DCI scheduling PDSCH reception for group #1 for this HARQ feedback occasion

In the NR-U Rel-16 CR for TS38.212 [5], there are some related descriptions as follows:

7.3.1.1.2	Format 0_1

……

-	1st downlink assignment index – 1, 2 or 4 bits:

-	1 bit for semi-static HARQ-ACK codebook;

-	2 bits for dynamic HARQ-ACK codebook, or for enhanced dynamic HARQ-ACK codebook without UL-TotalDAI-Included-r16 configured;

-	4 bits for enhanced dynamic HARQ-ACK codebook and with UL-TotalDAI-Included-r16 = “enable”.

-	2nd downlink assignment index – 0, 2 or 4 bits:

-	2 bits for dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks, or for enhanced dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks and without UL-TotalDAI-Included-r16 configured;

-	4 bits for enhanced dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks and with UL-TotalDAI-Included-r16 = “enable”;

-	0 bit otherwise.

……

In the NR-U Rel-16 CR for TS38.213 [6], there are some related descriptions as follows:

9.1.3.3	Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission

……

If the HARQ-ACK information is multiplexed in a PUSCH transmission, the HARQ-ACK information is determined as

-	……

-	for multiplexing in PUCCH transmission occasion i(g), if the PUSCH transmission is scheduled by a DCI format that includes one DCI field except that, for g=0, the DCI field with value  in the DCI format is used after the completion of the c and m loops for the pseudo-code for the HARQ-ACK codebook generation in Subclause 9.1.3.1

-	for multiplexing in PUCCH transmission occasion i(g), if the PUSCH transmission is scheduled by a DCI format that includes two DCI fields except that the DCI field with value  in the DCI format is used after the completion of the c and m loops for the pseudo-code for the HARQ-ACK codebook generation in Subclause 9.1.3.1

……

With respect to the case when the PUSCH transmission is scheduled by a DCI format that includes one DCI field, it can be understood that this DCI field corresponds to “1st downlink assignment index” field with 2 bits, and also to “2nd downlink assignment index” field with 2 bits when two HARQ-ACK sub-codebooks applied, in a DCI format 0_1. In addition, according to the agreement, this DCI field is not always used for g=0, which was captured in the CR incorrectly. In fact, when the HARQ-ACK information to be multiplexed in a PUSCH transmission refers to only one PDSCH group, this DCI field included in the DCI format scheduling the PUSCH transmission applies to the reported PDSCH group, otherwise it applies to g=0 when both PDSCH groups involved.

Proposal 6: It should be captured in the NR-U Rel-16 CR for TS38.213 correctly that for the case with indicated  value(s) for only one PDSCH group in a DCI format scheduling a PUSCH transmission, when the HARQ-ACK information to be multiplexed in the PUSCH transmission refers to only one PDSCH group, this DCI field applies to the reported PDSCH group, otherwise it applies to g=0 when both PDSCH groups involved.

With respect to the case when the PUSCH transmission is scheduled by a DCI format that includes two DCI fields, it can be understood that these two DCI fields correspond to “1st downlink assignment index” field with 4 bits, and also to “2nd downlink assignment index” field with 4 bits when two HARQ-ACK sub-codebooks applied, in a DCI format 0_1. In addition, each 4 bits should be divided into two DCI fields, i.e., the first 2 bits correspond to the first DCI field, and the last 2 bits correspond to the second DCI filed. Then, the first DCI field is used for g=0, and the second DCI field for g=1. This should be captured in the CR clearly.

Proposal 7: It should be captured in the NR-U Rel-16 CR for TS38.212 and TS38.213 clearly how to indicate and apply two DCI fields with  values in a DCI format 0_1.

- 2.1.2 One-shot feedback

- Codebook construction for spatial bundling

In the NR-U Rel-16 CR for TS38.213 [6], there are some descriptions about type-3 HARQ-ACK codebook determination as follows:

9.1.4	Type-3 HARQ-ACK codebook determination

……

Set NTB,cDL to the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided; else, set NTB,cDL=1

Set NHARQ-ACK,cCBG/TB,max to the number of HARQ-ACK information bits per TB for PDSCH receptions on serving cell c as described in Subclause 9.1.1 if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c and pdsch-HARQ-ACK-OneShotFeedbackCBG-r16 is provided; else, set NHARQ-ACK,cCBG/TB,max=0

Set NDIHARQ=0 if pdsch-HARQ-ACK-OneShotFeedbackNDI-r16 is provided; else set NDIHARQ=1

……

if NDIHARQ=0

if NHARQ-ACK,cCBG/TB,max>0

while t<NTB,cDL

while g<NHARQ-ACK,cCBG/TB,max

= HARQ-ACK information bit for CBG g of TB t for HARQ process number h of serving cell c

j=j+1

g=g+1

end while

= last NDI value for TB t for HARQ process number h on serving cell c, if any; else,

g=0

j=j+1

t=t+1

end while

else

while t<NTB,cDL

= HARQ-ACK information bit for TB t for HARQ process h of serving cell c

j=j+1

= last NDI value for TB t for HARQ process number h on serving cell c, if any; else,

j=j+1

t=t+1

end while

end if

t=0

else

……

end if

……

From above part of pseudo-code it can be seen that when spatial bundling is configured, NTB,cDL=1, and only HARQ-ACK information for the first TB, i.e. TB 0 is reported, so HARQ-ACK information for the second TB will be lost totally, which is not exactly the intention of spatial bundling.

A feasible way is that when CBG-based HARQ-ACK reporting is enabled for a serving cell, i.e.NHARQ-ACK,cCBG/TB,max>0, spatial bundling is always not enabled regardless of spatial bundling related configuration, which is aligned with NR Rel-15 behavior, since it is more important to report detailed HARQ-ACK information than to reduce signaling overhead. When CBG-based HARQ-ACK reporting is not enabled for a serving cell, i.e.NHARQ-ACK,cCBG/TB,max=0, spatial bundling is enabled based on spatial bundling related configuration, i.e., whether harq-ACK-SpatialBundlingPUCCH is provided for the cell group of this serving cell. When spatial bundling is enabled, the single HARQ-ACK value actually reported for HARQ process h of serving cell c is set to binary AND operation of the HARQ-ACK information bits corresponding to first and second TBs for HARQ process h of serving cell c.

Proposal 8: When CBG-based HARQ-ACK reporting is enabled for a serving cell, spatial bundling is always not enabled regardless of spatial bundling related configuration; otherwise spatial bundling is enabled based on whether harq-ACK-SpatialBundlingPUCCH is provided for the cell group of this serving cell.

Proposal 9: When spatial bundling is enabled, the single HARQ-ACK value actually reported for HARQ process h of serving cell c is set to binary AND operation of the HARQ-ACK information bits corresponding to first and second TBs for HARQ process h of serving cell c.

- Codebook construction when reporting NDI

When NDI is configured to be part of one-shot feedback, i.e. NDIHARQ=0, if spatial bundling is not enabled actually, latest NDI value or default value 0 for TB t for HARQ process number h on serving cell c will be reported right after HARQ-ACK information bit for TB t for HARQ process h of serving cell c. But if spatial bundling is enabled actually, how to place the result of binary AND operation of the HARQ-ACK information bits corresponding to two TBs and NDI value for each TB has no corresponding agreement so far. A straightforward way is that the result of binary AND operation is followed by NDI value for each TB in the order of increasing TB index.

Proposal 10: When NDI is configured to be part of one-shot feedback and spatial bundling is enabled actually, for a HARQ process of a serving cell, the result of binary AND operation of the HARQ-ACK information bits corresponding to two TBs is followed by NDI value for each TB in the order of increasing TB index.

- HARQ-ACK values when not reporting NDI

In the NR-U Rel-16 CR for TS38.213 [6], there are some descriptions about type-3 HARQ-ACK codebook determination when not reporting NDI as follows:

9.1.4	Type-3 HARQ-ACK codebook determination

……

Set NTB,cDL to the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided; else, set NTB,cDL=1

Set NHARQ-ACK,cCBG/TB,max to the number of HARQ-ACK information bits per TB for PDSCH receptions on serving cell c as described in Subclause 9.1.1 if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c and pdsch-HARQ-ACK-OneShotFeedbackCBG-r16 is provided; else, set NHARQ-ACK,cCBG/TB,max=0

Set NDIHARQ=0 if pdsch-HARQ-ACK-OneShotFeedbackNDI-r16 is provided; else set NDIHARQ=1

……

if NDIHARQ=0

……

else

if NHARQ-ACK,cCBG/TB,max>0

while t<NTB,cDL

if UE has reported ACK for TB t for HARQ process number h on serving cell c

while g<NHARQ-ACK,cCBG/TB,max

j=j+1

g=g+1

end while

else

while g<NHARQ-ACK,cCBG/TB,max

= HARQ-ACK information bit for CBG g of TB t for HARQ process number h of serving cell c

j=j+1

g=g+1

end while

end if

g=0

t=t+1

end while

else

while t<NTB,cDL

if UE has reported NACK for TB t for HARQ process number h on serving cell c

= HARQ-ACK information bit for TB t for HARQ process h of serving cell c

j=j+1

t=t+1

else

j=j+1

t=t+1

end if

end while

end if

t=0

end if

……

From above part of pseudo-code it can be seen that when NDI is not configured to be part of one-shot feedback, CBG-based HARQ-ACK feedback and TB-based HARQ-ACK feedback don’t take consistent operation regarding whether UE has reported a HARQ-ACK value for a TB for a HARQ process.

In RAN1#99 meeting [3], following agreement about reporting NDI or not has been achieved:

Agreement:

For one-shot HARQ feedback:

- NDI can be configured to be part of one-shot HARQ feedback.

  - When NDI is configured

    - The latest NDI value detected by the UE is reported along with HARQ-ACK for the corresponding HARQ process ID. The UE assumes NDI=0 if there is no prior NDI value for the HARQ process

    - NDI is included for each TB

  - When NDI is not configured:

    - NDI value is not reported along with HARQ-ACK for the corresponding PDSCH

    - UE is expected to reset HARQ-ACK state (as DTX or NACK) for a HARQ process ID once ACK is reported for the same HARQ process ID in the previous feedback

So, in above part of pseudo-code the operation for TB-based HARQ-ACK feedback should be revised to keep consistent with that for CBG-based HARQ-ACK feedback, that is, if UE has reported ACK for TB t for HARQ process number h on serving cell c,  for TB t for HARQ process number h on serving cell c is set to NACK.

Proposal 11: For type-3 HARQ-ACK codebook determination, when not reporting NDI, the operation for TB-based HARQ-ACK feedback in the NR-U Rel-16 CR for TS38.213 should be revised to keep consistent with that for CBG-based HARQ-ACK feedback.

- Text proposals

  - Text proposals for TS38.212

For the NR-U Rel-16 CR for TS38.212 [5], text proposals based on aforementioned proposals are collected as below:

-------------------------------------------------------Start text proposal-----------------------------------------------------------

##### 7.3.1.1.2	Format 0_1

DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback informatin (CG-DFI) to a UE.

……

-	Redundancy version –– number of bits determined by the following:

-	2 bits as defined in Table 7.3.1.1.1-2 if the number of scheduled PUSCH indicated by indicated by the Time domain resource assignment field is 1;

-	otherwise 2, 3, 4, 5, 6, 7 or 8 bits determined by the maximum number of schedulable PUSCHs among all entries in the higher layer parameter pusch-TimeDomainAllocationList-r16, where each bit corresponds to one scheduled PUSCH as defined in subclause 6.1.4 in [6, TS 38.214] and redundancy version is determined according to Table 7.3.1.1.2-34.

-	HARQ process number – 4 bits

-	1st downlink assignment index – 1, 2 or 4 bits:

-	1 bit for semi-static HARQ-ACK codebook;

-	2 bits for dynamic HARQ-ACK codebook, or for enhanced dynamic HARQ-ACK codebook without UL-TotalDAI-Included-r16 configured, in which case 2 bits correspond to one field for a PDSCH group;

-	4 bits for enhanced dynamic HARQ-ACK codebook and with UL-TotalDAI-Included-r16 = “enable”, in which case the 4 bits correspond to two fields with the first 2 bits as the first field corresponding to PDSCH group 0 and the last 2 bits as the second field corresponding to PDSCH group 1.

-	2nd downlink assignment index – 0, 2 or 4 bits:

-	2 bits for dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks, or for enhanced dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks and without UL-TotalDAI-Included-r16 configured, in which case 2 bits correspond to one field for a PDSCH group;

-	4 bits for enhanced dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks and with UL-TotalDAI-Included-r16 = “enable” , in which case the 4 bits correspond to two fields with the first 2 bits as the first field corresponding to PDSCH group 0 and the last 2 bits as the second field corresponding to PDSCH group 1;

-	0 bit otherwise.

-	TPC command for scheduled PUSCH – 2 bits as defined in Subclause 7.1.1 of [5, TS38.213]

……

--------------------------------------------------------End text proposal----------------------------------------------------------

Proposal 12: Adopt the text proposals listed in section 3.1 for Rel-16 TS38.212.

  - Text proposals for TS38.213

For the NR-U Rel-16 CR for TS38.213 [6], text proposals based on aforementioned proposals are collected as below:

-------------------------------------------------------Start text proposal-----------------------------------------------------------

#### 9.1.3.3	Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission

If a UE is provided pdsch-HARQ-ACK-Codebook = enhancedDynamic-r16, the UE determines HARQ-ACK information for multiplexing in a PUCCH transmission occasion according to the following procedure.

Set g to the value of a PDSCH_group indicator field in a DCI format. If the DCI format schedules PDSCH reception and does not include a PDSCH_group indicator field, set g=0.

Set i(g) to denote a PUCCH transmission occasion for multiplexing HARQ-ACK information

Set k to the value of a PDSCH-to-HARQ_feedback timing field, if any, in a DCI format providing a value of g. If the DCI format does not include a PDSCH-to-HARQ_feedback timing field, set k to the value provided by dl-DataToUL-ACK

Set h(g) to the value of a first New_Feedback indicator field, if any, in a DCI format providing a value of g

Set hg+1mod2(g) to a value of a second New_Feedback indicator field, if any, in a DCI format providing a value of g

Set VDAIg+1mod2 to the value of a total DAI field for group g+1mod2, if any, in a DCI format providing a value of g

Set q to the value of a HARQ-ACK information request field, if any

Generate first HARQ-ACK information for PUCCH transmission occasion i(g) in a slot, as described in Subclause 9.1.3.1, where

-	the first HARQ-ACK information corresponds only to detections of DCI formats each providing a same value of g, of h(g), if any, and at least one of the DCI formats providing a value of k indicating the slot

-	at least one of the DCI formats provides a h(g) value

-	m=0 corresponds to a PDCCH monitoring occasion, where the UE detects a DCI format that provides a value of g, that is the first PDCCH monitoring occasion after a PDCCH monitoring occasion where the UE detects another DCI format that provides a value different than h(g)

If hg+1mod2g=∅ or hg+1mod2g=h(g+1mod2), generate second HARQ-ACK information for PUCCH transmission occasion i(g+1mod2) in a slot, as described in Subclause 9.1.3.1, where

-	the second HARQ-ACK information corresponds to detections of DCI formats each providing a same value of g+1mod2, of h(g+1mod2), if any

-	at least one of the DCI formats provides a h(g+1mod2) value

-	m=0 corresponds to a PDCCH monitoring occasion, where the UE detects a DCI format that provides a value of g+1mod2, that is the first PDCCH monitoring occasion after a PDCCH monitoring occasion where the UE detects another DCI format that provides a value different than h(g+1mod2)

-	the PUCCH transmission occasion i(g+1mod2) is a last one for multiplexing second HARQ-ACK information and it is not after PUCCH transmission occasion i(g)

-	if VDAIg+1mod2≠∅, after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Subclause 9.1.3.1, set Vtemp2=VDAIg+1mod2 for both sub-codebooks, if any.

If hg+1mod2g≠∅ and hg+1mod2g≠h(g+1mod2), generate second HARQ-ACK information, as described in Subclause 9.1.3.1, by setting VC-DAI,c,mDL=0 for all c and all m and, after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Subclause 9.1.3.1, setting Vtemp2=VDAIg+1mod2.

If q=0, the UE

includes only the first HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g)

elseif q=1

if g = 1

appends the first HARQ-ACK information to the second HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g)

else

append the second HARQ-ACK information to the first HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g)

end if

end if

If the HARQ-ACK information is multiplexed in a PUSCH transmission, the HARQ-ACK information is determined as

-	for multiplexing in PUCCH transmission occasion i(g), if the PUSCH transmission is not scheduled by a DCI format or is scheduled by a DCI format that does not include a DCI field with value

-	for multiplexing in PUCCH transmission occasion i(g), if the PUSCH transmission is scheduled by a DCI format that includes one DCI field except that, for PDSCH group  g=0, the DCI field with value  in the DCI format is used after the completion of the c and m loops for the pseudo-code for the HARQ-ACK codebook generation in Subclause 9.1.3.1, and when the HARQ-ACK information refers to only one PDSCH group g is the index of this group, or when the HARQ-ACK information refers to both PDSCH groups g=0

-	for multiplexing in PUCCH transmission occasion i(g), if the PUSCH transmission is scheduled by a DCI format that includes two DCI fields except that the each DCI field with value  in the DCI format is used for corresponding PDSCH group after the completion of the c and m loops for the pseudo-code for the HARQ-ACK codebook generation in Subclause 9.1.3.1

For any case that HARQ-ACK information for both PDSCH groups is multiplexed in a PUCCH transmission or PUSCH transmission, HARQ-ACK information for g=1 is appended to that for g=0.

If a UE detects DCI formats with respective PDSCH-to-HARQ_feedback timing field values indicating a same PUCCH transmission occasion and none of the DCI formats includes a New_Feedback indicator field, the UE generates HARQ-ACK information as described in Subclause 9.1.3.1 or 9.1.3.2 for multiplexing in the PUCCH transmission occasion.

For PUCCH transmission occasion i(g), the UE determines a PUCCH or a PUSCH transmission to multiplex the HARQ-ACK information according to the procedures in Subclause 9.2.5.

### 9.1.4	Type-3 HARQ-ACK codebook determination

If a UE is provided pdsch-HARQ-ACK-OneShotFeedback-r16, the UE determines a Type-3 HARQ-ACK codebook according to the following procedure.

Set NcellsDL to the number of serving cells

Set NHARQ,cDL to the value of nrofHARQ-ProcessesForPDSCH for serving cell c, if provided; else, set NHARQ,cDL=8

Set NTB,cDL to the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided; else, set NTB,cDL=1

Set NHARQ-ACK,cCBG/TB,max to the number of HARQ-ACK information bits per TB for PDSCH receptions on serving cell c as described in Subclause 9.1.1 if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c and pdsch-HARQ-ACK-OneShotFeedbackCBG-r16 is provided; else, set NHARQ-ACK,cCBG/TB,max=0

Set NDIHARQ=0 if pdsch-HARQ-ACK-OneShotFeedbackNDI-r16 is provided; else set NDIHARQ=1

Set c=0 – serving cell index

Set h=0 – HARQ process number

Set t=0 – TB index

Set g=0 – CBG index

Set j=0

while c<NcellsDL

while h<NHARQ,cDL

if NDIHARQ=0

if NHARQ-ACK,cCBG/TB,max>0

while t<NTB,cDL

while g<NHARQ-ACK,cCBG/TB,max

= HARQ-ACK information bit for CBG g of TB t for HARQ process number h of serving cell c

j=j+1

g=g+1

end while

= last NDI value for TB t for HARQ process number h on serving cell c, if any; else,

g=0

j=j+1

t=t+1

end while

else

if harq-ACK-SpatialBundlingPUCCH is not provided

while t<NTB,cDL

= HARQ-ACK information bit for TB t for HARQ process h of serving cell c

j=j+1

= last NDI value for TB t for HARQ process number h on serving cell c, if any; else,

j=j+1

t=t+1

end while

else

if NTB,cDL = 2

= binary AND operation of the HARQ-ACK information bits corresponding to first and second transport blocks for HARQ process h of serving cell c - if the UE receives one transport block, the UE assumes ACK for the second transport block;

else

= HARQ-ACK information bit for TB 0 for HARQ process h of serving cell c

end if

j=j+1

while t<NTB,cDL

= last NDI value for TB t for HARQ process number h on serving cell c, if any; else,

j=j+1

t=t+1

end while

end if

end if

t=0

else

if NHARQ-ACK,cCBG/TB,max>0

while t<NTB,cDL

if UE has reported ACK for TB t for HARQ process number h on serving cell c

while g<NHARQ-ACK,cCBG/TB,max

j=j+1

g=g+1

end while

else

while g<NHARQ-ACK,cCBG/TB,max

= HARQ-ACK information bit for CBG g of TB t for HARQ process number h of serving cell c

j=j+1

g=g+1

end while

end if

g=0

t=t+1

end while

else

if harq-ACK-SpatialBundlingPUCCH is not provided

while t<NTB,cDL

if UE has reported NACK for TB t for HARQ process number h on serving cell c

= HARQ-ACK information bit for TB t for HARQ process h of serving cell c

j=j+1

t=t+1

else

= HARQ-ACK information bit for TB t for HARQ process h of serving cell c

j=j+1

t=t+1

end if

end while

else

if UE has reported ACK for HARQ process number h on serving cell c

j=j+1

else

if NTB,cDL = 2

= binary AND operation of the HARQ-ACK information bits corresponding to first and second transport blocks for HARQ process h of serving cell c - if the UE receives one transport block, the UE assumes ACK for the second transport block;

else

= HARQ-ACK information bit for TB 0 for HARQ process h of serving cell c

end if

j=j+1

end if

end if

end if

t=0

end if

h=h+1

end while

h=0

c=c+1

end while

If the UE detects a DCI format that includes a One-shot HARQ-ACK request field with value 1, the UE determines a PUCCH or a PUSCH to multiplex a Type-3 HARQ-ACK codebook for transmission in a slot as described in Subclause 9.2.5. The UE multiplexes only the Type-3 HARQ-ACK codebook in the PUCCH or the PUSCH for transmission in the slot.

--------------------------------------------------------End text proposal----------------------------------------------------------

Proposal 13: Adopt the text proposals listed in section 3.2 for Rel-16 TS38.213.

- Conclusion

In this contribution, some remaining issues on NR-U Rel-16 HARQ operation have been discussed, and the proposals made are summarized below:

Proposal 1: HARQ-ACK for SPS PDSCH, if any, is always placed at the end of enhanced HARQ-ACK codebook to be transmitted, no matter whether the transmitted HARQ-ACK codebook contains only the first HARQ-ACK information, or both the first HARQ-ACK information and the second HARQ-ACK information.

Proposal 2: It should be captured in the NR-U Rel-16 CR for TS38.213 that in any case when the first HARQ-ACK information and the second HARQ-ACK information are multiplexed in a same PUCCH transmission occasion, they should be placed in an ascending order of group index.

Proposal 3: It should be clarified in the NR-U Rel-16 CR for TS38.213 that VDAIg+1mod2 should be set to the value of a total DAI field for PDSCH group g+1mod2 to avoid ambiguity.

Proposal 4: It should be determined how to indicate and apply VDAIg+1mod2 when the second HARQ-ACK codebook contains two sub-codebooks.

Proposal 5: The corresponding operation when a DCI format scheduling PDSCH group g indicates an NFI value for PDSCH group g+1mod2 other than an NFI value indicated by a latest DCI format scheduling PDSCH group g+1mod2 for exactly the scheduled group should be clarified.

Proposal 6: It should be captured in the NR-U Rel-16 CR for TS38.213 correctly that for the case with indicated  value(s) for only one PDSCH group in a DCI format scheduling a PUSCH transmission, when the HARQ-ACK information to be multiplexed in the PUSCH transmission refers to only one PDSCH group, this DCI field applies to the reported PDSCH group, otherwise it applies to g=0 when both PDSCH groups involved.

Proposal 7: It should be captured in the NR-U Rel-16 CR for TS38.212 and TS38.213 clearly how to indicate and apply two DCI fields with  values in a DCI format 0_1.

Proposal 8: When CBG-based HARQ-ACK reporting is enabled for a serving cell, spatial bundling is always not enabled regardless of spatial bundling related configuration; otherwise spatial bundling is enabled based on whether harq-ACK-SpatialBundlingPUCCH is provided for the cell group of this serving cell.

Proposal 9: When spatial bundling is enabled, the single HARQ-ACK value actually reported for HARQ process h of serving cell c is set to binary AND operation of the HARQ-ACK information bits corresponding to first and second TBs for HARQ process h of serving cell c.

Proposal 10: When NDI is configured to be part of one-shot feedback and spatial bundling is enabled actually, for a HARQ process of a serving cell, the result of binary AND operation of the HARQ-ACK information bits corresponding to two TBs is followed by NDI value for each TB in the order of increasing TB index.

Proposal 11: For type-3 HARQ-ACK codebook determination, when not reporting NDI, the operation for TB-based HARQ-ACK feedback in the NR-U Rel-16 CR for TS38.213 should be revised to keep consistent with that for CBG-based HARQ-ACK feedback.

Proposal 12: Adopt the text proposals listed in section 3.1 for Rel-16 TS38.212.

Proposal 13: Adopt the text proposals listed in section 3.2 for Rel-16 TS38.213.

- References

- Chairman’s Notes, RAN#82, Sorrento, Italy, Dec. 10-13, 2018.

- Chairman's Notes, RAN1#98bis, Chongqing, China, Oct. 14-20, 2019

- Chairman's Notes, RAN1#99, Reno, USA, Nov. 18-22, 2019

- TR 38.889, “Study on NR-based access to unlicensed spectrum”, v16.0.0, Dec. 2018

- R1-1913631, Introduction of NR based access to unlicensed spectrum into 38.212, Huawei

- R1-1913632, Introduction of shared spectrum channel access (into 38.213), Samsung