3GPP TSG-RAN WG1 Meeting #100		R1-2000320

e-Meeting, February 24th – March 6th, 2020

Source:	vivo

Title:	Remaining issues on in-device coexistence between NR and LTE sidelinks

Agenda Item:	7.2.4.4

Document for:	Discussion and Decision

- Introduction

In this contribution, some remaining issues of in-device coexistence are discussed and the associated text proposals are provided.

- Discussion

In the RAN1 1901 Ad-Hoc meeting, the TDM based solutions for in-device coexistence between LTE and NR V2X are discussed. It was identified that timing alignment is required for TDM based solution according to the following agreement [1]:

Agreements: - For TDM solutions for in-device coexistence between LTE and NR V2X: - Time Alignment - Subframe boundary alignment is required between LTE and NR V2X sidelinks - Both LTE and NR V2X sidelinks are aware of the time resource index (e.g., DFN for LTE) in both carriers

In the current specification (i.e. 38.213 v16.0.0), the TDM solution is applied whenever LTE and NR sidelink signals/channels overlap in time. One example is shown below [2]:

… If a UE would transmit or receive an S-SS/PSBCH block or, for E-UTRA radio access, sidelink synchronization signals, and the transmission or reception would overlap in time with other transmissions and/or receptions on the sidelink, the UE transmits or receives the signal/channel with the higher priority. …

It is worth noting that the description is same as that for coexistence of MR-DC in Uu (one example is cited below).

… - If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC and - if UE transmission(s) in subframe of the MCG overlap in time with UE transmission(s) in slot of the SCG in FR1, and - if in any portion of slot of the SCG, the UE reduces transmission power in any portion of slot of the SCG so that in any portion of slot , where and are the linear values of the total UE transmission powers in subframe of the MCG and in slot of the SCG in FR1, respectively. The UE is not required to transmit in any portion of slot of the SCG if would need to be reduced by more than the value provided by XSCALE in order for in any portion of slot of the SCG. The UE is required to transmit in slot of the SCG if would not need to be reduced by more than the value provided by XSCALE in order for in all portions of slot . …

However, unlike the Uu case where asynchronous DC is supported, in sidelink it is only agreed to support TDM solutions with the prerequisite of explicit time alignment. Therefore, addition clarifications for TDM solution are required. The associated text proposals are provided as below:

## 16.1 Synchronization procedures … If a UE - would transmit or receive an S-SS/PSBCH block or, for E-UTRA radio access, sidelink synchronization signals, and - the transmission or reception would overlap in time with other transmissions and/or receptions on the sidelink, and - the time resource indexes of the transmission or reception of both NR and E-UTRA sidelinks are known to the UE, and - the subframe boundaries are aligned between NR and E-UTRA sidelinks the UE transmits or receives the signal/channel with the higher priority. … ### 16.2.4 Prioritization of transmissions/receptions #### 16.2.4.1 Simultaneous NR and E-UTRA transmission/reception If a UE - would transmit a first channel/signal using E-UTRA radio access and a second channel/signal using NR radio access, and - a transmission of the first channel/signal would overlap in time with a transmission of the second channel/signal, and - the time resource indexes of the transmission or reception of both NR and E-UTRA sidelinks are known to the UE, and - the subframe boundaries are aligned between NR and E-UTRA sidelinks, and - the priorities of the two channels/signals are known to the UE T msec prior to the start of the earlier of the two transmissions the UE transmits only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers. If a UE - would respectively transmit or receive a first channel/signal using E-UTRA radio access and receive or transmit a second channel/signal using NR radio access, and - a transmission or reception of the first channel/signal would respectively overlap in time with a reception or transmission of the second channel/signal, and - the time resource indexes of the transmission or reception of both NR and E-UTRA sidelinks are known to the UE, and - the subframe boundaries are aligned between NR and E-UTRA sidelinks, and - the priorities of the two channels/signals are known to the UE T msec prior to the start of the later transmission or reception the UE transmits or receives only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers.

Proposal 1: Agree the TP to clarify that time alignment is required for TDM solution of in-device coexistence.

- Conclusion

In the contribution, we provide our view on the coexistence issues of LTE and NR sidelink technologies with the following proposals:

Proposal 1: Agree the TP to clarify that time alignment is required for TDM solution of in-device coexistence.

- Reference

- Chairman Notes, 3GPP TSG RAN WG1 Ad-Hoc meeting #1901, Taipei, January 2019.

- 3GPP TS 38.213, “NR; Physical layer procedures for control”, v16.0.0 (2019-12).