3GPP TSG RAN WG1 #100                                                          R1-2000438

e-Meeting, February 24th – March 6th, 2020

Agenda item:	7.2.2.2.2

Source: 	MediaTek Inc.

Title: 	Remaining issues on initial access procedure for NR-U operation

Document for:	Discussion and decision

# Introduction

In this contribution, we discuss two remaining issues about initial access procedure and mobility measurements. One is about the allowed SSB transmission patterns in a DRS window. The other issue is the signaling of the Q value.

# SSB Transmission

At RAN1 #99 meeting, we made the following agreement to reduce UE implementation complexity.

Agreement: From a UE’s perspective, the number of transmitted SSBs within a DRS transmission window is not larger than Q.

In Figure 1, all the five cases of SS/PBCH block transmission patterns are allowed based on the above agreement. However, the transmission patterns in Cases 2, 3, and 4 command unnecessary UE power consumption in measurements and hence should not be allowed. In Case 2, it is not clear to us why network would want to transmit SS/PBCH blocks in this way. In Case 3, only one instead of two SS/PBCH blocks is transmitted within a slot for coverage enhancement. However, by selecting a proper Q value, the SS/PBCH block transmission pattern in Case 3 can be achieved by Case 5. As to Case 4 where SS/PBCH blocks with a same “beam” index are repeatedly transmitted within a discovery burst transmission window, it is not aligned with the Rel-15 SS/PBCH burst design. Therefore, we propose the following to exclude Case 2, 3, and 4 to reduce UE power consumption in measurements.

Proposal 1: Within a discovery burst transmission window, the number of candidate SS/PBCH blocks from the first transmitted SS/PBCH block and the last transmitted SS/PBCH block should not be greater than Q.

Proposal 2: Adopt the text proposal in this section to TS38.213.

Figure 1: Examples of SS/PBCH block transmission patterns within a discovery burst transmission window with different Q values. A block with solid line and filling means the SS/PBCH block is actually transmitted while a block with dashed line and slashed filling simply means a candidate SS/PBCH position not being used for actual SS/PBCH transmission.

============== Start of Text Proposal for Section 4.1 of TS 38.213 ==================================

For operation with shared spectrum channel access, a UE assumes that SS/PBCH blocks in a serving cell that are within a same discovery burst transmission window or across discovery burst transmission windows are quasi co-located with respect to average gain, QCL-TypeA, and QCL-TypeD properties, when applicable [6, TS 38.214], if a value of NDM-RSPBCHmodNSSBQCL is same among the SS/PBCH blocks. NDM-RSPBCH is an index of a DM-RS sequence transmitted in a PBCH of a corresponding SS/PBCH block, and NSSBQCL is either provided by ssbPositionQCL-Relationship-r16 or, if ssbPositionQCL-Relationship-r16 is not provided, obtained from a MIB provided by a SS/PBCH block according to Table 4-1. ssbSubcarrierSpacingCommon indicates SCS of RMSI only for the case of "operation without shared spectrum".The UE assumes that within a discovery burst transmission window, the number of candidate SS/PBCH blocks from the first transmitted SS/PBCH block to the last transmitted SS/PBCH block on a cell is not larger than NSSBQCL. The UE can determine an SS/PBCH block index according to NDM-RSPBCHmodNSSBQCL, or according to i modNSSBQCL where i is the candidate SS/PBCH block index.

============== End of Text Proposal for Section 4.1 of TS 38.213 ==================================

# Signalling of Q

For the signaling of the Q value, we made the following agreements at RAN1 #98bis and #99 meetings.

Agreement: (RAN1 #98bis, 1910) For RRM measurements of a neighbor cell in IDLE, INACTIVE and CONNECTED UE states, - Support signaling of a common Q value per frequency by broadcast RRC signaling (SIBx) and/or dedicated RRC signaling (measObjectNR) from the serving cell. - Support signaling from the serving cell of a Q value for a listed neighbour cell. - If Q is signaled for a listed cell, it overrides any common Q value per frequency

Agreement: (RAN1 #99, 1911) The Q value signalled in system information for a serving cell is also used for RRM measurements on the serving cell.

Agreement: (RAN1 #99, 1911) A serving cell can indicate Q via dedicated RRC signaling (e.g., for SCell or SCG addition).

Though the agreements state that gNB can indicate Q to UE for RRM measurements of neighboring cells, or SCell addition or SCG addition, it is not clear whether or not gNB will always indicate Q to UE so that UE does not have to read PBCH in the mentioned scenarios. For the clarity of UE behavior and to reduce UE complexity, we propose the following.

Proposal 3: For RRM measurements of a neighboring cell, RAN1 down-selects one of the following:

- Option 1: At least a common Q value per frequency is provided to UE.

- Option 2: If no Q value is provided, UE assumes Q=8.

Proposal 4: For SCell addition and SCG addition, RAN1 down-selects one of the following:

- Option 1: The Q value of the cell to be added is provide to UE via dedicated RRC signaling.

- Option 2: If no Q value is provided, UE assumes Q=8.

Proposal 5: Send an LS to RAN2 and RAN4 informing the down-selection results.

# Conclusion

In this contribution, we make the following proposals.

Proposal 1: Within a discovery burst transmission window, the number of candidate SS/PBCH blocks from the first transmitted SS/PBCH block and the last transmitted SS/PBCH block should not be greater than Q.

Proposal 2: Adopt the text proposal in this section to TS38.213.

Proposal 3: For RRM measurements of a neighboring cell, RAN1 down-selects one of the following:

- Option 1: At least a common Q value per frequency is provided to UE.

- Option 2: If no Q value is provided, UE assumes Q=8.

Proposal 4: For SCell addition and SCG addition, RAN1 down-selects one of the following:

- Option 1: The Q value of the cell to be added is provide to UE via dedicated RRC signaling.

- Option 2: If no Q value is provided, UE assumes Q=8.

Proposal 5: Send an LS to RAN2 and RAN4 informing the down-selection results.