3GPP TSG RAN WG1 #100			R1-2000450

e-Meeting, February 24th – March 6th, 2020

Agenda item:	7.2.4.7

Title:	Support of NR-Uu controlling LTE sidelink

Source:	MediaTek Inc.

Document for:	Discussion and Decision

# Introduction

In RAN1#97, the following agreement was made on the DCI payload size to activate/deactivate LTE SL-SPS [1].

Agreements: - RRC-based activation/deactivation is not supported - DCI-based activation/deactivation is supported - Support of LTE PC5 scheduling by NR Uu (mode 3-like ) is based on UE capability - NR DCI provides the fields of DCI 5A in LTE-V that are related to SPS schedulingd - The size of DCI for activation/deactivation is one of the DCI size(s) that will be defined for NR Uu scheduling NR V2V - FFS whether the DCI format is the same as one of the DCI formats that will be defined for NR Uu scheduling NR V2V - Activation/deactivation applies to the first LTE subframe after Z+X ms after receiving the DCI - Z is the same timing offset in current LTE V2X specs - X>0. FFS value(s) of X, and if one or multiple values of X are possible

In this contribution we discuss the remaining issue on NR-DCI format 3_1 which is used to activate/deactivate SL-SPS configurations in LTE sidelink controlled by NR Uu.

# DCI format 3_1

The payload size of DCI format 3_1 should be aligned with both DCI format 3_0 and DCI format 1_0/0_0 to minimize UE blind detection complexity. Compared with DCI format 3_0 and DCI format 0_0/0_1, DCI 3_1 payload size is smaller. Thus, zero padding can be used to match the DCI 3_0 payload size to DCI format 0_0/0_1.

We have the following proposal:

Proposal 1: Align DCI format 3_1 payload size with both DCI format 0_0/0_1 and DCI format 3_0 by zero padding.

# Conclusions

We have the following proposals:

Proposal 1: Align DCI format 3_1 payload size with both DCI format 0_0/0_1 and DCI format 3_0 by zero padding.

# References

- Chairman’s notes, RAN1#97, Reno, Nevada, May 2019.