3GPP TSG RAN WG1 Meeting #100							R1-2000540

e-Meeting, February 24th – March 6th, 2020

Source:	CATT

Title:	Remaining issues on UL SRS for NR Positioning

Agenda item:	7.2.8.2

Document for:	Discussion and Decision

Introduction

Most of the issues for Rel-16 NR Positioning Support WI were resolved in RAN1#98bis[1] and RAN1#99[2]. In this contribution, we discuss some remaining issues on UL SRS for positioning (SRS-Pos), especially the cyclic shifts of SRS-Pos.

Discussion

For Rl-16 positioning, staggered patterns were agreed for the transmission of the UL SRS for positioning (SRS-Pos) as shown in the following agreement[2]:

Agreement: (RAN1#99)

For each pair of comb size and number of symbols of SRS, there is one single RE pattern. The patterns in the table below are selected

| Number of symbols / Comb size | 1 | 2 | 4 | 8 | 12 |
| --- | --- | --- | --- | --- | --- |
| 2 | {0} | {0, 1} | {0, 1, 0, 1} | N/A | N/A |
| 4 | N/A | {0, 2} | {0,2,1,3}, | {0,2,1,3, 0,2,1,3} | {0,2,1,3, 0,2,1,3, 0,2,1,3} |
| 8 | N/A | N/A | {0,4,2,6} | {0,4,2,6,1,5,3,7} | {0,4,2,6,1,5,3,7,0,4,2,6} |

With the introduction of the staggered patterns for SRS-Pos, when the formula of the SRS cyclic shifts is re-used for the SRS-Pos, there are a number of potential issues. For example, there are phase discontinuities when a staggered SRS-Pos pattern is de-staggered (combined) for the SRS-Pos detection at the receiver. Also, when multiple UEs are multiplexed to the same REs for the transmission SRS-Pos based on the assignment of different cyclic shifts, the phase increments for different UEs on the REs are different. This may prevent the gNB from detecting the SRS-Pos signals from multiple UEs with the same correlation process. To overcome these issues, introducing different formulas for cyclic shifts for SRS-Pos was discussed in previous meetings without reaching a consensus[3].

In Rel16, the maximum number of SRS resources per set for positioning is defined to be16, and the maximum number of supported SRS resource sets for positioning can be up to 16 resource sets per BWP for a UE. In addition, for supporting the need to allocating many UEs simultaneously, the number of bits for the sequence ID for SRS-Pos is increased to 16. Furthermore, Rel16 supports the configuration of a SRS resource with NsymbSRS∈1,2,4,8,12 symbols located in anywhere is a UL slot and comb-size {2, 4, 8}. Therefore, the detection of SRS-Pos signals is expected to be a much more difficult than the detection of the SRS signals, especially for the SRS-Pos from remote neighboring TRPs.

According to TS 38.211, SRS/SRS-Pos sequence in a OFDM symbol l  will be generated according to

anKTC+k0(pi),l=βSRSejαnru,vnn=0,1,…,Msc,bRS-1l=0,1,...,NsymbSRS-10otherwise

α=2πnSRScsnSRScs,max

k0(pi)=nshiftNscRB+kTC(pi)+koffsetl mod KTC

Where KTC∈{2,4,8}  is the come-size, Msc,bSRS  is the SRS-Pos sequence length. The cyclic shift applies to each element of the base sequence ru,vn is related to the element index n and the configured cyclic shift index nSRScs∈{0, 1, …., nSRScs, max-1} , nSRScs, max={8, 12,6} for KTC∈{2,4,8} respectively. For SRS, koffsetl=0, and for SRS-Pos the quantity koffsetl is given the SRS-Pos staggering pattern, as shown in Table 1.

Table 1 The offset koffsetl  for SRS-Pos

| KTC | koffset0, …, koffsetNsymbSRS-1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | NsymbSRS=1 | NsymbSRS=2 | NsymbSRS=4 | NsymbSRS=8 | NsymbSRS=12 |
| 2 | 0 | 0,1 | 0,1,0,1 | - | - |
| 4 | - | 0, 2 | 0, 2, 1, 3 | 0, 2, 1, 3, 0, 2, 1, 3 | 0, 2, 1, 3, 0, 2, 1, 3, 0, 2, 1, 3 |
| 8 | - | - | 0, 4, 2, 6 | 0, 4, 2, 6, 1, 5, 3, 7 | 0, 4, 2, 6, 1, 5, 3, 7, 0, 4, 2, 6 |

For SRS, if we use comb-size 4 as an example, the maximum number of cyclic shifts M=nSRScs, max is 12. If we assume one UE i is configured for SRS transmission with cyclic shift indexes ni∈{0, . 1, …, 11} and base sequence run, we have the mapping of SRS sequences to the REs as shown in Table 2, where  αi=j2πni12

Table 2 Mapping of SRS sequences (UE i, Comb-size 4, Comb-size 4)

| … | … | … | … | … | … |
| --- | --- | --- | --- | --- | --- |
| 4m+3 |  |  |  |  |  |
| 4m+2 |  |  |  |  |  |
| 4m+1 |  |  |  |  |  |
| 4m | ejmairum | ejmairum | ejmairum | ejmairum | 4ejmairum |
| … |  |  |  |  |  |
| … | … | … | … | … | … |
| 11 |  |  |  |  |  |
| 10 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 8 | ej2airu2 | ej2airu2 | ej2airu2 | ej2airu2 | 4ej2airu2 |
| 7 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 4 | ejairu1 | ejairu1 | ejairu1 | ejairu1 | 4ejairu1 |
| 3 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 1 |  |  |  |  |  |
| 0 | ru0 | ru0 | ru0 | ru0 | 4ru0 |
| Subcarrier ↑ Symbols → | 0 | 1 | 2 | 3 | Combined |

As shown in Table 2, SRS sequence has the same phase incensement of αi=j2πni12 between the SRS REs for UE i.

For SRS-Pos, however, due to the staggered pattern with koffsetl={0, 2, 1, 3}, the mapping of SRS-Pos sequences is shown in Table 3. It shows that different phase offset incensements between the SRS-Pos REs for UE i.

Table 3 Mapping of SRS-Pos sequences (UE i, Comb-size 4, SRS-Pos pattern (0, 2, 1, 3))

| … | … | … | … | … | … |
| --- | --- | --- | --- | --- | --- |
| 4m+3 |  |  |  | ejmairum | ejmairum |
| 4m+2 |  | ejmairum |  |  | ejmairum |
| 4m+1 |  |  | ejmairum |  | ejmairum |
| 4m | ejmairum |  |  |  | ejmairum |
| … |  |  |  |  |  |
| … | … | … | … | … | … |
| 11 |  |  |  | ej2airu2 | ej2airu2 |
| 10 |  | ej2airu2 |  |  | ej2airu2 |
| 9 |  |  | ej2airu2 |  | ej2airu2 |
| 8 | ej2airu2 |  |  |  | ej2airu2 |
| 7 |  |  |  | ej2airu2 | ejairu1 |
| 6 |  | ejairu1 |  |  | ejairu1 |
| 5 |  |  | ejairu1 |  | ejairu1 |
| 4 | ejairu1 |  |  |  | ejairu1 |
| 3 |  |  |  | ru0 | ru0 |
| 2 |  | ru0 |  |  | ru0 |
| 1 |  |  | ru0 |  | ru0 |
| 0 | ru0 |  |  |  | ru0 |
| Subcarrier ↑ Symbols → | 0 | 1 | 2 | 3 | Combined |

Now let us assume there are total N UEs that are assigned to the same REs, but with different cyclic shift indexes ni∈{0, . 1, …, N-1}. Then, under the same assumption of comb-size 4, with the staggered pattern with koffsetl={0, 2, 1, 3}, the mappings of SRS-Pos sequences are shown in Table 4. It shows different phase offset incensements between the SRS-Pos subcarriers. It is known that the mapping makes it difficult to remove the cyclic shifts for the detection for the SRS-Pos, and causes aliasing problem of correlation peaks in SRS-Pos detection as discussed in previous meetings ([3-7]).

Table 4 Mapping of SRS-Pos sequences (N UEs, Comb-size 4, SRS-Pos pattern (0, 2, 1, 3), the ∑=i=0N-1

| … | … | … | … | … | … |
| --- | --- | --- | --- | --- | --- |
| 4m+3 |  |  |  | ∑ejmairum | rum∑ejmai |
| 4m+2 |  | ∑ejmairum |  |  | rum∑ejmai |
| 4m+1 |  |  | ∑ejmairum |  | rum∑ejmai |
| 4m | ∑ejmairum |  |  |  | rum∑ejmai |
| … |  |  |  |  |  |
| … | … | … | … | … | … |
| 11 |  |  |  | ∑ej2airu2 | ru2∑ej2ai |
| 10 |  | ∑ej2airu2 |  |  | ru2∑ej2ai |
| 9 |  |  | ∑ej2airu2 |  | ru2∑ej2ai |
| 8 | ∑ej2airu2 |  |  |  | ru2∑ej2ai |
| 7 |  |  |  | ∑ ejairu1 | ru1∑ejai |
| 6 |  | ∑ ejairu1 |  |  | ru1∑ejai |
| 5 |  |  | ∑ ejairu1 |  | ru1∑ejai |
| 4 | ∑ ejairu1 |  |  |  | ru1∑ejai |
| 3 |  |  |  | Nru0 | Nru0 |
| 2 |  | Nru0 |  |  | Nru0 |
| 1 |  |  | Nru0 |  | Nru0 |
| 0 | Nru0 |  |  |  | Nru0 |
| Subcarrier ↑ Symbols → | 0 | 1 | 2 | 3 | Combined |

A number of solutions were proposed to resolve above problem with the similar idea to introducing an adjustment of the cyclic shifts as a function of the staggered pattern, such that the phase offset incensement between the SRS-Pos subcarriers to be the same ([3-7]), which can be summarized into the following two options in [3]:

Option 1 : ru,v(αi,δ,l')n= ejαKTCn+ fl´maxKTC/NsymbSRS,1ru,vn if configured ru,vα,δn=ejαnru,vn,otherwise Option 2: ru,vαi,δ,l'n=ru,vα,δn ejfl´KTCα' α'=α+2πa α' configured via nSRSeff∈{0,…,KTCnmax-1} and a=nSRSeffnSRScs,max nSRSeff is configurable (range for cyclicshift is extended) Note: the maximum value of cyclic shift nSRScs,max is not changed

In this paper, we propose a similar, but a simplier solution to the cyclic shits of the SRS-Pos as follows:

anKTC+k0(pi),l=βSRSejαnru,vnejγl'n=0,1,…,Msc,bRS-1l=0,1,...,NsymbSRS-10otherwise

where

γl'=αimodk0pi,KTCKTc

The k0pi is the frequency-domain starting position of symbol l'.

With above change, we have the same phase increment for the SRS-Pos symbols, and the aliasing problem due to the uneven  phase increment should be resolved as shown in Table 5.

Table 5 Mapping of SRS-Pos sequences (Comb-size 4, SRS-Pos pattern (0, 2, 1, 3)

| … | … | … | … | … | … |
| --- | --- | --- | --- | --- | --- |
| 4m+3 |  |  |  | ej3/4αiejmairim | ej3/4αiejmairim |
| 4m+2 |  | ej2/4αi ejmairim |  |  | ej2/4αiejmairim |
| 4m+1 |  |  | ej1/4αiejmairim |  | ej1/4αiejmairim |
| 4m | ejmairim |  |  |  | ejmairim |
| … |  |  |  |  |  |
| … | … | … | … | … | … |
| 11 |  |  |  | ej3/4αiej2airi2 | ej3/4αiej2airi2 |
| 10 |  | ej2/4αi ej2airi2 |  |  | ej2/4αiej2airi2 |
| 9 |  |  | ej1/4αiej2airi2 |  | ej1/4αiej2airi2 |
| 8 | ej2airi2 |  |  |  | ej2airi2 |
| 7 |  |  |  | ej3/4αiejairi1 | ej3/4αiejairi1 |
| 6 |  | ej2/4αi ejairi1 |  |  | ej2/4αiejairi1 |
| 5 |  |  | ej1/4αiejairi1 |  | ej1/4αiejairi1 |
| 4 | ejairi1 |  |  |  | ejairi1 |
| 3 |  |  |  | ej3/4αiri0 | ej3/4αiri0 |
| 2 |  | ej2/4αi ejairi0 |  |  | ej2/4αiri0 |
| 1 |  |  | ej1/4αiri0 |  | ej1/4αiri0 |
| 0 | ri0 |  |  |  | ri0 |
| Subcarrier ↑ Symbols → | 0 | 1 | 2 | 3 | Combined |

| … | … | … | … | … | … |
| --- | --- | --- | --- | --- | --- |
| 4m+3 |  |  |  | ∑ej(m+3/4)airum | rum∑ej(m+3/4)ai |
| 4m+2 |  | ∑ej(m+2/4)airum |  |  | rum∑ej(m+2/4)ai |
| 4m+1 |  |  | ∑ej(m+1/4)airum |  | rum∑ej(m+1/4)ai |
| 4m | ∑ejmairum |  |  |  | rum∑ejmai |
| … |  |  |  |  |  |
| … | … | … | … | … | … |
| 11 |  |  |  | ∑ej(2+3/4)airu2 | ru2∑ej(2+3/4)ai |
| 10 |  | ∑ej(2+2/4)airu2 |  |  | ru2∑ej(2+2/4)ai |
| 9 |  |  | ∑ej(2+1/4)airu2 |  | ru2∑ej(2+1/4)ai |
| 8 | ∑ej2airu2 |  |  |  | ru2∑ej2ai |
| 7 |  |  |  | ∑ej(1+3/4)airu1 | ru1∑ej(1+3/4)ai |
| 6 |  | ∑ej(1+2/4)airu1 |  |  | ru1∑ej(1+2/4)ai |
| 5 |  |  | ∑ej(1+1/4)airu1 |  | ru1ej(1+1/4)ai |
| 4 | ∑ ejairu1 |  |  |  | ru1∑ejai |
| 3 |  |  |  | ∑ej(3/4)airu0 | ru0∑ej(3/4)ai |
| 2 |  | ∑ej(2/4)airu0 |  |  | ru0∑ej(2/4)ai |
| 1 |  |  | ∑ej(1/4)airu0 |  | ru0∑ej(1/4)ai |
| 0 | Nru0 |  |  |  | Nru0 |
| Subcarrier ↑ Symbols → | 0 | 1 | 2 | 3 | Combined |

Conclusions

In this contribution, we discuss some remaining issues on UL SRS for NR positioning. Based on the discussion, we propose:

Proposal 1: Adopt the following changes to Section 6.4.1.4.3 in TS 38.211

- 6.4.1.4.3 Mapping to physical resources When SRS is transmitted on a given SRS resource, the sequence rpi(n,l') for each OFDM symbol and for each of the antenna ports of the SRS resource shall be multiplied with the amplitude scaling factor in order to conform to the transmit power specified in [5, 38.213] and mapped in sequence starting with to resource elements in a slot for each of the antenna ports according to aKTCk'+k0(pi),l'+l0(pi)=1NapβSRSrpik',l'ejγl'k'=0,1,…,Msc,bRS-1l'=0,1,...,NsymbSRS-10otherwise γl'=αimodk0pi,KTCKTc The k0(pi) is the frequency-domain starting position of symbol l'.

References

- 3GPP RAN1 Chairman’s Notes of TSG RAN WG1 Meeting #98-bis

- 3GPP RAN1 Chairman’s Notes of TSG RAN WG1 Meeting #99

- R1-1913507, “Session notes #2 on offline discussion 7.2.10.2 UL Reference Signals for NR Positioning”,  Ericsson

- R1-1912113, “Uplink RS design for NR positioning”, MediaTek Inc.

- R1-1913101, “Remaining details on SRS enhancements for positioning”, Fraunhofer IIS, Fraunhofer HHI

- R1-1913136, “UL Reference Signals for NR Positioning”, Ericsson

- R1-1911897, “Remaining issues on SRS for NR positioning”, Huawei, HiSilicon

- R1-1912293, “Views on UL reference signals for NR Positioning”, Nokia, Nokia Shanghai Bell

- R1-1913675, “LS on updated Rel-16 LTE and NR parameter lists”, RAN1, Qualcomm

- R1-1913674, “Updated consolidated parameter list for Rel-16 NR”, Qualcomm