3GPP TSG-RAN WG1 Meeting #100              	R1-2000557

# e-Meeting, February 24th – March 6th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.215 | CR | draft | rev | - | Current version: | 16.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of UE UTRAN FDD measurements for SRVCC from NR to UMTS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | SRVCC_NR_to_UMTS-Core |  |  |  |  |  | Date: |  |  | 05/14/2020 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | UTRAN FDD measurement quantities need to be defined for UE in this specification to prepare SRVCC handover from NR to UTRAN FDD. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UTRAN FDD measurement quantities (UTRA FDD CPICH RSCP, UTRA FDD carrier RSSI, UTRA FDD CPICH Ec/No) for UE are introduced in subclause 5.1. The measurement support is only applicable for RRC_CONNECTED state for performing inter-RAT measurements with appropriate measurement gaps, in alignment with R4-1914447, clause 10.3, included in RP-193009 which has been approved at RAN #86. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No UE UTRAN FDD measurement support possible for the SRVCC from NR to UMTS feature (Rel-16). Inconsistency to RAN4 CR R4-1914447 included in RP-193009 approved at RAN #86, referring to these UE UTRAN FDD measurement quantities. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.x (new), 5.1.y (new), 5.1.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS38.133, TS38.331 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

### 5.1.x	UTRA FDD CPICH RSCP

| Definition | Received Signal Code Power, the received power on one code measured on the Primary CPICH. The reference point for the RSCP shall be the antenna connector of the UE. If Tx diversity is applied on the Primary CPICH the received code power from each antenna shall be separately measured and summed together in [W] to a total received code power on the Primary CPICH. If receiver diversity is in use by the UE, the reported value shall not be lower than the corresponding CPICH RSCP of any of the individual receive antenna branches. |
| --- | --- |
| Applicable for | RRC_CONNECTED inter-RAT |

### 5.1.y	UTRA FDD carrier RSSI

| Definition | The received wide band power, including thermal noise and noise generated in the receiver, within the bandwidth defined by the receiver pulse shaping filter. The reference point for the measurement shall be the antenna connector of the UE. If receiver diversity is in use by the UE, the reported value shall not be lower than the corresponding UTRA carrier RSSI of any of the individual receive antenna branches. |
| --- | --- |
| Applicable for | RRC_CONNECTED inter-RAT |

NOTE:	This definition does not correspond to a reported measurement. This definition is just an intermediate definition used in the definition of UTRA FDD CPICH Ec/No.

### 5.1.z	UTRA FDD CPICH Ec/No

| Definition | The received energy per chip divided by the power density in the band. If receiver diversity is not in use by the UE, the CPICH Ec/No is identical to CPICH RSCP/UTRA Carrier RSSI. Measurement shall be performed on the Primary CPICH. The reference point for the CPICH Ec/No shall be the antenna connector of the UE. If Tx diversity is applied on the Primary CPICH the received energy per chip (Ec) from each antenna shall be separately measured and summed together in [Ws] to a total received chip energy per chip on the Primary CPICH, before calculating the Ec/No. If receiver diversity is in use by the UE, the measured CPICH Ec/No value shall not be lower than the corresponding CPICH RSCPi/UTRA Carrier RSSIi of receive antenna branch i . |
| --- | --- |
| Applicable for | RRC_CONNECTED inter-RAT |

End of Change 1