3GPP TSG RAN WG1 #100                                                                                       R1-2000593

E-meeting, Feb. 24th – Mar. 6th, 2020

Agenda Item:	7.2.2.2.4

Source:	Lenovo, Motorola Mobility

Title:	Text proposals for configured grant enhancement for NR-U

Document for:	Discussion & Decision

# Introduction

The contribution is focused on text proposals for configured grant enhancement in Rel-16 NR-U.

# Discussion

## CG-DFI transmission

Regarding CG-DFI transmission, the related agreement is made in

Agreement: - For DFI design for configured grants, support at least the following - DFI including at least TB level HARQ-ACK bitmap for all UL HARQ processes - Note: Total number of HARQ processes is as defined in Rel-15 - FFS: CBG level HARQ-ACK feedback, if supported - RRC configured minimum duration, D, from the ending symbol of the PUSCH to the starting symbol of the DFI carrying HARQ-ACK for that PUSCH - Note: UE assumes HARQ-ACK is valid only for PUSCH transmissions ending before n-D, where n is the time corresponding to the beginning of the start symbol of the DFI. - FFS: the definition of minimum duration for the case of slot aggregation - UE blind decoding complexity shall not be increased due to DFI size

As mentioned above, HARQ-ACK bitmap in DFI includes TB-level HARQ-ACK bitmap for all UL HARQ processes and the total number of HARQ processes is as defined in Rel-15.

According to TS38.214-v15.8.0, for uplink, 16 HARQ processes per cell is supported by the UE. The related text is listed below:

6.1 UE procedure for transmitting the physical uplink shared channel <Unrelated parts are omitted> For uplink, 16 HARQ processes per cell is supported by the UE. <Unrelated parts are omitted>

Therefore, in Section 7.3.1.1.2 of TS38.212, the square brackets for the size of the HARQ-ACK bitmap should be removed. At the same time, we suggest fixing some typos:

-----------------------------------------------< BEGIN TEXT PROPOSAL >-------------------------------------------------

##### 7.3.1.1.2	Format 0_1

DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE.

The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

-	Carrier indicator – 0 or 3 bits, as defined in Subclause 10.1 of [5, TS38.213].

-	DFI flag – 0 or 1 bit

-	1 bit if the UE is configured to monitor DCI format 0_1 with CRC scrambled by CS-RNTI and for operation in a cell with shared spectrum channel access. For a DCI format 0_1 with CRC scrambled by CS-RNTI, the bit value of 0 indicates activating type 2 CG transmission and the bit value of 1 indicates CG-DFI. For a DCI format 0_1 with CRC scrambled by C-RNTI/SP-CSI-RNTI/MCS-C-RNTI, the bit is reserved.

-	0 bit otherwise;

If DCI format 0_1 is used for indicating CG-DFI, all the remaining fileds are set as follows:

- HARQ-ACK bitmap –16 bits , where the order of the bitmap to HARQ process index mapping is such that HARQ process indices are mapped in ascending order from MSB to LSB of the bitmap. For each bit of the bitmap, value 1 indicates ACK, and value 0 indicates NACK.

- TPC command for scheduled PUSCH – 2 bits as defined in Subclause 7.1.1 of [5, TS38.213]

- All the remaining bits in format 0_1 are set to zero.

If DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell or activating type 2 CG transmission, all the remaining fields are set as follows:

<Unrelated parts are omitted>

-----------------------------------------------< END TEXT PROPOSAL >-------------------------------------------------

## Early termination for TB repetition

In RAN1#99 meeting, regarding TB repetition, early termination is supported when UE receives an ACK in the CG-DFI for the giving HARQ process. The related agreement is listed below:

Agreement: When the UE is configured with repK > 1, repetition of a TB is mapped within a configuration in the case when an UE is configured with multiple active configurations - The UE repeats the TB in the earliest consecutive transmission occasion candidates within the same configuration instead of consecutive slots - The UE may drop repetition transmissions that fall into a subsequent configured period. - The UE terminates the repetitions if an explicit feedback indicating ACK in the DFI is received for the HARQ process. Based on the above agreement, UE behavior of early termination should be captured in TS38.214 to reflect the agreement. We have below text proposals for TS38.214:

-----------------------------------------------< BEGIN TEXT PROPOSAL >-------------------------------------------------

##### 6.1.2.3.1	Transport Block repetition for uplink transmissions with a configured grant

The higher layer configured parameters repK and repK-RV define the K repetitions to be applied to the transmitted transport block, and the redundancy version pattern to be applied to the repetitions. If the parameter repK-RV is not provided in the configuredGrantConfig, the redundancy version for uplink transmissions with a configured grant shall be set to 0. Otherwise, for the nth transmission occasion among K repetitions, n=1, 2, …, K, it is associated with (mod(n-1,4)+1)th value in the configured RV sequence. The initial transmission of a transport block may start at

-	the first transmission occasion of the K repetitions if the configured RV sequence is {0,2,3,1},

-	any of the transmission occasions of the K repetitions that are associated with RV=0 if the configured RV sequence is {0,3,0,3},

-	any of the transmission occasions of the K repetitions if the configured RV sequence is {0,0,0,0}, except the last transmission occasion when K=8.

For any RV sequence, the repetitions shall be terminated after transmitting K repetitions, or at the last transmission occasion among the K repetitions within the period P, or from the starting symbol of the repetition that overlaps with a PUSCH with the same HARQ process scheduled by DCI format 0_0 or 0_1, whichever is reached first. The UE is not expected to be configured with the time duration for the transmission of K repetitions larger than the time duration derived by the periodicity P. If the UE determines that, for a transmission occasion, the number of symbols available for the PUSCH transmission in a slot is smaller than transmission duration L, the UE does not transmit the PUSCH in the transmission occasion.

For both Type 1 and Type 2 PUSCH transmissions with a configured grant, when the UE is configured with repK > 1, the UE shall repeat the TB across the repK consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. A Type 1 or Type 2 PUSCH transmission with a configured grant in a slot is omitted according to the conditions in Subclause 11.1 of [6, TS38.213]. The UE shall terminate the transmission of repetitions if an ACK for the corresponding HARQ process is received in the CG-DFI before completion of transmitting all K repetitions.

<omitted text>

-----------------------------------------------< END TEXT PROPOSAL >-------------------------------------------------

## CG-UCI transmission

In RAN1#99 meeting, regarding CG-UCI transmission, CG-UCI is included in every CG-PUSCH transmission for simplicity. The related agreement is listed below:

Agreement: CG-UCI is included in every CG-PUSCH transmission (confirms working assumption from RAN1#98)

Based on the above agreement, UE behavior of CG-UCI transmission should be captured in TS38.213 to reflect the agreement. We have below text proposals for TS38.213:

-----------------------------------------------8< BEGIN TEXT PROPOSAL >8-------------------------------------------------

## 9.3 UCI reporting in physical uplink shared channel

Offset values are defined for a UE to determine a number of resources for multiplexing HARQ-ACK information and for multiplexing CSI reports in a PUSCH. Offset values are also defined for multiplexing CG-UCI [5, TS 38.212] in a CG-PUSCH. The offset values are signalled to a UE either by a DCI format scheduling the PUSCH transmission or by higher layers.

If DCI format 0_0, or DCI format 0_1 that does not include a beta_offset indicator field, schedules the PUSCH transmission from the UE and the UE is provided betaOffsets = 'semiStatic', the UE applies the , , and  values that are provided by betaOffsets = 'semiStatic' for the corresponding HARQ-ACK information, Part 1 CSI reports and Part 2 CSI reports.

HARQ-ACK information offsets  are configured to values according to Table 9.3-1. The betaOffsetACK-Index1, betaOffsetACK-Index2, and betaOffsetACK-Index3 respectively provide indexes , , and  for the UE to use if the UE multiplexes up to 2 HARQ-ACK information bits, more than 2 and up to 11 HARQ-ACK information bits, and more than 11 bits in the PUSCH, respectively.

Part 1 CSI report and Part 2 CSI report offsets  and , respectively, are configured to values according to Table 9.3-2. The betaOffsetCSI-Part1-Index1 and betaOffsetCSI-Part2-Index1 respectively provide indexes  and  for the UE to use if the UE multiplexes up to 11 bits for Part 1 CSI reports or Part 2 CSI reports in the PUSCH. The betaOffsetCSI-Part1-Index2 and betaOffsetCSI-Part2-Index2 respectively provide indexes  or  for the UE to use if the UE multiplexes more than 11 bits for Part 1 CSI reports or Part 2 CSI reports in the PUSCH.

If a DCI format 0_1 schedules the PUSCH transmission from the UE and if DCI format 0_1 includes a beta_offset indicator field, as configured by uci-OnPUSCH, the UE is provided by each of {betaOffsetACK-Index1, betaOffsetACK-Index2, betaOffsetACK-Index3} a set of four  indexes, by each of {betaOffsetCSI-Part1-Index1, betaOffsetCSI-Part1-Index2} a set of four  indexes and by each of {betaOffsetCSI-Part2-Index1, betaOffsetCSI-Part2-Index2} a set of four  indexes from Table 9.3-1 and 9.3-2, respectively, for multiplexing HARQ-ACK information, Part 1 CSI reports, and Part 2 CSI reports, respectively, in the PUSCH transmission. The beta_offset indicator field indicates a  value, a  value and a  value from the respective sets of values, with the mapping defined in Table 9.3-3.

For PUSCH transmissions configured by ConfiguredGrantConfig, or for activated PUSCH transmissions configured by semiPersistentOnPUSCH, associated CG-UCI is multiplexed on every CG-PUSCH transmission.

For a PUSCH transmission that is configured by a ConfiguredGrantConfig, or for an activated PUSCH transmission that is configured by semiPersistentOnPUSCH, and includes CG-UCI, the UE is provided by betaOffsetCG-UCI-r16 a IoffsetCG-UCI value, from a set of values, with the mapping defined in Table 9.3-4. If the UE multiplexes HARQ-ACK information in the PUSCH transmission, as described in Subclause 9.2.5, the UE jointly encodes the HARQ-ACK information and the CG-UCI [5, TS 38.212] and determines a corresponding IoffsetHARQ-ACK/CG-UCI value as TBD.

-----------------------------------------------8< END TEXT PROPOSAL >8-------------------------------------------------

# Conclusion

In this contribution, we focus on the configured grant PUSCH transmission issues and propose some text proposals for DCI format 0_1, early termination of repetitions, and CG-UCI transmission as above.

# References

TS38.211-g00

TS38.212-g00

TS38.213-g00

TS38.214-g00