3GPP TSG RAN WG1 #100			  R1-2000609

e-Meeting, February 24th – March 6th, 2020

Agenda item:	7.2.2.1.3

Source:	Samsung

Title:	Uplink signal and channel design for NR-U

Document for:	Discussion and Decision

- Introduction

In RAN1 99 meeting, most aspects of UL signals/channels have been finalized. Whether RB sets can be indicated by fallback DCI has not been resolved yet. RAN1 agreed the FDRA field for fallback DCI in CSS does not include Y bits for RB sets indication [1] while no conclusion for fallback DCI in USS.

Agreement: - For interlaced PUSCH transmission in a BWP, Y bits of the frequency domain resource allocation (FDRA) field indicate which RB sets (corresponding to LBT bandwidths) are allocated to the UE - This applies to PUSCH of the following types - PUSCH scheduled by at least non-fallback DCI - FFS: applicability to fallback DCI - Configured Grant PUSCH Type 2 (FDRA indicated by DCI) - Configured Grant PUSCH Type 1 (FDRA configured by RRC) - The UE determines the overall PUSCH frequency domain resource allocation by the intersection of the following: - Allocated interlaces (indicated by X bits of the FDRA field, as previously agreed) - Available PRBs derived at least from the allocated RB sets (indicated by Y bits of the FDRA field) and intra-carrier guard bands between RB sets corresponding to contiguous LBT bandwidths - Note: An RB set contains PRBs within an LBT bandwidth and does not include any inter or intra carrier guard PRBs - Note: The PRBs between adjacent RB sets comprise an intra-carrier guard - Y is determined by the number of RB sets contained in the BWP - The Y bits indicate a first RB set and a number of RB sets corresponding to contiguous LBT bandwidths - Note: The maximum possible value of Y is thus log2NN+12 where N is the number of RB sets contained in the BWP 3 Agreement: - The UE transmits PUSCH scheduled by fallback DCI in CSS within the initial BWP on the interlaces indicated by the X bits of the FDRA field - Note: The FDRA field for fallback DCI in CSS does not include Y bits

Besides, RAN1 agreed to support user-multiplexing for PUCH format 2/3 with OCC, but the rate matching for PUCCH format 2/3 in TS38.212 does not include the OCC length.

- Discussion

LBT bandwidth indication in DCI 0_0 in USS

For PUSCH scheduled by normal DCI, FDRA field includes Y bits for indicating the RB sets allocated to the UE in addition to X bits for interlace index. For PUSCH scheduled by fallback DCI in CSS, FDRA field does not include Y bits, considering PUSCH transmission is within initial BWP which is typically only one RB set. For PUSCH scheduled by fallback DCI in USS, PUSCH transmission is within active UL BWP, which may include multiple RB sets. It is beneficial to support PUSCH transmission over some of RB sets within the active UL BWP, if the indication of Y bits does not increase the DCI overhead. As is known, DCI 0_0 and DCI 1_0 size should be aligned. Comparing the payload of DCI 0_0 with X+Y bits and DCI 1_0, it can be seen that the payload of DCI 0_0 is still smaller than DCI 0_1 even if active UL BWP bandwidth is much larger than DL BWP bandwidth, e.g. UL BWP bandwidth =100MHz and DL BWP bandwidth =20MHz.

Table 1 DCI payload for DCI 0_0 and DCI 1_0 if interlace is configured

|  | DCI 0_0 | DCI 1_0 |
| --- | --- | --- |
| Identifier for DCI formats | 1 | 1 |
| FDRA | - 30KHz, 5+Y - 15KHz, 6+Y Y=0,1,2,3,4 depending on BWP bandwidth | Minimum bit length=11 (20MHz BWP and 30KHz SCS) |
| TDRA | 4 | 4 |
| Frequency Hopping | 1 | NA |
| VRB-to-PRB mapping | NA | 1 |
| MCS | 5 | 5 |
| NDI | 1 | 1 |
| RV | 2 | 2 |
| HARQ process number | 4 | 4 |
| DAI | NA | 2 |
| TPC for PUSCH | 2 | NA |
| TPC for PUCCH | NA | 2 |
| UL/SUL indicator | 0/1 | NA |
| PUCCH resource indicator | NA | 3 |
| PDSCH-to-HARQ_feedback timing indicator | NA | 3 |
| ChannelAccess-CPext | 2 | 2 |
| Total | Maximum payload= 33 bits (15KHz, Y=4 bits) | Minimum payload = 41 bits (20MHz BWP and 30KHz SCS) |

Proposal 1: For fallback DCI in USS, FDRA field includes both X bits for interlace index and Y bits for RB sets. Adopt the following TP for TS 38.212.

The corresponding TP proposal is provided as below,

TS 38.212 7.3.1.1.1 Format 0_0 DCI format 0_0 is used for the scheduling of PUSCH in one cell. The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: - Identifier for DCI formats – 1 bit - The value of this bit field is always set to 0, indicating an UL DCI format - Frequency domain resource assignment – bits where is defined in clause 7.3.1.0 - For PUSCH hopping with resource allocation type 1: - MSB bits are used to indicate the frequency offset according to Clause 6.3 of [6, TS 38.214], where if the higher layer parameter frequencyHoppingOffsetLists contains two offset values and if the higher layer parameter frequencyHoppingOffsetLists contains four offset values - bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - For non-PUSCH hopping with resource allocation type 1: - bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - If the higher layer parameter useInterlacePUSCH-Dedicated-r16 or useInterlacePUSCH-Common-r16 is configured, - 5 + Y bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 30 kHz - 6 + Y bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 15 kHz The value of Y is 0 if DCI format 0_0 is in CSS, otherwise, it is determined by log2NN+12 where N is the number of RB sets contained in the BWP as defined in clause x of [x]. - Time domain resource assignment – 4 bits as defined in Clause 6.1.2.1 of [6, TS 38.214] - Frequency hopping flag – 1 bit according to Table 7.3.1.1.1-3, as defined in Clause 6.3 of [6, TS 38.214] - Modulation and coding scheme – 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214] - New data indicator – 1 bit - Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2 - HARQ process number – 4 bits - TPC command for scheduled PUSCH – 2 bits as defined in Clause 7.1.1 of [5, TS 38.213] - Padding bits, if required. - UL/SUL indicator – 1 bit for UEs configured with supplementaryUplink in ServingCellConfig in the cell as defined in Table 7.3.1.1.1-1 and the number of bits for DCI format 1_0 before padding is larger than the number of bits for DCI format 0_0 before padding; 0 bit otherwise. The UL/SUL indicator, if present, locates in the last bit position of DCI format 0_0, after the padding bit(s). - If the UL/SUL indicator is present in DCI format 0_0 and the higher layer parameter pusch-Config is not configured on both UL and SUL the UE ignores the UL/SUL indicator field in DCI format 0_0, and the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config is configured; - If the UL/SUL indicator is not present in DCI format 0_0 and pucch-Config is configured, the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config is configured. - If the UL/SUL indicator is not present in DCI format 0_0 and pucch-Config is not configured, the corresponding PUSCH scheduled by the DCI format 0_0 is for the uplink on which the latest PRACH is transmitted. - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for operation in a cell with shared spectrum channel access; 0 bit otherwise. The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI: - Identifier for DCI formats – 1 bit - The value of this bit field is always set to 0, indicating an UL DCI format - Frequency domain resource assignment – number of bits determined by the following: - bits if the higher layer parameter useInterlacePUSCH-Common-r16 is not configured, where - is the size of the initial UL bandwidth part. - For PUSCH hopping with resource allocation type 1: - MSB bits are used to indicate the frequency offset according to Table 8.3-1 in Clause 8.3 of [5, TS 38.213], where if and otherwise - bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - For non-PUSCH hopping with resource allocation type 1: - bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] if the higher layer parameter useInterlacePUSCH-Common-r16 is configured - 5 bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 30 kHz - 6 bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 15 kHz - Time domain resource assignment – 4 bits as defined in Clause 6.1.2.1 of [6, TS 38.214] - Frequency hopping flag – 1 bit according to Table 7.3.1.1.1-3, as defined in Clause 6.3 of [6, TS 38.214] - Modulation and coding scheme – 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214] - New data indicator – 1 bit, reserved - Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2 - HARQ process number – 4 bits, reserved - TPC command for scheduled PUSCH – 2 bits as defined in Clause 7.1.1 of [5, TS 38.213] - Padding bits, if required. - UL/SUL indicator – 1 bit if the cell has two ULs and the number of bits for DCI format 1_0 before padding is larger than the number of bits for DCI format 0_0 before padding; 0 bit otherwise. The UL/SUL indicator, if present, locates in the last bit position of DCI format 0_0, after the padding bit(s). - If 1 bit, reserved, and the corresponding PUSCH is always on the same UL carrier as the previous transmission of the same TB - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for operation in a cell with shared spectrum channel access; 0 bit otherwise

PUCCH format 2 and 3 rate-matching length

In NR-U, in addition to interlace structure resource mapping, PUCCH format 2 and 3 are enhanced to be equipped with orthogonal cover code as a user-multiplexing mechanism. However, the orthogonal cover code length is not included in the rate matching output sequence length specified in section 6.3.1.4 TS 38.212. We propose the text proposal below for the correction.

Proposal 2: Rate matching for PUCCH format 2/3 should consider OCC-Length-r16, if provided. Adopt the following TP for TS 38.212.

TS 38.212 6.3.1.4 Rate matching For PUCCH formats 2/3/4, the total rate matching output sequence length is given by Table 6.3.1.4-1, where , , and are the number of symbols carrying UCI for PUCCH formats 2/3/4 respectively; and are the number of PRBs that are determined by the UE for PUCCH formats 2/3 transmission respectively according to Clause 9.2 of [5, TS38.213]; and is the spreading factor for PUCCH format 4. , NSFPUCCH,2 and NSFPUCCH,3 are orthogonal cover code lengths provided by OCC-Length-r16. Table 6.3.1.4-1: Total rate matching output sequence length PUCCH format; Modulation order. QPSK; π/2-BPSK. PUCCH format 2; or 16∙Nsymb,UCIPUCCH,2∙NPRBPUCCH,2 / NSFPUCCH,2, if NSFPUCCH,2 is provided; N/A. PUCCH format 3; or 24∙Nsymb,UCIPUCCH,3∙NPRBPUCCH,3 / NSFPUCCH,3, if NSFPUCCH,3 is provided; or 12∙Nsymb,UCIPUCCH,3∙NPRBPUCCH,3 / NSFPUCCH,3, if NSFPUCCH,3 is provided. PUCCH format 4.

- Conclusions

Proposal 1: For fallback DCI in USS, FDRA field includes both X bits for interlace index and Y bits for RB sets. Adopt the following TP for TS 38.212.

| TS 38.212 7.3.1.1.1 Format 0_0 DCI format 0_0 is used for the scheduling of PUSCH in one cell. The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: - Identifier for DCI formats – 1 bit - The value of this bit field is always set to 0, indicating an UL DCI format - Frequency domain resource assignment – bits where is defined in clause 7.3.1.0 - For PUSCH hopping with resource allocation type 1: - MSB bits are used to indicate the frequency offset according to Clause 6.3 of [6, TS 38.214], where if the higher layer parameter frequencyHoppingOffsetLists contains two offset values and if the higher layer parameter frequencyHoppingOffsetLists contains four offset values - bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - For non-PUSCH hopping with resource allocation type 1: - bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - If the higher layer parameter useInterlacePUSCH-Dedicated-r16 or useInterlacePUSCH-Common-r16 is configured, - 5 + Y bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 30 kHz - 6 + Y bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 15 kHz The value of Y is 0 if DCI format 0_0 is in CSS, otherwise, it is determined by log2NN+12 where N is the number of RB sets contained in the BWP as defined in clause x of [x]. - Time domain resource assignment – 4 bits as defined in Clause 6.1.2.1 of [6, TS 38.214] - Frequency hopping flag – 1 bit according to Table 7.3.1.1.1-3, as defined in Clause 6.3 of [6, TS 38.214] - Modulation and coding scheme – 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214] - New data indicator – 1 bit - Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2 - HARQ process number – 4 bits - TPC command for scheduled PUSCH – 2 bits as defined in Clause 7.1.1 of [5, TS 38.213] - Padding bits, if required. - UL/SUL indicator – 1 bit for UEs configured with supplementaryUplink in ServingCellConfig in the cell as defined in Table 7.3.1.1.1-1 and the number of bits for DCI format 1_0 before padding is larger than the number of bits for DCI format 0_0 before padding; 0 bit otherwise. The UL/SUL indicator, if present, locates in the last bit position of DCI format 0_0, after the padding bit(s). - If the UL/SUL indicator is present in DCI format 0_0 and the higher layer parameter pusch-Config is not configured on both UL and SUL the UE ignores the UL/SUL indicator field in DCI format 0_0, and the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config is configured; - If the UL/SUL indicator is not present in DCI format 0_0 and pucch-Config is configured, the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config is configured. - If the UL/SUL indicator is not present in DCI format 0_0 and pucch-Config is not configured, the corresponding PUSCH scheduled by the DCI format 0_0 is for the uplink on which the latest PRACH is transmitted. - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for operation in a cell with shared spectrum channel access; 0 bit otherwise. The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI: - Identifier for DCI formats – 1 bit - The value of this bit field is always set to 0, indicating an UL DCI format - Frequency domain resource assignment – number of bits determined by the following: - bits if the higher layer parameter useInterlacePUSCH-Common-r16 is not configured, where - is the size of the initial UL bandwidth part. - For PUSCH hopping with resource allocation type 1: - MSB bits are used to indicate the frequency offset according to Table 8.3-1 in Clause 8.3 of [5, TS 38.213], where if and otherwise - bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - For non-PUSCH hopping with resource allocation type 1: - bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] if the higher layer parameter useInterlacePUSCH-Common-r16 is configured - 5 bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 30 kHz - 6 bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 15 kHz - Time domain resource assignment – 4 bits as defined in Clause 6.1.2.1 of [6, TS 38.214] - Frequency hopping flag – 1 bit according to Table 7.3.1.1.1-3, as defined in Clause 6.3 of [6, TS 38.214] - Modulation and coding scheme – 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214] - New data indicator – 1 bit, reserved - Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2 - HARQ process number – 4 bits, reserved - TPC command for scheduled PUSCH – 2 bits as defined in Clause 7.1.1 of [5, TS 38.213] - Padding bits, if required. - UL/SUL indicator – 1 bit if the cell has two ULs and the number of bits for DCI format 1_0 before padding is larger than the number of bits for DCI format 0_0 before padding; 0 bit otherwise. The UL/SUL indicator, if present, locates in the last bit position of DCI format 0_0, after the padding bit(s). - If 1 bit, reserved, and the corresponding PUSCH is always on the same UL carrier as the previous transmission of the same TB - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for operation in a cell with shared spectrum channel access; 0 bit otherwise |  |
| --- | --- |

Proposal 2: Rate matching for PUCCH format 2/3 should consider OCC-Length-r16, if provided. Adopt the following TP for TS 38.212.

| TS 38.212 6.3.1.4 Rate matching For PUCCH formats 2/3/4, the total rate matching output sequence length is given by Table 6.3.1.4-1, where , , and are the number of symbols carrying UCI for PUCCH formats 2/3/4 respectively; and are the number of PRBs that are determined by the UE for PUCCH formats 2/3 transmission respectively according to Clause 9.2 of [5, TS38.213]; and is the spreading factor for PUCCH format 4. , NSFPUCCH,2 and NSFPUCCH,3 are orthogonal cover code lengths provided by OCC-Length-r16. Table 6.3.1.4-1: Total rate matching output sequence length PUCCH format; Modulation order. QPSK; π/2-BPSK. PUCCH format 2; or 16∙Nsymb,UCIPUCCH,2∙NPRBPUCCH,2 / NSFPUCCH,2, if NSFPUCCH,2 is provided; N/A. PUCCH format 3; or 24∙Nsymb,UCIPUCCH,3∙NPRBPUCCH,3 / NSFPUCCH,3, if NSFPUCCH,3 is provided; or 12∙Nsymb,UCIPUCCH,3∙NPRBPUCCH,3 / NSFPUCCH,3, if NSFPUCCH,3 is provided. PUCCH format 4. |  |
| --- | --- |

References

[1] RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting 99.