3GPP TSG RAN WG1#100-e			R1-2000632

e-Meeting, 24th February – 6th March, 2020

Agenda item:	7.2.6.1

Source: 	Samsung

Title: 	Correction on CSI part 2 payload in 38.212

Document for:	Discussion and Decision

# Issue

The following is the description of the PMI payload for the Rel. 16 Type II CSI codebook based on the current version of TS 38.212 V16.0.0 [1].

----Start of the description---

The bitwidth for PMI of codebookType=typeII-r16 is provided in Tables 6.3.2.1.2-1A, where the values of (N1, N2), (O1, O2), L, KNZ, TOT, N3, and Mll=1,…, υ are given by Clause 5.2.2.2.5 in [6, TS 38.214].

Table 6.3.2.1.2-1A: PMI of codebookType= typeII-r16

|  |  |  | Information fields X1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | i1,1 |  |  |  | i1,2 |  | i1,8,1 |  |  | i1,8,2 |  | i1,8,3 |  |  | i1,8,4 |  |
| Rank=1 N3≤19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2KNZ,TOT |  |  | N/A |  | N/A |  |  | N/A |  |
| Rank=2 N3≤19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2(2L) |  |  | log2(2L) |  | N/A |  |  | N/A |  |
| Rank=3 N3≤19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2(2L) |  |  | log2(2L) |  | log2(2L) |  |  | N/A |  |
| Rank=4 N3≤19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2(2L) |  |  | log2(2L) |  | log2(2L) |  |  | log2(2L) |  |
| Rank=1 N3>19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2KNZ,TOT |  |  | N/A |  | N/A |  |  | N/A |  |
| Rank=2 N3>19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2(2L) |  |  | log2(2L) |  | N/A |  |  | N/A |  |
| Rank=3 N3>19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2(2L) |  |  | log2(2L) |  | log2(2L) |  |  | N/A |  |
| Rank=4 N3>19 |  |  | log2(O1O2) |  |  |  | log2N1N2L |  | log2(2L) |  |  | log2(2L) |  | log2(2L) |  |  | log2(2L) |  |
|  | Information fields X2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | i2,3,1 | i2,3,2 |  | i2,3,3 | i2,3,4 | i1,5 |  | i1,6,1 |  | i1,6,2 | i1,6,3 |  | i1,6,4 |  | {i2,4,l}l=1,…,υ | {i2,5,l}l=1,…,υ |  | {i1,7,l}l=1,…,υ |
| Rank=1 N3≤19 | 4 | N/A |  | N/A | N/A | N/A |  | log2N3-1M1-1 |  | N/A | N/A |  | N/A |  | 3(KNZ,TOT-1) | 4(KNZ,TOT-1) |  | 2LM1 |
| Rank=2 N3≤19 | 4 | 4 |  | N/A | N/A | N/A |  | log2N3-1M2-1 |  | log2N3-1M2-1 | N/A |  | N/A |  | 3(KNZ,TOT-2) | 4(KNZ,TOT-2) |  | 4LM2 |
| Rank=3 N3≤19 | 4 | 4 |  | 4 | N/A | N/A |  | log2N3-1M3-1 |  | log2N3-1M3-1 | log2N3-1M3-1 |  | N/A |  | 3(KNZ,TOT-3) | 4(KNZ,TOT-3) |  | 6LM3 |
| Rank=4 N3≤19 | 4 | 4 |  | 4 | 4 | N/A |  | log2N3-1M4-1 |  | log2N3-1M4-1 | log2N3-1M4-1 |  | log2N3-1M4-1 |  | 3(KNZ,TOT-4) | 4(KNZ,TOT-4) |  | 8LM4 |
| Rank=1 N3>19 | 4 | N/A |  | N/A | N/A | log2(2M1) |  | log22M1-1M1-1 |  | N/A | N/A |  | N/A |  | 3(KNZ,TOT-1) | 4(KNZ,TOT-1) |  | 2LM1 |
| Rank=2 N3>19 | 4 | 4 |  | N/A | N/A | log2(2M2) |  | log22M2-1M2-1 |  | log22M2-1M2-1 | N/A |  | N/A |  | 3(KNZ,TOT-2) | 4(KNZ,TOT-2) |  | 4LM2 |
| Rank=3 N3>19 | 4 | 4 |  | 4 | N/A | log2(2M3) |  | log22M3-1M3-1 |  | log22M3-1M3-1 | log22M3-1M3-1 |  | N/A |  | 3(KNZ,TOT-3) | 4(KNZ,TOT-3) |  | 6LM3 |
| Rank=4 N3>19 | 4 | 4 |  | 4 | 4 | log2(2M4) |  | log22M4-1M4-1 |  | log22M4-1M4-1 | log22M4-1M4-1 |  | log22M4-1M4-1 |  | 3(KNZ,TOT-4) | 4(KNZ,TOT-4) |  | 8LM4 |

Note:	the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ shown in Table 6.3.2.1.2-1A is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = υ, respectively.

----End of the description---

A similar description is captured for the Rel. 16 Type II port selection codebook in Table Table 6.3.2.1.2-2A of TS 38.212 v16.0.0. There is an issue with the current description of the bitwidth for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ. The bitwidth in the current description is the total bitwidth (as highlighted above), and the per layer bitwidth of i1,7,l, i2,4,l, and i2,5,l for each value of l is not captured. This leads to the following discrepancy in the specification.

- First, it is not aligned with the RAN1 agreements. Based on the agreed UCI parameter list, provided in Table 4 of [2], which is copied in Table 1 below, it is clear that the bitwidths of bitmap, amplitude, and phase were agreed to be the per layer (not total across layers) in RAN1.

- Second, it is unclear how the per layer bitwidths can be obtained from the total bitwidths. For example, for rank 2, the current specification says that the total bitwidth is 4LM2. According to the agreement, the bitwidth for each layer l=1,2 is 2LM2. If the bitwidth for each layer is not specified, then it will lead to ambiguity since the bitwidths for the two layers can be a pair (X, Y) such that X+Y=4LM2. For example, (X, Y) can be 2LM2,2LM2 or LM2,3LM2.

Table 1 UCI parameter list

| Parameter | Location | Details/description |
| --- | --- | --- |
| # NZ coefficients | UCI part 1 | RI ({1,…, RIMAX}) and KNZ,TOT (the total number of non-zero coefficients summed across all the layers, where KNZ,TOT {1,2,…, 2K0} are reported in UCI part 1 |
| Wideband CQI | UCI part 1 | Same as R15 |
| Subband CQI | UCI part 1 | Same as R15 |
| Bitmap per layer | UCI part 2 | RI=1-2: for layer l, size-2LM For RI=3-4, bitmaps, each with size-2LMi (i=0,1,…, RI-1, where i denotes the i-th layer) are reported in UCI part 2 |
| Strongest coefficient indicator (SCI) | UCI part 2 | RI=1: the existing agreement on SCI: “A log2KNZ-bit indicator for the strongest coefficient index l*,m*” RI>1: See Table below |
| SD basis subset selection indicator | UCI part 2 | SD basis subset selection indicator is a -bit indicator. Details follow Rel.15 |
| FD basis subset selection indicator | UCI part 2 | See Table below |
| LC coefficients: phase | UCI part 2 | Quantized independently across layers |
| LC coefficients: amplitude | UCI part 2 | Quantized independently across layers (including reference amplitude for weaker polarization, for each layer) |
| SD oversampling (rotation) factor q1, q2 | UCI part 2 | Values of q1, q2 follow Rel.15 |

Observation: There are the following issues with the current description of the bitwidths for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ in TS 38.212:

- The current description is not aligned with previous RAN1 agreements, which included per layer bitwidths for these parameters.

- The current description leads to ambiguity in the values of the per layer bitwidths for these parameters.

Proposed solution

The abovementioned issues can be resolved by simply including a description of the per layer bidwidth for each of the three parameters. The proposed text proposal is as follows.

Proposal: Adopt the following text proposal to correct discrepancy between the RAN1 agreements on bandwidths of UCI parameters and the description in TS 38.212 V16.0.0:

#### 6.3.2.1.2 CSI --- start TP --- ---Unchanged text is omitted--- Proposed text after Table 6.3.2.1.2-1A: Note: the bitwidth for {i1,7,l}l=1,…,v, {i2,4,l}l=1,…,v and {i2,5,l}l=1,…,v shown in Table 6.3.2.1.2-1A is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = v, respectively, and the corresponding per layer bitwidths are 2LMυ, 3KNZ, l-1, and 4KNZ, l-1, where KNZ, l as defined in Subclause 5.2.2.2.5 in [6, TS 38.214] is the number of nonzero coefficients for layer l such that KNZ, TOT=l=1υKNZ, l. ---Unchanged text is omitted--- Proposed text after Table 6.3.2.1.2-2A: Note: the bitwidth for {i1,7,l}l=1,…,v, {i2,4,l}l=1,…,v and {i2,5,l}l=1,…,v shown in Table 6.3.2.1.2-2A is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = v, respectively, and the corresponding per layer bitwidths are 2LMυ, 3KNZ, l-1, and 4KNZ, l-1, where KNZ, l as defined in Subclause 5.2.2.2.6 in [6, TS 38.214] is the number of nonzero coefficients for layer l such that KNZ, TOT=l=1υKNZ, l. ---Unchanged text is omitted--- --- end TP ---

Conclusion

In this contribution, the following observation and proposal are given:

Observation: There are the following issues with the current description of the bitwidths for {i1,7,l}l=1,…,υ, {i2,4,l}l=1,…,υ and {i2,5,l}l=1,…,υ in TS 38.212:

- The current description is not aligned with previous RAN1 agreements, which included per layer bitwidths for these parameters.

- The current description leads to ambiguity in the values of the per layer bitwidths for these parameters

Proposal: Adopt the following text proposal to correct discrepancy between the RAN1 agreements on bandwidths of UCI parameters and the description in TS 38.212 V16.0.0:

#### 6.3.2.1.2 CSI --- start TP --- ---Unchanged text is omitted--- Proposed text after Table 6.3.2.1.2-1A: Note: the bitwidth for {i1,7,l}l=1,…,v, {i2,4,l}l=1,…,v and {i2,5,l}l=1,…,v shown in Table 6.3.2.1.2-1A is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = v, respectively, and the corresponding per layer bitwidths are 2LMυ, 3KNZ, l-1, and 4KNZ, l-1, where KNZ, l as defined in Subclause 5.2.2.2.5 in [6, TS 38.214] is the number of nonzero coefficients for layer l such that KNZ, TOT=l=1υKNZ, l. ---Unchanged text is omitted--- Proposed text after Table 6.3.2.1.2-2A: Note: the bitwidth for {i1,7,l}l=1,…,v, {i2,4,l}l=1,…,v and {i2,5,l}l=1,…,v shown in Table 6.3.2.1.2-2A is the total bitwidth of {i1,7,l}, {i2,4,l} and {i2,5,l} up to Rank = v, respectively, and the corresponding per layer bitwidths are 2LMυ, 3KNZ, l-1, and 4KNZ, l-1, where KNZ, l as defined in Subclause 5.2.2.2.6 in [6, TS 38.214] is the number of nonzero coefficients for layer l such that KNZ, TOT=l=1υKNZ, l. ---Unchanged text is omitted--- --- end TP ---

# References

- 3GPP RAN1, TS 38.212 V16.0.0

- R1-1911730, “Summary of agreements for NR_eMIMO up to RAN1#98”, Samsung