3GPP TSG RAN WG1 #100			R1-2000700

e-Meeting, February 24th – March 6th, 2020

Agenda Item:	6.2.1.1

Source:	Qualcomm Incorporated

Title:	Maintenance for UE-group wake-up signal

Document for:	Discussion and Decision

# 1	Introduction

The issues related UE-group wake-up signal for eMTC (MWUS) and also the proposed updates are listed below:

- Issue #1: The current specification ‘A BL/CE UE can be configured with up to two MWUS’ in Sect. 17 of TS36.213 needs to be clarified based on the following agreement. We suggest the modification as “A BL/CE UE can be configured by higher layers with up to two MWUS.” We assume the following agreements will be captured in TS36.304.

Agreement

- A UE is required to detect 2 sequences, the common WUS and the group WUS of the group to which it belongs

Agreement

A UE is required to monitor WUS(s) in only one WUS (time/frequency) resource location.

- Issue #2: The following agreement is not captured, which define the WUS multiplexing in one WUS resource. If eNB transmits more than one sequence using power boosting, it may still degrade the detection performance under freq/time error at receiver side. In order to align eNB transmission and UE assumption, we suggest adding “A BL/CE UE may assume that no more than one WUS sequence is sent per WUS resource at a given time”.

Agreement

- Single-sequence CDM is used in the same WUS resource if more than one UE group is configured to monitor the WUS resource.

- Note: Single sequence CDM is where different sequences share the same resource and only one sequence is transmitted at a given time

- Proposal 1	Agree on the TP presented in Sect. 2 and capture its contents on TS 36.211.

- Issue #3: As agreed, the frequency location of WUS resource 0 is signalled by higher layer. The frequency location of other WUS resources with ID=1, 2 or 3 (if configured) are predefined without additional signalling. The time location of WUS resources has been captured in TS36.213. Therefore, we suggest a minor change in Sect. 6.11B.2 of TS36.211 as “The MWUS bandwidth is 2 consecutive PRBs, the frequency location of the lowermost PRB determined by higher layers”. We assume the agreements for WUS resource pattern will be captured in TS36.304.

- Proposal 2	Agree on the TP presented in Sect. 3 and capture its contents on TS 36.213.

- Issue #4: We have agreed the resource ID mapping order for WUS resources with up to 2-FDM and up to 2-TDM. However, the resource ID mapping order for 3-FDM WUS resources is not defined yet. We propose the following (which should be captured together with WUS resource location in TS36.304).

- Proposal 3: If Rel-15 WUS is configured, predefine the time/freq location for 3-FDM WUS resources based on the 2-bit freqLocation of WUS resource 0 as

| freqLocation of WUS resource 0 | n0 | n2 | n4 |
| --- | --- | --- | --- |
| WUS resource locations | f; 0. 1. 2; t. | Alt1: f; 1. 0. 2; t. Alt2: f; 2. 0. 1; t. Alt1 if NB is below center carrier; otherwise Alt2. | f; 2. 1. 0; t. |

Section 2 and Section 3 contain the actual updates as they will appear on the TS 36.211 and 36.213.

# 2	Text Proposal for TS 36.211

---------------------------------------------------------------- Text Omitted -------------------------------------------------------------

## 6.11B	MTC wake-up signal (MWUS)

### 6.11B.1	Sequence generation

The MWUS sequence  in subframe x=0, 1, …, M-1 is defined by

wm= θnf, nsm'e-jπunn+1131ej2πgm132

m=0, 1, …, 131

m'=m+132x

n=m mod 132

θnf,nsm'=1 if cnf,ns2m'=0 and cnf,ns2m'+1=0 -1 if cnf,ns2m'=0 and cnf,ns2m'+1=1j if cnf,ns2m'=1 and cnf,ns2m'+1=0-1 if cnf,ns2m'=1 and cnf,ns2m'+1=1

u=NIDcell mod 126+3

where  is the actual duration of MWUS as defined in [4]. For a UE not configured with group MWUS, g=0. For a UE configured with group MWUS, g=14NgroupWUS+1 for 0≤NgroupWUS≤7, where NgroupWUS is determined by the UE group to which the UE is associated as determined by higher layers [10]. The common MWUS sequence shall be determined by  g=126 unless the resource is shared with non-group MWUS and common MWUS is configured to be non-group MWUS in which case g=0.

The scrambling sequence cnf,nsi, i=0, 1, …, 2∙132M-1 is given by clause 7.2, and shall be initialized at the start of the MWUS with

cinit_WUS=(NIDcell+1)10nf_start_PO+ns_start_PO2mod 2048+129+NIDcell+NIDresource∙229

where nf_start_PO is the first frame of the first PO to which the MWUS is associated, ns_start_PO is the first slot of the first PO to which the MWUS is associated and NIDresource indicates the group MWUS resource to which the UE is associated. For a UE not configured with group MWUS, NIDresource=0, whereas for a UE configured with group MWUS, NIDresource is determined by higher layers [10].

### 6.11B.2	Mapping to resource elements

The same antenna port shall be used for all symbols of the MWUS within a subframe. The UE shall not assume that the MWUS is transmitted on the same antenna port as any of the downlink reference signals or synchronization signals. If only one CRS port is configured by the eNB, the UE may assume the transmission of all MWUS subframes is using the same antenna port; otherwise, the UE may assume the same antenna port is used for MWUS transmission in downlink subframes w0 + 2n and w0 + 2n + 1, where w0 is the first downlink subframe of the MWUS transmission as specified in [4], and n=0, 1.

The MWUS bandwidth is 2 consecutive PRBs, the frequency location of the lowermost PRB determined by higher layers. For both PRB pairs in the frequency domain, for which MWUS is defined, the MWUS sequence wm shall be mapped to resource elements (k,l) in sequence, starting with w(0) in increasing order of first the index k=0, 1, …, NscRB-1, over the 12 assigned subcarriers and then the index l=3, 4, …, 2NsymbDL-1 in each subframe in which MWUS is transmitted.

The MWUS sequence is mapped to the set of subframes in the actual MWUS duration as defined in [4], where in a subframe in which an MWUS PRB pair overlaps with any PRB pair carrying PSS, SSS, RSS, PBCH or PDSCH associated with SI-RNTI is transmitted, the subframe is counted in the MWUS mapping but not used for transmission of MWUS.

A resource element (k,l) overlapping with resource elements where cell-specific reference signals according to clause 6.10 are transmitted shall not be used for MWUS transmission but is counted in the mapping process.

---------------------------------------------------------------- Text End -------------------------------------------------------------------

# 3	Text Proposal for TS 36.213

---------------------------------------------------------------- Text Omitted -------------------------------------------------------------

## 17	Wake-up signal related procedures for BL/CE UE

A BL/CE UE can be configured by higher layers with up to two MWUS. A BL/CE UE may assume that no more than one WUS sequence is sent per WUS resource at a given time.

A BL/CE UE using MWUS can assume the actual duration of MWUS, starting in subframe w0, is one of the values in the set listed in Table 17-1 corresponding to the maximum duration of MWUS, LMWUS_max , configured by higher layers. The maximum duration of MWUS starts in subframe w0, where w0 is the latest subframe such that there are LMWUS_max  BL/CE DL subframes in the maximum duration and a total of k∙LMWUS_max BL/CE DL subframes in the duration that ends in subframe (g0-1), where g0 is defined by [14], k=1 if FDM-only MWUS resource configuration [14], k=NIDresource+12 otherwise, and NIDresource is the MWUS resource that the UE is associated to as defined in [3]. The UE may assume that MWUS and its first associated paging occasion subframes are in the same narrowband.

Table 17-1: Actual MWUS durations in BL/CE DL subframes.

| LMWUS_max | Actual MWUS durations set |
| --- | --- |
| 1 | {1} |
| 2 | {1, 2} |
| 4 | {1, 2, 4} |
| 8 | {1, 2, 4, 8} |
| 16 | {1, 2, 4, 8, 16} |
| 32 | {1, 2, 4, 8, 16, 32} |
| 64 | {1, 2, 4, 8, 16, 32, 64} |

---------------------------------------------------------------- Text End -------------------------------------------------------------------

# 4.	Conclusion

Upon reviewing the content of this Text Proposal, it is proposed:

Proposal 1	Agree on the TP presented in Sect. 2 and capture its contents on TS 36.211.

Proposal 2	Agree on the TP presented in Sect. 3 and capture its contents on TS 36.213.

Proposal 3: 	If Rel-15 WUS is configured, predefine the time/freq location for 3-FDM WUS resources based on the 2-bit freqLocation of WUS resource 0 as

| freqLocation of WUS resource 0 | n0 | n2 | n4 |
| --- | --- | --- | --- |
| WUS resource locations | f; 0. 1. 2; t. | Alt1: f; 1. 0. 2; t. Alt2: f; 2. 0. 1; t. Alt1 if NB is below center carrier; otherwise Alt2. | f; 2. 1. 0; t. |

# References

R1-1913594, RAN1 agreements for Rel-16 Additional MTC Enhancements for LTE, Reno, USA, RAN1#99

TS 36.211, “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation”, version 16.0.0

TS 36.213, “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures”, version 16.0.0