3GPP TSG RAN WG1 Meeting #100-E                                                         	   R1-2000723

e-Meeting, February 24th – March 6th, 2020

Agenda Item:	7.2.2.1.2

Source: 	Intel Corporation

Title:	DL Signals and Channels for NR-unlicensed

Document for: 	Discussion/Decision

# Introduction

The first version of Rel-16 specifications were released reflecting agreements from NR-U WI discussions. However, there are some remaining issues to be clarified in the specs based on the previous agreements and this contribution is discussing those aspects.

# Discussions

## DPSCH mapping type B

There are some editorial changes needed for TS 38.211 [1] and the following text proposal is proposed to be incorporated into the official CR for 38.211.

Text proposal for TS 38.211 ##### 7.4.1.1.2 Mapping to physical resources *** Unchanged text is omitted *** For PDSCH mapping type B - if the PDSCH duration ld ∈2,3,4,5,6,7,8,9,10,11,12,13 OFDM symbols for normal cyclic prefix or ld∈2,4,6 OFDM symbols for extended cyclic prefix, and the front-loaded DM-RS of the PDSCH allocation collides with resources reserved for a search space set associated with a CORESET, shall be incremented such that the first DM-RS symbol occurs immediately after the CORESET and until no collision with any CORESET occurs, and - if the PDSCH duration ld is 2 symbols, the UE is not expected to receive a DM-RS symbol beyond the second symbol, - if the PDSCH duration ld is 7 symbols for normal cyclic prefix or 6 symbols for extended cyclic prefix: - the UE is not expected to receive the front-loaded DM-RS beyond the fourth symbol, and - if one additional single-symbol DM-RS is configured, the UE only expects the additional DM-RS to be transmitted on the 5th or 6th symbol when the front-loaded DM-RS symbol is in the 1st or 2nd symbol, respectively, of the PDSCH duration, otherwise the UE should expect that the additional DM-RS is not transmitted; - if the PDSCH duration ld is 12 or 13 symbols, the UE is not expected to receive a DM-RS symbol mapped to symbol 12 or later in the slot; - for all other values of the PDSCH duration ld, the UE is not expected to receive a DM-RS symbol beyond the (ld-1):th symbol; - if the PDSCH duration ld is less than or equal to 4 OFDM symbols, only single-symbol DM-RS is supported. - if the higher-layer parameter lte-CRS-ToMatchAround or additionalLTE-CRS-ToMatchAroundList is configured, the PDSCH duration ld=10 symbols for normal cyclic prefix, the subcarrier spacing configuration μ=0, single-symbol DM-RS is configured, and at least one PDSCH DM-RS symbol in the PDSCH allocation collides with a symbol containing resource elements as indicated by the higher-layer parameter lte-CRS-ToMatchAround or additionalLTE-CRS-ToMatchAroundList, then l shall be incremented by one in all slots. *** Unchanged text is omitted *** Table 7.4.1.1.2-3: PDSCH DM-RS positions for single-symbol DM-RS. ld in symbols; DM-RS positions. PDSCH mapping type A; PDSCH mapping type B. dmrs-AdditionalPosition; dmrs-AdditionalPosition. pos0; pos1; pos2; pos3; pos0; pos1; pos2; pos3. 2; -; -; -; -; l0; l0. 3; l0; l0; l0; l0. 4; l0; l0. 5; l0; l0,4; l0,4; l0,4. 6; l0,4; l0,4. 7; l0,4; l0,4. 8; , 7; , 7; , 7; l0; l0, 6; l0,3, 6; l0,3, 6. 9; , 7; , 7; , 7; l0; l0,7; l0, 4, 7; l0, 4, 7. 10; , 9; , 6, 9; , 6, 9; l0; l0, 7; l0, 4, 7; l0, 4, 7. 11; , 9; , 6, 9; , 6, 9; l0; l0, 8; l0, 4, 8; l0, 3, 6, 9. 12; , 9; , 6, 9; , 5, 8, 11; l0; l0,9; l0,5, 9; l0, 3, 6, 9. 13; , l1; , 7, 11; , 5, 8, 11; l0; l0,9; l0,5, 9; l0, 3, 6, 9. 14; , l1; , 7, 11; , 5, 8, 11; -; -; -; -. Table 7.4.1.1.2-4: PDSCH DM-RS positions for double-symbol DM-RS. ld in symbols; DM-RS positions. PDSCH mapping type A; PDSCH mapping type B. dmrs-AdditionalPosition; dmrs-AdditionalPosition. pos0; pos1; pos2; pos0; pos1; pos2. <4; -; -. 4; -; -. 5; l0; l0. 6. 7. 8; l0; l0,5. 9; l0; l0,5. 10; , 8; l0; l0,7. 11; , 8; l0; l0,7. 12; , 8; l0; l0,8. 13; , 10; l0; l0,8. 14; , 10; -; -.

## Switching between groups of search space sets

We have the following agreement with respect to the PDCCH monitoring switching between two groups of search space sets. As highlighted below, the switching operation has to be applied at the earliest start of a slot that is at least P1/P2 symbols later than the last symbol of the corresponding PDCCH. However, current version of the TS 38.213 does not correctly reflect the switching time as shown in [2].

Agreement: When a UE is configured with two groups of search space sets for PDCCH - The presence of a monitoring group flag in DCI format 2_0 is configurable - A timer is configured as a number of slots, which is decremented at the end of each slot. - If the monitoring group flag is present in DCI format 2_0 (Note: Explicit switching between Group1 and Group2 and vice versa, plus implicit switching from Group2 to Group1) - If the flag = 1 - Switch to (or keep) monitoring Group2 at the next applicable slot boundary with respect to the detected DCI format 2_0 - The UE starts (or restarts) the configurable timer - If the flag = 0 - Switch to (or keep) monitoring Group1 at the next applicable slot boundary with respect to the detected DCI format 2_0 - The UE switches from Group2 to Group1 at the earliest slot boundary that is at least P1 symbols after - the end of the slot in which the timer expires, or - an indicated COT duration is exceeded, whichever comes first - If the monitoring group flag is not present in DCI format 2_0 (Note: Implicit switching between Group1 and Group2 and vice versa) - If any PDCCH in Group1 is successfully detected - Switch from Group1 to Group2 at the next applicable slot boundary with respect to the detected PDCCH - The UE starts (or restarts) the configurable timer if it receives any PDCCH - The UE switches from Group2 to Group1 at the earliest slot boundary that is at least P2 symbols after - the end of the slot in which the timer expires, or - an indicated COT duration is exceeded, whichever comes first - If the UE does not monitor for DCI format 2_0 (Note: Implicit switching between Group1 and Group2 and vice versa) - If any PDCCH in Group1 is successfully detected - Switch from Group1 to Group2 at the next applicable slot boundary with respect to the detected PDCCH - The UE starts (or restarts) the configurable timer if it receives any PDCCH - The UE switches from Group2 to Group1 at the earliest slot boundary that is at least P2 symbols after the end of the slot in which the timer expires - The "next applicable slot boundary" is the earliest start of a slot that is at least P1/P2 symbols later than the last symbol of the corresponding PDCCH. P1 and P2 should be no less than the processing time required by the UE for performing search space switching. - Search space switching is a new UE capability - Processing time is fixed in the specification - FFS: Whether P1 and P2 need to be different - The groups of serving cells to which the above behaviour is applicable can be configured via RRC

In order to reflect the above agreement in the specification more precisely, following text proposal is proposed to be incorporated into the official CR for 38.213.

Text proposal for TS 38.213 ## 10.4 Search space set switching A UE can be provided a group index for a respective search space set by searchSpaceGroupIdList-r16 for PDCCH monitoring on a serving cell indicated by searchSpaceSwitchingGroup-r16. If the UE is not provided searchSpaceGroupIdList-r16 for a search space set, or for PDCCH monitoring on a serving cell that is not indicated by searchSpaceSwitchingGroup-r16, the following procedures are not applicable for PDCCH monitoring according to the search space set. A UE can be provided, by searchSpaceSwitchingTimer-r16, a timer value. The UE decrements the timer value by one after each slot in the active DL BWP of the serving cell where the UE monitors PDCCH for detection of DCI format 2_0. If a UE is provided by SearchSpaceSwitchTrigger-r16 a location of a search space set switching field for a serving cell in a DCI format 2_0, as described in Clause 11.1.1, and detects DCI format 2_0 in a slot - if the UE is not monitoring PDCCH according to search space sets with group index 0, the UE starts monitoring PDCCH according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, on the serving cell at a first slot that is at least P1 symbols after the last symbol of the PDCCH where the DCI format 2_0 is detected in the active DL BWP of the serving cell, if a value of the search space set switching field is 0 - if the UE is not monitoring PDCCH according to search space sets with group index 1, the UE monitors PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, on the serving cell at a first slot that is at least P1 symbols after the last symbol of the PDCCH where the DCI format 2_0 is detected in the active DL BWP of the serving cell, and the UE sets the timer value to the value provided by searchSpaceSwitchingTimer-r16, if a value of the search space set switching field is 1 - if the UE monitors PDCCH on a serving cell according to search space sets with group index 1, the UE starts monitoring PDCCH on the serving cell according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, on the serving cell at the beginning of the first slot that is at least P1 symbols after a slot where the timer expires or after a last slot of a remaining channel occupancy duration for the serving cell that is indicated by DCI format 2_0 If a UE is not provided SearchSpaceSwitchTrigger-r16 for a serving cell, - if the UE detects a DCI format by monitoring PDCCH according to a search space set with group index 0 in a slot, the UE starts monitoring PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, on the serving cell at a first slot that is at least P2 symbols after the last symbol of the PDCCH where the DCI format is detected in the active DL BWP of a serving cell, the UE sets the timer value to the value provided by searchSpaceSwitchingTimer-r16 if the UE detects a DCI format by monitoring PDCCH in any search space set - if the UE monitors PDCCH on a serving cell according to search space sets with group index 1, the UE starts monitoring PDCCH on the serving cell according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, on the serving cell at the beginning of the first slot that is at least P2 symbols after a slot where the timer expires or, if the UE is provided a search space set to monitor PDCCH for detecting a DCI format 2_0, after a last slot of a remaining channel occupancy duration for the serving cell that is indicated by DCI format 2_0

# Conclusion

This contribution provides two text proposals in order to correctly reflect the agreements on PDSCH mapping type B and dynamic switching between two groups of search space sets. It is proposed that two text proposals are incorporated into the official CR for 38.211 and 38.213.

# References

[1]	3GPP TS 38.211 V16.0.0

[2]	3GPP TS 38.213 V16.0.0