3GPP TSG RAN WG1 Meeting #100-E                                                                      	        R1-2000726

e-Meeting, February 24th – March 6th, 2020

Agenda Item:		7.2.2.2.3

Source: 	Intel Corporation

Title:	Enhancements to HARQ for NR-unlicensed

Document for: 	Discussion/Decision

# Introduction

The Rel-16 WI on NR-U was completed at RAN1#99 meeting. However, there are still several remaining issues need further clarification. Therefore, we provide our views on the remaining issues of HARQ-ACK transmission and multi-TTI PUSCH scheduling.

# HARQ-ACK transmission

In the last RAN1 meeting, the behaviour regarding fallback DCI format 1_0 was agreed. According to the agreement, the PDSH group ID is 0 by default. The NFI value associated with a DCI format 1_0 follows the DCI format 1_1 which indicates the PDSCH group #0 and the same slot for HARQ-ACK transmission.

Agreement:

If enhanced dynamic codebook is configured, for a PDSCH scheduled by DL DCI 1_0:

- NFI for group #0 is not signalled in DCI 1_0

- If the UE detects a DCI that indicates an NFI corresponding to group #0 since the last scheduled PUCCH that includes feedback for group #0 and before the PUCCH occasion that includes feedback corresponding to PDSCH scheduled with the DCI 1_0

  - UE follows the indicated NFI (in DCI 1_1) to determine the codebook.

- If the UE does not detect a DCI that indicates an NFI corresponding to group #0 since the last scheduled PUCCH that includes feedback for group #0 and before the PUCCH occasion that includes feedback corresponding to PDSCH scheduled with the DCI 1_0

  - UE assumes that the PDSCHs scheduled by DCI 1_0 since the latest PUCCH occasion do not belong to any group, and the UE only reports HARQ-ACK feedback for the PDSCHs scheduled by DCI 1_0 since the latest PUCCH occasion (as in Rel-15)

  - If the C-DAI received in the first DCI format 1_0 received after the latest PUCCH occasion is not equal to 1, and if the UE didn’t detect a DCI format 1_1 since the latest PUCCH occasion, the UE assumes that missed DCIs are DCI formats 1_0

For a group of PDSCH with the same group ID and associated with same NFI value, following two special cases need further consideration.

- Case 1: the first DCI of a PDSCH group is DCI format 1_0 without NFI field, for which the associated NFI value is temporarily not available until the next DCI format 1_1 of the same PDSCH group is received.

- Case 2: the last DCI of a PDSCH group is DCI format 1_0 without NFI field, for which the associated NFI value is derived by a previous DCI format 1_1 of the same PDSCH group.

The current specification needs to be revised to capture the clear operations of the two cases.

Proposal 1: Revise the specification to clarify the operations of two cases of DCI format 1_0.

Text proposal for section 9.1.3.3 in 38.213-g00.

Set g to the value of a PDSCH_group indicator field in a DCI format. If the DCI format schedules PDSCH reception and does not include a PDSCH_group indicator field, set g=0.

Set i(g) to denote a PUCCH transmission occasion for multiplexing HARQ-ACK information

Set k to the value of a PDSCH-to-HARQ_feedback timing field, if any, in a DCI format providing a value of g. If the DCI format does not include a PDSCH-to-HARQ_feedback timing field, set k to the value provided by dl-DataToUL-ACK

Set h(g) to the value of a first New_Feedback indicator field, if any, in a DCI format providing a value of g and indicating a slot for HARQ-ACK transmission. If the DCI format schedules PDSCH reception and does not include the first New_Feedback indicator field, set h(g) to the value of h(g) provided by another DCI format, if any, providing the same value of g and indicating the same slot for HARQ-ACK transmission

Set hg+1mod2(g) to a value of a second New_Feedback indicator field, if any, in a DCI format providing a value of g

Set VDAIg+1mod2 to the value of a total DAI field, if any, in a DCI format providing a value of g

Set q to the value of a HARQ-ACK information request field, if any

Generate first HARQ-ACK information for PUCCH transmission occasion i(g) in a slot, as described in Clause 9.1.3.1, where

-	the first HARQ-ACK information corresponds only to detections of DCI formats each providing a same value of g, of h(g), if any, and at least one of the DCI formats providing a value of k indicating the slot

-	at least one of the DCI formats provides a h(g) value

-	m=0 corresponds to a PDCCH monitoring occasion, where the UE detects a DCI format that provides a value of g, that is the first PDCCH monitoring occasion after a PDCCH monitoring occasion where the UE detects another DCI format that provides the same value of g and a different h(g) value

If hg+1mod2g=∅ or hg+1mod2g=h(g+1mod2), generate second HARQ-ACK information for PUCCH transmission occasion i(g+1mod2) in a slot, as described in Clause 9.1.3.1, where

-	the second HARQ-ACK information corresponds to detections of DCI formats each providing a same value of g+1mod2, of h(g+1mod2), if any

-	at least one of the DCI formats provides a h(g+1mod2) value

-	m=0 corresponds to a PDCCH monitoring occasion, where the UE detects a DCI format that provides a value of g+1mod2, that is the first PDCCH monitoring occasion after a PDCCH monitoring occasion where the UE detects another DCI format that provides the same value of g and a different h(g+1mod2) value

-	the PUCCH transmission occasion i(g+1mod2) is a last one for multiplexing second HARQ-ACK information and it is not after PUCCH transmission occasion i(g)

-	if VDAIg+1mod2≠∅, after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Clause 9.1.3.1, set Vtemp2=VDAIg+1mod2 for both sub-codebooks, if any.

For enhanced dynamic HARQ-ACK codebook, the main reason to introduce more than 1 PDSCH groups is to account for the UE processing delay. As shown in Figure 1, the first 2 blue PDSCHs could use PDSCH group 1 since there is no enough UE processing time between the two PDSCHs and PUCCH U1. Since different PDSCH groups are used, NFI/C-DAI/T-DAI runs independently.

Figure 1: Enhanced dynamic codebook with 2 PDSCH groups

However, there is no explicit agreement that whether PDSCH group 0 could be used in the first two blue PDSCHs or same group can be also indicated to both green and blue PDSCHs. If PDSCH group 0 is used, it become problematic regardless of whether  NFI is set to 0 or 1. As shown in Figure 2, if NFI for the blue PDSCH equals to 1, i.e. toggled compared to green PDSCHs, there is no chance to retransmit HARQ-ACK for green PDSCH if PUCCH U2 is failed. On the other hand, if NFI for the blue PDSCH equals to 0, C-DAI/T-DAI has to be continuous for all green and blue PDSCHs, and then the HARQ-ACK transmission on PUCCH U1 becomes useless.

Figure 2: Enhanced dynamic codebook using single PDSCH group

Based on the above analysis, it is needed to explicitly clarify that UE does not expect to receive a DL grant which schedules a PDSCH of the same PDSCH group and indicates a PUCCH resource in a slot later than a pending PUCCH transmission of the same PDSCH group.

Proposal 2: A UE does not expect to receive a DL grant which schedules a PDSCH of the same PDSCH group and indicates a PUCCH resource in a slot later than a pending PUCCH transmission of the same PDSCH group.

Text proposal for section 9.1.3.3 in 38.213-g00.

If hg+1mod2g=∅ or hg+1mod2g=h(g+1mod2), generate second HARQ-ACK information for PUCCH transmission occasion i(g+1mod2) in a slot, as described in Clause 9.1.3.1, where

-	the second HARQ-ACK information corresponds to detections of DCI formats each providing a same value of g+1mod2, of h(g+1mod2), if any

-	at least one of the DCI formats provides a h(g+1mod2) value

-	m=0 corresponds to a PDCCH monitoring occasion, where the UE detects a DCI format that provides a value of g+1mod2, that is the first PDCCH monitoring occasion after a PDCCH monitoring occasion where the UE detects another DCI format that provides a value different than h(g+1mod2)

-	the PUCCH transmission occasion i(g+1mod2) is a last one for multiplexing second HARQ-ACK information. The UE does not expect that  i(g+1mod2) is later than PUCCH transmission occasion i(g)

-	if VDAIg+1mod2≠∅, after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Clause 9.1.3.1, set Vtemp2=VDAIg+1mod2 for both sub-codebooks, if any.

Regarding a SPS PDCCH release, the HARQ process ID field in the DCI is already occupied for PDCCH validation. Since TDRA field in DCI 1_0 for SPS PDSCH release is not used and it has a fixed size of 4 bits, the TDRA field could be reused to indicate a HARQ process ID for one-shot feedback.

Proposal 3: For SPS PDSCH release, the associated HARQ process ID is indicated by TDRA field in DCI 1_0 for SPS PDSCH release.

Text proposal for section 9.1.4 in 38.213-g00.

### 9.1.4	Type-3 HARQ-ACK codebook determination

If a UE is provided pdsch-HARQ-ACK-OneShotFeedback-r16, the UE determines a Type-3 HARQ-ACK codebook according to the following procedure.

…

For a SPS PDSCH release, the HARQ process number used in Type-3 HARQ-ACK codebook determination is indicated by time domain resource assignment field in the DCI carrying the SPS PDSCH release. If the UE detects a DCI format that includes a One-shot HARQ-ACK request field with value 1, the UE determines a PUCCH or a PUSCH to multiplex a Type-3 HARQ-ACK codebook for transmission in a slot as described in Clause 9.2.5. The UE multiplexes only the Type-3 HARQ-ACK codebook in the PUCCH or the PUSCH for transmission in the slot.

One-shot feedback can be triggered by a DCI 1_1 with or without PDSCH scheduling. For one-shot feedback, it was agreed that one value of the frequency domain resource assignment field indicates that this DCI does not schedule a PDSCH.

Agreement: for NR-U

If a UE is configured to monitor feedback request for one-shot HARQ-ACK codebook feedback and the feedback is requested in DL DCI 1_1

- This DL DCI can either schedule or not schedule a PDSCH

  - Working assumption: One value of the frequency domain resource assignment field indicates that this DCI does not schedule a PDSCH

  - If the DL DCI does not schedule a PDSCH, the HARQ process ID and NDI fields are ignored by the UE

- If UE is triggered to report both one-shot and other HARQ-ACK feedback in the same slot, the UE reports only the one-shot feedback.

The special value could be all ‘1’ for FDRA type 1 and all ‘0’ for FDRA type 0. However, the same special values were already agreed in MR-DC&eCA WI. Therefore, joint consideration for the two WIs are needed. The simplest way is to avoid the simultaneous support of one-shot feedback triggering and SCell dormancy switching indication in a DCI if the DCI doesn’t schedule a PDSCH. Table 1 summarize the potential combinations. The combination of special value of FDRA and value ‘0’ for one-shot HARQ-ACK request is a reserved state for NR-U. If this reserved value is used for SCell dormancy switching indication, one-shot feedback in NR-U is not impacted by the behaviour of SCell dormancy switching. When FDRA indicates a valid frequency resource, PDSCH transmission is scheduled. One-shot feedback triggering and SCell dormancy switching indication are controlled by one-shot HARQ-ACK request field and SCell dormancy indication field respectively.

Table 1: Indication of one-shot feedback and SCell dormancy switching

| DCI |  | Data transmission | One-shot feedback | Dormancy switching |
| --- | --- | --- | --- | --- |
| FDRA | one-shot HARQ-ACK request |  |  |  |
| Special value | 1 | N | Y | N |
| Special value | 0 | N | N | Y |
| valid FDRA | 1 | Y | Y | by SCell dormancy indication field |
| valid FDRA | 0 | Y | N | By SCell dormancy indication field |

Proposal 4: Confirm the working assumption and the special value triggering one-shot feedback w/o scheduled PDSCH in DCI format 1_1 are.

- FDRA field set to all 1s when type 1 RA is used for UE;

- FDRA field set to all 0s when type 0 RA is used for UE.

Text proposal for section 9.1.4 in 38.213-g00.

If the UE detects a DCI format that includes a One-shot HARQ-ACK request field with value 1, the UE determines a PUCCH or a PUSCH to multiplex a Type-3 HARQ-ACK codebook for transmission in a slot as described in Clause 9.2.5. The UE multiplexes only the Type-3 HARQ-ACK codebook in the PUCCH or the PUSCH for transmission in the slot. If the UE detects a DCI format that includes a one-shot HARQ-ACK request field with value 1, and if

-	resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0, or

-	resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 1

the UE determines that the DCI format triggers the transmission of Type-3 HARQ-ACK codebook and does not schedule a PDSCH transmission.

fdfdgf

Multi-TTI PUSCH scheduling

NR-U supports that a single DCI format 0_1 could schedule multiple consecutive PUSCHs. The LBT type and start offset is indicated in the UL grant even for Multi-TTI PUSCH scheduling. It is straightforward that the indicated LBT type and start offset applies to the first scheduled PUSCH. In other words, an indicated gap selected from {0us, 16us+TA, 25us, 25us+TA} applies to the first scheduled PUSCH. However, it is possible that LBT operation of the indicated LBT type is not successful immediately before the first scheduled PUSCH. In this case, the UE may need to continue LBT operation until the next scheduled PUSCH if CAT-4 LBT is indicated. If other LBT type is indicated by the DCI, UE could do 25us CAT-2 LBT for the channel access of the next scheduled PUSCH.

One question is whether the same CP extension should be used by the UE for this case. Two options could be considered,

- Option 1: the indicated CP extension applies to the subsequent PUSCH if the LBT of the previous PUSCH scheduled by the same DCI format fails.

- Option 2: CP extension of 0 us applies to all scheduled PUSCHs except the first PUSCH scheduled by the same DCI format.

Assuming CAT-2 LBT applies to the next scheduled PUSCH, it is not desirable to use a gap 16us+TA which is normally associated with CAT-1 LBT. Further, a gap 25us+TA is also for the case that a UL transmission immediately follows a DL transmission. Therefore, a gap 25us+TA is not suitable for a next scheduled PUSCH which doesn’t follow a DL transmission. With the above analysis, Option 2 could be the correct choice since it is always applicable independent from indicate LBT type and CP extension by UL grant.

Proposal 5: If LBT fails for the first scheduled PUSCH for multi-PUSCH transmission,

- if the CAT-4 LBT is indicated, the UE continues CAT-4 LBT operation for the next scheduled PUSCH;

- if the CAT-1 or CAT-2 LBT is indicated, UE performs 25us CAT-2 LBT for the subsequent PUSCH;

- CP extension of 0 us applies to the next scheduled PUSCH.

Corresponding text proposal for section 4.2.1.0.1 in 37.213-g00

##### 4.2.1.0.1 Channel access procedures for consecutive UL transmission(s) For contiguous UL transmission(s), the following are applicable: - If a UE is scheduled to transmit a set of UL transmissions including PUSCH using a UL grant, and if the UE cannot access the channel for a transmission in the set prior to the last transmission, the UE shall attempt to transmit the next transmission according to the channel access type indicated in the UL grant. if the indicated channel access type is either 1 or 2A. - If a UE is scheduled to transmit a set of UL transmissions including PUSCH using a UL grant, and if the UE cannot access the channel for a transmission in the set prior to the last transmission, the UE shall attempt to transmit the next transmission according to the channel access type 2A if the indicated channel access type is 2B or 2C - If a UE is scheduled to transmit a set of consecutive UL transmissions without gaps including PUSCH using one or more UL grant(s) and the UE transmits one of the scheduled UL transmissions in the set after accessing the channel according to one of Type 1 or Type 2 UL channel access procedures, the UE may continue transmission of the remaining UL transmissions in the set, if any. - A UE is not expected to be indicated with different channel access types for any consecutive UL transmissions without gaps in between the transmissions.

Another text proposal for section 6.1.2.1 in 38.214-g00 especially for CP extension

#### 6.1.2.1 Resource allocation in time domain ###### *** Unchanged text is omitted *** If pusch-TimeDomainAllocationList in pusch-Config contains row indicating resource allocation for two to eight contiguous PUSCHs, K2 indicates the slot where UE shall transmit the first PUSCH of the multiple PUSCHs. Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is signalled by the number of indicated valid SLIVs in the row of the pusch-TimeDomainAllocationList signalled in DCI format 0_1. For operation with shared spectrum channel access, if the UE fails to access the channel for the first PUSCH, 0 s of a cyclic prefix extension Text to be applied for the transmission of subsequent PUSCHs according to [4, TS 38.211].

# Conclusion

In this contribution, we discussed the remaining issues on HARQ-ACK transmission and mulit-TTI PUSCH scheduling for operations on unlicensed spectrum, and we derived the following proposals. It is proposed to agree on the text proposals related to the following proposals.

Proposal 1: Revise the specification to clarify the operations of two cases of DCI format 1_0.

Proposal 2: UE does not expect to receive a DL grant which schedules a PDSCH of the same PDSCH group and indicates a PUCCH resource in a slot later than a pending PUCCH transmission of the same PDSCH group.

Proposal 3: For SPS PDSCH release, the associated HARQ process ID is indicated by TDRA field in DCI 1_0 for SPS PDSCH release.

Proposal 4: Confirm the working assumption and the special value triggering one-shot feedback w/o scheduled PDSCH in DCI format 1_1 are.

- FDRA field set to all 1s when type 1 RA is used for UE;

- FDRA field set to all 0s when only type 0 RA is used for UE.

Proposal 5: If LBT fails for the first scheduled PUSCH for multi-PUSCH transmission,

- if the CAT-4 LBT is indicated, the UE continues CAT-4 LBT operation for the next scheduled PUSCH;

- if the CAT-1 or CAT-2 LBT is indicated, UE performs 25us CAT-2 LBT for the subsequent PUSCH;

- CP extension of 0 us applies to the next scheduled PUSCH.