3GPP TSG-RAN WG1 Meeting #100-e	R1-2000831

e-Meeting, 24th February – 6th March, 2020

Agenda Item:	7.2.2.1.3

Source:	Ericsson

Title:	Feature lead summary for Maintenance of UL Signals and Channels

Document for:	Discussion, Decision

# 1	Introduction

This document summarizes the contributions made under the “UL Signals and Channels” sub-agenda item for 7.2.2 Rel-16 Maintenance of NR-based Access to Unlicensed Spectrum.

The following guidance was given to feature leads by the RAN1 chairman for the preparation of this summary for initial input to the e-Meeting preparation phase (2/18 – 2/21):

      - For the summary, due to limited time for preparation, each feature lead is not required to provide a detailed summary of companies positions and their proposals, but rather based on the contributions, to provide a list of issues/proposals and an indication of priority

From Tuesday 2/18, three email threads will be started to finalize the set of critical issues from those listed in this document. The titles of these email threads will be

      - [100e-Prep-NR_unlic-ULSignalsChannels-A]

      - [100e-Prep-NR_unlic-ULSignalsChannels-B]

      - [100e-Prep-NR_unlic-ULSignalsChannels-C]

By the end of Friday 2/21, once all critical issues have been identified and the group agrees on the topics to be discussed over email, a revised feature lead summary will be submitted with Tdoc#.

# 2	UL Signals and Channels Issues and Priority

To organize the email discussion, the issues have been grouped into maximum three threads according to the chairman’s guidance. Thread A will likely require some discussion to cover all cases of USS, CSS, initial BWP, active BWP, BWP switching, etc., so this thread is mostly focused on the configuration of the FDRA field (X and Y bits) in DCI 0_0 and DCI 0_1. Threads B and C contain several issues each, but the needed corrections are probably fairly straightforward.

Note: Several companies have proposals related to open issues on CP extension for PUSCH and PUCCH. These issues are not summarized here, since that topic has been primarily discussed within the channel access agenda item (7.2.2.2.1). Further discussions should be taken within that AI.

## 2.1	Critical Issues (High Priority)

### 2.1.1	PUSCH Related

| Issue # | Description | Tdoc References | Email Discussion Thread |
| --- | --- | --- | --- |
| 1 | For Type 2 UL resource allocation, the frequency domain resource allocation (FDRA) field for DCI 0_0 with CRC scrambled by C-RNTI / CS-RNTI / MCS-RNTI is unspecified. Discuss whether FDRA field should be be X bits or X+Y bits, considering both CSS and USS. TPs needed to 38.212 §7.3.1.1.1 and 38.214 §7.1.2. | [11], [12], [13], [3] | A |
| 2 | For DCI 0_1, ordering of X and Y bits in FDRA field is not specified. TP 38.212 §7.3.1.1.2 | [2], [5], [6], [10] | A |
| 3 | For DCI 0_1, if a BWP switch is indicated, and new BWP has different #RB-Sets and/or different SCS, how should X and Y bits be interpreted? | [5], [13] | A |
| 4 | For 15 kHz SCS, currently specified RIV-based interlace allocation for PUSCH implicitly assumes that index of first interlace in an RB set or carrier is always 0, which is inconsistent with interlace definition which uses Point A as a reference. TP needed to 38.214 §6.1.2.2.3. | [5] | B |
| 5 | For interlaced PUSCH with DFT-s-OFDM, UE behavior is unspecified if the number of PRBs does not equal 2a×3b×5c where a,b,c are non-negative integers. TP to 38.214 §6.1.2.2.3 | [5], [9], [10] | B |
| 6 | Clarification of UE interpretation of frequency domain resource allocation field in RAR UL grant accouting for different field sizes when interlacing configured/not-configured and operation with/without shared spectrum channel acces. TP to 38.213 §8.3. | [5], [13], [3] | B |

### 2.1.2	PUCCH Related

| Issue # | Description | Tdoc References | Email Discussion Thread |
| --- | --- | --- | --- |
| 7 | Specifications are not clear on the following points related to frequency domain resource allocation for interlaced PUCCH - Missing procedure, analogous to PUSCH, that PUCCH resource occupies RBs given by intersection of the RB set indicated by rb-SetIndex and the interlace(s) indicated by interlace0, interlace1 - Handling of case if intersection results in fewer than 10 PRBs - For PF3, handling of case if intersection has 11 PRBs - Clarification on definition of variables MInterlace,0PUCCH and MInterlace,1PUCCH (# of PRBs in interlace 0, 1) - For PF0/1, clarification that cyclic shift cycling starts with lowest indexed IRB in the allocated interlace TP to 38.213 §9.2.1. | [4], [8], [9], [10], [3] | C |
| 8 | Eroneous RRC parameter for the RB-set index of a PUCCH resource. Should be rb-SetIndex. TP to 38.213 §9.2.1. | [3] | C |
| 9 | PUCCH rate matching for Interlaced PF2 and PF3 does not take into account spreading factor. TP 38.212 §6.3.1.4 | [11] | C |

## 2.2 Issues with Lower Priority

Issues listed as lower priority do not mean that corrections are not required; rather, corrections can be discussed later. If all of the issues in Section 2.1 can be solved, this will be very good progress.

### 2.2.1	PUCCH Related

| Issue # | Description | Tdoc References |
| --- | --- | --- |
| 10 | Specifications are not clear that when useInterlaceConfig-Dedicated is configured, only PUCCH formats 0,1,2,3 are allowed. TP 38.213 §9.2.1 and 38.331. | [7] |
| 11 | Clarification on the conditions for using PF0,1,2,3,4. TP to 38.213 §9.2.2 | [8] |

### 2.2.2	SRS Related

| Issue # | Description | Tdoc References |
| --- | --- | --- |
| 12 | Procedure text capturing that SRS resource can start at any OFDM symbol in a slot is not specified. TP to 38.214 §6.2.1. | [1], [10] |
| 13 | Support CP extension for SRS (in addition to PUCCH/PUSCH) | [2] |
| 14 | Allow aperiodic-SRS starting position in a slot to be later than slot n + k (n = slot containing trigger, k = RRC configured slot offet) depending on LBT outcome | [2] |
| 15 |  |  |

### 2.2.3	Other

| Issue # | Description | Tdoc References |
| --- | --- | --- |
| 16 | Specifications do not capture that interlacing is not supported for bandwidths less than 20 MHz (e.g., 10 MHz carrier). TP to 38.211 §4.4.4.6 | [4] |
| 17 | Discuss whether or not clarification is needed on the behavior related to the RRC parameter enableConfiguredUL-r16. Any relationship to indicated CO duration? | [9] |

### 2.2.4	Editorial Issues

| Issue # | Description | Tdoc References |
| --- | --- | --- |
| 18 | The parameter interlace1 is contained in PUCCH-format{2 and 3}-r16 not in PUCCH-Resource-r16. Number of interlaces is not specified explicitly. TP needed to 38.213 §9.2.1 to align with 38.331. | [7] |
| 19 | For PUCCH resource set prior to dedicated configuration, reference to OCC index in 38.211 would be helpful. | [10] |

# References

- R1-2000647, “TP for SRS configuration,” CATT, RAN1#100-e, February, 2020.

- R1-2000770, “UL signals and channels,” ETRI, RAN1#100-e, February, 2020.

- R1-2000825, “UL signals and channels,” Ericsson, RAN1#100-e, February, 2020.

- R1-2000196, “Corrections on uplink signals and channels,” Huawei, HiSilicon RAN1#100-e, February, 2020.

- R1-2000662, “Remaining issues of UL signals and channels for NR-U,” LG Electronics, RAN1#100-e, February, 2020.

- R1-2000591, “Text proposals for UL signals and channels for NR-U,” Lenovo, Motorola Mobility, RAN1#100-e, February, 2020.

- R1-2000436, “Remaining issues on UL signals and channels for NR-U operation,” MediaTek Inc., RAN1#100-e, February, 2020.

- R1-2000410, “Remaining Issues on UL Signals and Channels for NR-U,” Nokia, Nokia Shanghai Bell, RAN1#100-e, February, 2020.

- R1-2000469, “Discussion on the remaining issues of UL signals and channels,” OPPO, RAN1#100-e, February, 2020.

- R1-2000955, “TP for UL signals and channels for NR-U,” Qualcomm Incorporated, RAN1#100-e, February, 2020.

- R1-2000609, “UL signals and channels for NR-U,” Samsung, RAN1#100-e, February, 2020.

- R1-2000872, “Remaining issues and corrections on UL signals and channels for NR-U,” Sharp, RAN1#100-e, February, 2020.

- R1-2000308, “Remaining issues on physical UL channel design in unlicensed spectrum,” vivo, RAN1#100-e, February, 2020.