3GPP TSG RAN WG1 Meeting #100	R1-2000878

Athens, Greece, February 24th – 28th, 2020

Source:	Sharp

Title:	Remaining issues on resource allocation mode 2 for NR sidelink

Agenda Item:	7.2.4.2.1

Document for:	Discussion and Decision

Introduction

In this document, we share our views on a few remaining issues on resource allocation mode 2 for NR sidelink.

Discussion

Time domain resource allocation

The following was agreed in RAN1#99,

Agreements: - The first proposal under Wed. session in R1-1913450 is agreed, with one clarification that S is the number of sub-channels in the resource pool The agreed proposal is as follows: For mode 1 and mode 2, for the time-frequency resource indication in the SCI: - NMAX = 2 - Frequency - m=1S(S+1-m)=SS+12 code-points, indicating starting sub-channel of the second resource and number of sub-channels of both resources - log2(SS+12) bit - Time - 1 code-point indicates no reserved resource - 31 code-points indicate different time position of the second resource within 32 slots - 5 bit - NMAX = 3 - Frequency - Option 2-f-a: joint coding - m=1SS+1-m2=SS+12S+16 code-points indicating starting sub-channel of the second resource, starting sub-channel of the third resource, and the number of sub-channels of all resources - log2(SS+12S+16) bit - Time - Option 2-t-a: joint coding - 1 code-point indicates no reserved resource - 31 code-points indicate different time position of the second resource within 32 slots, when no third resource is reserved - 30 + 29 +…+ 1 = 465 code-points indicate different time position of two resources within 32 slots - 9 bit

The size of the “Time resource allocation” field in 1st-stage SCI has been correctly reflected in TS 38.212. However, it remains to be specified in TS 38.214 how to map each code-point to the slot index(es) of the reserved resource(s) within the resource reservation window.

The case of NMAX=2 is straightforward, e.g. 0 is mapped to single resource (i.e. no reserved resource), and 1 to 31 is respectively mapped to the slot index of the 2nd resource (i.e. 1st reserved resource) within the resource reservation window.

In case of NMAX=3, the values of the slot index for the 2nd resource (denoted t2 here) and the 3rd resource (denoted t3 here) are like the following,

- For t2=1, t3={2, 3, …, 31};

- For t2=2, t3={3, …, 31};

- …

It can be seen that the restriction of t2 and 31-t3 is the same as that of (LRBs, RBstart) in Downlink/Uplink resource allocation type 1, assuming NBWPsize=30.

Proposal 1: Mapping of “Time resource assignment” in 1st-stage SCI to the slot index of the 2nd assigned resource within the resource reservation window (t2), if any, and the slot index of the 3rd assigned resource within the resource reservation window (t3), if any:

- For NMAX=2,

  - 0 corresponds to the case of a single assigned resource (i.e. no t2);

  - 1 to 31 correspond to the case of two assigned resources, and RIV=t2.

- For NMAX=3,

  - 0 corresponds to the case of a single assigned resource (i.e. no t2 and no t3);

  - 1 to 31 correspond to the case of two assigned resources, and RIV=t2;

  - 32 and above correspond to the case of three assigned resources, and the mapping for Downlink/Uplink resource allocation type 1 is reused by assuming LRBs=t2, RBstart=31-t3, and NBWPsize=30.

Frequency domain resource allocation

Similarly to time domain resource allocation, it remains to be specified in TS 38.214 how to map each code-point of “Frequency resource allocation” field in 1st-stage SCI to the number of allocated subchannels and the starting subchannel of each allocated resource.

For NMAX=2, the RIV-based mapping for Downlink/Uplink resource allocation type 1 can be reused.

For NMAX=3, a given code-point is mapped to three indicated values (i.e. number of allocated subchannels, denoted as m here, starting subchannel of the 2nd resource, denoted as f2 here, and the starting subchannel of the 3rd resource, denoted as f3 here), hence the RIV-based mechanism cannot be directly reused. Since the number of code-points for each value of m is only dependent on m (i.e. S+1-m2, the code-points can be allocated in such a way that the starting code-point for each value of m is i=1m-1S+1-i2, as follows,

RIV=f2⋅S+1-m+f3+i=1m-1S+1-i2

Note that the same mechanism can also be used for NMAX=2, i.e.

RIV=f2+i=1m-1S+1-i

Proposal 2: Mapping of “Frequency resource assignment” in 1st-stage SCI to the number of assigned subchannels (m), the starting subchannel of the 2nd assigned resource (f2), and the starting subchannel of the 3rd assigned resource (f3):

- For NMAX=2,

  - Alt-1: the RIV-based mapping for Downlink/Uplink resource allocation type 1 is reused, assuming LRBs=m, RBstart=f2 and NBWPsize=S;

  - Alt-2: RIV=f2+i=1m-1S+1-i.

- For NMAX=3,

  - RIV=f2⋅S+1-m+f3+i=1m-1S+1-i2

Conclusion

In this contribution, we discuss some remaining issues on resource allocation mode 2 for NR sidelink, and make the following proposals:

Proposal 1: Mapping of “Time resource assignment” in 1st-stage SCI to the slot index of the 2nd assigned resource within the resource reservation window (t2), if any, and the slot index of the 3rd assigned resource within the resource reservation window (t3), if any:

- For NMAX=2,

  - 0 corresponds to the case of a single assigned resource (i.e. no t2);

  - 1 to 31 correspond to the case of two assigned resources, and RIV=t2.

- For NMAX=3,

  - 0 corresponds to the case of a single assigned resource (i.e. no t2 and no t3);

  - 1 to 31 correspond to the case of two assigned resources, and RIV=t2;

  - 32 and above correspond to the case of three assigned resources, and the mapping for Downlink/Uplink resource allocation type 1 is reused by assuming LRBs=t2, RBstart=31-t3, and NBWPsize=30.

Proposal 2: Mapping of “Frequency resource assignment” in 1st-stage SCI to the number of assigned subchannels (m), the starting subchannel of the 2nd assigned resource (f2), and the starting subchannel of the 3rd assigned resource (f3):

- For NMAX=2,

  - Alt-1: the RIV-based mapping for Downlink/Uplink resource allocation type 1 is reused, assuming LRBs=m, RBstart=f2 and NBWPsize=S;

  - Alt-2: RIV=f2+i=1m-1S+1-i.

- For NMAX=3,

  - RIV=f2⋅S+1-m+f3+i=1m-1S+1-i2

References

RP-190766, “New WID on 5G V2X with NR sidelink”, LG Electronics, Huawei, 3GPP RAN#83.