3GPP TSG RAN WG1 #100                                                                                   R1-2000920

e-Meeting, February 24th – March 6th, 2020

Source:	NTT DOCOMO, INC.

Title:	Discussion on the priority index for the HARQ-ACK codebook

Agenda Item:	7.2.5.2

Document for: 	Discussion and Decision

- Introduction

In this contribution, we discussed one issue related to the priority index for the HARQ-ACK codebook when two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE.

- The priority index for the HARQ-ACK codebook

A UE with multiple services can be indicated to generate two HARQ-ACK codebooks associated with different priorities. RRC parameters in PUCCH configuration related to HARQ-ACK feedback can be separately configured for two HARQ-ACK codebooks. This is captured in TS 38.213 section 9.1 [1] as following:

## 9.1 HARQ-ACK codebook determination If a UE is provided pdsch-HARQ-ACK-Codebook-List, the UE can be indicated by pdsch-HARQ-ACK-Codebook-List to generate one or two HARQ-ACK codebooks - a first HARQ-ACK codebook is associated with a PUCCH of priority index either 0 or 1 and a second HARQ-ACK codebook is associated with a PUCCH of priority index either 1 or 0, respectively - the UE is provided first and second for each of {PUCCH-Config, UCI-OnPUSCH, PDSCH-codeBlockGroupTransmission} by { PUCCHConfigurationList, UCI-OnPUSCH-List, PDSCH-CodeBlockGroupTransmission-List}, respectively, for use with the first and second HARQ-ACK codebooks, respectively […]

With above description, the UE with multiple services can be indicated by pdsch-HARQ-ACK-Codebook-List to generate two HARQ-ACK codebooks, e.g., one HARQ-ACK codebook for eMBB associated with a PUCCH of priority index 0, one HARQ-ACK codebook for URLLC associated with a PUCCH of priority index 1. There are two possible associations between the priority index and the HARQ-ACK codebook based on the above spec as shown in Fig.1. Different pdsch-HARQ-ACK-Codebook and PUCCH-Config can be used for the same HARQ-ACK codebook with a given priority index. That means when the UE receives a dynamic scheduled PDSCH, it can identify which HARQ-ACK codebook (i.e., for eMBB HARQ-ACK codebook or for URLLC HARQ-ACK codebook), the HARQ-ACK for this PDSCH belongs to by priority indicator field indication in the scheduled DCI. However, the UE is not able to identify the association between the priority index and the HARQ-ACK codebook for this HARQ-ACK. Then the UE is not able to determine which {PUCCH-Config, UCI-OnPUSCH, PDSCH-codeBlockGroupTransmission} of {PUCCHConfigurationList, UCI-OnPUSCH-List, PDSCH-CodeBlockGroupTransmission-List} is for HARQ-ACK for eMBB and which is for HARQ-ACK for URLLC. Therefore, following TP is proposed:

Fig.1 Example for the two association between the priority index and the HARQ-ACK codebook

Proposal 1:

- Adopt following TP in TS 38.213 section 9.1

## 9.1 HARQ-ACK codebook determination If a UE is provided pdsch-HARQ-ACK-Codebook-List, the UE can be indicated by pdsch-HARQ-ACK-Codebook-List to generate one or two HARQ-ACK codebooks. When UE is indicated to generate two HARQ-ACK codebooks, - a first HARQ-ACK codebook is associated with a PUCCH of priority index 0 and a second HARQ-ACK codebook is associated with a PUCCH of priority index 1, respectively - the UE is provided first and second for each of {PUCCH-Config, UCI-OnPUSCH, PDSCH-codeBlockGroupTransmission} by { PUCCHConfigurationList, UCI-OnPUSCH-List, PDSCH-CodeBlockGroupTransmission-List}, respectively, for use with the first and second HARQ-ACK codebooks, respectively […]

- Conclusion

In this contribution, we discussed one issue related to the priority index for the HARQ-ACK codebook when at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE. Following is the proposal:

Proposal 1:

- Adopt following TP in TS 38.213 section 9.1

## 9.1 HARQ-ACK codebook determination If a UE is provided pdsch-HARQ-ACK-Codebook-List, the UE can be indicated by pdsch-HARQ-ACK-Codebook-List to generate one or two HARQ-ACK codebooks. When UE is indicated to generate two HARQ-ACK codebooks, - a first HARQ-ACK codebook is associated with a PUCCH of priority index 0 and a second HARQ-ACK codebook is associated with a PUCCH of priority index 1, respectively - the UE is provided first and second for each of {PUCCH-Config, UCI-OnPUSCH, PDSCH-codeBlockGroupTransmission} by { PUCCHConfigurationList, UCI-OnPUSCH-List, PDSCH-CodeBlockGroupTransmission-List}, respectively, for use with the first and second HARQ-ACK codebooks, respectively […]

References

- R1-1913649, ‘Introduction of Ultra Reliable Low Latency Communications Enhancements in TS 38.213’, Samsung

- 3GPP, RAN1 #99 meeting, chairman’s notes.