3GPP TSG RAN WG1 #100-e																			R1- 2000932

Feb 24 – Mar 06, 2020

Source:	CATT

Title:	FL Summary of Remaining issues on NR Positioning Measurements

Agenda Item:	7.2.8.3

Document for:	Discussion and Decision

# Introduction

This document provides a summary of the remaining issues and proposals for AI 7.2.8.3 related to UE and gNB measurements for NR Positioning based on the contributions submitted under this AI [1-10]. According to the guidance of RAN1 Chairman, the intention of this summary is to provide a list of issues/proposals and also the indication of priority based on the contributions.

To facilitate the discussion, more details on the issues/proposals from the submitted contributions and the FL views are presented in Section 2. The summary of the issues/proposals is provided in Section 3, where the feature lead has also provided his assessment on the critical issues/proposals. Interested companies are welcome to present their views on the priority of these issues.

Issues/proposals for UE/gNB measurements

## TPs for the corrections/changes based on existing agreements

A number of TPs were submitted for the corrections and changes for TS 38.214 and TS 38.215 [1-10] based on existing agreements.

Submitted Proposals

- (Huawei) Proposal 1

    - Adopt the following TP for TS 38.215

============================ Unchanged part omitted ========================== ### 5.1.29 DL reference signal time difference (DL RSTD) Definition; DL reference signal time difference (DL RSTD) is defined as the DL relative timing difference between the positioning node j and the reference positioning node i, is defined as TSubframeRxj – TSubframeRxi, Where: TSubframeRxj is the time when the UE receives the start of one subframe from positioning node j. TSubframeRxi is the time when the UE receives the corresponding start of one subframe from positioning node i that is closest in time to the subframe received from positioning node j. Multiple DL PRS resources can be used to determine the start of one subframe from a positioning node. For frequency range 1, the reference point for the DL RSTD shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSTD shall be the antenna of the UE.. Applicable for; RRC_CONNECTED intra-frequency RRC_CONNECTED inter-frequency. ============================ Unchanged part omitted ==========================

- (Qualcomm) Proposal 1

    - Change the title of the section 5.1.29 of 38.215 from “DL relative signal time difference (DL RSTD)” to “DL Reference relative Signal Time Difference (DL RSTD)

- (vivo) Proposal 2:

  - Rename ‘DL-PRS-RstdReferenceInfo’ to ‘DL-PRS-ReferenceInfo’ and add descriptions in the specification to make this parameter applicable to DL PRS RSRP and UE Rx-Tx time difference methods.

- (vivo) Proposal 3:

  - Adopt the following text proposals into TS 38.214 for RSTD, PRS-RSRP and UE Rx-TX time difference measurement reference

TS 38.214-g00 5.1.6.5 PRS reception procedure < Unchanged parts are omitted > The UE may be indicated by the network that a DL PRS resources can be used as the reference for the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements in a higher layer parameter DL-PRS-RstdReferenceInfo. The reference time indicated by the network to the UE can also be used by the UE to determine how to apply higher layer parameters DL-PRS-expectedRSTD and DL-PRS-expectedRSTD-uncertainty. The UE expects the reference time to be indicated whenever it is expected to receive the DL PRS. This reference time provided by DL-PRS-RstdReferenceInfo may include an [ID], a PRS resource set ID, and optionally a single PRS resource ID or a list of PRS resource IDs. The UE may use different DL PRS resources or a different DL PRS resource set to determine the reference time for the RSTD measurement as long as the condition that the DL PRS resources used belong to a single DL PRS resource set is met. If the UE chooses to use a different reference time than indicated by the network, then it is expected to report the [TRP ID], the DL PRS resource ID(s) or the DL PRS resource set ID used to determine the reference. < Unchanged parts are omitted >

  - (vivo) Proposal 4:

    - Adopt the following text proposals into TS 38.214 for RSTD, PRS-RSRP and UE Rx-TX time difference measurement reference

- TS 38.214-g00 - 5.1.6.5 PRS reception procedure - < Unchanged parts are omitted > - For the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements the UE can report an associated higher layer parameter Timestamp. The Timestamp can include the SFN and the slot number for a subcarrier spacing. These values correspond to the reference which is provided by DL-PRS-RSTDReferenceInfo. - < Unchanged parts are omitted >

- (Qualcomm)  Proposal 2

  - Update the text in Section 5.1.6.5 of 38.214 as follows:

The UE can be configured in higher layer parameter UE Rx-Tx Time-MeasRequestInfo to report, subject to UE capability, up to 4 UE Rx-Tx time difference measurements corresponding to a single configured SRS resource or resource set for positioning. Each measurement corresponds to a single received DL PRS resource or resource set which can be in difference positioning frequency layers.

- (vivo)  Proposal 7

  - The higher layer parameters in TS38.211-TS38.215 should be aligned with TS37.855 and TS38.331.

FL Comments

TPs for the corrections and changes based on existing agreements should be considered in this meeting.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## Introducing the definition of positioning node

Submitted Proposals

- (Nokia)  Proposal 1: Define a positioning node as a TRP, UL-positioning only RP, or DL-positioning only TP which transmits and/or receives positioning signals.

- Nokia)  Proposal 2: Adopt the TP in the Appendix for TS 38.215.

### 5.1.29 DL relative signal time difference (DL RSTD) Definition; DL relative timing difference (DL RSTD) between the positioning node j and the reference positioning node i, is defined as TSubframeRxj – TSubframeRxi, Where: TSubframeRxj is the time when the UE receives the start of one subframe from positioning node j. TSubframeRxi is the time when the UE receives the corresponding start of one subframe from positioning node i that is closest in time to the subframe received from positioning node j. A positioning node is defined as as a TRP, UL-positioning only RP, or DL-positioning only TP which transmits and/or receives positioning signals. Multiple DL PRS resources can be used to determine the start of one subframe from a positioning node. For frequency range 1, the reference point for the DL RSTD shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSTD shall be the antenna of the UE.. Applicable for; RRC_CONNECTED intra-frequency RRC_CONNECTED inter-frequency.

FL Comments

Positioning node may be better defined in higher layer specs (e.g., together with TRP). If the term is only used in TS 38.215, maybe we can replace “positioning node” with TRP or TP for DL RSTD.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## Measurement time occasions for DL PRS-RSRP

Submitted Proposals

- (OPPO) Proposal 5

  - Correct the definition of DL PRS-RSRP and UL RTOA in TS 38.215

Text proposal for Proposals 5: In TS 38.215: <omitted text> ### 5.1.aa DL PRS reference signal received power (DL PRS-RSRP) Definition; DL PRS reference signal received power (DL PRS-RSRP), is defined as the linear average over the power contributions (in [W]) of the resource elements that carry DL PRS reference signals configured for RSRP measurements within the considered measurement frequency bandwidth in the configured measurement time occasions. For frequency range 1, the reference point for the DL PRS-RSRP shall be the antenna connector of the UE. For frequency range 2, DL PRS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported DL PRS-RSRP value shall not be lower than the corresponding DL PRS-RSRP of any of the individual receiver branches.. Applicable for; RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency. <omitted text>

FL Comments

Measurement time occasion for positioning was discussed before. The conclusion, after long offline/online discussions was not mention it, since the term is undefined. Suggest not reopen the discussion in this meeting.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## Correction of UL Relative Time of Arrival definition

Submitted Proposals

- (OPPO) Proposal 5

  - Correct the definition of DL PRS-RSRP and UL RTOA in TS 38.215

### 5.2.aa UL Relative Time of Arrival (TUL-RTOA) Definition; [The UL Relative Time of Arrival (TUL-RTOA) is received timing of the beginning start of subframe i containing SRS received in positioning node #j, relative to the configurable reference time.] Multiple SRS resources for positioning can be used to determine the beginning of one subframe containing SRS received at a positioning node. The reference point for TUL-RTOA shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna, - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector.. <omitted text>

FL Comments

The current wording for the definition of UL Relative Time of Arrival seems clear (Note: The bracket can be removed see Issue 2.1).

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## Reference point of timing related measurements for FR2

Submitted Proposals

(Futurewei) Proposal 1:

Remove the use of ‘antenna of the UE’ and ‘Rx antenna/Tx antenna’ in the definition of DL RSTD and UE Rx-Tx time difference. The alternative is TBD. A possible clarification, which has been used extensively within 38.215 is:

  - “measured for each receiver branch based on the combined signal from antenna elements corresponding to the receiver branch”

(Futurewei) Proposal 2:

- Remove the following phrases below since the exact reference point definitions are provided in TS38.104 which are not aligned with the existing terminologies ‘Rx antenna’ or ‘Tx antenna’ used in TS38.215.

UL Relative Time of Arrival:

“The reference point for TUL-RTOA shall be:

- for type 1-C base station TS 38.104 [14]: the Rx antenna connector,

- for type 1-O or 2-O base station TS 38.104 [14]: the Rx antenna,

for type 1-H base station TS 38.104 [14]: the Rx Transceiver Array Boundary connector.”

gNB Rx-Tx time difference:

“The reference point for TgNB-RX shall be:

- for type 1-C base station TS 38.104 [14]: the Rx antenna connector,

- for type 1-O or 2-O base station TS 38.104 [14]: the Rx antenna,

- for type 1-H base station TS 38.104 [14]: the Rx Transceiver Array Boundary connector.

The reference point for TgNB-TX shall be:

- for type 1-C base station TS 38.104 [14]: the Tx antenna connector,

- for type 1-O or 2-O base station TS 38.104 [14]: the Tx antenna,

for type 1-H base station TS 38.104 [14]: the Tx Transceiver Array Boundary connector”

FL comments:

The current definitions of  the reference points for timing-related measurements in FR2 in TS 38.215 are based on the reply LS from RAN4 (R4-1915801[12]). Since RAN4 is responsible for the definition and also the usage of reference points of the measurement, suggest not opening the discussion in RAN1.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## RTOA reference time

In RAN1#98b, we made the following offline consensus [5]. However, this offline consensus was left out during the online discussion, and the LS had not been sent. As a result, RAN3 is not aware of the role of defining the reference time for UL RTOA measurement.

Offline Consensus

- It is RAN1’s understanding that the TRPs are SFN time synchronized or the SFN offsets are known to the TRPs.

- It is up to RAN3 to define the reference time for UL RTOA measurements,

- Send LS to RAN3

Submitted Proposals

- (Huawei) Proposal 2:

  - It is RAN1’s understanding that the TRPs are SFN time synchronized or the SFN offsets are known to the TRPs.

  - It is up to RAN3 to define the reference time for UL RTOA measurements,

  - Include it in the LS to RAN3

FL comments

We should recapture this left out offline consensus in this e-meeting.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## Details on the definition of RTOA reference time

Submitted Proposals

- (Huawei) Proposal 3:

      - If SFN initialization time is provided in the SRS configuration or implicitly assumed the same for the neighbouring gNB to measure SRS, the RTOA reference time is defined as T0+tSRS, where

      - T0 is the SFN initialization time (time of start of subframe#0 of SFN#0)

      - tSRS is the offset of the start of the subframe that contains the target SRS to the start of subframe#0 of SFN#0

FL comments

Based on the Offline Consensus in RAN1#98bis, the details on the definition of the reference time for UL RTOA measurements can be handled in RAN3.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## FFSs on Maximum numbers of DL PRS resources and UE capability

Following agreements were made on Rel-16 UE/gNB positioning measurements in RAN1#99. There are a number FFSs, which needs to be decided.

Agreement: UE can be configured for DL PRS processing according to the following table: Description; Maximum numbers for DL PRS resources; Values that can be signaled as part of UE Capability. Max number of frequency layers (X1); X1=4; Values = {1,4} FFS: other values. Max number of TRPs per frequency layer (X2); X2=64. Max number of PRS resource sets per TRP (X3) per frequency layer; X3=2; Values = {1,2}. Max number of Resources per PRS resource set (X4); X4=64; FFS: values. Max number of DL PRS Resources per UE (X5); NA; FFS: values. Max number of TRPs for all frequency layers (X6) per UE; 256; FFS: values. Max number of Resources per frequency layer (X7); NA; FFS: values. Note: This does not constrain in any way how features and feature sets are defined. The values in the table above may or may not be signalled to be different for different features or feature sets.

Submitted Proposals

- (CATT) Proposal 1

      - UE can be configured for DL PRS processing according to the following table:

| Description | Maximum numbers for DL PRS resources | Values that can be signaled as part of UE Capability |
| --- | --- | --- |
| Max number of frequency layers (X1) | X1=4 | Values = {1, 2, 4} FFS: other values |
| Max number of Resources per PRS resource set (X4) | X4=8 for FR1 X4=64 for FR2 | FFS: values NA |
| Max number of DL PRS Resources per UE (X5) | NA | FFS: values {32, 64, 128, 264}*8*2 for FR1 {32, 64, 128, 264}*64*2 for FR2 |
| Max number of TRPs for all frequency layers (X6) per UE | 256 | FFS: values {32, 64, 128, 264} |
| Max number of Resources per frequency layer (X7) | NA | FFS: values {32, 64, 128, 264}*8 for FR1 {32, 64, 128, 264}*64 for FR2 |

- (Samsung) Proposal 1

      - The following values for X1 to X7 should be supported.:

| Description | Maximum numbers for DL PRS resources | Values that can be signaled as part of UE Capability |
| --- | --- | --- |
| Max number of frequency layers (X1) | X1=4 | Values = {1,4} No other values |
| Max number of TRPs per frequency layer (X2) | X2=64 |  |
| Max number of PRS resource sets per TRP (X3) per frequency layer | X3=2 | Values = {1,2} |
| Max number of Resources per PRS resource set (X4) | X4=64 | Values = {64, 32, 16, 8 ,4, 2,1} |
| Max number of DL PRS Resources per UE (X5) | NA | Values = {16, 8, 4, 2 ,1} |
| Max number of TRPs for all frequency layers (X6) per UE | 256 | Values = {1,…,256} |
| Max number of Resources per frequency layer (X7) | NA | Values = {128, 64, 32, 16, 8 ,4, 2, 1} |

- (Samsung) Proposal 2

      - There is no need to support FR1/FR2 differentiation for the values of X1 to X7

FL Comments

The FFSs in previous agreement should be determined in this meeting.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## UE capability on DL PRS RSRP

Following agreements were made on Rel-16 UE/gNB positioning measurements in RAN1#99, where it is FFS on whether N is a UE capability

Agreement: The prior agreement is updated as given below: UE can be configured to measure and report up to N=8 (> 1) DL PRS RSRP measurements on different DL PRS resources from the same TRP - N=[3] FFS: N is a UE capability

Submitted Proposals

- (vivo) Proposal 5:

      - N DL PRS RSRP measurements being performed on different DL PRS resources from the same TRP subject to UE capability

- (vivo) Proposal 6:

      - Adopt the following text proposal into TS 38.214 for PRS RSRP measurement and report related description

TS 38.214-g00 5.1.6.5 PRS reception procedure < Unchanged parts are omitted > The UE may be configured to measure and report, subject to UE capability, up to 8 DL PRS RSRP measurements on different DL PRS resources from the same cell. When the UE reports DL PRS RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS RSRP measurements have been performed using the same spatial domain filter for reception. < Unchanged parts are omitted >

- (CATT) Proposal 2

      - Update the prior agreement as given below

        - UE can be configured to measure and report up to N=8 DL PRS RSRP measurements on different DL PRS resources from the same TRP.

    - FFS: N is a UE capability

- (Samsung) Proposal 3

      - The number of reported DL PRS RSRP measurements N is not a UE capability

FL Comments

The FFS in previous agreement should be determined in this meeting.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## Reporting TRP ID of DL PRS reference

Submitted Proposals

- Issue 1: PRS reference for measurement and report

  - (vivo) Proposal 1:

    - The TRP ID of the reference DL PRS Resource or DL PRS Resource set shall be reported by the UE if it chooses to use a different reference whose TRP is different from the one indicated by the network

FL Comments

Yes. This can be clarified.  Previous agreement is only say report the DL PRS Resource ID(s) and/or the information on the DL PRS Resource set.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## Working assumption on multiple Rx–Tx time difference measurements

The following working assumption was made in RAN1#98bis

Working assumption: A UE can be configured to report multiple Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL PRS resource/resource set. The DL PRS resource/resource sets can be in different positioning frequency layers - FFS: Reporting of SRS for positioning resource/resource set ID corresponding to a UE Rx-Tx time difference measurement - Note: This agreement does not introduce any new behavior for the transmission of SRS for positioning.

During RAN1#99, there was length online and online discussion on the issue and the following agreement was made:

Agreement: Confirm the working assumption from RAN1#98bis on reporting of multiple Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning. The FFS item in the working assumption is removed.

Submitted Proposals

(OPPO)  Proposal 4: Confirm the working assumption with the revision:

- A UE can be configured to report multiple Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL PRS resource/resource set. The DL PRS resource/resource sets can be in different positioning frequency layers.

  - The UE does not report the SRS for positioning resource or set ID used to measure UE Rx-Tx time difference.

Text proposal for Proposals 4: In TS 38.213: Section 5.1.6.4 PRS reception procedure <omitted text> The UE can be configured in higher layer parameter UE Rx-Tx Time-MeasRequestInfo to report multiple UE Rx-Tx time difference measurements.corresponding to a single configured SRS resource or resource set for positioning. Each measurement corresponds to a single received DL PRS resource or resource set which can be in difference positioning frequency layers. <omitted text>

FL Comments

UL SRS can be sent in different frequency layers and there can also be different TA groups in UE. Thus, we should keep the original agreement. We have lengthy online/offline discussions in RAN1#99 to reach the agreement. Suggest not opening the discussion again.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## Differential measurement reporting

Submitted Proposals

A number of companies have discussed the issues related to differential measurement reporting:

- (Huawei) Proposal 4:

  - Support differential report for the following measurement types if multiple measurements of the same type are reported for a single TRP/UE

    - PRS-RSRP

    - DL RSTD

    - UE Rx – Tx time difference

    - SRS-RSRP

    - UL RTOA

    - gNB Rx – Tx time difference

  - Two measurements of the same type can be associated with the same or different

    - PRS resource ID for DL, or

    - PRS resource ID or SSB index for UL

  - Include it in the LS to RAN2 and RAN3.

- (vivo) Proposal 8:

  - When a UE is configured to report up to multiple DL PRS RSTD measurements with each measurement between a different pair of DL PRS resources or DL PRS resource sets, and those multiple measurements being performed on the same pair of TRPs, the UE can be configured to report one full RSTD measurement and some additional differential RSTD measurement(s).

- (LGE) Proposal 1:

  - For the report of multiple RSTD measurements for a pair of two TRPs, support differential RSTD measurement reporting.

  - For the report of multiple UE RX-TX time difference measurements for multiple PRS resources or multiple PRS resource sets transmitted from a single TRP/cell, support differential UE RX-TX time difference measurement reporting.

FL Comments

While it is beneficial to support differential reporting, RAN1 does not have the time to work on in previous meetings due to the time constraint. Maybe this issue can be handled by RAN2 without RAN1 involvement.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## Determination of RSTD measurement

Submitted Proposals

- (LGE) Proposal 2

  - In case the UE reports a single RSTD measurement with a DL PRS resource ID and a DL PRS resource set ID, the UE chooses a DL PRS resource within a PRS resource set from which the UE could obtain the minimum propagation time of the first-arrival signal path. If two or more PRS resources show the same minimum propagation time of the first-arrival signal path, then the UE selects a PRS resource corresponding to the larger received signal power.

FL Comments

How determine and report RSTD measurement was discussed in previous meetings. The conclusion was that it will be up to UE’s implementation. Suggest not opening the discussion in this meeting.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## Determination of the Time Stamp for UE measurements

The following agreements were made about the time Stamp for UE measurements:

Agreement: A UE measurement can be associated with a time stamp. - For DL RSTD and RSRP measurements, the time stamp can include at least the SFN - FFS: SFN corresponds to the reference DL PRS resource or DL PRS resource sets (can be determined by the UE) - For Rx-Tx measurements, the time stamp can include the SFN, as well as the slot number for a subcarrier spacing - FFS: these values correspond to the reference DL PRS resource or DL PRS resource set (can be determined by the UE) Agreement: Modify the previous agreement on the definition of the time stamp as follows: A UE measurement can be associated with a time stamp. For UE RSTD, DL PRS RSRP and UE Rx-Tx time difference measurement report, the time stamp can include the SFN, as well as the slot number for a subcarrier spacing. These values correspond to the reference provided by the DL-PRS-RstdReferenceInfo.

Submitted Proposals

- (OPPO) Proposal 1

  - For DL RSTD, PRS-RSRP and UE Rx-Tx time difference measurement, the time stamp includes the SFN and slot number of the DL PRS resource or the earliest PRS resource in the PRS resource set.

Text proposal for Proposals 1 and 2: In TS 38.214: Section 5.1.6.4 PRS reception procedure <omitted text> For the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements the UE can report an associated higher layer parameter Timestamp. The Timestamp can include the SFN and the slot number for a subcarrier spacing. If one DL PRS resource is used for the measurement, the Timestamp can include the SFN and slot number of the DL PRS resource. If one PRS resource set is used for the measurement, the Timestamp can include the SFN and slot number of the earliest DL PRS resource in the PRS resource set. These values correspond to the reference which is provided by DL-PRS-RSTDReferenceInfo. <omitted text>

FL Comments

The timestamp for measurement indicates when the measurement is valid. It should be up to the UE to determine the valid timestamp. It may not need to be associated with the start of DL PRS resources.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## Indication of the confidence level of uncertainty

Submitted Proposals

- (Huawei) Proposal 5:

  - For each measurement associated with quality report, UE or NG-RAN should also indicate the confidence by which the measurement is known to be within the uncertainty range, using 7 bits expressed as a percentage.

    - Include it in the LS to RAN2 and RAN3

FL Comments

While the indication of the confidence level of uncertainty can be useful for positioning applications, it may be too late to open the discussion of this issue in R16.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## Clarify the quality metrics of RSTD and UE Rx – Tx time difference

Submitted Proposals

- (Huawei) Proposal 6:

  - Clarify the quality measurement of RSTD and UE Rx – Tx time difference is the quality of timing measurement of a TRP, rather than the quality of timing difference measurement.

    - Note that all TRPs, including reference TRP and neighbouring TRPs should have its own timing measurement quality

FL Comments

It is unclear on the necessity for this clarification.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## UE/gNB RX-TX time difference measurements for NR E-CID

Submitted Proposals

- (LGE) Proposal 3:

      - Support UE RX-TX time difference measurement and gNB RX-TX time difference measurement for NR positioning.

      - Support DL PRS for NR ECID technique

- (Ericsson) Proposal 1:

      - Support reuse of Rel-15 SRS resource set for gNB Rx-Tx and UE Rx-Tx measurements for positioning in NR.

      - Send an LS to RAN4 regarding UE Rx-Tx requirement

Note: There is no impact to specifications managed by RAN1

FL Comments

Supporting UE/gNB RX-TX time difference measurement with R15 SRS may be beneficial for NR E-CID, However, supporting RTT with R15 NR signal is not in the scope of R16 WI. It may be too late to open the discussion in RAN1.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## UE Measurement gap

Submitted Proposals

- (OPPO): Proposal 2:

  - The UE can request a measurement gap if the UE is expected to measure DL PRS resource with a numerology different from that of the active DL BWP or on symbols configured as UL symbol.

Text proposal for Proposals 1 and 2: In TS 38.214: Section 5.1.6.4 PRS reception procedure <omitted text> The UE is expected to measure the DL PRS resource outside the active DL BWP or with a numerology different from the numerology of the active DL BWP if the measurement is made during a configured measurement gap. When not configured with a measurement gap, the UE is only required to measure DL PRS within the active DL BWP and with the same numerology as the active DL BWP. If the UE is not provided with a measurement gap, the UE is not expected to process DL PRS resources on serving or non-serving cells on any symbols indicated as UL by the serving cell. When the UE is expected to measure the DL PRS resource outside the active DL BWP or with a numerology different from the numerology of the active DL BWP or on any symbols indicated as UL symbol by the serving cell, it may request a measurement gap in higher layer parameter [XYZ].

FL Comments

The discussion of measurement gap may be brought up to AI 8.1.2.4 for measurement procedure.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

## Rx beam sweeping

Submitted Proposals

- (OPPO): Proposal 3:

  - For one DL PRS RSRP measurement, the UE may indicate whether Rx beam sweeping is performed for receiving the DL PRS resource.

Text proposal for Proposals 1 and 2: In TS 38.214: Section 5.1.6.4 PRS reception procedure <omitted text> The UE may be configured to measure and report up to 8 DL PRS RSRP measurements on different DL PRS resources from the same cell. When the UE reports DL PRS RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS RSRP measurements have been performed using the same spatial domain filter for reception. When UE reports one DL PRS RSRP measurement, the UE may indicate whether the DL PRS RSRP measurement has been performed using multiple different spatial domain receive filters for receiving the corresponding PRS resource. <omitted text>

The following table includes one text proposal for RSTD measurement:

Text proposal for Proposals 1 and 2: In TS 38.213: 5.1.6.4 PRS reception procedure <omitted text> The UE may be indicated by the network that a DL PRS resources or a subset of DL PRS resources or a DL PRS resource set can be used as the reference for the RSTD measurement in a higher layer parameter DL-PRS-RstdReferenceInfo. The reference time indicated by the network to the UE can also be used by the UE to determine how to apply higher layer parameters DL-PRS-expectedRSTD and DL-PRS-expectedRSTD-uncertainty. The UE expects the reference time to be indicated whenever it is expected to receive the DL PRS. This reference time provided by DL-PRS-RstdReferenceInfo may include an [ID], a PRS resource set ID, and optionally a single PRS resource ID or a list of PRS resource IDs. The UE may use different DL PRS resources or a different DL PRS resource set to determine the reference time for the RSTD measurement as long as the condition that the DL PRS resources used belong to a single DL PRS resource set is met. If the UE chooses to use a different reference time than indicated by the network, then it is expected to report the DL PRS resource ID(s) or the DL PRS resource set ID used to determine the reference. <omitted text>

FL Comments

The discussion of beam sweeping procedure may be brought up to in AI 8.1.2.4 for measurement procedure.

Comments from interested companies

| Company |  | Comments |
| --- | --- | --- |

Summary on the Priority of the Issues/Proposals

Interested companies please indicate their views on the priority of the Issues/Proposals for this meeting in the following table.

•	High priority: Critical issues/Proposals need to be discussed and resolved in this AI in this meeting.

•	Low priority: Issues/Proposals may be discussed in this meeting with low priority.

•	Others: Issues/Proposals may not necessarily be discussed in this AI.

| Issues/Proposals | High Priority |  | Low Priority |  | Others |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Total# | FL/Companies | Total# | FL/Companies | Total# | FL/Companies |
| 2 Issues/proposals for UE/gNB measurements |  |  |  |  |  |  |
| - TPs for the corrections/changes based on existing agreements |  | FL |  |  |  |  |
| - Introducing the definition of positioning node |  |  |  |  |  |  |
| - Measurement time occasions for DL PRS-RSRP |  |  |  |  |  |  |
| - Correction of UL Relative Time of Arrival definition |  |  |  |  |  |  |
| - Reference point of timing related measurements for FR2 |  |  |  |  |  |  |
| - RTOA reference time |  | FL |  |  |  |  |
| - Details on the definition of RTOA reference time |  |  |  |  |  |  |
| - FFSs on Maximum numbers of DL PRS resources and UE capability |  | FL |  |  |  |  |
| - UE capability on DL PRS RSRP |  | FL |  |  |  |  |
| - Reporting TRP ID of DL PRS reference |  | FL |  |  |  |  |
| - Working assumption on multiple Rx–Tx time difference measurements |  |  |  |  |  |  |
| - Differential measurement reporting |  |  |  |  |  |  |
| - Determination of RSTD measurement |  |  |  |  |  |  |
| - Determination of the Time Stamp for UE measurements |  |  |  |  |  |  |
| - Indication of the confidence level of uncertainty |  |  |  |  |  |  |
| - Clarify the quality metrics of RSTD and UE Rx – Tx time difference |  |  |  |  |  |  |
| - UE/gNB RX-TX time difference measurements for NR E-CID |  |  |  |  |  |  |
| - UE Measurement gap |  |  |  |  |  |  |
| - Rx beam sweeping |  |  |  |  |  |  |

References

- R1-2000192	Maintenance of NR positioning measurements	Huawei, HiSilicon

- R1-2000343	Discussion on remaining issues on UE and gNB measurements for NR positioning	vivo

- R1-2000464	Remaining Issues on Measurements for NR Positioning	OPPO

- R1-2000541	Remaining issues on NR Positioning Measurements	CATT

- R1-2000577	TP on Measurements	Futurewei

- R1-2000641	UE and gNB measurements for NR Positioning	Samsung

- R1-2000688	Remaining issues of UE and gNB measurements for NR Positioning	LG Electronics

- R1-2000805	Maintenance of Measurement Definitions	Nokia, Nokia Shanghai Bell

- R1-2000978	Maintenace on UE and gNB measurements for NR Positioning	Qualcomm Incorporated

- R1-2001004	Maintenance of rel16 Physical-layer procedures to support UE - gNB measurements	Ericsson

- R4-1915801	Response LS on Reference Point for Timing Related Measurements in FR2, CATT, Ericsson