3GPP TSG RAN WG1 #100 eMeeting 	R1- 2000969

February 24th – March 6th, 2020

Agenda item:	7.2.5.2

Source: 	Qualcomm

Title: 	Remaining Issues on UCI Enhancements for URLLC

Document for:	Discussion and Decision

# 1	Introduction

In this document, the text proposals for the following topics are presented. Each corresponding section also includes a reason for the proposed addition or modification.

| Section 2 | Timeline for cancellation due to intra-UE prioritization |
| --- | --- |
| Section 3 | Timeline extension for the HP channel |
| Section 4 | Multiplexing versus cancellation order |
| Section 5 | PUCCH resource selection for PRI < 3 bits |
| Section 6 | Collision handling amongst multiple HARQ-ACK PUCCHs and PUSCH |
| Section 7 | Type-2 HARQ-ACK codebook determination with different DAI bitwidths |
| Section 8 | CPU release with uplink interruption |

# 2	Timeline for Cancellation due to Intra-UE Prioritization

During the last meeting, the following agreement was reached:

Agreement:

When a high-priority UL transmission overlaps with a low-priority UL transmission in a slot,

- The UE is expected to cancel the low-priority UL transmission starting from Tproc,2 +d1 after the end of PDCCH scheduling the high-priority transmission, where

  - Tproc,2 is correponding to UE processing time capability for the carrier.

  - Value d1 is the time duration corresponding to 0,1,2 symbols reported by UE capability

  - Note: d_2,1=0 is for cancellation

- The minimum processing time of the high priority channel is extended by d2 symbols

  - Value d2 is the time duration corresponding to 0,1,2 symbols reported by UE capability

The overlapping condition is per repetition of the uplink transmission

This agreement applies to the scenarios where the high-priority channel is scheduled by DCI. For this case, and from the agreement, it is not clear how the Tproc,2 should be defined. It should be noted that since handling collision involves cancelling a transmission and replacing it with another transmission, this case is more similar to PUCCH overriding as captured in Section 9.2.3 of TS 38.213 than cancellation due to SFI.

Besides the collision handling scenarios that are involved a dynamically granted high-priority channel, there are other scenarios that a configured channel with a high priority, e.g., SR or PUCCH carrying SPS HARQ-ACK, can cancel a low priority channel. The cancellation time for these scenarios have not been specified yet. In addition, since there is no agreement on how to handle the collision between DG-PUSCH and CG-PUSCH, the related parts in the current specification can be removed (Note that these parts were added by the editor as a placeholder in case there will be future agreements.) In fact, in our view, the handling of collision in such a scenario can be done similar to Rel. 15 NR without creating over-the-air interruption. Finally, since no consensus was reached after the RAN1 [99-NR-13] email discussion, two PUCCHs carrying HARQ-ACK with different priorities cannot overlap, fully or partially, in time; hence, the corresponding parts, that were left as a placeholder by the editor, from TS 38.213 should be removed.

To address the abovementioned issues, we propose the following additions and modifications:

Modified clause (Section 6.1 of TS 38.214)

A UE is not expected to be scheduled by a PDCCH ending in symbol i to transmit a PUSCH on a given serving cell overlapping in time with a transmission occasion, where the UE is allowed to transmit a PUSCH with configured grant according to [10, TS38.321], starting in a symbol j on the same serving cell if the end of symbol i is not at least N2 symbols before the beginning of symbol j. The value N2 in symbols is determined according to the UE processing capability defined in Subclause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH with configured grant and the subcarrier spacing of the PDCCH scheduling the PUSCH.

If the UE reports the capability of [intra-UE prioritization], and if the UE would transmit PUSCHs corresponding to a configured grant with a priority either not provided or set to 0 in configuredGrantConfig or PUSCHs scheduled by PDCCHs where the priority indicator field in the corresponding DCI is either not configured or set to 0 overlapping fully or partially in time with a PUCCH of a priority index 1

- If the UE is transmitting PUCCH to report HARQ-ACK information for a PDSCH scheduled by a PDCCH or in response to receiving PDCCH indicating a SPS PDSCH release:

  - the UE shall cancel the transmission of the PUSCHs starting from M symbols after the end of the last symbol of the PDCCH indicating PUCCH transmission at the latest, where

    - M = Tproc,2 +d1, where Tproc,2=N2.2048+144.κ2-μ.Tc,  and d1 is determined by the reported UE capability [XXXXX], and μ corresponds to the smallest SCS configuration among SCS configuration of the PDCCHs providing the DCI formats, and the SCS configuration of the PUCCH and the PUSCHs. If processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the DCI scheduling PDSCH and if processingType2Enabled of PUSCH-ServingCellConfig is set to enabled for all the serving cells with DCI scheduling PUSCH, N2 is selected based on the UE PUSCH preparation procedure time according to Table 6.4-2 and SCS configuration μ; otherwise, N2 is selected based on the UE PUSCH preparation procedure time according to Table 6.4-1 and SCS configuration μ.

- If the UE is transmitting PUCCH conveying SR, the UE shall cancel the transmission of the PUSCHs starting from the first overlapping symbol at the latest.

- In case of PUSCH repetitions, the overlapping handling is performed for each PUSCH repetition separately.

The UE is not expected to be scheduled by a PDCCH with another PUSCH/PUCCH with a priority index 0 or with SRS where the transmission of the PUSCH/PUCCH/SRS starts before the end of the time domain allocation of the cancelled PUSCH.

The UE is not expected to be scheduled with PUSCH where the priority indicator field in the corresponding DCI is either not configured or set to 0 overlapping, either partially or fully in time, with a PUCCH with priority index 1 carrying the HARQ-ACK information in response to PDSCH without a corresponding PDCCH.

End

Modified clause (Section 9 of TS 38.213)

A PUSCH or a PUCCH, including repetitions if any, can be of priority index 0 or of priority index 1. If a priority index is not provided for a PUSCH or a PUCCH, the priority index is 0. If in an active DL BWP a UE monitors PDCCH either for detection of DCI format 0_1 and DCI format 1_1 or for detection of DCI format 0_2 and DCI format 1_2, a priority index can be provided by a priority indicator field. If a UE indicates a capability to monitor, in an active DL BWP, PDCCH for detection of DCI format 0_1 and DCI format 1_1 and for detection of DCI format 0_2 and DCI format 1_2, a DCI format 0_1 or a DCI format 0_2 can schedule a PUSCH transmission of any priority and a DCI format 1_1 or a DCI format 1_2 can schedule a PDSCH reception and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority.

If a UE reports the capability of [intra-UE prioritization], and if, after resolving overlapping for PUCCH and/or PUSCH transmissions of a same priority index, the UE determines to transmit

- a first PUCCH of larger priority index carrying SR, a second PUCCH of smaller priority index, and the transmission of the first PUCCH would overlap in time with the transmission of the second PUCCH, , the UE shall cancel the transmission of the second PUCCH starting from the first overlapping symbol at the latest.

- a first PUCCH of a larger priority index reporting HARQ-ACK information for a PDSCH scheduled by a PDCCH or a PUSCH of a larger priority index scheduled by PDCCH, a second PUCCH of a smaller priority index, and the transmission of the first PUCCH or PUSCH would overlap in time with the transmission of the second PUCCH, the UE shall cancel the transmission of the second PUCCH starting from M symbols after the end of the last symbol of the PDCCH indicating the first PUCCH transmission or the PUSCH transmission at the latest, where M = Tproc,2 +d1, where Tproc,2=N2.2048+144.κ2-μ.Tc,  and d1 is determined by the reported UE capability [XXXXX], and μ corresponds to the smallest SCS configuration among SCS configuration of the PDCCHs providing the DCI formats, and the SCS configuration of the first PUCCH or the PUSCH and the second PUCCH. If processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the DCI scheduling the first PUCCH or PUSCH, N2 is selected based on the UE PUSCH preparation procedure time according to Table 6.4-2 and SCS configuration μ; otherwise, N2 is selected based on the UE PUSCH preparation procedure time according to Table 6.4-1 and SCS configuration μ.

- a PUSCH corresponding to a configured grant of larger priority index, a PUCCH of smaller priority index, and a transmission of the PUSCH would overlap in time with a transmission of the PUCCH, the UE shall cancel the transmission of the PUCCH starting from the first overlapping symbol at the latest.

-

The cancellation of the PUSCH with a smaller priority index when overlapping with a PUCCH of a larger priority index is described in [6.1, TS 38.214].

The UE is not expected to be scheduled with a PUCCH of a larger priority carrying the HARQ-ACK information in response to PDSCH without a corresponding PDCCH overlapping with a PUCCH of a smaller priority index on the same serving cell.

The UE is not expected to be scheduled with a PUCCH of a larger priority index carrying the HARQ-ACK information for a PDSCH scheduled by a PDCCH overlapping with a PUCCH of a smaller priority index carrying the HARQ-ACK information for a PDSCH scheduled by a PDCCH on the same serving cell.

The UE is not expected to be scheduled by a PDCCH with another PUSCH/PUCCH with a priority index 0 or with SRS where the transmission of the PUSCH/PUCCH/SRS starts before the end of the time domain allocation of the cancelled PUCCH.

End

Modified clause (Section 6.2.1 of TS 38.214)

The UE may be configured by the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying  adjacent symbols within the last 6 symbols of the slot, where all antenna ports of the SRS resources are mapped to each symbol of the resource. When the SRS is configured with the higher layer parameter [SRS-for-positioning] the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying NS∈1,2,4,8,12 adjacent symbols anywhere within the slot.

If a UE is not configured with [intra-UE Prioritization] and PUSCH and SRS are transmitted in the same slot on a serving cell, the UE may only be configured to transmit SRS after the transmission of the PUSCH and the corresponding DM-RS.

If a UE is configured with [intra-UE Prioritization] and  the transmission of a PUSCH/PUCCH with a priority index 1  would overlap in time with an SRS transmission on a serving cell, the UE does not transmit SRS in the overlapping symbol(s).

End

# 3	Timeline Extension for High-Priority Channels

In RAN1 #99, it was agreed that the minimum processing time of the high priority channel can be extended by d2 symbols based on the UE capability if the high priority channel overlaps with a low priority channel according to the following agreement:

Agreement:

When a high-priority UL transmission overlaps with a low-priority UL transmission in a slot,

- The UE is expected to cancel the low-priority UL transmission starting from Tproc,2 +d1 after the end of PDCCH scheduling the high-priority transmission, where

  - Tproc,2 is correponding to UE processing time capability for the carrier.

  - Value d1 is the time duration corresponding to 0,1,2 symbols reported by UE capability

  - Note: d_2,1=0 is for cancellation

- The minimum processing time of the high priority channel is extended by d2 symbols

  - Value d2 is the time duration corresponding to 0,1,2 symbols reported by UE capability

The overlapping condition is per repetition of the uplink transmission

However, the timeline extension is not captured in the specification. Hence, we propose the following additions:

Modified clause (Section 5.3 of TS 38.214)

If the first uplink symbol of the PUCCH which carries the HARQ-ACK information, as defined by the assigned HARQ-ACK timing K1 and the PUCCH resource to be used and including the effect of the timing advance, starts no earlier than at symbol L1, where L1 is defined as the next uplink symbol with its CP starting after Tproc,1=(N1+d1,1+d2)(2048+144)⋅κ2-μ⋅TC after the end of the last symbol of the PDSCH carrying the TB being acknowledged, then the UE shall provide a valid HARQ-ACK message.

-	N1 is based on µ of table 5.3-1 and table 5.3-2 for UE processing capability 1 and 2 respectively, where µ corresponds to the one of (µPDCCH, µPDSCH, µUL) resulting with the largest Tproc,1, where the µPDCCH corresponds to the subcarrier spacing of the PDCCH scheduling the PDSCH, the µPDSCH corresponds to the subcarrier spacing of the scheduled PDSCH, and µUL corresponds to the subcarrier spacing of the uplink channel with which the HARQ-ACK is to be transmitted, and κ is defined in clause 4.1 of [4, TS 38.211].

-	If the PDSCH DM-RS position l1 for the additional DM-RS in Table 7.4.1.1.2-3 in clause 7.4.1.1.2 of [4, TS 38.211] is l1=12 then N1,0=14 in Table 5.3-1, otherwise N1,0=13.

-	If the UE is configured with multiple active component carriers, the first uplink symbol which carries the HARQ-ACK information further includes the effect of timing difference between the component carriers as given in [11, TS 38.133].

-	For the PDSCH mapping type A as given in clause 7.4.1.1 of [4, TS 38.211]: if the last symbol of PDSCH is on the i-th symbol of the slot where i < 7, then d1,1 = 7 - i, otherwise d1,1 = 0.

- If the UE reports the capability of [intra-UE prioritization], and if the PUCCH of a larger priority index is overlapping with PUCCH/PUSCH of a smaller priority index, d2 is determined by the reported UE capability [XXXXX].

End

Modified clause (Section 6.4 of TS 38.214)

If the first uplink symbol in the PUSCH allocation for a transport block, including the DM-RS, as defined by the slot offset K2 and the start and length indicator SLIV of the scheduling DCI and including the effect of the timing advance, is no earlier than at symbol L2, where L2 is defined as the next uplink symbol with its CP starting Tproc,2=max(N2+d2,1+d2)(2048+144)⋅κ2-μ⋅TC,d2,2after the end of the reception of the last symbol of the PDCCH carrying the DCI scheduling the PUSCH, then the UE shall transmit the transport block.

-	N2 is based on µ of Table 6.4-1 and Table 6.4-2 for UE processing capability 1 and 2 respectively, where µ corresponds to the one of (µDL, µUL) resulting with the largest Tproc,2, where the µDL corresponds to the subcarrier spacing of the downlink with which the PDCCH carrying the DCI scheduling the PUSCH was transmitted and µUL corresponds to the subcarrier spacing of the uplink channel with which the PUSCH is to be transmitted, and κ is defined in clause 4.1 of [4, TS 38.211].

-	If the first symbol of the PUSCH allocation consists of DM-RS only, then d2,1 = 0, otherwise d2,1 = 1.

-	If the UE is configured with multiple active component carriers, the first uplink symbol in the PUSCH allocation further includes the effect of timing difference between component carriers as given in [11, TS 38.133].

-	If the scheduling DCI triggered a switch of BWP, d2,2 equals to the switching time as defined in [11, TS 38.133], otherwise d2,2=0.

- If the UE reports the capability of [intra-UE prioritization], and if the PUSCH of a larger priority index is overlapping with a PUCCH of a smaller priority index, d2 is determined by the reported UE capability [XXXXX].

-	For a UE that supports capability 2 on a given cell, the processing time according to UE processing capability 2 is applied if the high layer parameter processingType2Enabled in PUSCH-ServingCellConfig is configured for the cell and set to enable,

-	If the PUSCH indicated by the DCI is overlapping with one or more PUCCH channels, then the transport block is multiplexed following the procedure in clause 9.2.5 of [6, TS 38.213], otherwise the transport block is transmitted on the PUSCH indicated by the DCI.

End

# 4	Multiplexing and Cancelation order

During the last RAN 1 meeting, the following agreement was reached:

Agreement:

- To resolve collision between UL transmissions, a UE performs the following:

  - Step 1: Resolve collision between UL transmissions with same priority.

  - Step 2: Resolve collision between UL transmissions with different priorities.

However, this agreement is not always consistent with the cancelation timeline agreement reviewed in Section 3. For example, consider the scenario illustrated in Fig. 1.

Fig. 1 Multiplexing vs cancellation order

According to this agreement, the UE would first resolve the collision between high priority channels, and then resolve the collision between the remaining high priority channel (in this case, it’s the PUSCH) and the low priority channel. However, according to the cancellation timeline agreement, the UE would need to prepare cancelling the low priority PUSCH starting from Tproc,2+d1 after receiving the first high priority PDCCH. In order to meet the cancellation timeline, the UE cannot wait to check whether the high priority PUCCH is to be cancelled by another high priority PDCCH scheduling a PUSCH transmission.

To solve the problem explained in Fig. 1, we propose to let the UE cancel the low priority channel. In other words, the UE first resolves the overlapping between the high priority PUCCH and the low priority channel, and then solve the overlapping between the high priority PUCCH and the high priority PUSCH.

Modified clause (Section 9 of TS 38.213)

If, after resolving overlapping for PUCCH and/or PUSCH transmissions of a same priority index, a UE determines to transmit

-	a first PUCCH of larger priority index, a PUSCH or a second PUCCH of smaller priority index, and a transmission of the first PUCCH would overlap in time with a transmission of the PUSCH or the second PUCCH, the UE does not transmit the PUSCH or the second PUCCH

-	a PUSCH of larger priority index, a PUCCH of smaller priority index, and a transmission of the PUSCH would overlap in time with a transmission of the PUCCH, the UE does not transmit the PUCCH

-	a first PUSCH of larger priority index on a serving cell, a second PUSCH of smaller priority index on the serving cell, and a transmission of the first PUSCH would overlap in time with a transmission of the second PUSCH, the UE does not transmit the second PUSCH, where at least one of the two PUSCH is not scheduled by a DCI format

If a UE determines to transmit

- a first PUCCH of larger priority index in response to a first PDCCH, the first PUCCH overlaps with a second PUCCH or a PUSCH of smaller priority index,

- a PUSCH of larger priority index scheduled by a second PDCCH after the first PDCCH, and the UE would multiplex the UCI of the first PUCCH on the PUSCH of larger priority index,

the UE cancels the second PUCCH or the PUSCH of smaller priority index as described above.

End

# 5	Collision handling amongst multiple HARQ-ACK PUCCHs and PUSCH

In NR Rel-16, sub-slot baesd HARQ-ACK feedback and CSI feedback is supported. In this case, it may be possible that multiple HARQ-ACK feedbacks or CSI feedbacks in respective sub-slots overlap with a PUSCH transmission in the slot of the subslots (see Fig. 1). In this case, the existing rule for piggybacking UCI on PUSCH in 38.213 does not work. To solve this issue, we propose to treat this as an error case.

Figure 1. Sub-slot based UCI transmissions of the same type overlapping with a slot-based PUSCH

Modified clause (Section 9.1.3 of 38.213)

A UE does not expect to multiplex in a PUSCH transmission in one slot with SCS configuration  UCI of same type that the UE would transmit in PUCCHs in different slots with SCS configuration  if .

If a UE is configured with subslotLength-ForPUCCH, the UE does not expect to multiplex in a PUSCH transmission in one slot UCI of the same type that the UE would transmit in PUCCHs in different sub-slots of the slot.

End

# 5	PUCCH resource selection for PRI < 3 bits

In previous RAN1 meetings, it was agreed that the size of the PUCCH resource indicator (PRI) in DCI format 1_2 can be configurable between 0 to 3 bits.

Agreements:

Support configurable size for “PUCCH resource indicator (0 or 1 or 2 or 3 bits)” for the new DCI format for DL scheduling.

• New RRC parameter is introduced to for the configuration

However, how to determine the PUCCH resource when the number of PUCCH resources in a PUCCH resource set is greater than 2NPRI has not been discussed in the meetings, where NPRI denotes the number of bits for the PRI. In our view, the same mechanism used to indicate the PUCCH resource from the first PUCCH resource set in NR Rel-15 can be reused. Namely, the UE may determine the PUCCH resource based on both the PRI and the index of the starting CCE of the PDCCH.

Modified clause (Section 9.1.3 of 38.213)

For a PUCCH transmission with HARQ-ACK information, a UE determines a PUCCH resource after determining a set of PUCCH resources for  HARQ-ACK information bits, as described in Clause 9.2.1. The PUCCH resource determination is based on a PUCCH resource indicator field [5, TS 38.212], if present, in a last DCI format, among the DCI formats that have a value of a PDSCH-to-HARQ_feedback timing indicator field, if present, or a value of dl-DataToUL-ACK, or a value of dl-DataToUL-ACKForDCIFormat1_2 for DCI format 1_2, indicating a same slot for the PUCCH transmission, that the UE detects and for which the UE transmits corresponding HARQ-ACK information in the PUCCH where, for PUCCH resource determination, detected DCI formats are first indexed in an ascending order across serving cells indexes for a same PDCCH monitoring occasion and are then indexed in an ascending order across PDCCH monitoring occasion indexes. For indexing DCI formats within a serving cell for a same PDCCH monitoring occasion, if the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs and is provided CORESETPoolIndex with value 1 for one or more second CORESETs on an active DL BWP of a serving cell, and with ACKNACKFeedbackMode = JointFeedback for the active UL BWP, detected DCI formats from PDCCH receptions in the first CORESETs are indexed prior to detected DCI formats from PDCCH receptions in the second CORESETs.

The PUCCH resource indicator field values map to values of a set of PUCCH resource indexes, as defined in Table 9.2.3-2 for a PUCCH resource indicator field of 3 bits, provided by resourceList for PUCCH resources from a set of PUCCH resources provided by PUCCH-ResourceSet with a maximum of eight PUCCH resources.

Denote by NPRI the number of bits for the PUCCH resource indicator and set nPRI=2NPRI. When the size  of resourceList is larger than nPRI, when a UE provides HARQ-ACK information in a PUCCH transmission in response to detecting a last DCI format in a PDCCH reception, among DCI formats with a value of the PDSCH-to-HARQ_feedback timing indicator field, if present, or a value of dl-DataToUL-ACK, or a value of dl-DataToUL-ACKForDCIFormat1_2 for DCI format 1_2, indicating a same slot for the PUCCH transmission, the UE determines a PUCCH resource with index , , as

rPUCCH=nCCE,p⋅⌈RPUCCH/nPRI⌉NCCE,p+ΔPRI⋅RPUCCHnPRI                                          if  ΔPRI<RPUCCH mod nPRInCCE,p⋅⌊RPUCCH/nPRI⌋NCCE,p+ΔPRI⋅RPUCCHnPRI+RPUCCH mod nPRI     if ΔPRI≥ RPUCCH mod nPRI

where  is a number of CCEs in CORESET  of the PDCCH reception for the DCI format as described in Clause 10.1,  is the index of a first CCE for the PDCCH reception, and  is a value of the PUCCH resource indicator field in the DCI format. When NPRI=0, then UE determines that ΔPRI=0.

Table 9.2.3-2: Mapping of PUCCH resource indication field values to a PUCCH resource in a PUCCH resource set with maximum 8 PUCCH resources for a PUCCH resource indicator field of 3 bits

| PUCCH resource indicator | PUCCH resource |
| --- | --- |
| 3 bits |  |
| '000' | 1st PUCCH resource provided by pucch-ResourceId obtained from the 1st value of resourceList |
| '001' | 2nd PUCCH resource provided by pucch-ResourceId obtained from the 2nd value of resourceList |
| '010' | 3rd PUCCH resource provided by pucch-ResourceId obtained from the 3rd value of resourceList |
| '011' | 4th PUCCH resource provided by pucch-ResourceId obtained from the 4th value of resourceList |
| '100' | 5th PUCCH resource provided by pucch-ResourceId obtained from the 5th value of resourceList |
| '101' | 6th PUCCH resource provided by pucch-ResourceId obtained from the 6th value of resourceList |
| '110' | 7th PUCCH resource provided by pucch-ResourceId obtained from the 7th value of resourceList |
| '111' | 8th PUCCH resource provided by pucch-ResourceId obtained from the 8th value of resourceList |

End

# 6	Type-2 HARQ-ACK codebook determination with different DAI bitwidths

In NR Rel-16, it was agreed that both DCI format 1_1 and 1_2 can be used to schedule high and low priority HARQ-ACK codebook. On the other hand, the size of the DAI in DCI format 1_2 may be different in DCI format 1_1. In this case, the pseudo code in Section 9.1.3 in 38.213 for Type 2 codebook determination may not work if the UE needs to multiplex HARQ-ACKs corresponding to DCI format 1_1 and 1_2 in the same codebook. Another issue for HARQ-ACK codebook determination is that, when the size of the counter DAI in DCI format 1_2 is 0 bit, the UE can not detect missing DCI(s) corresponding to DCI format 1_2. To solve the above issues, we propose the following TP.

Modified clause (Section 9.1.3 of 38.213)

Denote by NC-DAIDL the number of bits for the counter DAI and set TD=2NC-DAIDL. Denote by  the value of the counter DAI in a DCI format scheduling PDSCH reception or SPS PDSCH release on serving cell  in PDCCH monitoring occasion  according to Table 9.1.3-1 or Table 9.1.3-1A. Denote by  the value of the total DAI in a DCI format in PDCCH monitoring occasion  according to Table 9.1.3-1. The UE assumes a same value of total DAI in all DCI formats that include a total DAI field in PDCCH monitoring occasion .

A UE does not expect to multiplex in a HARQ-ACK codebook first HARQ-ACK information scheduled by DCI format 1_2 and second HARQ-ACK information bit scheduled by DCI format 1_0 or 1_1 if the bitwidth of the counter DAI  in DCI format 2_1 is different from that of the DCI format 1_0 or 1_1. If the bitwidth of counter DAI field in DCI format 1_2 is configured to be 0 bit, a UE does not expect to multiplex in a HARQ-ACK codebook the first HARQ-ACK information scheduled by a DCI format 1_2 and second HARQ-ACK information scheduled by DCI format 1_0 or 1_1 or 1_2.

End

# 7	CPU Release with Uplink Interruption

In the current specification, the occupied CPUs are assumed to remain occupied until the last symbol of PUCCH/PUSCH that carries the CSI report. However, in Rel. 16, and for a UE that supports the intra-UE prioritization capability or uplink cancellation indication, the channels that carry CSI may need to be cancelled. Hence, in order to not impose any additional burden on the UE for supporting these features, we propose to keep this behaviour unchanged even in cases when the uplink channels are interrupted. Hence, the following clarification is proposed to be added:

Modified clause (Section 5.2.1.6 of 38.214)

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', the CPU(s) are occupied for a number of OFDM symbols as follows:

-	A periodic or semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource for channel or interference measurement, respective latest CSI-RS/CSI-IM/SSB occasion no later than the corresponding CSI reference resource, until the last symbol of the PUSCH/PUCCH carrying the report.

-	An aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the PUSCH carrying the report.

-	An initial semi-persistent CSI report on PUSCH after the PDCCH trigger occupies CPU(s) from the first symbol after the PDCCH until the last symbol of the PUSCH carrying the report.

If a UE reports the capability of [intra-UE prioritization] and if the PUCCH/PUSCH carrying the report is overlapping with a PUCCH/PUSCH of a larger priority index in time, the occupied CPUs are assumed to remain occupied until the last symbol of PUCCH/PUSCH carrying the report before cancellation.

End