3GPP TSG RAN WG1 #100-e			R1-2000978

February 24th – March 6th, 2020

Agenda item:	7.2.8.3

Source: 	Qualcomm Incorporated

Title: 	Maintenance on UE & gNB measurements for NR Positioning

Document for:		Discussion and Decision

# Introduction

In the last meeting (RAN1#99), the following agreements were made with regards to measurements for positioning:

Agreement: Modify the previous agreement on the definition of the AoA (ϕ) and ZoA (θ) as follows: - AoA (ϕ) and ZoA (θ) define the estimated angles of a user with respect to a reference direction which are determined at the TRP antenna for an UL channel corresponding to this UE. - The reference directions can be defined according to either one of the following options (both options are supported in specifications) - Option 1: - The reference directions are defined with respect to the Global Coordinate System (GCS) as follows: - For AoA, the geographical North, positive in a counter-clockwise direction. - For ZoA, the zenith/vertical - Note: θ=0 pointing to the zenith and θ=90o pointing to the horizon. - Option 2: - For AoA and ZoA, tThe reference directions are defined is with respect to the Local Coordinate System (LCS) defined in TR 38.901 as follows: - For AoA, the x-axis, positive in a counter-clockwise direction. - For ZoA, the z-axis - Note: θ=0 pointing to the z-axis and θ=90o pointing to the x-y plane. - The translation of the GCS to LCS uses the set of angles α (bearing angle), β (downtilt angle), γ (slant angle) ,which are reported together with the AoA (ϕ) and ZoA (θ) in LCS. Agreement: UE can be configured for DL PRS processing according to the following table: Description; Maximum numbers for DL PRS resources; Values that can be signaled as part of UE Capability. Max number of frequency layers (X1); X1=4; Values = {1,4} FFS: other values. Max number of TRPs per frequency layer (X2); X2=64. Max number of PRS resource sets per TRP (X3) per frequency layer; X3=2; Values = {1,2}. Max number of Resources per PRS resource set (X4); X4=64; FFS: values. Max number of DL PRS Resources per UE (X5); NA; FFS: values. Max number of TRPs for all frequency layers (X6) per UE; 256; FFS: values. Max number of Resources per frequency layer (X7); NA; FFS: values. Note: This does not constrain in any way how features and feature sets are defined. The values in the table above may or may not be signalled to be different for different features or feature sets. Agreement: The maximum number of SRS resources for positioning across all SRS resource sets per BWP is 64. Agreement: The prior agreement is updated as given below: UE can be configured to measure and report up to [M] DL PRS RSTD measurements per pair of TRPs with each measurement between a different pair of DL PRS resources or DL PRS resource sets, and the M measurements being performed on the same pair of TRPs subject to UE capability - All the RSTD measurements in a single report should have a single reference timing - Note: Each RSTD measurement is between DL PRS Resources corresponding to different TRP IDs. - M=[3] 4 Agreement: The prior agreement is updated as given below: UE can be configured to measure and report up to N=8 (> 1) DL PRS RSRP measurements on different DL PRS resources from the same TRP - N=[3] FFS: N is a UE capability Agreement: The ID (previously agreed for TRP) of the reference DL PRS Resource or DL PRS Resource set is provided in DL-PRS-RSTD-ReferenceInfo. Agreement: - Exclude SS-RSRPB from the list of UE RRM measurement for E-CID - Include this agreement in an LS to other RAN 2/4 WGs Agreement: Modify the previous agreement on the definition of the time stamp as follows: A UE measurement can be associated with a time stamp. For UE RSTD, DL PRS RSRP and UE Rx-Tx time difference measurement report, the time stamp can include the SFN, as well as the slot number for a subcarrier spacing. These values correspond to the reference provided by the DL-PRS-RstdReferenceInfo. Agreement: When reporting a gNB measurement (UL RTOA measurements, gNB Rx-Tx time difference, SRS RSRP, or UL AoA/ZoA), gNB can be requested to report the ID of DL PRS resource and the ID of the DL PRS resource set, or SSB Index, which is used to identify the Rx beam that is used for determining the gNB measurements. Agreement: - The reporting granularity for the gNB angle measurements (AoA, ZOA) is defined as 0.1 degree. - RAN1 assumes that the details of the reporting granularity and ranges for the gNB angle measurements (AoA, ZoA) will be determined by RAN4. - The reporting granularity for the set of angles α (bearing angle), β (downtilt angle) and γ (slant angle) for the translation of the GCS to LCS is defined as 0.1 degree. - RAN1 assumes that the reporting granularity, the range of values and mapping for DL PRS RSRP and UL SRS RSRP measurements will be determined by RAN4 - Include above information in an LS to RAN2/RAN3/RAN4 Agreement: - The reporting granularity for the UE/gNB timing measurements (DL RSTD, the UE Rx-Tx time difference, UL RTOA, gNB Rx-Tx time difference) is defined as T=Tc2k, where k is a configuration parameter with a minimum value of at most 0. - Note: RAN4 can determine if -1 can be a minimum value - RAN1 assumes that the details of the reporting granularity and ranges for the UE/gNB timing measurements (DL RTSD, the UE Rx-Tx time difference, UL RTOA, gNB Rx-Tx time difference) will be determined by RAN4, including the potential relation of the parameter k to DL PRS bandwidth. - Include above information in an LS to RAN2/RAN3/RAN4 Agreement: The quality metrics for both DL and UL timing based measurements have a resolution of {0.1m, 1m, 10m, 30m} and are reported with a 5-bit value field Agreement: The quality metrics for angular measurements (AoA and ZoA) in either LCS or GCS have a resolution of {0.1} deg and are reported with an 8-bit value field. Agreement: Confirm the working assumption from RAN1#98bis on reporting of multiple Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning. The FFS item in the working assumption is removed. Agreement: A gNB measurement can be associated with a time stamp. For UL RTOA, UL SRS RSRP, AoA and gNB Rx-Tx time difference measurement report, the time stamp can include the SFN, as well as the slot number for a reported subcarrier spacing.

# Naming of RSTD in NR

The following texts are shown as titles of the Sections 5.1.13 and 5.1.29 in 38.215 with respect to the Reference Signal time difference (RSTD):

### 5.1.13 Reference signal time difference (RSTD) for E-UTRA Definition; The relative timing difference between the E-UTRA neighbour cell j and the E-UTRA reference cell i, defined as TSubframeRxj – TSubframeRxi, where: TSubframeRxj is the time when the UE receives the start of one subframe from E-UTRA cell j TSubframeRxi is the time when the UE receives the corresponding start of one subframe from E-UTRA cell i that is closest in time to the subframe received from E-UTRA cell j. The reference point for the observed subframe time difference shall be the antenna connector of the UE.. Applicable for; RRC_CONNECTED inter-RAT.

### 5.1.29 DL relative signal time difference (DL RSTD) Definition; DL relative timing difference (DL RSTD) between the positioning node j and the reference positioning node i, is defined as TSubframeRxj – TSubframeRxi, Where: TSubframeRxj is the time when the UE receives the start of one subframe from positioning node j. TSubframeRxi is the time when the UE receives the corresponding start of one subframe from positioning node i that is closest in time to the subframe received from positioning node j. Multiple DL PRS resources can be used to determine the start of one subframe from a positioning node. For frequency range 1, the reference point for the DL RSTD shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSTD shall be the antenna of the UE.. Applicable for; RRC_CONNECTED intra-frequency RRC_CONNECTED inter-frequency.

We observe that in LTE, RSTD is called Reference Signal Time difference, which is the correct name, whereas in NR, it is called Relative signal time difference. To avoid further confusion across RAN WGs and specifications, we propose to correct the latter to “Reference Signal Time Difference”.

Proposal 1: Change the title of the section 5.1.29 of 38.215 from “DL relative signal time difference (DL RSTD)” to “DL Reference Signal Time Difference (DL RSTD)”

# Multiple Measurements Reporting

The following working assumption was made in RAN1 #98 Bis:

Working assumption: A UE can be configured to report multiple Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL PRS resource/resource set. The DL PRS resource/resource sets can be in different positioning frequency layers - FFS: Reporting of SRS for positioning resource/resource set ID corresponding to a UE Rx-Tx time difference measurement - Note: This agreement does not introduce any new behavior for the transmission of SRS for positioning.

Then, in the endorsed high layer signaling list (shown in R1-1913674), the following parameter was added:

| NR_pos-Core | NR DL PRS Report Configuration | UE can be configured to report up to M = 4 UE-Rx-Tx time difference measurements for different DL PRS resources or DL PRS resource sets per TRP A UE can be configured to report multiple Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL PRS resource/resource set. The DL PRS resource/resource sets can be in different positioning frequency layers | {1,2,3,4} |
| --- | --- | --- | --- |

However, in 38.214, the number of the maximum number of Rx-Tx time differences is missing:

The UE can be configured in higher layer parameter UE Rx-Tx Time-MeasRequestInfo to report multiple UE Rx-Tx time difference measurements corresponding to a single configured SRS resource or resource set for positioning. Each measurement corresponds to a single received DL PRS resource or resource set which can be in difference positioning frequency layers.

Proposal 2: Update the text in Section 5.1.6.5 of 38.214 as follows:

The UE can be configured in higher layer parameter UE Rx-Tx Time-MeasRequestInfo to report, subject to UE capability, up to 4 UE Rx-Tx time difference measurements corresponding to a single configured SRS resource or resource set for positioning. Each measurement corresponds to a single received DL PRS resource or resource set which can be in difference positioning frequency layers.

# Conclusion

In this contribution, we make the following proposals:

Proposal 1: Change the title of the section 5.1.29 of 38.215 from “DL relative signal time difference (DL RSTD)” to “DL Reference Signal Time Difference (DL RSTD)”.

Proposal 2: Update the text in Section 5.1.6.5 of 38.214 as follows:

The UE can be configured in higher layer parameter UE Rx-Tx Time-MeasRequestInfo to report, subject to UE capability, up to 4 UE Rx-Tx time difference measurements corresponding to a single configured SRS resource or resource set for positioning. Each measurement corresponds to a single received DL PRS resource or resource set which can be in difference positioning frequency layers.

# References

[1] RP-190752, “New WID: NR Positioning Support”, Intel, Ericsson

[2] 36.214, “3GPP E-UTRA  Physical layer Measurements”

[3] 38.215, “3GPP NR Physical layer measurements”