3GPP TSG-RAN WG1 Meeting #100-e	R1-2001007

e-Meeting, February 24th- March 6th, 2020

Agenda Item:	7.2.4.1

Source:	Ericsson

Title:	Physical layer structure for NR sidelink

Document for:	Discussion, Decision

# 1	Introduction

This paper addresses remaining issues for physical layer structure of NR SL Rel-16 including the following listed in RP-193198.

| A1 | TBS determination |
| --- | --- |
| A2 | Initialization of the scrambling sequence generators for 2nd stage SCI and data transmitted on PSSCH, 1st stage SCI transmitted on PSCCH, and sidelink master information transmitted on PSBCH |
| A3 | PSSCH DMRS pattern in frequency domain (whether to support either or both Type 1 and Type 2, details on multiplexing of different ports for PSSCH DMRS) |
| A4 | Cyclic shift determination for PSFCH sequence |
| A5 | Details on 2nd stage SCI format(s) |
| A6 | Frequency domain OCC design for PSCCH DMRS |
| A7 | Remaining details of time and frequency resource indication in 1st SCI |
| A8 | Details of ECP |

# 2	Initialization of scrambling sequence generator

This section completes missing details of initialization of the scrambling sequence generators for different physical channels.

## 2.1	Scrambling of PSCCH

Since the PSCCH should be blindly decoded by all UEs in the pool, it is straightforward to fix the c_init for the scrambling sequence generator to a constant, like in LTE V2X. We propose to use a value larger than the maximum number of NR cells (which is 1008) for this constant, e.g., c_init=1010.

Use cinit= 1010 to initialize the scrambling sequence generator for PSCCH.

## 2.2	Scrambling of the 2nd-stage SCI and PSSCH

Agreements in RAN1 #98bis: The 2nd stage SCI is carried within the resource of the corresponding PSSCH. Scrambling operation for the 2nd stage SCI is applied separately with PSSCH.

In TS 38.211, the block of bits of each codeword q, q=0,1, of the PDSCH is scrambled by a scrambler which is initialized as follows:

The scrambling sequence generator shall be initialized with cinit=nRNTI⋅215+q⋅214+nID where - equals the higher-layer parameter dataScramblingIdentityPDSCH if configured and the RNTI equals the C-RNTI, MCS-C-RNTI, or CS-RNTI, and the transmission is not scheduled using DCI format 1_0 in a common search space, - nID=NIDcell otherwise and where corresponds to the RNTI associated with the PDSCH transmission as described in clause 5.1 of [6, TS 38.214].

In our view it is simplest to reuse the above procedure for the 2nd-stage SCI as well as the PSSCH. However, some modifications need to be made for it to work in the SL, as follows:

- nRNTI needs to be replaced. In our view it is most straightforward to replace it by the CRC of the associated PSCCH.

- nID can be set to a constant, e.g., 1010, as in Section 2.1.

- Finally, for simplicity, we propose that the same cinit is used for the 2nd-stage SCI and the PSSCH.

The same value cinit is used for initializing the scrambling sequence generator of the 2nd-stage SCI and the PSSCH. cinit=nX.27+q.214+1010 with nX equals the decimal representation of the associated PSCCH CRC.

## 2.3	Scrambling of the PSBCH

We propose to initialize the scrambling sequence generator for PSBCH with c_init equal to SLSS_ID, like in LTE V2X.

The scrambling sequence generator for the PSBCH is initialized with cinit=SLSS_ID.

# 3	Support of 256QAM and signalling of MCS tables

In RAN1 #98bis, the following agreements related to MCS tables and modulation order were made:

Agreements: - 256QAM is supported for SL. - Support of 256QAM by a UE is FFS between mandatory vs. based on UE capability from the Rx perspective - Support of 256QAM is based on UE capability from the Tx perspective - 64QAM is mandatory Agreements: - Three MCS tables supported in Rel-15 NR Uu CP-OFDM are also used for SL. - Support of the the low-spectral efficiency 64QAM MCS table is an optional UE feature in SL as in the Uu link - For each resource pool, at least one MCS table is (pre)-configured - FFS whether or not to introduce a case where the MCS table can be overwritten by PC5 RRC or indicated in SCI - Each resource pool is only configured with one 1st stage SCI PSCCH format.

On the open issue of whether supporting 256QAM is mandatory from the Rx UE perspective, in our opinion it should be mandatory. This is to avoid any mismatch between the modulation order (or the MCS table) that is (pre)configured for a pool and Rx UE capability in that pool. Without this, extra signalling is required for those Rx UE which cannot support 256QAM, which should be avoided since vehicle UEs can move across different geographical areas with different pool configurations.

Support of 256 QAM is mandatory from Rx UE perspective.

Regarding the remaining FFS on the signalling the MCS table(s), in our view the overwriting by PC5 RRC should not be allowed. The MCS table selected by the Tx UE is signalled in the second stage SCI.

Do not support MCS table to be overwritten by PC5 RRC.

The MCS table selected by the Tx UE is signaled in the 2nd-stage SCI.

# 4	TBS determination for the 2nd-stage SCI

Agreement: (from [99-NR-08]) - For determination of the number of coded bits, the determination of coded modulation symbols of HARQ-ACK with UL-SCH in Rel-15 NR is a baseline. - (Working assumption) The number of coded modulation symbols per layer for 2nd SCI is determined as follows. - QSCI2'=minOSCI2+LSCI2∙βoffsetSCI2∙l=0NsymbolPSSCH-1MscSCI2(l)r=0CSL-SCH-1Kr, αl=0NsymbolPSSCH-1MscSCI2(l)+γ - OSCI2 is the number of the 2nd SCI bits - LSCI2 is the number of CRC bits for 2nd SCI, which is 24 bits. - LSCI2 value is FFS to be resolved at RAN1 anniversary meeting, i.e. RAN1#100 - βoffsetSCI2 is indicated by the corresponding 1st SCI. - α is (pre-)configured per resource pool. - NsymbolPSSCH is the number of allocated symbols for the PSSCH except AGC symbol. - MscSCI2(l) is the number of REs that can be used for transmission of the 2nd SCI. - γ is determined to ensure that there is no remaining RE in the RB having the last coded symbol of the SCI 2 after mapping the SCI2. - Kr is the r-th code block size for SL-SCH of the PSSCH transmission. - CSL-SCH is the number of code blocks for SL-SCH of the PSSCH transmission. - The first symbol that can be used for 2nd SCI mapping is the first PSSCH DMRS symbol. - Note: This symbol is not the symbol for AGC training. However, since the first SL symbol in a slot is a copy of the second SL symbol in the slot, the first SL symbol may have REs for the 2nd SCI.

For the FFS above we propose to use 24 bits is used for the CRC of stage-2 SCI.

24 bits is used for the CRC of stage-2 SCI.

# 5	OCC design for PSCCH DMRS

Agreement: (from [99-NR-07]) - NR PDCCH DMRS sequence is the baseline for PSCCH DMRS sequence at least with the following modification. - n_ID is determined by a (pre-)configured value per resource pool - Frequency-domain OCC is applied, one of the [2 or 3 or 4] OCCs is randomly selected by the Tx UE. - Note: there is no (pre-)configuration on the number of OCCs.

In LTE V2X 4 cyclic shifts can be applied to a base sequence to obtain 4 orthogonal PSCCH DMRS sequences. This was instrumental in achieving good system performance. In our view, NR V2X should have at least the same performance as LTE V2X in this regard. Therefore, we propose to use 4 OCCs.

Frequency-domain OCC is applied, one of the 4 OCCs is randomly selected by the Tx UE.

# 6	Frequency-domain patterns for PSSCH DMRS

The following working assumption was reached in RAN1#97 on this aspect:

Working assumption: - Rel-15 PDSCH DMRS Configuration type 1 and/or type 2 are reused for frequency-domain pattern of PSSCH DMRS. - FFS whether to support either one or both types - FFS details on multiplexing of different ports for PSSCH DMRS

In our view, only a single type of DMRS pattern in the frequency domain should be supported. Specifically, we do not see the need of supporting both NR Rel-15 PDSCH/PUSCH DMRS type 1 and type 2. This is because the major use case of DMRS type 2 in Uu is to maximize the number of orthogonal DMRS ports so that MU-MIMO can be efficiently supported, but such a goal is not in relevant for NR SL. Additionally, by restricting to a single type, particularly type 1, we can not only reduce the signaling overhead but also minimize the DMRS alignment issue.

Support a single DMRS pattern in frequency domain for PSSCH, where the PDSCH DMRS Configuration type 1 (including multiplexing of DMRS ports) is reused.

# 7	Sequence generation for PSFCH

The sequence generation of the PUCCH format 0 can be reused for the PSFCH but some details of cyclic shift determination are missing.

Several agreements and working assumptions related to PSFCH were made in RAN1#99. The following working assumption is most relevant to the cyclic shift determination of the PSFCH.

Working assumption: - For the PSFCH candidate resource set with Z PRBs and Y cyclic shift pairs in each PRB, - Each PSFCH resource is indexed in the manner of frequency first and cyclic shift second. - FFS the order of cyclic shift indexing in a PRB. - PSFCH resource with the index ((K+M) mod (Z*Y)) is used for PSFCH transmission of a RX UE. - K is the L1 source ID of the associated PSCCH/PSSCH. - M is 0 for unicast and groupcast feedback option 1 and M is the member ID of the RX UE for groupcast feedback option 2. - FFS whether to have the following restriction. - Groupcast HARQ feedback option 2 is not used if X > Z*Y (Y denotes the number of PSFCH in a PRB). - Note: RAN1 assumes that the member ID M is an integer between 0 and X-1.

We are now ready to determine the cyclic shift for PSFCH. To start with, in 38.211, the cyclic shift α of PUCCH is determined as follows:

where - ns,fμ is the slot number in the radio frame - l is the OFDM symbol number in the PUCCH transmission where l=0 corresponds to the first OFDM symbol of the PUCCH transmission, - l' is the index of the OFDM symbol in the slot that corresponds to the first OFDM symbol of the PUCCH transmission in the slot given by [5, TS 38.213] - is given by [5, TS 38.213] for PUCCH format 0 and 1 while for PUCCH format 3 and 4 is defined in subclause 6.4.1.3.3.1 - except for PUCCH format 0 when it depends on the information to be transmitted according to subclause 9.2 of [5, TS 38.213]. The function is given by ncsns,fμ,l=m=072mc8Nsymbslotns,fμ+8l+m where the pseudo-random sequence is defined by subclause 5.2.1. The pseudo-random sequence generator shall be initialized with , where is given by the higher-layer parameter hoppingId if configured, otherwise nID=NIDcell.

In our view, the above procedure should be modified in the following ways for PSFCH:

- m0 is set to the value equal to the index of the cyclic shift pair which corresponds to the PSFCH resource index ((K+M+C) mod (Z*Y)). In particular, m0=K+M+CZmod Y   (suppose the indices starts from 0), where K, M, Z, Y are defined in the cited working assumption above and C is a parameter of the associated PSCCH (see our contribution [2] for detail).

- mcs is the same as in the case of one information bit of PUCCH format 0 [Table 9.2.3-3, TS 38.213].

- ncsns,fμ,l+l' should be omitted because the cyclic shift of PSFCH is fully determined from m_0 and m_cs above. Furthermore, the PSFCH includes two identical OFDM symbols, hence there should be no dependence on OFDM symbol index.

Formula for cyclic shift of NR PUCCH is used for determining cyclic shift of PSFCH with the following modifications:

  - m0=K+M+CZmod Y, where

    - Z is the number of RBs in the corresponding PSFCH resource set

    - Y is the number of cyclic shift pairs in the resource pool

    - K is the L1 source ID of the associated PSCCH/PSSCH

    - M is 0 for unicast and groupcast feedback option 1 and M is the member ID of the RX UE for groupcast feedback option 2.

    - C is a parameter of the associated PSCCH.

  - mcs is the same as in the case of one information bit of PUCCH format 0 [Table 9.2.3-3, TS 38.213]

  - ncsns,fμ,l+l' is omitted.

# 8	Remaining details of time and frequency resource indication in 1st SCI

RAN1#99 made the following agreements on the content of the 1st-stage SCI:

Agreements: In the 1st stage SCI, there are [2] reserved bits (for future compability) where a Rel-16 Tx UE shall set the bits to all zeros while a Rel-16 Rx UE does not make any assumption about these bits - To down-select: - Alt 1: Some additional explicit bits (e.g., [2-4] bits) in the 1st stage SCI to indicate part of L1 destination ID while the remaining is indicated in the 2nd stage SCI - This implies that the 16-bit destination ID is not carried in the 1st stage SCI - FFS the additional explicit bits can be further (pre)-configured - Alt 2: all 16-bit L1 destination ID is indicated by 2nd stage SCI - FFS whether or not the number of reserved bits can be further (pre)-configured Agreements: - 1st SCI includes at least - Priority (QoS value), - PSSCH resource assignment (frequency/time resource for PSSCH), - Resource reservation period (if enabled), - PSSCH DMRS pattern (if more than one patterns are (pre-)configured), - 2nd SCI format (e.g. information on the size of 2nd SCI), - [2]-bit information on amount of resources for 2nd SCI (e.g. beta offset or aggregation level) - Number of PSSCH DMRS port(s) - 5-bit MCS - FFS on some part of destination ID Agreement: (from [99-NR-06]) - For Rel-16 NR sidelink, only wideband precoding is assumed for PSSCH transmission. - Note: This implies that PRG size equal to scheduled PSSCH BW is assumed in Rel-16. - TX UE behavior for wideband precoding cycling is not specified - The number of reserved bits in the 1st stage SCI is configurable - [2-4] bits

Thus, a remaining detail to be resolved is the signalling of time and frequency resources for the 2nd-stage SCI.

When it comes to the code rates of the 2nd-stage SCI, we think that providing the flexibility of 2 different target code rates (which for a given payload size translate into varying resource allocation) is enough for Rel-16. Additionally, two code rates could be reserved for future use. This will result in the need of 2 bits in the 1st stage SCI. For simplicity, we propose to reuse target code rates from MCS index table 2 for PDSCH in TS 38.211, at least for Rel-16, with details given in our contribution [1].

Support four different target code rates for 2nd-stage SCI, with two code rates specified in Rel-16 (values are taken from MCS index table 2 for PDSCH.)

Furthermore, to allow for a future proof design, 2 bits can be used in the 1st stage SCI to indicate different payload sizes of the 2nd-stage SCI. For Rel-16, we propose specifying two different payload sizes; one for groupcast option 1 and one for unicast/groupcast option 2/broadcast. Additional sizes can be specified in future releases.

Support multiple formats/sizes of the 2nd stage SCI, with two formats specified in Rel-16: one for groupcast option 2 and one for unicast/groupcast option 2/broadcast.

Given the above analysis, we propose using 2 bits in the 1st-stage SCI for signalling resources for the 2nd-stage SCI, and 4 bits reserved for future use.

Rel-16 1st-SCI includes the following:

- 2-bit information on amount of resources for the 2nd-stage SCI.

- 4 reserved bits.

# 9	Details of 2nd-stage SCI format(s)

- Based on the agreements so far in RAN1 and the discussion on SCI formats in the preceding section, we summarize the two SCI formats for the 2nd-stage SCI in Rel-16 in Table 1 below.

Table 1: Content of the two formats of the 2nd-stage SCI

| SCI fields |  | Broad-cast | Group-cast | Uni- cast | Format 1 | Format 2 | Size |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Transport block related information | MCS table indicator | X | X | X | X | X | 2 bits |
|  | New data indicator | X | X | X | X | X | 1 bit |
|  | Redundancy version | X | X | X | X | X | 2 bits |
| HARQ-related information | Process number | X | X | X | X | X | 4 bits |
|  | Presence of HARQ feedback |  | X | X | X | X | 1 bit |
|  | Groupcast FB Option 1 or Option 2 |  | X |  | X | X | 1 bit |
|  | Zone ID |  | Option 1 only |  | X |  | Up to RAN2 |
|  | Communication range requirement |  | Option 1 only |  | X |  | 4 bits |
| CSI-related information | CSI-RS indication (implicitly CSI report request) |  |  | X | X | X | 1 bit |
| Layer-1 IDs | Source ID | X | X | X | X | X | 8 bits |
|  | Destination ID | X | X | X | X | X | 16 bits |

Use 2 formats for the 2nd-stage SCI as specified in Table 1.

# 10	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Use cinit= 1010 to initialize the scrambling sequence generator for PSCCH.

Proposal 2	The same value cinit is used for initializing the scrambling sequence generator of the 2nd-stage SCI and the PSSCH. cinit=nX.27+q.214+1010 with nX equals the decimal representation of the associated PSCCH CRC.

Proposal 3	The scrambling sequence generator for the PSBCH is initialized with cinit=SLSS_ID.

Proposal 4	Support of 256 QAM is mandatory from Rx UE perspective.

Proposal 5	Do not support MCS table to be overwritten by PC5 RRC.

Proposal 6	The MCS table selected by the Tx UE is signaled in the 2nd-stage SCI.

Proposal 7	24 bits is used for the CRC of stage-2 SCI.

Proposal 8	Frequency-domain OCC is applied, one of the 4 OCCs is randomly selected by the Tx UE.

Proposal 9	Support a single DMRS pattern in frequency domain for PSSCH, where the PDSCH DMRS Configuration type 1 (including multiplexing of DMRS ports) is reused.

Proposal 10	Formula for cyclic shift of NR PUCCH is used for determining cyclic shift of PSFCH with the following modifications:

a.	m0=K+M+CZmod Y, where

i.	Z is the number of RBs in the corresponding PSFCH resource set

ii.	Y is the number of cyclic shift pairs in the resource pool

iii.	K is the L1 source ID of the associated PSCCH/PSSCH

iv.	M is 0 for unicast and groupcast feedback option 1 and M is the member ID of the RX UE for groupcast feedback option 2.

v.	C is a parameter of the associated PSCCH.

b.	mcs is the same as in the case of one information bit of PUCCH format 0 [Table 9.2.3-3, TS 38.213]

c.	ncsns,fμ,l+l' is omitted.

Proposal 11	Support four different target code rates for 2nd-stage SCI, with two code rates specified in Rel-16 (values are taken from MCS index table 2 for PDSCH.)

Proposal 12	Support multiple formats/sizes of the 2nd stage SCI, with two formats specified in Rel-16: one for groupcast option 2 and one for unicast/groupcast option 2/broadcast.

Proposal 13	Rel-16 1st-SCI includes the following:

-	2-bit information on amount of resources for the 2nd-stage SCI.

-	4 reserved bits.

Based on the agreements so far in RAN1 and the discussion on SCI formats in the preceding section, we summarize the two SCI formats for the 2nd-stage SCI in Rel-16 in Table 1 below.

Proposal 14	Use 2 formats for the 2nd-stage SCI as specified in Table 1.

# References

R1-1912597, PHY layer structure for NR sidelink, Ericsson, RAN1#99, Nov. 2019

R1-2001012, PHY layer procedures for NR sidelink, Ericsson, RAN1#100-e, Feb. 2020.