3GPP TSG-RAN WG1 Meeting #100-e	R1- 2001015

e-Meeting, February 24th- March 6th, 2020

Agenda Item:	7.2.4.8

Source:	Ericsson

Title:	Remaining details on resource pool determination

Document for:	Discussion, Decision

# 1	Introduction

RAN1#98bis has made the following agreement:

Agreements: - A slot is the time-domain granularity for resource pool configuration. - To down-select: - Alt 1. Slots for a resource pool is (pre-)configured with bitmap, which is applied with periodicity - Alt 2. Slots for a resource pool is (pre-)configured, where the slots are applied with periodicity. - FFS: signaling details - FFS: how to apply the above bitmap signaling, e.g., to all slots or only to a set of slots - FFS: symbols for sidelink in the slot, how to indicate for the case when not all symbols are for SL

There remaining details are still left for discussion. This contribution presents our view on those details.

# 2	NR V2X resource pool determination

Regarding Alt.1 and Alt.2 above, in our view Alt.1 is the obvious choice because that is the procedure used in LTE V2X and there is no fundamental difference between LTE V2X and NR V2X in this regard.

Slots for a resource pool is (pre-)configured with bitmap, which is applied with periodicity.

As a result, the corresponding procedure for LTE V2X (Section 14.1.5 in TS 36.213) can be reused with some adjustments. In LTE, the following procedure is followed to determine the set of subframes that belong to PSSCH resource pool.

The set of subframes that may belong to a PSSCH resource pool for sidelink transmission mode 3 or 4 is denoted by t0SL,t1SL,...,tTmaxSL() where - 0≤tiSL<10240, - the subframe index is relative to subframe#0 of the radio frame corresponding to SFN 0 of the serving cell or DFN 0 (described in [11]), - the set includes all the subframes except the following subframes, - subframes in which SLSS resource is configured, - downlink subframes and special subframes if the sidelink transmission occurs in a TDD cell, - reserved subframes which are determined by the following steps: 1) the remaining subframes excluding Nslss and Ndssf subframes from the set of all the subframes are denoted by l0, l1,..., l(10240-Nslss-Ndssf-1) arranged in increasing order of subframe index, where Nslss is the number of subframes in which SLSS resource is configured within 10240 subframes and Ndssf is the number of downlink subframes and special subframes within 10240 subframes if the sidelink transmission occurs in a TDD cell. 2) a subframe lr0≤r<10240-Nslss-Ndssf belongs to the reserved subframes if r=m⋅10240-Nslss-NdssfNreserved where m=0, ..., Nreserved-1 and Nreserved=10240-Nslss-Ndssf mod Lbitmap. Here, Lbitmap the length of the bitmap is configured by higher layers. - the subframes are arranged in increasing order of subframe index. The UE determines the set of subframes assigned to a PSSCH resource pool as follows: - A bitmap b0,b1,...,bLbitmap associated with the resource pool is used where Lbitmap the length of the bitmap is configured by higher layers. - A subframe tkSL0≤k<10240-Nslss-Ndssf-Nreserved belongs to the subframe pool if bk'=1 where k'=kmodLbitmap .

Translating the above procedure to NR VX, the set of slots that may belong to the PSSCH/PSCCH resource pool include all the slots except the following:

- Slots in which SLSS resource is configured.

- Slots containing downlink symbols and X symbols (in other words, UL only slots are included in the set). Note that this aligns with RAN1#99 agreement on supporting SL transmissions in cell-specific UL resources in Uu.

- The set of reserved slots is determined in the same way as in LTE V2X.

Reuse the procedure for resource pool determination of LTE V2X (i.e., Section 14.1.5 in TS 36.213) for NR V2X. The set of slots used PSCCH/PSSCH resource pool includes all the slots except:

- Slots configured for SLSS resource

- Slots containing downlink or X symbols

- Reserved slots (determined by using a procedure similar to that of LTE V2X.)

# 3	Conclusion

Based on the discussion in the previous section we propose the following:

Proposal 1	Slots for a resource pool is (pre-)configured with bitmap, which is applied with periodicity.

Proposal 2	Reuse the procedure for resource pool determination of LTE V2X (i.e., Section 14.1.5 in TS 36.213) for NR V2X. The set of slots used PSCCH/PSSCH resource pool includes all the slots except:

-	Slots configured for SLSS resource

-	Slots containing downlink or X symbols

-	Reserved slots (determined by using a procedure similar to that of LTE V2X.)