3GPP TSG RAN WG1 Meeting #100-e	R1-2001041

February 24 - March 6, 2020

Agenda Item:	7.2.6.4

Source:	Huawei, HiSilicon

Title:	Remaining issues on UL full power transmission in R16

Document for:	Discussion and Decision

Introduction

Based on the progress of full power transmission in R16, there is still one open issue to be decided [1].

Agreement

For 4 ports, number of bits to indicate TPMI(s) which can deliver UL full power:

- Non Coherent 2 bits

- Partial coherent 4 bits

  - Additional entries on top of existing entries may be added to table 1 and table 2

- Whether is this capability reporting is optional or not will be discussed as part of UE capability discussions

Table 1.

| 4Tx, nonCoherent | 4Tx, partial coherent (4bit) |
| --- | --- |
| G0 | G0 |
| G1 | G1 |
| G2 | G2 |
| G3 | G3 |
|  | G4 |
|  | G5 |
|  | G6 |

Definition of G0~G6 can be found in the table below.

Table 2.

In this contribution, we discuss this remaining issue for the full power transmission for partial-coherent UE.

TPMI reporting for Partial coherent UE

In last meeting, detailed solution on TPMI reporting for full power transmission has been agreed. As can be seen in the agreed Table 1 and Table 2 in the introduction, one or multiple TPMIs can be reported by UE for full power transmission according to the UE antenna implementation. In Table 3, we summary the UE antenna structure assumption for each full power TPMI group as we agreed.

Table 3. Potential UE antenna architecture related to Table 2

| TPMI Groups | Potential UE antenna architecture |
| --- | --- |
| G0 | 23dBm+17dBm+17dBm+17dBm |
| G1 | 23dBm+17dBm+23dBm+17dBm |
| G2 | 23dBm+23dBm+23dBm+17dBm |
| G3 | 20dBm+20dBm+20dBm+17dBm |
| G4 | 20dBm+17dBm+20dBm+17dBm |
| G5 | 20dBm+20dBm+20dBm+17dBm |
| G6 | 20dBm+20dBm+20dBm+20dBm |

It is worth noting that the possible UE antenna architectures for each TPMI group is the at least antenna capability, if the antenna capability is higher than it, the TPMI group still can be used. For example, for a UE with 23+20+20+20 dBm, G0 still can be used.

For partial-coherent UE, some important cases are still missing. For the first case, UE is equipped with 23+17+23+17 dBm (or 23+20+23+20 dBm), where  121000, 120010,1210000010 can be used as shown in G1. However, 1210000010 is equivalent to 121010,1210-10,1210j0,1210-j0 for full power transmission for partial coherent UEs. So, the TPMI groups should include both 121000, 120010,1210000010 and 121010,1210-10,1210j0,1210-j0, where the case is missed in the current agreement.

For the second case, UE is equipped with 23+23+23+17dBm. In the current table, TPMIs in G2 are used. Actually, 121010,1210-10,1210j0,1210-j0 are also available in the case for partial coherent UE. So, the case of TPMI group with G2+121010,1210-10,1210j0,1210-j0 is missed in current agreement.

To include these important cases, we propose to add two entries into TPMI groups for the missing cases shown in Table 4 and Table 5.

Table 4. Revised Table 1

| 4Tx, nonCoherent | 4Tx, partial coherent (4bit) |
| --- | --- |
| G0 | G0 |
| G1 | G1 |
| G2 | G2 |
| G3 | G3 |
|  | G4 |
|  | G5 |
|  | G6 |
|  | G7 |
|  | G8 |

Table 5. Revised Table 2

| TPMI Groups |  |
| --- | --- |
| G0 | 121000 |
| G1 | 121000, 120010,1210000010 |
| G2 | 121000,120100,120010,1210000010,1210010000,1200100010,12100010000010 |
| G3 | 1210000010,1210010000,1200100010,12100010000010 |
| G4 | 121010,1210-10,1210j0,1210-j0,1210000010 |
| G5 | 121010,1210-10,1210j0,1210-j0,1210000010,1210010000,1200100010,12100010000010 |
| G6 | 121010,1210-10,1210j0,1210-j0,120101,12010-1,12010j,12010-j,1210000010,1210010000,1200100010,1210000001,1200100001,1200001001,12100010000010 |
| G7 | 121000, 120010,121010,1210-10,1210j0,1210-j0,1210000010 |
| G8 | 121000,120100,120010,121010,1210-10,1210j0,1210-j0,1210000010,1210010000,1200100010,12100010000010 |

Proposal 1: Add two more entries in TPMI groups for full power transmission for the partial coherent UEs.

The related text proposals are provided in appendix.

Conclusions

Based on the discussion, we have the following proposal:

Proposal 1: Add two more entries in TPMI groups for full power transmission for the partial coherent UEs.

- References

3GPP, “RAN1#99 Meeting Chairman’s Notes”, Reno, USA, November 18th – 22nd, 2019.

- Appendix

Text Proposal for Section 4.2.7.2 of 38.306

--------------------------------------------------------------------------------------------------------------------------

< Unchanged parts are omitted >

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| pusch-fullPowerTPMI Defines support of the full power TPMI for PUSCH transmission as described in clause 7.1 of TS 38.213. If UE indicates support of non-coherent codebook subset, 4 values can be indicated. - Value 1 indicates 121000; - Value 2 indicates 121000, 120010,1210000010; - Value 3 indicates 121000,120100,120010,1210000010,1210010000,1200100010,12100010000010; - Value 4 indicates 1210000010,1210010000,1200100010,12100010000010 If UE indicates support of partial-coherent codebook subset, 9 values can be indicated. - Value 1 indicates 121000; - Value 2 indicates 121000, 120010,1210000010; - Value 3 indicates 121000,120100,120010,1210000010,1210010000,1200100010,12100010000010; - Value 4 indicates 1210000010,1210010000,1200100010,12100010000010; - Value 5 indicates 121010,1210-10,1210j0,1210-j0,1210000010; - Value 6 indicates 121010,1210-10,1210j0,1210-j0,1210000010,1210010000,1200100010,12100010000010; - Value 7 indicates 121010,1210-10,1210j0,1210-j0,120101,12010-1,12010j,12010-j,1210000010,1210010000,1200100010,1210000001,1200100001,1200001001,12100010000010; - Value 8 indicates 121000, 120010,121010,1210-10,1210j0,1210-j0,1210000010; - Value 9 indicates 121000,120100,120010,121010,1210-10,1210j0,1210-j0,1210000010,1210010000,1200100010,12100010000010 | Band | No | No | No |

< Unchanged parts are omitted >

--------------------------------------------------------------------------------------------------------------------------