3GPP TSG RAN WG1 Meeting #100-e	R1-2001179

February 24 – March 6, 2020

Agenda Item:	6.2.3.1

Source:	Huawei

Title:	Feature summary #2 on LTE DL MIMO efficiency enhancement

Document for:	Discussion and Decision

Introduction

Agreements and conclusions in previous meeting for the LTE DL MIMO efficiency enhancements (WI code LTE_DL_MIMO_EE-Core; WID in RP-182901) are summarized in [1].

In this paper, the inputs from companies submitted to RAN1#100-e meeting ([2]-[9]) on remaining issues are summarized.

Discussion

  - Transmission pattern

### Issue 1: Capturing of the number of antenna switching.

- As explained in [2], the following agreement is not correctly captured, especially for additional SRS without frequency hopping.

Agreement #1

The number of antenna switching is:

•	2 for 1T2R is configured or the number of pairs is configured as 2 for 2T4R

•	3 if the number of pairs is configured as 3 for 2T4R

•	4 for 1T4R

- Proposal: Endorse the following text proposal to TS 36.213

Text proposal to 36.213 <Unchanged parts are omitted> For trigger type 2 SRS transmission, when closed-loop or open-loop UE transmit antenna selection is enabled for a given serving cell for a UE that supports antenna selection, or for UE that can be configured with ue-TxAntennaSelection-SRS-1T4R-Config or ue-TxAntennaSelection-SRS-2T4R-NrOfPairs - when higher layer parameter ‘ue-TXAntennaSelection-SRS-1T4R-Config’ is configured for a given serving cell, the number of antenna switches NAS = 4, and the index anSRS of the UE antenna that transmits the SRS at time nSRS is given by anSRS=nSRSnSRS mod 4 when frequency hopping is disabled, and anSRS=nSRSNFHnSRSNFH mod 4 when frequency hopping is enabled. UE is not expected to be configured with anSRS>4. - when higher layer parameter 'ue-TXAntennaSelection-SRS-2T4R-Config’ is configured for a given serving cell for a UE configured with Λ UE antenna pairs, where Λ={2 or 3} is given by higher layer parameter ‘ue-TXAntennaSelection-SRS-2T4R-Config’, the number of antenna switches NAS = 3, and the index for the UE antenna pair is 2anSRS, 2anSRS+1 when Λ =2, or 0, anSRS+1 when Λ =3 that transmits the SRS at time nSRS, where anSRS is given by anSRS=nSRSnSRS mod Λ when frequency hopping is disabled, and anSRS=nSRSNFHnSRSNFH mod Λ when frequency hopping is enabled. UE is not expected to be configured with anSRS>Λ. - otherwise, the number of antenna switches NAS = 2, and the index of the UE antenna that transmits the SRS at time nSRS is given by anSRS=nSRSnSRS mod 2 when frequency hopping is disabled, and anSRS=nSRSNFHnSRSNFH mod 2 when frequency hopping is enabled. UE is not expected to be configured with anSRS>2. where NFH is the number of frequency hops defined in Subclause 5.5.3.2.2 of [3]. <Unchanged parts are omitted>

### Issue 2: The order of repetition, antenna switching and frequency hopping for additional SRS.

- As explained in [4], the repetition should always be performed before frequency hopping and/or antenna switching since it never requires a guard symbol between repeated symbols. The order of antenna switching/frequency hopping depends on the guard period.

- Proposal:

Proposal 1: Repetition is always performed before any frequency hopping and/or antenna switching in all cases. Proposal 2: In order to reduce the additional SRS overhead including any guard symbols, frequency hopping should be performed before antenna switching, except when guard symbol is required for frequency hopping but not antenna switching, in which case frequency hopping is performed after antenna switching.

### Issue 3: guard symbol for antenna switching.

- As explained in [5], guard symbol for antenna switching is not captured in spec.

- As explained in [7], it needs to clarified that the UE is not expected to send any uplink signal in guard symbol.

- Proposal: Endorse the following text proposal to TS 36.211

Text proposal to 36.211 ---- Unchanged parts are omitted ---- ##### 5.5.3.2.2 Mapping to physical resources for additional SRS An additional SRS spans one or more OFDM symbols in the time domain, where - the starting OFDM symbol l0 within the subframe is given by the higher-layer parameter additionalSRS-startPos; - the duration N in number of OFDM symbols, including potential guard symbols, is given by the higher-layer parameter additionalSRS-duration; Mapping to physical resources shall be done according to clause 5.5.3.2.1 with the following exceptions: - frequency hopping between OFDM symbols is supported and if a UE is configured by higher layer parameter additionalSRS-GuardSymbolFH, a guard symbol is added between every frequency hop; - antenna switching within a subframe is supported and if a UE is configured by higher layer parameter additionalSRS-GuardSymbolAS, a guard symbol is added between every antenna switching; - nSRS=lR where l is the index of the OFDM symbol number carrying additional SRS within the subframe not counting guard symbol(s), and R∈1,2,3,4,6,7,8,9,12,13 is the repetition factor given by the higher-layer parameter additionalSRS-RepNum; - BSRS is given by the higher-layer parameter additionalSRS-Bandwidth; - bhop is given by the higher-layer parameter additionalSRS-HoppingBandwidth; - NFH is the number of frequency hops for additional SRS, derived from N=RNFH+NFH-1GFH if antenna switching is not configured for additional SRS, and from N=RNASNFH+NAS-1GAS+NFH-1GFH if antenna switching is configured for additional SRS, where R is the repetition factor given by the higher-layer parameter additionalSRS-RepNum, NAS is the number of antenna switches for additional SRS defined in 8.2 of [4], GAS∈0, 1 is the guard-symbol configuration for antenna switching given by the higher-layer parameter additionalSRS-GuardSymbolAS, GFH∈0, 1 is the guard symbol configuration for frequency hopping given by the higher-layer parameter additionalSRS-GuardSymbolFH, and N is given by the higher-layer parameter additionalSRS-duration; ---- Unchanged parts are omitted ----

### Issue 4: Error in the equation to derive the number of frequency hopping.

- As explained in [4], the current equation doesn’t work for the case that guard symbols for both frequency hopping and antenna switching are configured, then the following is proposed:

Replace the agreed equation

N=R*NAS*NFH+(NAS*GSAS-1)*GSAS+(NFH*GSFH-1)*GSFH

with

N=R*NAS*NFH +(NAS -1) * GSAS*( 1-GSFH)+ ( NFH -1) * GSFH*( 1-GSAS)+ ( NAS*NFH-1) * GSAS* GSFH

- As explained in [6], NFH should be “derived from N=RNASNFH+NAS-1GAS+NFH-1GFHNAS”, where NFH-1GFHNAS is used to correctly count the total number of frequency hops on every antenna/antenna pair if GFH=1  is configured.

- As explained in [7], there is an error in current equation as the number of frequency hopping for every antenna/antenna pair should all be counted, then it is proposed to endorse following text proposal

Text proposal to 36.211 ##### 5.5.3.2.2 Mapping to physical resources for additional SRS An additional SRS spans one or more OFDM symbols in the time domain, where - … - NFH is the number of frequency hops for additional SRS, derived from N=RNFH+NFH-1GFH if antenna switching is not configured for additional SRS, and from N=RNASNFH+NAS-1GAS+NFH-1NASGFH if antenna switching is configured for additional SRS, where R is the repetition factor given by the higher-layer parameter additionalSRS-RepNum, NAS is the number of antenna switches for additional SRS defined in 8.2 of [4], GAS∈0, 1 is the guard-symbol configuration for antenna switching given by the higher-layer parameter additionalSRS-GuardSymbolAS, GFH∈0, 1 is the guard symbol configuration for frequency hopping given by the higher-layer parameter additionalSRS-GuardSymbolFH, and N is given by the higher-layer parameter additionalSRS-duration;

### Issue 5: Subbands for frequency hopping.

- As explained in [4], the subband bandwidth for frequency hopping can be derived from the number of frequency hopping:

If the subband bandwidth per hop BWhopis not configured, UE derives it from NFH as 
BWhop=BWSRSNFH

- As explained in [6], it is proposed that the same set of subbands is used per antenna/antenna pair , and it is proposed to endorse the following text proposal to TS 36.211

Text proposal to 36.211 ---------------------------------------------------------------- Text Omitted ------------------------------------------------------------- ##### 5.5.3.2.2 Mapping to physical resources for additional SRS An additional SRS spans one or more OFDM symbols in the time domain, where - the starting OFDM symbol l0 within the subframe is given by the higher-layer parameter additionalSRS-startPos; - the duration N in number of OFDM symbols, including potential guard symbols, is given by the higher-layer parameter additionalSRS-duration; Mapping to physical resources shall be done according to clause 5.5.3.2.1 with the following exceptions: - frequency hopping between OFDM symbols is supported and if a UE is configured by higher layer parameter additionalSRS-GuardSymbolFH, a guard symbol is added between every frequency hop; - nSRS=lR where l is the index of the OFDM symbol number carrying additional SRS within the subframe not counting guard symbol(s), and R∈1,2,3,4,6,7,8,9,12,13 is the repetition factor given by the higher-layer parameter additionalSRS-RepNum; - BSRS is given by the higher-layer parameter additionalSRS-Bandwidth; - bhop is given by the higher-layer parameter additionalSRS-HoppingBandwidth; - NFH is the number of frequency hops for additional SRS with a same antenna/antenna pair, derived from N=RNFH+NFH-1GFH if antenna switching is not configured for additional SRS, and from N=RNASNFH+NAS-1GAS+NFH-1GFHNAS if antenna switching is configured for additional SRS, where R is the repetition factor given by the higher-layer parameter additionalSRS-RepNum, NAS is the number of antenna switches for additional SRS defined in 8.2 of [4], GAS∈0, 1 is the guard-symbol configuration for antenna switching given by the higher-layer parameter additionalSRS-GuardSymbolAS, GFH∈0, 1 is the guard symbol configuration for frequency hopping given by the higher-layer parameter additionalSRS-GuardSymbolFH, and N is given by the higher-layer parameter additionalSRS-duration; - If SRS antenna switching and frequency hopping are both enabled, the same set of subbands is used per antenna/antenna pair with subband index order reversed when switching the antenna/antenna pair. - nRRC is given by the higher-layer parameter freqDomainPosition-additionalSRS; - Nap is given by the higher-layer parameter additionalSRS-AntennaPort; - nSRScs is given by the higher-layer parameter additionalSRS-cyclicShift; - KTC is given by the higher-layer parameter additionalSRS-transmissionCombNum; - kTC is given by the higher-layer parameter additionalSRS-transmissionComb; ---------------------------------------------------------------- Text End -------------------------------------------------------------------

### Issue 6: SRS symbol index.

- As explained in [7], the indexing of SRS symbols follows the index of OFDM symbol l in uplink subframe not accounting the guard symbol, which has drawback of partial SRS repetition when frequency hopping and/or antenna selection is used.

- Proposal: modify the description of the parameter l  as the SRS transmission occasion index within the subframe and endorse the text proposal to 36.211

Text proposal to 36.211 Mapping to physical resources shall be done according to clause 5.5.3.2.1 with the following exceptions: - frequency hopping between OFDM symbols is supported and if a UE is configured by higher layer parameter additionalSRS-GuardSymbolFH, a guard symbol is added between every frequency hop; - nSRS=l'R where l' is the index of the OFDM symbol number carrying additional SRS transmission occasion within the subframe not counting guard symbol(s), and R∈1,2,3,4,6,7,8,9,12,13 is the repetition factor given by the higher-layer parameter additionalSRS-RepNum;

### Issue 7: Clarification for frequency hopping for aperiodic SRS.

- As explained in [7], frequency hopping should also be supported for aperiodic additional SRS symbols.

- Proposal: Endorse the text proposal to 36.211

Text proposal to 36.211 The frequency hopping of the sounding reference signal is configured by the parameter , provided by higher-layer parameter srs-HoppingBandwidth. Frequency hopping is not supported for aperiodic transmission, except for additional SRS. If frequency hopping of the sounding reference signal is not enabled (i.e., ), the frequency position index remains constant (unless re-configured) and is defined by where the parameter is given by higher-layer parameters freqDomainPosition and freqDomainPosition-ap for periodic and each configuration of aperiodic transmission, respectively. If frequency hopping of the sounding reference signal is enabled (i.e., ), the frequency position indexes are defined by …

  - Additional SRS and legacy SRS

### Issue 8: transmission in case of periodic legacy and additional SRS in one subframe.

- As explained in [3], RAN1 has agreed that transmission of legacy periodic/aperiodic SRS and additional SRS in the same subframe is supported. Because of this, a statement that trigger type 0 and trigger type 2 can be transmitted in the same subframe, should be added.

- Proposal: Endorse the following text proposal to TS 36.213

## 8.2 UE sounding procedure If the UE is configured with a PUCCH-SCell, the UE shall apply the procedures described in this clause for both primary PUCCH group and secondary PUCCH group unless stated otherwise - When the procedures are applied for the primary PUCCH group, the terms 'secondary cell', 'secondary cells', 'serving cell', and 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell or serving cells belonging to the primary PUCCH group respectively unless stated otherwise. - When the procedures are applied for secondary PUCCH group, the terms 'secondary cell', 'secondary cells', 'serving cell' and 'serving cells' in this clause refer to secondary cell, secondary cells (not including the PUCCH-SCell), serving cell, serving cells belonging to the secondary PUCCH group respectively unless stated otherwise. The term 'primary cell' in this clause refers to the PUCCH-SCell of the secondary PUCCH group. A UE shall transmit Sounding Reference Symbol (SRS) on per serving cell SRS resources based on three trigger types: - trigger type 0: higher layer signalling - trigger type 1: DCI formats 0/0A/0B/4/4A/4B/1A/6-0A/6-1A for FDD, TDD, and frame structure type 3 and DCI formats 2B/2C/2D/3B for TDD, and frame structure type 3, and DCI format 7-0A/7-0B/7-1E/7-1F/7-1G for TDD if the UE is configured by higher layers for SRS triggering via DCI format 7-0A and has indicated the capability srs-DCI7-Triggering-FS2-r15/ srs-DCI7-Triggering-FS2-r16 and the UE is configured for SRS triggering with srs-DCI7-TriggeringConfig-r15/ srs-DCI7-Triggering-FS2-r16. - trigger type 2: DCI formats 0/4/1A/6-0A/6-1A for FDD and TDD, and DCI formats 2B/2C/2D/3B for TDD, and DCI format 7-0A/7-0B/7-1E/7-1F/7-1G for TDD if the UE is configured by higher layers for SRS triggering via DCI format 7-0A and has indicated the capability srs-DCI7-Triggering-FS2-r16 and the UE is configured for SRS triggering with srs-DCI7-TriggeringConfig-r16. A UE is not expected to be configured with SRS trigger type 0 on a LAA SCell. In case both trigger type 0 and trigger type 1 SRS transmissions would occur in the same subframe in the same serving cell, the UE shall only transmit the trigger type 1 SRS transmission. This prioritization rule shall be applied before other prioritization rules defined in this subclause. In case both trigger type 1 and trigger type 2 SRS transmissions would occur in the same subframe, the UE shall transmit both the trigger type1 and type 2 SRS transmissions. In case both trigger type 0 and trigger type 2 SRS transmissions would occur in the same subframe, the UE shall transmit both the trigger type 0 and type 2 SRS transmissions. If higher layer parameter specialSubframePatterns-v1430 indicates ssp10, or if higher layer parameter specialSubframePatterns-v1450 indicates ssp10-CRS-LessDwPTS, the UE shall assume for the purpose of determining kSRS that the special subframe configuration is that signalled by specialSubframePatterns (without suffix).

  - Collision handling

### Issue 9: Clarification of collision between additional SRS and PUSCH/PUCCH.

- As explained in [8], UEs not capable of sPUSCH/sPUCCH, are not expected to be triggered with additional SRS on subframes in which PUCCH/PUSCH are to be transmitted. The current spec may be misleading.

- Proposal: Endorse the following text proposal to TS 36.213

Text proposal to 36.213 -------------------------- Start of proposed TP2 for TS 36.213 ------------------------------------ ## 8.2 UE sounding procedure ----------------------------------------------- Unchanged text omitted ------------------------------------------- A UE shall not transmit SRS whenever SRS and a PUSCH transmission corresponding to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure coincide in the same subframe. A UE not configured with higher layer parameter ul-STTI-Length is not expected to be triggered with trigger type 2 SRS transmission in the same subframes symbols as a PUSCH/PUCCH. -------------------------------------------End of proposed TP 2 ----------------------------------------------------

### Issue 10: Collision between additional SRS and short PUSCH/PUCCH.

- As explained in [9], as short PUSCH/PUCCH are transmitted in a slot of subframe, there may still be collision with additional SRS as additional SRS may requires more symbols than a slot, and this case needs to be discussed.

- Proposal: Endorse the following text proposal to TS 36.213

Text proposal to 36.213 =========================Text proposal to TS 36.213============================= 8.2 UE sounding procedure <unchanged parts are omitted> The following prioritization rules shall be applied in case of collision between a transmission of SRS over serving cell d and transmission of a physical signal/channel over a serving cell in set S(d):: - If PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/PTI/CRI/wideband PMI only (PUCCH reporting type 2a in Subclause 7.2.2) and/or PRACH on a serving cell in set S(d) overlaps in the same symbol with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) on serving cell d, then the UE shall not transmit SRS. Otherwise, - if PUSCH transmission carrying aperiodic CSI on a serving cell in set S(d) overlaps in the same symbol with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) in serving cell d, and if the SRS transmission is a type 0 SRS transmission, then the UE shall not transmit the type 0 SRS. Otherwise, - if PUSCH transmission on a serving cell in set S(d) overlaps in more than one symbol with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) in serving cell d, then the UE shall drop the PUSCH transmission. If PUCCH/SRS transmission on a serving cell in set S(d) overlaps in the same symbol with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) on serving cell d, the UE shall drop the PUCCH/SRS transmission. In case an SRS transmission in subframe N on serving cell d is dropped due to a collision with a higher priority transmission (as defined above) in subframe N+1, and there is a lower priority transmission (as defined above) in subframe N that would have been dropped had the transmission in subframe N+1 not occurred, the UE is not required to transmit the lower priority transmission in subframe N. An additional SRS symbol configured by higher layer parameter additionalSRS-Config shall be dropped if it collides with a slot/subslot-PUSCH. =========================End of text proposal to TS 36.213=========================

  - Power control

### Issue 11: Capturing of close-loop power control for additional SRS symbols.

- As explained in [3], in the spec, the close loop power control only applies to additional SRS symbols for PUSCH-less carriers. In addition, the close-loop power control of trigger type 2 should be independent of trigger type 0/1.

- Proposal: Endorse the following text proposal to TS 36.213

-------------------------- <Unchanged parts are omitted> ---------------------------------------- ### 5.1.3 Sounding Reference Symbol (SRS) #### 5.1.3.1 UE behaviour The setting of the UE Transmit power for the SRS transmitted on subframe i for serving cell is defined by: for SRS transmission given trigger type 2 or for serving cell with frame structure type 2, and not configured for PUSCH/PUCCH transmission [dBm] otherwise [dBm] where - is the configured UE transmit power defined in [6] in subframe i for serving cell . - is semi-statically configured by higher layers for m=0 and m=1 for serving cell . For SRS transmission given trigger type 0 then m=0 and for SRS transmission given trigger type 1 then m=1. - is the bandwidth of the SRS transmission in subframe i for serving cell expressed in number of resource blocks. - is the current PUSCH power control adjustment state for serving cell , see Subclause 5.1.1.1. - and are parameters as defined in Subclause 5.1.1.1 for subframe , where . - is the higher layer parameter alpha-SRS for SRS transmission given trigger type 0, 1, or for high layer parameter alpha-additionalSRS given trigger type 2, configured by higher layers for serving cell . - is a parameter composed of the sum of a component which is p0-Nominal-PeriodicSRS, p0-Nominal-AperiodicSRS, or p0_Nominal_AdditionalSRS provided from higher layers for m=0, 1 or 2 respectively, and a component which is p0-UE-PeriodicSRS, p0-UE-AperiodicSRS, or p0-Nominal-AdditionalSRS provided by higher layers for m=0, 1 or 2 respectively, for serving cell . For SRS transmission given trigger type 0 then m=0 and for SRS transmission given trigger type 1 then m=1 and for SRS transmission given trigger type 2 then m=2. - For SRS transmission given trigger type 2 or for serving cell with frame structure type 2, and not configured for PUSCH/PUCCH transmission, the current SRS power control adjustment state is given by and is defined by: - if accumulation is enabled, and if accumulation is not enabled based on higher layer parameter accumulation-Enabled, for SRS transmission given trigger type 0 or 1, or for higher layer parameter accumulationEnabled-additionalSRS given trigger type 2, where - is a correction value, also referred to as a SRS TPC command signalled on PDCCH with DCI format 3 or 3A for SRS transmission given trigger type 2, or DCI format 3B for SRS transmission for trigger type 0, 1, 2 SRS in the most recent subframe , where . - The UE is not expected to receive different SRS TPC command values for serving cell in the same subframe for SRS transmission given trigger type 0 or 1. - The UE is not expected to receive different SRS TPC command values for serving cell in the same subframe for SRS transmission given trigger type 2.

### Issue 12: Power scaling factor for additional SRS symbols.

- As explained in [6], for CA, whether the power scaling factor is independent or not between legacy and additional SRS needs to be discussed.

- Proposal: Endorse the following text proposal to TS 36.213

Text proposal to 36.213 ---------------------------------------------------------------- Text Omitted ------------------------------------------------------------- ### 5.1.3 Sounding Reference Symbol (SRS) #### 5.1.3.1 UE behaviour The setting of the UE Transmit power for the SRS transmitted on subframe i for serving cell is defined by: for SRS transmission given trigger type 2 or for serving cell with frame structure type 2, and not configured for PUSCH/PUCCH transmission [dBm] otherwise [dBm] where - is the configured UE transmit power defined in [6] in subframe i for serving cell . - is semi-statically configured by higher layers for m=0 and m=1 for serving cell . For SRS transmission given trigger type 0 then m=0 and for SRS transmission given trigger type 1 then m=1. - is the bandwidth of the SRS transmission in subframe i for serving cell expressed in number of resource blocks. - is the current PUSCH power control adjustment state for serving cell , see Subclause 5.1.1.1. - and are parameters as defined in Subclause 5.1.1.1 for subframe , where . - is the higher layer parameter alpha-SRS for SRS transmission given trigger type 0, 1, or for high layer parameter alpha-additionalSRS given trigger type 2, configured by higher layers for serving cell . - is a parameter composed of the sum of a component which is p0-Nominal-PeriodicSRS, p0-Nominal-AperiodicSRS, or p0_Nominal_AdditionalSRS provided from higher layers for m=0, 1 or 2 respectively, and a component which is p0-UE-PeriodicSRS, p0-UE-AperiodicSRS, or p0-Nominal-AdditionalSRS provided by higher layers for m=0, 1 or 2 respectively, for serving cell . For SRS transmission given trigger type 0 then m=0 and for SRS transmission given trigger type 1 then m=1 and for SRS transmission given trigger type 2 then m=2. - For serving cell with frame structure type 2, and not configured for PUSCH/PUCCH transmission, the current SRS power control adjustment state is given by and is defined by: - if accumulation is enabled, and if accumulation is not enabled based on higher layer parameter accumulation-Enabled, for SRS transmission given trigger type 0 or 1, or for higher layer parameter accumulationEnabled-additionalSRS given trigger type 2, where - is a correction value, also referred to as a SRS TPC command signalled on PDCCH with DCI format 3 or 3A for SRS transmission given trigger type 2, or DCI format 3B for SRS transmission for trigger type 0, 1, 2 SRS in the most recent subframe , where . - The UE is not expected to receive different SRS TPC command values for serving cell in the same subframe for SRS transmission given trigger type 0, 1, or 2. - The UE attempts to decode a PDCCH of DCI format 3B with CRC scrambled by higher layer parameter srs-TPC-RNTI-r14in every subframe except where serving cell is deactivated. - The UE attempts to decode a PDCCH of DCI format 3 or 3A with CRC scrambled by higher layer parameter TPC-PUSCH-RNTI in every subframe except where service cell is deactivated. - dB for a subframe where no TPC command in PDCCH with DCI format 3/3A/3B is decoded for serving cell or i is not an uplink/special subframe in TDD or FDD-TDD and serving cell c frame structure type 2. - If higher layer parameter fieldTypeFormat3B or fieldTypeFormat3B-additionalSRS indicates 2-bit TPC command, the dB values signalled on PDCCH with DCI format 3B are given in Table 5.1.1.1-2 by replacing with , or if higher layer parameter fieldTypeFormat3B or fieldTypeFormat3B-additionalSRS indicates 1-bit TPC command, the dB values signalled on PDCCH with DCI format 3B are given in Table 5.1.1.1-3 by replacing with . - The dB values signalled on PDCCH with DCI format 3 are given in Table 5.1.1.1-2 by replacing with , and the dB values signalled on PDCCH with DCI format 3A are given in Table 5.1.1.1-3 by replacing with . - If accumulation is enabled, is the first value after reset of accumulation. The UE shall reset accumulation - For serving cell , when value is changed by higher layers - For serving cell , when the UE receives random access response message for serving cell . - For both types of (accumulation or current absolute) the first value is set as follows: - If value is received by higher layers - - else - if the UE receives the random access response message for a serving cell - , where is the TPC command indicated in the random access response corresponding to the random access preamble transmitted in the serving cell , see Subclause 6.2, and and is provided by higher layers and corresponds to the total power ramp-up requested by higher layers from the first to the last preamble in the serving cell , is the bandwidth of the SRS transmission expressed in number of resource blocks valid for the subframe of first SRS transmission in the serving cell . If the UE is not configured with an SCG or a PUCCH-SCell, and if the total transmit power of the UE for the Sounding Reference Symbol in an SC-FDMA symbol would exceed , the UE scales for the serving cell and the SC-FDMA symbol in subframe i such that the condition is satisfied where is the linear value of , is the linear value of defined in [6] in subframe i and is a scaling factor of for serving cell where . Note thatvalues are the same across serving cells. If the UE is not configured with an SCG or a PUCCH-SCell, and if the UE is configured with multiple TAGs and the SRS transmission of the UE in an SC-FDMA symbol for a serving cell in subframe in a TAG overlaps with the SRS transmission in another SC-FDMA symbol in subframe for a serving cell in another TAG, and if the total transmit power of the UE for any Sounding Reference Symbol in the overlapped portion would exceed , the UE scales for the serving cell and each of the overlapped SRS SC-FDMA symbols in subframe such that the condition is satisfied for all the overlapped SRS SC-FDMA symbol(s) in subframe i where is the linear value of , is the transmit power of SRS trigger type 2 or trigger type 0/1 in subframe i for serving cell , is the linear value of defined in [6] in subframe i and is a scaling factor of for serving cell where . Note thatvalues are the same across serving cells. If the UE is configured with a LAA SCell for uplink transmissions, the UE may compute the scaling factor assuming that the UE performs a SRS transmission on the LAA SCell in subframe i irrespective of whether the UE can access the LAA SCell for the SRS transmission in subframe i according to the channel access procedures described in Subclause 4.2.1 of [13]. If the UE is configured with higher layer parameter UplinkPowerControlDedicated-v12x0 for serving cell and if subframe belongs to uplink power control subframe set 2 as indicated by the higher layer parameter tpc-SubframeSet-r12, the UE shall use instead of to determine for subframe i and serving cell , where is defined in Subclause 5.1.1.1. ---------------------------------------------------------------- Text End -------------------------------------------------------------------

  - Other

### Issue 13: Term alignment between specs.

- As explained in [5][8], the term used in 36.211 is additional SRS symbol while in 36.213 it’s trigger type 2.

- Proposal: Endorse the following text proposal to TS 36.211

Text proposal to 36.211 ---- Unchanged parts are omitted ---- ### 5.5.3 Sounding reference signal Two types of sounding reference signals can be configured: - basic sounding reference signal, supporting periodic or aperiodic transmission - additional sounding reference signal, supporting aperiodic transmission only Basic SRS corresponds to either SRS trigger type 0 or type 1 in 8.2 of [4]. Additional SRS corresponds to SRS trigger type 2 in 8.2 of [4]. ---- Unchanged parts are omitted ----

Or

Text proposal to 36.211 -------------------------- Start of proposed TP1 for TS 36.211 ------------------------------------ #### 5.5.1.5 Determining virtual cell identity for sequence generation The definition of depends on the type of transmission. Transmissions associated with PUSCH: - if no value for is configured by higher layers or if the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure, - otherwise. Transmissions associated with SPUCCH/PUCCH: - if no value for is configured by higher layers, - otherwise. Basic sSounding reference signals transmissions of types 0 and 1 as defined in 8.2 of [4]: - nIDRS=nIDSRS if the higher-layer parameters nSRS-Identity-Legacy and nSRS-Identity both are configured, where nIDSRS equals the higher-layer parameter nSRS-Identity - nIDRS=NIDcell otherwise. Additional sSounding reference signals transmissions of type 2 as defined in 8.2 of [4]: - nIDRS=NIDcell if no value for nIDSRS is configured by the higher-layer parameter nSRS-Identity - nIDRS=nIDSRS otherwise. ----------------------------------------------- Unchanged text omitted ------------------------------------------- ### 5.5.3 Sounding reference signal Three types of sounding reference signals can be configured as defined in 8.2 of [4]: SRS trigger type 0, SRS trigger type 1, and SRS trigger type 2. Two types of sounding reference signals can be configured: - basic sounding reference signal, supporting periodic or aperiodic transmission - additional sounding reference signal, supporting aperiodic transmission only #### 5.5.3.1 Sequence generation ##### 5.5.3.1.1 Sequence generation for SRS trigger types 0 and 1 basic SRS The sounding reference signal sequence is defined by clause 5.5.1, where is the sequence-group number defined in clause 5.5.1.3, is the base sequence number defined in clause 5.5.1.4, and . The cyclic shift of the sounding reference signal is given as , where is configured separately for periodic and each configuration of aperiodic sounding by the higher-layer parameters cyclicShift and cyclicShift-ap, respectively, for each UE and is the number of antenna ports used for sounding reference signal transmission. The parameter if , otherwise . The parameter is given by the higher layer parameter transmissionCombNum if configured, otherwise . ##### 5.5.3.1.2 Sequence generation for SRS trigger type 2 additional SRS The sounding reference signal rSRS(p)n is defined by clause 5.5.3.1.1 with the following exceptions - nSRScs is given by the higher-layer parameter additionalSRS-cyclicShift - Nap is given by the higher-layer parameter additionalSRS-AntennaPort - KTC is given by the higher-layer parameter additionalSRS-transmissionComb - the function fgh in clause 5.5.1.3 is given by fghns,l=0if group hopping is disabledi=072ic8nsNsymbUL+l+i mod 30if group hopping is enabled where l is the OFDM symbol index within the slot ns and NsymbUL is the number of OFDM symbols per slot - the function v in clause 5.5.1.4 is given by v=cnsNsymbUL+lif group hopping is disabled and sequence hopping is enabled0otherwise #### 5.5.3.2 Mapping to physical resources ##### 5.5.3.2.1 Mapping to physical resources for SRS trigger types 0 and 1 basic SRS The sequence shall be multiplied with the amplitude scaling factor in order to conform to the transmit power specified in clause 5.1.3.1 in 3GPP TS 36.213 [4], and mapped in sequence starting with to resource elements on antenna port according to ----------------------------------------------- Unchanged text omitted ------------------------------------------- ##### 5.5.3.2.2 Mapping to physical resources for SRS trigger type 2 additional SRS An additional SRS of trigger type 2 spans one or more OFDM symbols in the time domain, where - the starting OFDM symbol l0 within the subframe is given by the higher-layer parameter additionalSRS-startPos; - the duration N in number of OFDM symbols, including potential guard symbols, is given by the higher-layer parameter additionalSRS-duration; Mapping to physical resources shall be done according to clause 5.5.3.2.1 with the following exceptions: - frequency hopping between OFDM symbols is supported and if a UE is configured by higher layer parameter additionalSRS-GuardSymbolFH, a guard symbol is added between every frequency hop; - nSRS=lR where l is the index of the OFDM symbol number carrying additional SRS of trigger type 2 within the subframe not counting guard symbol(s), and R∈1,2,3,4,6,7,8,9,12,13 is the repetition factor given by the higher-layer parameter additionalSRS-RepNum; - BSRS is given by the higher-layer parameter additionalSRS-Bandwidth; - bhop is given by the higher-layer parameter additionalSRS-HoppingBandwidth; - NFH is the number of frequency hops for additional SRS of trigger type 2, derived from N=RNFH+NFH-1GFH if antenna switching is not configured for additional SRS of trigger type 2, and from N=RNASNFH+NAS-1GAS+NFH-1GFH if antenna switching is configured for additional SRS of trigger type 2, where R is the repetition factor given by the higher-layer parameter additionalSRS-RepNum, NAS is the number of antenna switches for additional SRS of trigger type 2 defined in 8.2 of [4], GAS∈0, 1 is the guard-symbol configuration for antenna switching given by the higher-layer parameter additionalSRS-GuardSymbolAS, GFH∈0, 1 is the guard symbol configuration for frequency hopping given by the higher-layer parameter additionalSRS-GuardSymbolFH, and N is given by the higher-layer parameter additionalSRS-duration; - nRRC is given by the higher-layer parameter freqDomainPosition-additionalSRS; - Nap is given by the higher-layer parameter additionalSRS-AntennaPort; - nSRScs is given by the higher-layer parameter additionalSRS-cyclicShift; - KTC is given by the higher-layer parameter additionalSRS-transmissionCombNum; - kTC is given by the higher-layer parameter additionalSRS-transmissionComb. ------------------------------------------- End of proposed TP 1 ----------------------------------------------------

### Issue 14: Typos for power control in 36.213.

- As explained in [5], there are several typos in 36213 for power control.

- Proposal: Endorse the following text proposal to TS 36.213

Text proposal to 36.213 ---- Unchanged parts are omitted ---- ### 5.1.3 Sounding Reference Symbol (SRS) #### 5.1.3.1 UE behaviour The setting of the UE Transmit power for the SRS transmitted on subframe i for serving cell is defined by: for SRS transmission given trigger type 2 or for serving cell with frame structure type 2, and not configured for PUSCH/PUCCH transmission [dBm] otherwise [dBm] where - is the configured UE transmit power defined in [6] in subframe i for serving cell . - is semi-statically configured by higher layers for m=0 and m=1 for serving cell . For SRS transmission given trigger type 0 then m=0 and for SRS transmission given trigger type 1 then m=1. - is the bandwidth of the SRS transmission in subframe i for serving cell expressed in number of resource blocks. - is the current PUSCH power control adjustment state for serving cell , see Subclause 5.1.1.1. - and are parameters as defined in Subclause 5.1.1.1 for subframe , where . - is the higher layer parameter alpha-SRS for SRS transmission given trigger type 0, 1, or is the higher layer parameter alpha-additionalSRS for SRS transmission given trigger type 2, configured by higher layers for serving cell . - is a parameter composed of the sum of a component which is p0-Nominal-PeriodicSRS, p0-Nominal-AperiodicSRS, or p0_Nominal_AdditionalSRS provided from higher layers for m=0, 1 or 2 respectively, and a component which is p0-UE-PeriodicSRS, p0-UE-AperiodicSRS, or p0-Nominal-AdditionalSRS provided by higher layers for m=0, 1 or 2 respectively, for serving cell . For SRS transmission given trigger type 0 then m=0 and for SRS transmission given trigger type 1 then m=1 and for SRS transmission given trigger type 2 then m=2. ---- Unchanged parts are omitted ----

Summary

FL’s view on the issues that are prioritized for discussion in this e-meeting are as following:

- Capturing, correction and clarification of transmission pattern for additional SRS:

    - Issues 1, 3, 4, 7

- Omitted issues or agreements on power control

    - Issues 11, 12

- Term alignment between specs

    - Issue 13

Other issues can be discussed in future f2f meetings.

Issues

Issue 1: Capturing of the number of antenna switching.

Issue 2: The order of repetition, antenna switching and frequency hopping for additional SRS.

Issue 3: guard symbol for antenna switching.

Issue 4: Error in the equation to derive the number of frequency hopping.

Issue 5: Subbands for frequency hopping.

Issue 6: SRS symbol index.

Issue 7: Clarification for frequency hopping for aperiodic SRS.

Issue 8: transmission in case of periodic legacy and additional SRS in one subframe.

Issue 9: Clarification of collision between additional SRS and PUSCH/PUCCH.

Issue 10: Collision between additional SRS and short PUSCH/PUCCH.

Issue 11: Capturing of close-loop power control for additional SRS symbols.

Issue 12: Power scaling factor for additional SRS symbols.

Issue 13: Term alignment between specs.

Issue 14: Typos for power control in 36.213.

- References

- R1-1913596, “RAN1 agreements for DL MIMO efficiency enhancements for LTE”, Huawei, Reno, USA, November 2019.

- R1-2000225	Corrections on additional SRS symbols	Huawei, HiSilicon

- R1-2000559	Maintenance for additional SRS symbols	Nokia, Nokia Shanghai Bell

- R1-2000594	Remaining issues on additional SRS symbols	Lenovo, Motorola Mobility

- R1-2000655	Text proposals on additional SRS symbols	LG Electronics

- R1-2000710	Additional SRS symbols	Qualcomm Incorporated

- R1-2000717	Corrections to SRS	Intel Corporation

- R1-2000942	Maintenance for additional SRS symbols	Ericsson

- R1-2000226	Remaining issues of additional SRS symbols	Huawei, HiSilicon