3GPP TSG RAN WG1 #100E						R1-2001233

e-Meeting, February 24th – March 6th, 2020

Source:	Intel Corporation

Title:	Output of email thread [100e-NR-Pos-DL-PRS-01]

Agenda item:	7.2.8.1

Document for:	Discussion and Decision

- Introduction

In this contribution, we provide summary of the RAN1-email discussion [100e-NR-Pos-DL-PRS-01] organized based on submitted text proposals [1] and aiming to provide corrections to the latest description of DL PRS in the TS 38.211.

- Summary of RAN1 E-Mail Discussion

In this section, we provide summary of views from companies on tentative proposals selected for RAN1 email discussion [100e-NR-Pos-DL-PRS-01].

## DL PRS Reference-Signal Sequence Generation

- Proposal #1:

  - Correct equation for DL PRS sequence generation as shown in text proposal below:

##### 7.4.1.7.2 Sequence generation The UE shall assume the reference-signal sequence rm is defined by rm=121-2c2m+j121-2c2m+1 where the pseudo-random sequence ci is defined in clause 5.2.1.

Comments:

| ZTE: | Support |
| --- | --- |
| OPPO | Support |
| QC | Support |
| Futurewei | Support |
| CATT | Support |
| Huawei/HiSilicon | Support |
| Nokia | Support |
| ericsson | Support |
| vivo | Support |
| LGE | Support |
| Intel Corporation | Support |

Based on discussion, all companies agreed with the proposal #1 above.

## DL PRS Sequence ID

- Proposal #2:

  - Update section 7.4.1.7.2 of the TS 38.211, to cover the case when dl-PRS-SequenceId-r16 is not configured. Text proposal is provided below:

The pseudo-random sequence generator shall be initialised with cinit=222nID,seqPRS1024+210Nsymbslotns,fμ+l+12nID,seqPRS mod 1024+1+nID,seqPRS mod 1024 mod 231 where ns,fμ is the slot number, the downlink PRS sequence ID nID,seqPRS∈0,1,…,4095 is given by the higher-layer parameter dl-PRS-SequenceId-r16, unless otherwise configured, nID,seqPRS is equal to physical cell ID, and l is the OFDM symbol within the slot to which the sequence is mapped.

Comments:

| ZTE | The new spec 37.355 is under discussion in RAN2. I noticed that the parameter ‘dl-PRS-SequenceId-r16’ is not optional in the latest version. In such case, we may not need the default value for nID,seqPRS . |
| --- | --- |
| OPPO | Share the same view as ZTE |
| QC | Share the same view as ZTE and OPPO. There is no need for a default value here. |
| Futurewei | If the specs is revised to state that ‘..configuration is expected..’, then there is no need in our view to define something for ‘unless otherwise configured’. |
| CATT | Share the same view as ZTE |
| Huawei/HiSilicon | Regarding the changes of parameter name, we suggest to postpone it, as currently RAN2 is deciding whether to remove “dl-”. Regarding default value of sequence ID, we do not see it really necessary, and it cannot be used in case of multiple TRPs under the same cell. |
| Nokia | We share the same view as most companies above that this is not needed at this stage. |
| Ericsson | We support the idea of a default value. If we do not want to introduce changes into the ran1 specs, the default value could be specified in RAN2. |
| vivo | No need. |
| LGE | If sequence ID configuration is not optional, then the default value might not be needed. After RAN2 discussion on TS 37.355, this could be discussed. In our view, optional configuration might be reasonable. There are many PRS resources need to be configured considering many TRPs and beams, and not all PRS resources need to use different sequence IDs all the time. For example, PRS resources transmitted from a TRP can have the common sequence ID, which might be possible scenario if the time-frequency resources are not overlapped between PRS resources and this can reduce redundant signaling overhead. |
| Intel Corporation | We do not see benefits of this proposal from physical layer perspective. If default value is needed discussion may continue in RAN2. |

Based on discussion there is no RAN1 WG consensus to adopt proposal #2. Discussion may continue in RAN2 WG.

## DL PRS Seed Initialization - cinit

- Proposal #3:

  - Conclude on whether the following change is needed to cinit DL PRS initialization equation

Current equation cinit=222nID,seqPRS1024+210Nsymbslotns,fμ+l+12nID,seqPRS mod 1024+1+nID,seqPRS mod 1024 mod 231 Proposed revision cinit=229nID,seqPRS1024+210Nsymbslotns,fμ+l+12nID,seqPRS mod 1024+1+nID,seqPRS mod 1024 mod 231

Comments:

| MTK | 1, In the beginning, 229 was proposed by Huawei and MTK to support PRS ID number= 4096. At the end, 222 was agreed for future extension to ID number >= 65536, due to QC’s suggestion 2, In R1-2000341, it is mentioned that there would be more repeated sequences when the parameter is equal to 222. And then it is suggested to consider using 229. We doubt the results. In our analysis, no matter 222 or 229 are applied, all the c_init are unique. We provide the matlab code in below for the reference |
| --- | --- |
| ZTE | As this issue has been discussed. We prefer not to discuss again. |
| QC | Same view as MTK. Our analysis (e.g. similar matlab code with what MTK shows above) shows that there are no repeated cinit. We encourage the proponent of this proposal to recheck their code, and/or provide further details. |
| CATT | Same view as MTK. |
| Huawei/HiSilicon | We do not see overlapping sequences in our simulation for either 2^22 or 2^29. |
| Nokia | We do not support the change. |
| Ericsson | The change does not seem warranted by any technical issue (the sequences are unique already). |
| vivo | Apologies for our mistake. We had error in our code. We verified that no repeated cinit for all SCS cases. No need to change. |
| LGE | Share the same view with MTK |
| Intel Corporation | Do not see the need in proposed change |

Based on discussion, the reported issue is not confirmed and there is no need to adopt proposal #3.

## Single Port DL PRS Transmission

- Proposal #4:

  - Update section “7.4.1.7.3 Mapping to physical resources in a downlink PRS resource” of the TS 38.211 to cover single port assumption for DL PRS transmission. Text proposal is provided below.

- the resource-element offset koffsetPRS∈0,1,…,KcombPRS-1 is given by the higher-layer parameter combOffset; - the quantity k' is given by Table 7.4.1.7.3-1. - The UE shall assume that a DL PRS is transmitted using a single antenna port p=5000.

Comments:

| ZTE | Support |
| --- | --- |
| OPPO | Support |
| QC | Note, this is not what the UE assumes, but what really happens from the transmission side. Note also the text used in 38.211 in Section 7.2: The following antenna ports are defined for the downlink: - Antenna ports starting with 1000 for PDSCH - Antenna ports starting with 2000 for PDCCH - Antenna ports starting with 3000 for channel-state information reference signals - Antenna ports starting with 4000 for SS/PBCH block transmission - Antenna ports starting with 5000 for positioning reference signals Therefore, suggest to just write as follows: <--------------------------------Unchanged Text Omitted------------------------------> and where - lstartPRS is the first symbol of the downlink PRS within a slot and given by the higher-layer parameter DL-PRS-ResourceSymbolOffset; - the size of the downlink PRS resource in the time domain LPRS∈2,4,6,12 is given by the higher-layer parameter DL-PRS-NumSymbols; - the comb size KcombPRS∈2, 4, 6,12 is given by the higher-layer parameter transmissionComb; - the resource-element offset koffsetPRS∈0,1,…,KcombPRS-1 is given by the higher-layer parameter combOffset; - the quantity k' is given by Table 7.4.1.7.3-1; - p=5000. <--------------------------------Unchanged Text Omitted------------------------------> |
| Futurewei | Support |
| CATT | Share the similar view as QC. It may be clearer to say “DL PRS is transmitted using a single antenna port p=5000.” |
| Huawei/HiSilicon | Should it be better added here? ************************ Unchanged part ************************ when the following conditions are fulfilled: - the resource element k,lp,μ is within the resource blocks occupied by the downlink PRS resource for which the UE is configured; - the antenna port p=5000; - the symbol l is not used by any SS/PBCH block used by the serving cell for downlink PRS transmitted from the serving cell or indicated by the higher-layer parameter SSB-positionInBurst for downlink PRS transmitted from a non-serving cell; - the slot number satisfies the conditions in clause 7.4.1.7.4. ************************ Unchanged part ************************ |
| Nokia | We support the proposal by QC and CATT. |
| Ericsson | In the definition of the CSI RS port numbering, the same sentence is used to describe the port mapping: The UE shall assume that a CSI-RS is transmitted using antenna ports numbered according to We would like to keep a similar wording for consistency over the different RSs. |
| vivo | Support |
| LGE | Support |
| Intel Corporation | We prefer the wording proposed by Huawei or Qualcomm |

Based on discussion, it seems all companies are in favor of correction. As an actual TP it can be recommended to take one of the wordings in the above table.

- Outcome of E-Mail Discussion

RAN1 WG discussed proposal provided in this document and agreed to prepare TP for the TS 38.211, in order to address the following identified issues:

- Correction to DL PRS reference-signal sequence generation (please refer to section 2.1 for details)

- Capturing assumption on single port DL PRS transmission (please refer to section 2.4 for details)

The corresponding text proposals were submitted in [2].

- References

- R1-2001176, “Feature Lead Summary#2 for AI 7.2.8.1 – DL Reference Signals for NR Positioning”, Intel Corporation

- R1-2001234, “Text proposal to TS 38.211 based on outcome of [100e-NR-Pos-DL-PRS-01]”, Intel Corporation