3GPP TSG RAN WG1 #100-e					  	R1-2000767

e-Meeting, February 24th – March 6th, 2020

Agenda item:	7.2.2.2.1

Source:	NEC

Title:              	Remaining issues on CP extension

Document for:	Discussion and Decision

- Introduction

In RAN1#98bis meeting, it has been agreed that:

Agreement:

- For the CP extension prior to at least a dynamically scheduled PUSCH transmission, the CP extension is located in the symbol(s) immediately preceding the PUSCH allocation indicated by SLIV. The supported durations for CP extension at the UE are:

  - 0 (i.e. no CP extension)

  - C1*symbol length – 25 us

  - C2*symbol length – 16 us - TA

  - C3*symbol length – 25 us – TA

And TS38.211-g00 and TS38.212-g00 realized that agreements. In this contribution, we have some text proposal to specifications based on the remaining issues of the agreement of CP extension.

- Discussion

As different OFDM symbol may have different CP duration, e.g. OFDM symbol #0 of a subframe has longer CP duration, the different OFDM symbol will have different symbol duration. The intention of CP extension is to make sure that the sum of gap for LBT and CP extension duration should occupy the entire OFDM symbol(s). Meanwhile, the CP extension can start at any OFDM symbol, hence the text in current specification may lead to a longer or shorter CP extension duration then expected one.

So we propose that:

Proposal 1: Adopt the following text proposals for the duration of CP extension.

- Text proposal 1

>>> Text proposal for 38.211 Section 5.3.1 >>>

< Unchanged parts are omitted >

In case of cyclic prefix extension of the first OFDM symbol l allocated for PUSCH transmission, the time-continuous signal sext(p,μ)t for the interval tstart,lμ-Text≤t<tstart,lμ preceding the first OFDM symbol for PUSCH is given by

sext(p,μ)t=sl(p,μ)t

where t<0 refers to the signal in the previous subframe and Text is given by

-	Table 5.3.1-1 with the index, C1=1 for μ=0,1 and C1=2 for μ=2;

-	Table 5.3.1-1 with the index, C2 and C3 given by the higer-layer parameters CP-ExtensionC2-r16 and CP-ExtensionC3-r16, respectively, and TTA given by clasue 4.3.1 for dynamically scheduled PUSCH transmissions. UE does not expect Text determined by C2 and C3 less than 0;

-	the procedure in [6, TS 38.214] for a PUSCH transmission using configured grant.

The starting position of OFDM symbol  for subcarrier spacing configuration in a subframe is given by

Table 5.3.1-1: The cyclic prefix extension Text.

|  | μ=0 | μ=1 | μ=2 |
| --- | --- | --- | --- |
|  | Tsymb,lμ-25∙10-6 | Tsymb,lμ-25∙10-6 | 2Tsymb,lμ-25∙10-6 |
|  | C2Tsymb,lμ-16∙10-6-TTA | C2Tsymb,lμ-16∙10-6-TTA | C2Tsymb,lμ-16∙10-6-TTA |
|  | C3Tsymb,lμ-25∙10-6-TTA | C3Tsymb,lμ-25∙10-6-TTA | C3Tsymb,lμ-25∙10-6-TTA |

| Index | μ=0,1,2 |
| --- | --- |
| 0 | - |
| 1 | k=1C1Tsymb, l-k mod 7∙2μ μ-25∙10-6 |
| 2 | k=1C2Tsymb, l-k mod 7∙2μ μ-16∙10-6-TTA |
| 3 | k=1C3Tsymb, l-k mod 7∙2μ μ-25∙10-6-TTA |

>>> End Text proposal >>>

- Text proposal 2

>>> Text proposal for 38.212 Section 7.3.1.1.1 >>>

< Unchanged parts are omitted >

Table 7.3.1.1.1-4: Channel access type & CP extension for DCI format 0_0 and DCI format 1_0

| Bit field mapped to index | Channel Access Type | The CP extension Text index defined in [clause 5.3.1 in 38.211] |
| --- | --- | --- |
| 0 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 2 |
| 1 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 3 |
| 2 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 1 |
| 3 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 0 |

>>> End Text proposal >>>

- Text proposal 3

>>> Text proposal for 38.212 Section 7.3.1.1.2 >>>

< Unchanged parts are omitted >

Table 7.3.1.1.2-35: Allowed entries for DCI format 0_1, configured by high layer parameter ULDCI-trigerred-UL-ChannelAccess-CPext-CAPC-List-r16

| Entry index | Channel Access Type | The CP extension Text index defined in [clause 5.3.1 in 38.211] | CAPC |
| --- | --- | --- | --- |
| 0 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 | 1 |
| 1 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 | 2 |
| 2 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 | 3 |
| 3 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 | 4 |
| 4 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 2 | 1 |
| 5 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 2 | 2 |
| 6 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 2 | 3 |
| 7 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 2 | 4 |
| 8 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 | 1 |
| 9 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 | 2 |
| 10 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 | 3 |
| 11 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 | 4 |
| 12 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 2 | 1 |
| 13 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 2 | 2 |
| 14 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 2 | 3 |
| 15 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 2 | 4 |
| 16 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 0 | 1 |
| 17 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 0 | 2 |
| 18 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 0 | 3 |
| 19 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 0 | 4 |
| 20 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 1 | 1 |
| 21 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 1 | 2 |
| 22 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 1 | 3 |
| 23 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 1 | 4 |
| 24 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 3 | 1 |
| 25 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 3 | 2 |
| 26 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 3 | 3 |
| 27 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 3 | 4 |
| 28 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 0 | 1 |
| 29 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 0 | 2 |
| 30 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 0 | 3 |
| 31 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 0 | 4 |
| 32 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 1 | 1 |
| 33 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 1 | 2 |
| 34 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 1 | 3 |
| 35 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 1 | 4 |
| 36 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 2 | 1 |
| 37 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 2 | 2 |
| 38 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 2 | 3 |
| 39 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 2 | 4 |
| 40 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 3 | 1 |
| 41 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 3 | 2 |
| 42 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 3 | 3 |
| 43 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 3 | 4 |

>>> End Text proposal >>>

- Text proposal 4

>>> Text proposal for 38.212 Section 7.3.1.2.2 >>>

< Unchanged parts are omitted >

Table 7.3.1.2.2-6: Allowed entries for DCI format 1_1, configured by high layer parameter DLDCI-trigerred-UL-ChannelAccess-CPext-CAPC-List-r16

| Entry index | Channel Access Type | The CP extension Text index defined in [clause 5.3.1 in 38.211] |
| --- | --- | --- |
| 0 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 |
| 1 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 2 |
| 2 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 |
| 3 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 2 |
| 4 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 0 |
| 5 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 1 |
| 6 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 3 |
| 7 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 0 |
| 8 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 1 |
| 9 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 2 |
| 10 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 3 |

>>> End Text proposal >>>

- Conclusion

In this contribution, we provided our views on some remaining issues of CP extension, and we proposed that:

Proposal 1: Adopt the following text proposals for the duration of CP extension.

- References

- TS38.211 v16.0.0

- TS38.212 v16.0.0