3GPP TSG RAN WG1 #100							R1- 2000839

e-Meeting, February 24th – March 6th, 2020

______________________________________________________________________Agenda item: 7.2.4.5

Source: LG Electronics

Title: Summary of critical issues for AI 7.2.4.5 Physical layer procedures for sidelink

Document for: Discussion and decision

- List of critical issues

- SL power control

  - [Prioritized] Remaining issues in power control formula for PSCCH/PSSCH/PSFCH

    - PSD of PSCCH, e.g., whether to apply power boosting

    - Whether to support multiple PSFCH TXs and, if supported, PSD of each PSFCH

    - How to derive reference PSSCH DMRS power for SL PL estimation

- SL HARQ feedback

  - [Prioritized] Details of the PSFCH resource determination

    - Whether candidate PSFCH resource is the set of PRBs associated with the starting sub-channel and slot used for that PSSCH or the sub-channel(s) and slot used for that PSSCH

    - Details on how to define the values of m0 and mCS for a PSFCH TX

    - Whether/how to handle the case when rbSetPSFCH is not a multiple of numSubchannel*periodPSFCHresource

  - [Prioritized] Details of the TX-RX distance determination

    - Details on length/width of each geographic zone and number of zones configured with respect to longitude/latitude

    - Details of calculating TX-RX distance for GC HARQ feedback Option 1, including whether/how to handle the case when the location information is not available at TX UE and/or RX UE

  - How to indicate GC HARQ feedback Option 1/2 to RX UE.

  - UE behavior on which HARQ status information is reported to MAC layer in SL HARQ-ACK reception via PSFCH

  - Whether or not to introduce restriction on the size of group in GC HARQ feedback Option 2

- Handling SL and UL transmissions

  - [Prioritized] Details of SL/UL prioritization and power sharing

    - Prioritization in the cases mentioned in RAN2 LS (R1-2000161)

    - When to prioritize which TX in case of simultaneous TXs of UL and SL across difference carriers

- SL CSI

  - [Prioritized] How to configure a latency bound for SL CSI reporting MAC CE

  - Details of SL CSI reference resource