3GPP TSG-RAN WG1 Meeting #100-e	R1-2001008

e-Meeting, February 24th- March 6th, 2020

Agenda Item:	7.2.4.2.1

Source:	Ericsson

Title:	Resource Allocation Mode 1 for NR SL

Document for:	Discussion, Decision

# 1	Introduction

In this paper, open issues related to the resource allocation mode 1 AI in NR SL Rel-16 are discussed.

# 2		Configured grant

## 2.1	HARQ process ID determination

One aspect that is not clear in the specifications is the HARQ process ID determination for configured grants. This functionality is essential as, otherwise, the gNB cannot provide additional resources for retransmission using a dynamic grant. In our view, the simplest approach is to have the HARQ process ID as part of the RRC configuration of the grant (type-1) or signalled in DCI (type-2).

The HARQ process ID used for sidelink transmission scheduled by a configured grant is provided by the gNB as part of the RRC configuration (for type-1) or in the DCI activating the grant (for type-2).

## 2.2	Timing aspects

For uplink, the RRC configuration for a configured grant type-1 includes the following field:

timeDomainOffset Offset related to SFN=0, see TS 38.321 [3], clause 5.8.2.

In principle, the UE could derive the slots corresponding to the configured grant type-1 by applying the timeDomainOffset to SFN=0 of the carrier in which the SL transmission takes place. If only configured grants type-1 were used, then there would be no collisions between different UEs due to timing ambiguities. However, if configured grant type-1 are used in the same carrier as other DCI-based grants collision may take place due to the timing ambiguities with the latter ones. For this reason, to derive the slots used for configured grant type-1 we propose to follow a rule like the one used for DCI-based grants.

For configured grant type-1 the slot corresponding to the first SL transmission is the first SL slot of the corresponding resource pool that starts no earlier than SFN0 + timeDomainOffset – TTA/2 x TC, where SFN0 corresponds to SFN=0 in the DL carrier, TTA is the timing advance value, and Tc is as defined in 38.211.

# 3	DCI aspects

Section 7.3.1.4.1 in 38.212 specifies DCI format 3_0 as follows:

DCI format 3_0 is used for scheduling of NR PSCCH and NR PSSCH in one cell. The following information is transmitted by means of the DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI: - Time gap – [x] bits determined by higher layer parameter timeGapFirstSidelinkTransmission, as defined in clause x.x.x of [6, TS 38.214] - HARQ process ID – [x] bits as defined in clause x.x.x of [6, TS 38.214] - New data indicator – 1 bit as defined in clause x.x.x of [6, TS 38.214] - Lowest index of the subchannel allocation to the initial transmission –log2(N subChannel SL) bits as defined in clause x.x.x of [6, TS 38.214] - SCI format 0-1 fields according to clause 8.3.1.1: - Frequency resource assignment. - Time resource assignment. - PSFCH-to-HARQ feedback timing indicator – 3 bits as defined in clause x.x.x of [6, TS 38.214]. - PUCCH resource indicator – 3 bits as defined in clause x.x.x of [6, TS 38.214]. - Configuration index – 0 bit if the UE is not configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI; otherwise [x] bits as defined in clause x.x.x of [6, TS 38.214]. If the UE is configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI, this field is reserved for DCI format 3_0 with CRC scrambled by SL-RNTI.

## 3.1	Contents

As seen above, some of the fields are not completely specified.

For time gap, we propose that the RRC table can be configured with up to 8 values and that the bitwidth is chosen accordingly.

Up to 8 values may be configured by higher layer for timeGapFirstSidelinkTransmission, which indicate the number of SL (logical) slots. The field time gap in DCI format 3_0 has variable bitwidth that depends on the number of configured values.

For HARQ process ID, we propose to use 4 bits.

16 SL HARQ processes are defined. The field HARQ process ID in DCI format 3_0 consists of 4 bits.

RAN2#108 made the following working assumption

Up to 8 configured grants (including both configured grant type1 and type2) in mode1 can be configured and active for NR sidelink communication (working assumption). Running 38.321 will specify something like “[8]”

Based on this, a 3-bit field is necessary in DCI.

The field ‘Configuration index’ consists of 3 bits, when applicable.

## 3.2	Size alignment

| Field |  | Format 0-0 | Format 0-1 |
| --- | --- | --- | --- |
| Identifier |  | 1 bit | 1 bit |
| Resource information | CFI | – | 0 or 3 bits |
|  | UL/SUL | 0 or 1 bit | 0 or 1 bit |
|  | BWP indicator | – | 0-2 bits |
|  | Freq.-domain allocation | Variable (only type 1) | variable |
|  | Time-domain allocation | 0-4 bits | 0-4 bits |
|  | Frequency hopping | 0 or 1 bit | 0 or 1 bit |
| TB-related | MCS | 5 bits | 5 bits |
|  | NDI | 1 bit | 1 bit |
|  | RV | 2 bis | 2 bis |
| HARQ-related | Process number | 4 bits | 4 bits |
|  | DAI | – | 1-4 bits |
|  | CBGTI | – | 0, 2, 4, or 6 bits |
| Multi-antenna related | DMRS seq initialization | – | 1 bit |
|  | Antenna ports | – | 2-5 bits |
|  | SRI | – | Variable |
|  | Precoding information | – | 0-6 bits |
|  | PTRS-DMRS assoc. | – | 0 or 2 bits |
|  | SRS request | – | 2 bits |
|  | CSI request | – | 0-6 bits |
| Power control | PUSCH power control | 2 bits | 2 bits |
|  | Beta offset | – | 0 or 2 bits |

Even when only 2 sub-channels are configured, the size exceeds 30 bits, meaning that alignment with DCI 0_0 is in general not possible.

When both DCI 0_1 and DCI 3_0 are configured, DCI 3_0 is zero-padded to match the size of DCI 0_1.

Similarly, if in a search space a UE is configured with format 3_0 with CRC scrambled with SL-RNTI and with format 3_0 with CRC scrambled with SL-CS-RNTI, then the former is zero-padded until it has the same size as the latter.

When both DCI 3_0 with CRC scrambled with SL-RNTI and DCI 3_0 with CRC scrambled with SL-CS-RNTI are configured, DCI 3_0 with CRC scrambled with SL-RNTI is zero-padded to match the size of DCI 3_0 with CRC scrambled with SL-CS-RNTI.

We propose to use the following text from 36.212 (with the corresponding changes to format numbers and RNTI names):

If the number of information bits in format 3_0 mapped onto a given search space is less than the payload size of format 0_1 mapped onto the same search space, zeros shall be appended to format 3_0 until the payload size equals that of format 0_1 including any padding bits appended to format 0_1. If the format 3_0 CRC is scrambled by SL-RNTI and if the number of information bits in format 3_0 mapped onto a given search space is less than the payload size of format 3_0 with CRC scrambled by SL-CS-RNTI mapped onto the same search space and format 0_1 is not defined on the same search space, zeros shall be appended to format 3_0 until the payload size equals that of format 3_0 with CRC scrambled by SL-CS-RNTI.

# 4	LTE Uu controlling NR sidelink

For LTE Uu controlling NR sidelink, only configured grant type-1 is supported. The issue with timing discussed in Section 3 also applies here and the same solution can be used.

For a configured grant type-1 configured over LTE Uu, the slot corresponding to the first SL transmission is the first SL slot of the corresponding resource pool that starts no earlier than SFN0 + timeDomainOffset – TTA/2 x TC, where SFN0 corresponds to SFN=0 in the DL carrier, TTA is the timing advance value, and Tc is as defined in 38.211.

# 5	HARQ procedure over Uu

## 5.1	UE behaviour for generating the HARQ reports

The agreements on the UE behaviour for generating HARQ reports have some FFS bullets that have led to an incomplete specification. In this section, we discuss how to complete them.

Agreements: - For unicast: - TX UE reports contents received in PSFCH (i.e., ACK/NACK) to gNB. - TX UE reports NACK if PSFCH is not detected - when generating the HARQ-ACK report for the transmissions corresponding to a grant, the TX UE uses the most recent PSFCH occasion (as working assumption) associated with the transmissions. - For groupcast option 1: - TX UE reports ACK to the gNB if no PSFCH is detected. - TX UE reports NACK to the gNB if at least one PSFCH (i.e., NACK) is detected. - when generating the HARQ-ACK report for the transmissions corresponding to a grant, the TX UE uses the most recent PSFCH occasion (as a working assumption) associated with the transmissions. - FFS the cases when TX UE does not transmit/receive due to prioritization. - For groupcast option 2: - TX UE reports ACK if all expected PSFCH resources are received and carry ACK. - TX UE reports NACK if at least one received PSFCH resource carries NACK or if no PSFCH is detected. - FFS behavior when TX UE does not detect some expected PSFCH. - FFS the case with PSFCHs corresponding to multiple PSCCH/PSSCH transmissions before generating the HARQ-ACK report. - FFS if no PSCCH/PSSCH is transmitted in a set of resources for configured grant. - FFS whether/how to deal with the case of reaching the maximum number of HARQ re-transmissions for a TB.

For groupcast option 1, the current agreements would result in ACK being reported to the gNB if the TX UE does not transmit PSCCH/PSSCH or if it does not receive PSFCH due to prioritization. This would result in the UE not being granted more resources for transmission of that TB although it would require them.

If prioritization rules in the UE result in dropping a PSCCH/PSSCH transmission or not being able to receive a PSFCH, the TX UE reports NACK to the gNB.

The LS by SA2 [6] states that the V2X application layer provides “a group size” and “a member ID”. Moreover, RAN1 has assumed that the member ID M is an integer between 0 and X-1, where X corresponds to the group size. Thus, the TX UEs can differentiate the PSFCH transmissions for the different RX UEs and identify whether some expected PSFCH is missing.

In groupcast option 2, when SL HARQ-ACK reporting to the gNB is used the TX UE reports NACK to the gNB if does not detect some expected PSFCH.

For case with PSFCHs corresponding to multiple PSCCH/PSSCH transmissions before generating the HARQ-ACK report, which was also left FFS, our view is that the TX UE should also use the most recent PSFCH occasion (like for unicast and groupcast). In this way, the TX UE sends ACK/NACK based on the already-agreed conditions.

For groupcast option 2, when generating the HARQ-ACK report for the transmissions corresponding to a grant, the TX UE uses the most recent PSFCH occasion associated with the transmissions.

Regarding the case that no PSCCH/PSSCH is transmitted in a set of resources for configured grant, we distinguish three cases.

- For groupcast option 1, the existing agreement would result in ACK being conveyed to the gNB because no PSFCH would be detected.

- For unicast and groupcast option 2, there would be no “PSFCH occasion associated with the transmissions” and, consequently, the behavior is unclear. We propose to report ACK in that case.

For unicast groupcast option 2, the TX UE reports ACK to the gNB if no PSCCH/PSSCH is transmitted in a set of resources for configured grant.

Our view is that, for Mode-1, whether to use SL HARQ FB or not should be controlled by the gNB. However, this is to be discussed in RAN2. From RAN1 perspective it is sufficient to say that, if SL HARQ-ACK reporting to the gNB is enabled but SL HARQ feedback is not used, ACK is reported.

If SL HARQ-ACK reporting to the gNB is enabled but SL HARQ feedback is not used, the UE reports ACK.

## 5.2	SL HARQ multiplexing

For DL HARQ acknowledgements, NR provides a framework for transmitting multiple DL HARQ acknowledgements in UL. Multiplexing can be done using a semi-static (Type-1) codebook or a dynamic (Type-2) codebook.

The semi-static (Type-1) codebook is configured based on a few RRC parameters (number of carriers, CBG configuration, configured PDSCH-to-HARQ_feedback timing values, etc.). In TS 38.213, two procedures are described in terms of pseudo code to generate the codebook:

- The first pseudo-code determines a set of occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in PUCCH (for an active cell, and a DL-BWP, and an UL-BWP).

- The second pseudo-code determines the HARQ-ACK information bits (for the serving cells configured for the UE).

The legacy codebook for DL HARQ acknowledgements can be described as a table where rows correspond to cells/CBGs/TBs and columns correspond to configured K1 values. Each of the entries in the table corresponds to a potential scheduling of a TB (or CBG). ACK is used if the TB (or CBG) was correctly decoded and NACK is used otherwise.

As agreed, the SL HARQ-ACK bits are generated using the Rel-15 procedures. To this end, the following changes are necessary to the procedure:

- Determine candidate PDCCH occasions carrying DCI dynamic grant or PDCCH releasing SPS configured grant type-1.

- Codebook size is determined by PSFCH-to-PUCCH configured values.

Some simplifications to the procedure are possible. For example, HARQ-ACK spatial bundling is not necessary, single HARQ-ACK bit is reported, etc. In addition, it may also be reasonable to restrict SL scheduling to come from a single DL carrier.

For Type-1 codebook:

- UE determines occasions for candidate PDCCH receptions carrying SL grant or PDCCH releasing SPS configured grant type-1 with PUCCH in the same slot based on PDCCH-to-PUCCH configured values.

- For each candidate, one HARQ-ACK bit is reported.

Table 1. Semi-static (Type-1) codebook for conveying multiple SL HARQ acknowledgements in UL.

| Sidelink | PDCCH-to-PUCCH values (SL) |  |  |
| --- | --- | --- | --- |
|  | Value A | Value B | Value C |
|  | SL codebook |  |  |

The dynamic (Type-2) codebook is determined based on the monitoring occasions for DCI formats correctly received in a corresponding PDCCH monitoring occasion and that indicate transmission of HARQ acknowledgements in a given PUCCH slot (i.e., through PDSCH-to-HARQ_feedback timing and K0 values). In addition, at the end of the codebook, the HARQ acknowledgements for SPS PDSCH receptions are added.

As agreed, the SL HARQ-ACK bits are generated using the Rel-15 procedures. To this end, the following changes are necessary to the procedure:

- Monitor occasions for PDCCH with the corresponding SL dynamic grant or PDCCH releasing SL configured grant type-1.

- Timing is given by PSFCH-to-PUCCH gap signalled in DCI.

As for type-1, some simplifications to the procedure are possible. Again, it may also be reasonable to restrict SL scheduling to come from a single DL carrier.

For Type-2 codebook:

- UE monitors occasions for PDCCH carrying SL grant or PDCCH releasing SL configured grant type-1 with PUCCH in the same slot based on PSFCH-to-PUCCH gap signalled in DCI.

- For each PDCCH occasion at most one HARQ-ACK bit is reported.

Table 2. Dynamic (Type-2) codebook for conveying multiple SL and DL HARQ acknowledgements in UL.

| HARQ-ACK bits for SL |  |
| --- | --- |
| HARQ-ACK bits corresponding to received DCI scheduling SL transmission or releasing SL Type-2 configured grant | HARQ-ACK bits corresponding to received DCI scheduling SL transmission or releasing SL Type-2 configured grant |

# 6	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	The HARQ process ID used for sidelink transmission scheduled by a configured grant is provided by the gNB as part of the RRC configuration (for type-1) or in the DCI activating the grant (for type-2).

Proposal 2	For configured grant type-1 the slot corresponding to the first SL transmission is the first SL slot of the corresponding resource pool that starts no earlier than SFN0 + timeDomainOffset – TTA/2 x TC, where SFN0 corresponds to SFN=0 in the DL carrier, TTA is the timing advance value, and Tc is as defined in 38.211.

Proposal 3	Up to 8 values may be configured by higher layer for timeGapFirstSidelinkTransmission, which indicate the number of SL (logical) slots. The field time gap in DCI format 3_0 has variable bitwidth that depends on the number of configured values.

Proposal 4	16 SL HARQ processes are defined. The field HARQ process ID in DCI format 3_0 consists of 4 bits.

Proposal 5	The field ‘Configuration index’ consists of 3 bits, when applicable.

Proposal 6	When both DCI 0_1 and DCI 3_0 are configured, DCI 3_0 is zero-padded to match the size of DCI 0_1.

Proposal 7	When both DCI 3_0 with CRC scrambled with SL-RNTI and DCI 3_0 with CRC scrambled with SL-CS-RNTI are configured, DCI 3_0 with CRC scrambled with SL-RNTI is zero-padded to match the size of DCI 3_0 with CRC scrambled with SL-CS-RNTI.

Proposal 8	For a configured grant type-1 configured over LTE Uu, the slot corresponding to the first SL transmission is the first SL slot of the corresponding resource pool that starts no earlier than SFN0 + timeDomainOffset – TTA/2 x TC, where SFN0 corresponds to SFN=0 in the DL carrier, TTA is the timing advance value, and Tc is as defined in 38.211.

Proposal 9	If prioritization rules in the UE result in dropping a PSCCH/PSSCH transmission or not being able to receive a PSFCH, the TX UE reports NACK to the gNB.

Proposal 10	In groupcast option 2, when SL HARQ-ACK reporting to the gNB is used the TX UE reports NACK to the gNB if does not detect some expected PSFCH.

Proposal 11	For groupcast option 2, when generating the HARQ-ACK report for the transmissions corresponding to a grant, the TX UE uses the most recent PSFCH occasion associated with the transmissions.

Proposal 12	For unicast groupcast option 2, the TX UE reports ACK to the gNB if no PSCCH/PSSCH is transmitted in a set of resources for configured grant.

Proposal 13	If SL HARQ-ACK reporting to the gNB is enabled but SL HARQ feedback is not used, the UE reports ACK.

Proposal 14	For Type-1 codebook:

	UE determines occasions for candidate PDCCH receptions carrying SL grant or PDCCH releasing SPS configured grant type-1 with PUCCH in the same slot based on PDCCH-to-PUCCH configured values.

	For each candidate, one HARQ-ACK bit is reported.

Proposal 15	For Type-2 codebook:

	UE monitors occasions for PDCCH carrying SL grant or PDCCH releasing SL configured grant type-1 with PUCCH in the same slot based on PSFCH-to-PUCCH gap signalled in DCI.

	For each PDCCH occasion at most one HARQ-ACK bit is reported.