3GPP TSG-RAN WG1 Meeting #100-e	  R1-2001289

Online, February 24th – March 6th, 2020

Agenda Item:	7.2.8.2

Source:	Ericsson

Title:	Outcome of email thread [100e-NR-Pos-ULRS-04]

Document for:	Discussion

# 1 Introduction

This document tracks the email discussion thread #4 for the agenda item UL Reference Signals for NR Positioning, approved by the chairman under:

[100e-NR-Pos-ULRS-04] E-mail discussion/approval to resolve the issue of spatial relation information for SRS for positioning (default, MAC CE update) by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2 – Florent (Ericsson)

The email discussion runs from 02/24 to 02/27. Once concluded, this document will be uploaded to provide a summary of the discussion.

At the conclusion of the email discussion, the following email discussion consensus was reached:

Proposal for email discussion consensus:

The feature of default spatial relation info is not applicable to SRS for positioning

The feature of using one MAC CE to activate one same spatial relation info for multiple SRS resources in multiple CCs is not applicable to SRS for positioning.

The text proposal of Section 3.1 in [R1-20NNNN email discussion #4 document] is endorsed.

# 2 Proposals

## 2.1 Default Spatial relation information for SRS for positioning

The proposals on this discussion is from R1-2000463 proposals 2 and 3.

The feature of default spatial relation info is not applicable to SRS for positioning

The feature of using one MAC CE to activate one same spatial relation info for multiple SRS resources in multiple CCs is not applicable to SRS for positioning.

Endorse the corresponding text proposal below, corresponding to proposal 1 and 2

Text proposal for Proposal 1 and 2: In TS 38.214: Section 6.2.1 UE sounding procedure <omitted text> When a spatialRelationInfo is activated/updated for a semi-persistent or aperiodic SRS resource by a MAC CE for a set of CCs/BWPs, where the applicable list of CCs is indicated by higher layer parameter [applicableCellList], the spatialRelationInfo is applied for the semi-persistent or aperiodic SRS resource(s), except for the SRS resource configured by the higher layer parameter [SRS-for-positioning], with the same SRS resource ID for all the BWPs in the indicated CCs. When the higher layer parameter enableDefaultBeamPlForSRS is set ‘enabled’, and if the higher layer parameter spatialRelationInfo for the SRS resource, except for the SRS resource with the higher layer parameter usage in SRS-ResourceSet set to 'beamManagement' or for the SRS resource with the higher layer parameter usage in SRS-ResourceSet set to ‘nonCodebook’ with configuration of associatedCSI-RS or for the SRS resource configured by the higher layer parameter [SRS-for-positioning], is not configured in FR2 and if the UE is not configured with higher layer parameter(s) pathlossReferenceRS, the UE shall transmit the target SRS resource - with the same spatial domain transmission filter used for the reception of the CORESET with the lowest controlResourceSetId in the active DL BWP in the CC. - with the same spatial domain transmission filter used for the reception of the activated TCI state with the lowest ID applicable to PDSCH in the active DL BWP of the CC if the UE is not configured with any CORESET in the CC <omitted text>

| Company | Comment |
| --- | --- |
| Huawei/HiSilicon | Support in general. However, we suggest to change the first paragraph as below. The second paragraph is OK as is. When a spatialRelationInfo is activated/updated for a semi-persistent or aperiodic SRS resource configured by the higher layer parameter SRS-Resource by a MAC CE for a set of CCs/BWPs, where the applicable list of CCs is indicated by higher layer parameter [applicableCellList], the spatialRelationInfo is applied for the semi-persistent or aperiodic SRS resource(s), except for the SRS resource configured by the higher layer parameter [SRS-for-positioning], with the same SRS resource ID for all the BWPs in the indicated CCs. In response to Oppo’s comments: The current running CR for TS 38.331 provides SRS for positioning as below. To follow the RAN1 recommendation of a new IE for configuring SRS for positioning, the existing IEs SRS-Resource and SRS-ResourceSet clearly will not be used to configure SRS for positioning for sure. Names in TS 38.214 should be aligned with ASN.1. SRS-Config ::= SEQUENCE { srs-ResourceSetToReleaseList SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSetId OPTIONAL, -- Need N srs-ResourceSetToAddModList SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet OPTIONAL, -- Need N srs-ResourceToReleaseList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-ResourceId OPTIONAL, -- Need N srs-ResourceToAddModList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource OPTIONAL, -- Need N tpc-Accumulation ENUMERATED {disabled} OPTIONAL, -- Need S ..., [[ srs-Type2-ResourceSetToReleaseList-r16 SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-Type2-ResourceSetId-r16 OPTIONAL, -- Need N srs-Type2-ResourceSetToAddModList-r16 SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-type2-ResourceSet-r16 OPTIONAL, -- Need N srs-Type2-ResourceToReleaseList-r16 SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Type2-ResourceId-r16 OPTIONAL, -- Need N srs-Type2-ResourceToAddModList-r16 SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Type2-Resource-r16 OPTIONAL -- Need N ]] Editor’s Note: The name of the IE is FFS. } |
| ZTE | Either the proposed TP or the change by HW is fine. After getting the new version of 38.331, editor will update RAN1 specs anyway. |
| OPPO | Support the TP. Regarding Huawei’s revision, we think it can be discussed later. The SRS-Resource may be configured in [SRS-for-positioning] based on the current 214 or the parameter list. Thus SRS-Resource cannot be used to differentiate normal SRS from SRS for positioning based on the current version of 214. Anyway, just as ZTE said, it can be updated according to 38.331 |
| CATT | Fine with the proposals and TP. |
| Nokia | We support the TP, we are okay with the suggested change by HW as well. |
| Qualcomm | Fine with the proposals 1 and 2 and the TPs. |
| vivo | We’re okay with the proposal 1, 2 and the TPs. |
| Futurewei | Ok for both proposals |
| LGE | We are fine with the proposal 1, 2, and the TPs. |
| Fraunhofer | We are fine with both proposals. |
| Intel Corporation | Agree with proposals |
| MTK | The wording provided by Huawei is kind of odd to me (I am not a native English speaker ): When a spatialRelationInfo is activated/updated for a semi-persistent or aperiodic SRS resource configured by the higher layer parameter SRS-Resource by a MAC CE for a set of CCs/BWPs Where, ”by the higher layer parameter SRS-Resource” is for ”SRS resource” ”by a MAC CE” is for ”spatialinfo is activated/updated” So, ”by a MAC CE” jumps quite far for ”spatialinfo is activated/updated” We suggest to change the wording in below, When a spatialRelationInfo is activated/updated by a MAC CE for a semi-persistent or aperiodic SRS resource configured by the higher layer parameter SRS-Resource for a set of CCs/BWPs |
| Sony | Support the proposals. |
| SS | Support |
| Ericsson | Support, with 1st paragraph chances as proposed by HW. |

## 2.2 MAC CE update of spatial relation info for SRS for positioning

The proposals on this discussion is from  R1-2000463 proposal  4.

The spatial relation info and pathloss RS of semi-persistent or aperiodic SRS for positioning can only be configured in RRC signaling.

Endorse the corresponding text proposal below, corresponding to  proposal 4

Text proposal for Proposal 4: In TS 38.214: Section 6.2.1 UE sounding procedure <omitted text> For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource is set to 'semi-persistent': - when a UE receives an activation command, as described in clause 6.1.3.17 of [10, TS 38.321], for an SRS resource, and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command is transmitted in slot n, the corresponding actions in [10, TS 38.321] and the UE assumptions on SRS transmission corresponding to the configured SRS resource set shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. The activation command also contains spatial relation assumptions provided by a list of references to reference signal IDs, one per element of the activated SRS resource set, except for the SRS resource configured with the higher layer parameter [SRS-for-Positioning]. Each ID in the list refers to a reference SS/PBCH block, NZP CSI-RS resource configured on serving cell indicated by Resource Serving Cell ID field in the activation command if present, same serving cell as the SRS resource set otherwise, or SRS resource configured on serving cell and uplink bandwidth part indicated by Resource Serving Cell ID field and Resource BWP ID field in the activation command if present, same serving cell and bandwidth part as the SRS resource set otherwise. When the SRS is configured with the higher layer parameter [SRS-for-positioning], each ID in the list of reference signal IDs may also refer to a reference SS/PBCH block of a non-serving cell or DL PRS of a serving or non-serving cell indicated by a higher layer parameter. - if an SRS resource in the activated resource set is configured with the higher layer parameter spatialRelationInfo, the UE shall assume that the ID of the reference signal in the activation command overrides the one configured in spatialRelationInfo. <omitted text>

| Company | Comment |
| --- | --- |
| Huawei/HiSilicon | The functionality of MAC CE for SP SRS for positioning is not clear yet. So far based on our understanding, since the Rel-15 MAC for SP-SRS activation and spatial relation info update indicates the resource set ID of SRS-ResourceSet, instead of SRS for positioning, it may not be reused for activation of SP SRS for positioning. If RAN1 assumes a new MAC CE should be introduced for SP SRS for positioning, RAN1 should provide the input to RAN2 so as to clarify the following functionalities: - Activation of transmission at resource level or resource set level - Spatial relation initialization/update supported or not - Pathloss RS initialization/update supported or not - How to assign the ID to spatial relation and pathloss RS in the MAC CE. Note that the LCID pool for the MAC CE is quite limited. For simplicity, we suggest postpone SP-SRS support to Rel-17 when we could have a better design and make full use of a MAC CE. In response to Oppo’s comments: The whole text is related to SRS configured by SRS-Resource, not SRS configured by [SRS-for-positioning]. It means without change, activation/deactivation of SP-SRS has not been specified yet. Note that RAN2 already agreed that SP-SRS for positioning is supported today, but the functionality of the MAC CE associated with SP-SRS needs more RAN1 discussion. [Further update] Considering RAN2 is designing the new MAC CE for activating and potentially updating spatial relation and pathloss reference for the SP-SRS for positioning, we think that adopting the second change (deleting the context related to ID referring to the spatial relation indication for SP-SRS) is reasonable, also because the whole context is addressing SP-SRS configured by SRS-Resource, instead of SP-SRS configured by [SRS-for-positioning]. At the meantime, we suggest RAN1 should discuss the following issues regarding the MAC CE for SP-SRS for positioning, as we do not think RAN2 can handle this without RAN1’s guidance. - Applicability of the MAC CE to - Single resource set, or - Multiple resource sets across CCs/BWPs - Whether or not the MAC CE can update the spatial relations for each resource - Whether or not the MAC CE can update the pathloss reference for each resource set - How to indicate the resource used for the spatial relations and the pathloss reference in the MAC CE |
| ZTE | We doubt SP SRS for positioning can work in Rel-16. Based on Rel-15 spec, e.g. 38.321, one MACCE is used to activate/deactivate a SRS resource set and update the spatial relation info for each SRS resource within the resource set. In other words, MACCE has to be used for activating SP SRS even we agree the proposal 4. Since SRS resource list for positioning is separate with legacy SRS resource list based on the RAN2’s discussion, a new MACCE should be supported for positioning. If RAN2 cannot complete the new MACCE design, SP SRS will not work for positioning. Therefore, we suggest not to agree this proposal and TP for now. Let’s wait for RAN2’s decision. |
| OPPO | Support the TP. We understand the intention of Huawei and ZTE. However, at least the 2nd change in the TP should be agreed no matter SP-SRS for positioning is supported or not. |
| CATT | In our point of view, Proposal 4 try to solve the issue of potential huge overhead of MAC CE supporting the semi-persistent SRS-Pos, considering the spatial relation information in the MAC CE should include the configuration information of reference signals of neighboring cells. Currently, RAN2 is discussing how to modify the legacy MAC CE or introduce a new MAC CE to support the activation and deactivation of SP SRS-Pos, including how to update or define the field of spatial relation info for Rel-16 SRS-Pos. We are fine for proposal 4 and think this issue also needs to be discussed in RAN2. |
| Nokia | Given the current discussion ongoing in RAN2 we think it would be best to postpone this discussion until future meetings where we may have a better chance to resolve this issue. It seems unlikely we will solve SP-SRS issues during this e-meeting. |
| Qualcomm | The SP-SRS feature would be much more useful if we can update the spatial relation and pathloss reference with a MAC CE command (as shown now in the Ran1 specification) and not just with RRC. Note that this would be useful for a UE supporting just 1 SP-SRS resource set. Then, this 1 set could be used for the transmission towards different TRPs with different Tx Beam and Tx power. We understand that the SP-SRS for positioning is being discussed in RAN2 now, but from RAN1 perspective, changing the spatial relation Info and pathloss RS using MAC CE makes sense technically. Therefore, we are OK with the current specification, and we don’t agree with this TP. Lets leave RAN2 to progress in this issue. |
| vivo | We don’t support proposal 4 and TP. If MAC CE is used to active/deactive SP-SRS, we don’t quite understand why it cannot be used for spatial relation info and pathloss RS. Furthermore, on the concern of overhead, whether there’s too much overhead is up to RAN2 to decide if any new MAC CE design or other ways. Similarly, the spatial relation info of aperiodic SRS configured by MAC CE is introduced in Rel-16 for SRS for other usages. We don’t see any problem to support for positioning. |
| Futurewei | Should wait for RAN2 to complete its discussion. No TP in RAN1 at the moment. |
| LGE | Similar to Qualcomm and vivo, we also think that SP-SRS feature could be more useful with MAC-CE command updating the spatial relation information and path-loss reference, and we have not found a clear reason why MAC-CE update should be prevented. |
| Fraunhofer | We do not support this TP. There is a need to update the spatial relation and pathloss with a MAC CE command. |
| Intel Corporation | This aspect is under RAN2 scope/discussion. We do not see the need for this change now. |
| MTK | We think that the MAC CE usage for SP SRS for positioning can be further analyzed in Rel-17. |
| Sony | Keep the current spec as it is and continue the discussion after RAN2 finalized the on-going SP-SRS discussion. |
| SS | OK with the TP |
| Ericsson | If MAC CE based SRS transmission is to be used for positioning (depending on RAN2 outcome), then we can discuss the content of the MAC CE to include spatial relation updates. We prefer wanting for the RAN2 outcome, and if RAN1 input is needed to design an updated / new MAC CE we can provide it in a future meeting. |

# 3 Text Proposal

## 3.1 Default Spatial relation information for SRS for positioning

******** begin Text proposal for 38.214 ******

## 6.2	UE reference signal (RS) procedure

### 6.2.1	UE sounding procedure

<omitted text>

When a spatialRelationInfo is activated/updated for a semi-persistent or aperiodic SRS resource configured by the higher layer parameter SRS-Resource by a MAC CE for a set of CCs/BWPs, where the applicable list of CCs is indicated by higher layer parameter [applicableCellList], the spatialRelationInfo is applied for the semi-persistent or aperiodic SRS resource(s), except for the SRS resource configured by the higher layer parameter [SRS-for-positioning], with the same SRS resource ID for all the BWPs in the indicated CCs.

When the higher layer parameter enableDefaultBeamPlForSRS is set ‘enabled’, and if the higher layer parameter spatialRelationInfo for the SRS resource, except for the SRS resource with the higher layer parameter usage in SRS-ResourceSet set to 'beamManagement' or for the SRS resource with the higher layer parameter usage in SRS-ResourceSet set to ‘nonCodebook’ with configuration of associatedCSI-RS or for the SRS resource configured by the higher layer parameter [SRS-for-positioning], is not configured in FR2 and if the UE is not configured with higher layer parameter(s) pathlossReferenceRS, the UE shall transmit the target SRS resource

-	with the same spatial domain transmission filter used for the reception of the CORESET with the lowest controlResourceSetId in the active DL BWP in the CC.

-	with the same spatial domain transmission filter used for the reception of the activated TCI state with the lowest ID applicable to PDSCH in the active DL BWP of the CC if the UE is not configured with any CORESET in the CC

<omitted text>

******** end  Text proposal for 38.214 ******