3GPP TSG RAN WG1 Meeting #100bis-e	R1-2001571

E-meeting, April 20 - April 30, 2020

Agenda Item:	6.2.2.3

Source:	Huawei, HiSilicon

Title:	Corrections on scheduling of multiple DL/UL transport blocks

Document for:	Discussion and Decision

- Introduction

In NB-IoT Rel-15, the agreements related to multiple DL/UL transport blocks are summarized in [1]. In this contribution, we provide our views on the remaining issues.

- SC-PTM

In Rel-14, UE is not required to monitor Type2A-CSS (for SC-MTCH scheduling) while receiving NPDSCH carrying SC-MTCH. For SC-MTCH multiple TBs scheduling, the same NPDCCH monitoring behavior can be reused. Since UE has received DCI scheduling multiple SC-MTCH TBs, there is no need to monitor Type2A-CSS during the scheduling gap or the processing gap while the other CSS (type1/1A/2) are still be monitored.

Proposal 1: For SC-MTCH multiple TBs scheduling, UE is not required to monitor Type2A-CSS during a scheduling gap or a processing gap.

Proposal 2: Adopt the following text proposals in Annex A for TS 36.213.

- Unicast

In RAN1#99, the following agreement is achieved on how to handle multiple TBs scheduling for SPS transmission. However, it has not been captured yet in current spec.

Agreement A UE is not expected to receive a DCI scheduling more than one TB with CRC scrambled by SPS-C-RNTI

Proposal 3: Adopt the following text proposals in Annex B for TS 36.213.

- Conclusion

In this contribution, our views on the remaining issues of multiple TB scheduling are provided. The following proposals are made.

Proposal 1: For SC-MTCH multiple TBs scheduling, UE is not required to monitor Type2A-CSS during a scheduling gap or a processing gap.

Proposal 2: Adopt the following text proposals in Annex A for TS 36.213.

Proposal 3: Adopt the following text proposals in Annex B for TS 36.213.

- References

R1-1913595, “RAN1 agreements for Rel-16 Additional Enhancements for NB-IoT”, Futurewei, RAN1#99, Reno, USA, 18th – 22nd November 2019.

# Annex A

---------------------------------------------- Start of Text Proposal ------------------------------------------

-------------------------------------------- Unchanged parts omitted -----------------------------------------

## 16.6	Narrowband physical downlink control channel related procedures

-------------------------------------------- Unchanged parts omitted -----------------------------------------

An NB-IoT UE is not required to monitor NPDCCH candidates of an NPDCCH search space during an NPUSCH UL gap.

An NB-IoT UE is not required to monitor NPDCCH candidates of a Type2A-NPDCCH common search space during the scheduling gap or the processing gap.

-------------------------------------------- Unchanged parts omitted -----------------------------------------

----------------------------------------------- End of Text Proposal ------------------------------------------

# Annex B

---------------------------------------------- Start of Text Proposal ------------------------------------------

-------------------------------------------- Unchanged parts omitted -----------------------------------------

### 16.5.1	UE procedure for transmitting format 1 narrowband physical uplink shared channel

-------------------------------------------- Unchanged parts omitted -----------------------------------------

-	For ,

-	if the UE is configured with higher layer parameter multi-TB-UL-Unicast-Interleaving-config, and NPUSCH corresponding to a NPDCCH with DCI CRC scrambled by C-RNTI, and  where  for ,  otherwise.

-	NB-IoT UL slots  with  are associated with TBr+1 ,

-	otherwise,

-	NB-IoT UL slots  with  are associated with TBr+1 ,

-	A UE may assume that a DCI with CRC scrambled by SPS-C-RNTI schedules a single TB.

Table 16.5.1-1: for DCI format N0 for FDD.

| 0 | 8 |
| --- | --- |
| 1 | 16 |
| 2 | 32 |
| 3 | 64 |

-------------------------------------------- Unchanged parts omitted -----------------------------------------

----------------------------------------------- End of Text Proposal ------------------------------------------