3GPP TSG RAN WG1 Meeting #100bis-e	R1-2001604

e-Meeting, April 20th – 30th, 2020

Source: 	ZTE

Title: 	NR eMIMO UE features

Agenda item:	7.2.11.6

Document for:	Discussion/Decision

# Introduction

After RAN1#99 meeting, all the Rel-16 RAN1-led WIs have been declared as finished. To facilitate the UE features discussion, two email discussions on NR and LTE Rel-16 UE features were agreed to collect companies’ inputs to update the current UE feature list. In this contribution, we give our views on NR eMIMO UE features.

# Discussion on each feature group

In this section, each of the MIMO feature groups is discussed and the corresponding proposed updates are done in the UE feature table shown in the Appendix.

16-1: Multi-beam operation

The following includes more detailed comments for features of 16-1 multi-beam operation.

Component-2 in 16-1a: FFS: Support of group-based reporting for L1-SINR

Group-based reporting has been agreed to be supported optionally in Rel-16. The following modification for the component shall be made, in order to support a UE capability reporting of support of group-based reporting for L1-SINR. Further, it should be a separate  feature group from maximum number of ZP-IMR or NZP-IMR configuration for a normal L1-SINR reporting, which is the requirement for both group or non-group based L1-SINR reporting.

“2.	FFS: Support of group-based reporting for L1-SINR”

Proposal 1: Split FG16-1a into two FGs separately, and support a capability reporting of support of group-based reporting.

16-1b: TCI state activation and spatial relation update

The motivation for “maximum number of Simultaneous TCI state activation across multiple CCs/Simultaneous spatial relation update across multiple CCs/PUCCH resource groups per BWP for simultaneous spatial relation update” is unclear. Considering the flexibility of TCI/spatial relation configuration and backward compatibility for Rel-15 UE, we suggest to define this FG as whether to support these features directly rather than the maximum values, and the maximum values are based on the existing agreements or Rel-15 UE capability reporting for active TCI/spatial relation in a single CC.

- The maximum number of simultaneous TCI state activation across multiple CCs is determined according to Component 2-4: TCI states for PDSCH.

- The maximum number of simultaneous spatial relation update across multiple CCs is determined according to Component 2-59: Configured spatial relations.

- The maximum number of PUCCH resource groups is 4 according to agreements.

Besides, considering that the above three features are independent, FG-16-1b should be split into three FGs separately.

16-1d: [Support of / The maximum number of] spatial relation update for AP-SRS via MAC CE

It is confusing about "the maximum number" of spatial relation update for AP-SRS. We prefer to update the component as "Support of spatial relation update for AP-SRS via MAC CE" as we do for other components for TCI/spatial relation update.

- For component 16-1d, the maximum number of spatial relation update for AP-SRS via MAC CE is based on the recent Component 2-59: Configured spatial relations.

16-1e: Pathloss reference RS activation via MAC CE

We prefer to have a fixed value for maximum number of activated pathloss reference RS(s) rather than according to UE capability. As a baseline, up to 4 activated pathloss reference RS(s) can be supported mandatory without capability signaling, which is the same as in Rel-15.

Component-5 in 16-1f: FFS: Support of PUCCH-BFR

The support of PUCCH-BFR shall be mandatory with capability signaling for FR2 and optional for FR1 (as we did for PCell-BFR), in order to guarantee the effectiveness/low-latency of SCell-BFR. Hence, this component 5 in 16-1f should be removed, and the UE supporting the basic feature group FG 16-1f should support both SR based and PUCCH based-BFR.

Component-7 in 16-1f: FFS: Density of CSI-RS for new beam identification for SCell BFR

For frequency density of CSI-RS for SCell BFR, we prefer to align it with PCell-BFR, i.e., there is not any further restriction for frequency density of CSI-RS for SCell BFR.

16-2: Multi-TRP

In FG16-2a: Support of out of order

Regarding the component 4-6 about out of order operation in FG 16-2a, we think they should be basic components. Once Multi-DCI is supported by UE, out of order operation for PDCCH to PDSCH, PDSCH to HARQ-ACK and PDCCH to PUSCH must be supported because of independent scheduling between two TRPs. Otherwise, it is impossible for gNB to implement MTRP in the case of non-ideal backhaul which is the typical scenario for Multi-DCI based MTRP. Therefore, the component 4-6 in FG16-2a, i.e. out of order operation for PDCCH to PDSCH, PDSCH to HARQ-ACK and PDCCH to PUSCH should be the basic components.

In FG16-2a: Support of two PDSCH scrambling sequences

Individual capability report is not needed for the support of two scrambling IDs. The support of two scrambling IDs is the basic feature for Multi-DCI base MTRP. Two PDSCH can be from two TRPs which may belong to different cells. So whether to support separate scrambling ID just depends on the support of  FG 16-2a. Therefore, we propose to make this component be the basic component.

In FG16-2a: Support of separate HARQ-ACK

Both separate and joint ACK/NACK feedback are agreed in Rel-16, one of them should be the basic component. Otherwise, M-DCI based MTRP cannot work if UE does not support both. Because the main scenario is non-ideal backhaul for M-DCI based MTRP, we suggest to make the optional component 4, i.e. support of separate feedback be the basic component.

Merge 16-2b-1 and 16-2b

We think 16-2b-1, i.e., the support of SDM scheme, should be merged into 16-2b. Further, the support of DCI indicating 2 TCI states by a codepoint and DMRS ports within two CDM groups and the support of DMRS entry {0,2,3} should be basic components in the merged FG. The reason is that SDM scheme is the basic functionality of single DCI based MTRP based on Rel-16 discussion, and other schemes including FDM and TDM schemes have been agreed as optional features for single-DCI based MTRP. If UE reports not to support any of these schemes, 16-2b is not supported as well. Further, there is no RRC signaling to disable SDM scheme if UE supports two TCI states indicated by one codepoint. Hence we have the following proposal. It is okay to put the support of two PTRS as an optional feature for FR1, but this feature should be mandatory with capability signaling if 16-2b is supported.

Proposal 2: Merge 16-2b-1 into 16-2b. The support of DCI indicating 2 TCI states by a codepoint and DMRS ports within two CDM groups and the support of DMRS entry {0,2,3} should be basic components in the merged FG.

In 16-2b: Support of MAC CE to activate multiple TCI states for a TCI codepoint

To activate two TCI states for one TCI codepoint is a basic feature to implement single-DCI based multi-TRP transmission. Without this feature, multi-TRP transmission cannot be implemented. Hence we think the “FFS” should be removed as: FFS: Support of MAC CE to activate multiple TCI states for a TCI codepoint.

16-3 eType II codebook

Since it is agreed that eType II codebook and eType II port selection codebook will have separate UE capabilities, it makes more sense to have separate feature groups for eType II and eType II port selection, which is same as Rel-15.

Proposal 3: Split 16-3 into two separate feature groups: eType II and eType II port selection.

Number of PMI sub-bands in 16-3a and 16-3b

We have the following agreement in RAN1#99.

Agreement

On the # of PMI subbands:

- Mandatory support for R=1

- Optional support for N3>19

FFS: whether it is mandatory or optional for R=2 and N3<=19

- To be discussed during the Rel-16 UE capabilitity discussions

Following the above agreement, the decision is to choose from the following two alternatives.

- Alt 1: R=2 and N3<=19 is mandatory if eType-II codebook is supported

- Alt 2: R=2 and N3<=19 is optional if eType-II codebook is supported

The selection between the above two alternatives is also related to the detailed parameter for this component.

- If Alt 1 is selected, it implies that N3<=19 is mandatory regardless of R, and N3>19 is optional. Then the UE capability parameter should be N3.

- Otherwise, R=1 is mandatory, whereas R=2 is optional. Then the UE capability parameter should be R.

The UE complexity on supporting large number of N3 is caused by the hardware (e.g., computation complexity and buffering) limits.

- Since Rel-15 sub-band reporting has already required 19 sub-bands, supporting N3 <= 19 now should have no extra UE complexity compared with what we already have for Rel-15 commercial UEs. Furthermore, for N3<=19, NW can still realize the performance gain from R=2 for CQI sub-band number < 10. Especially for large CQI sub-band size cases(e.g. 16 and 32), which may not need to configure more than 10 CQI sub-bands, allowing R=2 configuration will significantly improve compression efficiency. Hence to make N3<=19 mandatory will make the best utilization of current UE hardware resource limits and allow more NW flexibility for small CQI sub-band number.

- For N3>19, we can depend on UE capability signaling to further realize the performance gain of R=2 for large bandwidth in future.

Considering the above, we think Alt 1 is a better trade-off between UE complexity and NW performance than Alt 2. In addition, in the Rel-16 Type II CSI framework, one-step FD indicator is used for N3<=19, and two-step FD indicator is used for N3>19. Hence Alt 1 can avoid mandatory UEs implement two different schemes and align with the general design principle for eType-II CSI.

Therefore, UE signals the maximum N3 value for this component. This value shall be larger than 19. It also makes sense to have this signaling jointly reported with the maximum number of CSI-RS ports per resource, the maximum number of active CSI-RS resources per band and the maximum number of active CSI-RS ports per band. All these parameter are related to the hardware resource UE needs to prepare for computing eType II or eType II port selection CSI.

Proposal 4: On the number of PMI subbands, UE signals the maximum N3 value, which shall be larger than 19. Support to report it jointly with the maximum number of CSI-RS ports per resource, the maximum number of active CSI-RS resources per band and the maximum number of active CSI-RS ports per band.

Supported codebook parameter combinations

It has agreed previously that for regular eType II, codebook parameter combinations 1-6 are basic components, and codebook parameter combination 7-8 is optional. It is not reflected in the current UE feature list.

Supported ranks

In current UE feature list, there is a basic component for eType II and eType II PS called rank restriction. We believe it is a mistake to have it here. What should be here is the support of rank 1 and 2, since we have agreed that rank 1 and 2 are basic, and rank 3 and 4 are optional.

Maximum number of AP CSI settings

It has been agreed that the maximum number of AP CSI settings for one BWP is increased from 4 to 8. It is not related to either eType II or eType II port selection. Hence we suggest to add one more FG 16-3c to indicate the support of maximum 8 AP CSI report settings for one BWP, and it does not reply on the support of eType II or eType II port selection.

16-5 Power-efficient UL transmission

We have the following views on the detailed components in this feature group.

16-5a/5b/5c:	Number of Tx to support mode 0/1/2: {2Tx, 4Tx, 2Tx_4Tx}

As a basic principle, for full Tx power transmission, the corresponding PUSCH power scaling factor and extension of UL codebook should be determined only according to the full Tx mode and TPMI-group reporting for full Tx power in mode2. The motivation for “16-5a/5b/5c:	Number of Tx to support mode 0/1/2: {2Tx, 4Tx, 2Tx_4Tx}” is unclear with the following analysis, and those three components also strongly restrict the flexibility of gNB configuration.

- For mode 0, all PAs of the UE shall support full Tx power, regardless of 2Tx, or 4Tx.

- For mode 1, the scaling factor is the same as Rel-15, and only extension of the UL codebook is supported herein. It does not make sense that the extension of codebook is only applied to a specific number of Tx.

- For mode 2, this component is also redundant, considering that the component 16-5-6: TPMI group which delivers full power can fully provide the UE capability of which TPMIs can support full power.

Component-3 in 16-5b: FFS: New UL codebook set(s) per supported Tx

The new UL codebook set(s) are fixed as specified in current specification. Consequently, the bullet should be removed. The new UL codebook sets should be supported if mode 1 is supported.

Component-4 in 16-5c: FFS: Number of ports per SRS resource

We are not sure why this component is needed. The motivation is not clear to us.

Component-5 in 16-5c: FFS: Maximum number of different spatial relation info for all SRS resources for usage set to ‘codebook’ in a resource set

We think this component is needed. We can consider the candidate values of "1", "2" or "4" for this component.

16-6a low PAPR RS for PUSCH

Since the design on Low PAPR DMRS for CP-OFDM and for DFT-S-OFDM are completely different, we think FG 16-6a should be split into two FGs. One is for CP-OFDM, the other is for DFT-S-OFDM.

The reason is that for CP-OFDM, the enhanced low PAPR DMRS is still inserted in frequency domain, the only change compared with Rel-15 is just on sequence initialization.

However, for DFT-S-OFDM, the enhanced low PAPR DMRS is inserted in time domain. Then, DFT process is needed to transform DMRS from the time domain to the frequency domain. The final DMRS sequence in frequency domain is not constant amplitude anymore. Hardware implementation change at both gNB side and UE side may be needed for this feature group. Some UEs with less capability maybe only support the new DMRS for CP-OFDM.

Therefore, we suggest to report the UE capability separately on low PAPR DMRS with and without transform precoding.

Proposal 5: Split FG16-6a into two FGs separately for CP-OFDM and DFT-s-OFDM.

# Conclusion

In this contribution, the following proposals on NR Rel-16 MIMO UE features are given.

Proposal 1: Split FG16-1a into two FGs separately, and support a capability reporting of support of group-based reporting.

Proposal 2: Merge 16-2b-1 into 16-2b. The support of DCI indicating 2 TCI states by a codepoint and DMRS ports within two CDM groups and the support of DMRS entry {0,2,3} should be basic components in the merged FG.

Proposal 3: Split 16-3 into two separate feature groups: eType II and eType II port selection.

Proposal 4: On the number of PMI subbands, UE signals the maximum N3 value, which shall be larger than 19. Support to report it jointly with the maximum number of CSI-RS ports per resource, the maximum number of active CSI-RS resources per band and the maximum number of active CSI-RS ports per band.

Proposal 5: Split FG16-6a into two FGs separately for CP-OFDM and DFT-s-OFDM.

Based on these proposals, an updated UE feature list on MIMO is provided in the Appendix. The principle is that each FG is associated with one supported feature.

# Reference

[1] R1-2001484, RAN1 UE features list for Rel-16 NR after RAN1#100-E, AT&T, NTT DOCOMO, INC

# Appendix: NR_eMIMO

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16. NR_eMIMO | 16-1 | Multi-beam operation | The components in FG 16-1 are separated into individual FGs from 16-1a to 16-1g as follows. |  |  |  |  |  |  |  |  |  | Optional |
|  | 16-1a | L1-SINR reporting | - The maximum number of L1-SINR based beam measurement and reporting based on ZP IMR and/or NZP IMR (FFS details on the sub-components, e.g., FG 2-24) | TBD |  | N/A |  | TBD [Per band] | N | N |  |  | TBD |
|  | 16-1b-1 | TCI state activation and spatial relation update | - Support of Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH (FFS whether to be a separate UE feature, e.g. 16-1b) - FFS: details on whether/how to indicate band pairs which can share the same DL TCI state | Component 1: 2-1, 2-4 |  | N/A |  | TBD [Per BC or per band] | N | Y |  |  | TBD |
|  | 16-1b-2 | Spatial relation update across multiple CCs | - Support of Simultaneous spatial relation update across multiple CCs: AP-SRS, SP-SRS - FFS: details on whether/how to indicate band pairs which can share the same UL spatial relation info | Component 1: 2-59, 2-60 |  | N/A |  | TBD [Per BC or per band] | N | Y |  |  | TBD |
|  | 16-1b-3 | Spatial relation update for PUCCH group | Support of PUCCH resource groups per BWP for simultaneous spatial relation update | 2-53, 2-59, 4-24 |  | N/A |  | TBD [Per BC or per band] | N | Y |  |  | TBD |
|  | 16-1c | Default spatial relation | Support of default spatial relation and pathloss reference RS for dedicated-PUCCH/SRS and PUSCH scheduled by DCI format 0_0 | 2-53, 2-59 |  | N/A |  | TBD [Per band] | N | Y |  |  | TBD |
|  | 16-1d | MAC CE spatial relation update for AP-SRS | Support of spatial relation update for AP-SRS via MAC CE (FFS whether to be a separate UE feature, e.g. 16-1c) | 2-53, 2-59 |  | N/A |  | TBD [Per UE] | N | N |  |  | TBD |
|  | 16-1e | Pathloss reference RS activation via MAC CE | - The support of pathloss reference RS activation for PUSCH/SRS/PUCCH via MAC CE | 8-2, 8-3 |  | N/A |  | TBD [Per UE] | N | N |  |  | TBD |
|  | 16-1f | SCell beam failure recovery | - The maximum number of SCells for SCell beam failure recovery (FFS whether to be a separate UE feature, e.g. 16-1d) - FFS: The maximum number of CSI-RS and/or SSB resources for new beam identification of SCell BFR [across all CCs / within a slot across all CCs / per CC] (FFS to replace this component to 14) | 2-31 |  | N/A |  | TBD | N |  |  |  | TBD |
|  | 16-1g | FFS: Resources for beam management, pathloss measurement, and BFR | - FFS: The maximum number of SSB/CSI-RS resources across all CCs for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, and new beam identification. - FFS: The maximum number of SSB/CSI-RS resources across all CCs for pathloss measurement - FFS: The maximum number of SSB/CSI-RS resources across all CCs for BFD - FFS: The maximum number of SSB/CSI-RS resources across all CCs for new beam identification |  |  | N/A |  | TBD | N |  |  |  | TBD |
|  |  | 16-1h |  | Group-based reporting for L1-SINR |  | Support of group-based reporting for L1-SINR |  | 16-1a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 16-2 | Multi-TRP operation | The previous components in FG 16-2 are separated into individual FGs from 16-2a to 16-2b-6 as follows. | TBD |  | N/A |  | TBC [Per band] | N | TBD |  |  | TBD |
|  | 16-2a | Multi-DCI based multi-TRP | - The maximum number of CORESETs configured per “PDCCH-Config” - The maximum number of CORESETs configured per CORESETPoolIndex ( if CORESETPoolIndex is not configured, it is assumed CORESETPoolIndex = 0) per “PDCCH-Config” - The value of R=[1,2] for BD/CCE - Support of fully/partially time/frequency overlapped PDSCH reception (PDSCHs overlapping types in time and frequency domain) - Support of out-of-order operation for PDCCH to PDSCH - Support of out-of-order operation for PDSCH to HARQ-ACK - Support of out-of-order operation for PDCCH to PUSCH - FFS: The maximum number of activated TCI states - FFS: The maximum number of MIMO layers of scheduled PDSCHs - FFS: the maximum number of CCs supporting multi-DCI based multi-TRP - Support of two PDSCH scrambling sequences per serving cell - Support of separate HARQ-ACK | TBD |  | N/A |  | TBD [per band / per FSPC] | N | TBD |  |  | TBD |
|  | 16-2a-1 | Separate CRS rate matching | Whether the UE shall rate match around configured CRS patterns which is associated with CORESETPoolIndex (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex | 16-2a, TBD |  | N/A |  | TBD [per band / per FSPC] | N | TBD |  |  | TBD |
|  | 16-2a-3 | Default QCL enhancement fr multi-DCI based multi-TRP | Support of default QCL assumption per CORESETPoolIndex | 16-2a, TBD |  | N/A |  | TBD [per band / per FSPC] | N | TBD |  |  | TBD |
|  | 16-2a-4 | Joint HARQ-ACK for multi-DCI based multi-TRP | - Support of joint HARQ-ACK | 16-2a, TBD |  | N/A |  | TBD [per band / per FSPC] | N | TBD |  |  | TBD |
|  | 16-2a-5 | Two TDMed long PUCCHs in a slot | Support of two TDMed long PUCCHs in a slot | 16-2a |  |  |  |  |  |  |  |  |  |
|  | 16-2b | Single-DCI based multi-TRP | - Support of MAC CE to activate multiple TCI states for a TCI codepoint - Support of DCI indication of of 2 TCI states by a codepoint and DMRS ports within two CDM groups - Support of DMRS entry {0, 2, 3} | TBD |  | N/A |  |  | N | TBD |  |  | TBD |
|  | 16-2b-1 | Two PTRS ports for single-DCI based multi-TRP | - Whether supporting two PTRS ports | 16-2b, TBD |  | N/A |  |  | N | TBD |  |  | TBD |
|  | 16-2b-2 | Single-DCI based FDMSchemeA | Support of FDMSchemeA | 16-2b, TBD |  | N/A |  |  | N | TBD |  |  | TBD |
|  | 16-2b-3 | Single-DCI based FDMSchemeB | - Support of FDMSchemeB - For FDMSchemeB, whether the UE can support CW soft combining | 16-2b, TBD |  | N/A |  | TBD [per FSPC] | N | TBD |  |  | TBD |
|  | 16-2b-4 | Single-DCI based TDMSchemeA | - Support of TDMSchemeA - Supported maximum TBS size for TDMSchemeA | 16-2b, TBD |  | N/A |  |  | N | TBD |  |  | TBD |
|  | 16-2b-5 | Single-DCI based inter-slot TDM | - Support of RepNumR16 in PDSCH-TimeDomainResourceAllocation and the maximum value of RepNumR16 - Supported maximum TBS size according to RepNumR16 in PDSCH-TimeDomainResourceAllocation - FFS: TCI state mapping to PDSCH transmission occasions (Cyclical mapping or Sequential mapping) | 16-2b, TBD |  | N/A |  |  | N | TBD |  |  | TBD |
|  | 16-2b-6 | Default QCL enhancement fr single-DCI based multi-TRP | Support of default QCL assumption with two TCI states | 16-2b, TBD |  | N/A |  |  | N | TBD |  |  | TBD |
|  | 16-3 | eType-II codebook | The components in FG 16-3 are separated into individual FGs from 16-3a to 16-3b-4 as follows. | TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
|  | 16-3a | Regular eType-II | - Support of parameter combinations 1-6 - Support of PMI sub-bands with value N3<=19 - Rank 1 and 2 - UCI omission | TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
|  | 16-3a-1 | CSI-RS and number of PMI subbands for eType-II | For regular eType-II: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources across all CCs simultaneously, total # of Tx ports across all CCs simultaneously, Max # of PMI subbands N3}, where N3>=19 | 16-3a, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
|  | 16-3a-2 | Rank for eType-II | Support of rank 3,4 for regular eType-II | 16-3a, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
|  | 16-3a-3 | CBSR for eType-II | Support of CBSR for regular eType II | 16-3a, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
|  | 16-3a-4 | Codebook parameter combination 7-8 for eType II | Support of codebook parameter combinations 7-8 | 16-3a, TBD |  |  |  |  |  |  |  |  |  |
|  | 16-3a-5 | FFS: Mixed codebook type for eType-II | FFS: Support of mixed codebook types | 16-3a, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
|  | 16-3b | Port selection eType-II | - FFS: {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II - 6 parameter combinations (combos with L=6 don’t apply) (FFS: Value of L per the number of antenna ports) - Number of PMI sub-bands (R=1 is mandatory, FFS: R=2 is mandatory or optional) Support of PMI sub-bands with value N3<=19 - Rank restriction - UCI omission | TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
|  | 16-3b-1 | CSI-RS and number of PMI subbands for port selection eType-II | For port selection eType-II: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources across all CCs simultaneously, total # of Tx ports across all CCs simultaneously, Max # of PMI subbands N3}, where N3>=19 | 16-3b, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
|  | 16-3b-2 | Rank for port selection eType-II | Support of rank 3,4 for port selection eType-II | 16-3b, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
|  | 16-3b-4 | FFS: Mixed codebook type for port selection eType-II | FFS: Support of mixed codebook types | 16-3b, TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
|  | 16-3c | Number of AP-CSI report settings per BWP | Support up to 8 configured aperiodic CSI report setting per BWP | TBD |  |  |  |  |  |  |  |  |  |
|  | 16-4 | Low PAPR DMRS for DL | Low PAPR DMRS for PDSCH | TBD |  | N/A |  | Per UE | N | N |  |  | [Optional] |
|  | 16-5 | Power-efficient UL transmission | UL full power transmission (mode [0], 1, 2) - Supported UL full power transmission mode ([mode 0], mode 1, mode 2, [mode1AndMode2]) - FFS: New UL codebook set(s) per supported mode - The maximum number of SRS resources in set with different number of ports [for usage set to ‘codebook’] - FFS: Number of ports per SRS resource - FFS: Maximum number of different spatial relationship info for all SRS resources for usage set to ‘codebook’ in a resource set - TPMI group which delivers full power Note: UE indicating mode 2 shall support full power transmission for 1 antenna port The components in FG 16-5 are separated into individual FGs from 16-5a to 16-5c as follows. | [2-13, 2-14] for #1-#3 |  | N/A |  | Per UE | N | N |  |  | TBD |
|  | 16-5a | UL full power transmission mode 0 | - Supported UL full power transmission [mode 0] | 2-13, 2-14 |  | N/A |  | FFS: Per FS or Per band or Per band per BC | N | N |  |  | TBD |
|  | 16-5b | UL full power transmission mode 1 | - Supported UL full power transmission mode 1 | 2-13, 2-14 |  | N/A |  | FFS: Per FS or Per band or Per band per BC | N | N |  |  | TBD |
|  | 16-5c | UL full power transmission mode 2 | - Supported UL full power transmission mode 2 - The maximum number of SRS resources in set with different number of ports [for usage set to ‘codebook’]. FFS on details for supported number of Tx. - Maximum number of different spatial relation info for all SRS resources for usage set to ‘codebook’ in a resource set - TPMI group which delivers full power. FFS on details for supported number of Tx. Note: UE indicating mode 2 shall support full power transmission for 1 antenna port | 2-13, 2-14 |  | N/A |  | FFS: Per FS or Per band or Per band per BC | N | N |  |  | TBD |
|  | 16-6 | Low PAPR DMRS for UL | - For PUSCH without transform precoding - For PUSCH with transform precoding and with pi/2 BPSK modulation - For PUCCH format 3 and/or PUCCH format 4 with transform precoding and with pi/2 BPSK modulation The components in FG 16-6 are separated into individual FGs 16-6a and 16-6b as follows. | TBD for #1-#3 |  | N/A |  | Per UE | N | N |  |  | TBD |
|  | 16-6a | Low PAPR DMRS for PUSCH without transform precoding | For PUSCH without transform precoding | TBD |  | N/A |  | FFS: Per band | N | N |  |  | FFS: Optional with capability signalling |
|  | 16-6b | Low PAPR DMRS for PUCCH | For PUCCH format 3 and/or PUCCH format 4 with transform precoding and with pi/2 BPSK modulation | TBD |  | N/A |  | FFS: Per band | N | N |  |  | FFS: Optional with capability signalling |
|  |  | 16-6c |  | Low PAPR DMRS for PUSCH with transform precoding and with pi/2 BPSK |  | For PUSCH with transform precoding and with pi/2 BPSK modulation |  | TBD |  |  |  | N/A |  |  |  | FFS: Per band |  | N |  | N |  |  |  |  |  | FFS: Optional with capability signalling |  |