3GPP TSG RAN WG1 #100bis		R1-2001656

e-Meeting, April 20th – 30th, 2020

Source:	vivo

Title:	Remaining issues on wideband operation in NRU

Agenda Item:	7.2.2.2.5

Document for:	Discussion and Decision

- Introduction

This contribution discusses remaining aspects on wideband operation for NRU.

- Discussion

  - RB set definition

Referring to the latest running RRC CR [1], RAN2 decides to use start CRB and number of CRBs to configure the intra carrier guard band. Accordingly, the corresponding text in 38.214 to define the RB sets should be updated based on this change.

Proposal 1: RAN1 adopts the following TP1 for RB set definition.

--------------------------------------------- Start TP1 for Section 7 of TS 38.214 ----------------------------------------

For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with  NRB-set-1  intra-cell guard bands on a carrier, each defined by start CRB and end number of CRBs, GB sstart,μ  and  NGB sμ , respectively. The intra-cell guard bands separate  NRB-set RB-sets, each defined by start and end CRB, RB sstart,μ and  RB send,μ, respectively.  UE determines RB 0start,μ=Ngridstart,μ, RBNRB-set-1end,μ=Ngridstart,μ+Ngridsize,μ  , and the remaining start and end CRBs as RB send,μ=GB sstart,μ-1 and RB s+1start,μ=GB send,μ+1GB sstart,μ+NGB sμ. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ].

---------------------------------------------- End TP1 for Section 7 of TS 38.214 ----------------------------------------

  - SS set configuration with freqMonitorLocations-r16

Agreement:

For a search space set configuration associated with multiple monitoring locations in the frequency domain (as per the previous agreement defining such a search space set associated with a CORESET confined within an LBT bandwidth):

- PRBs allocated by frequencyDomainResources in the CORESET configuration are confined within one of LBT bandwidths within the BWP corresponding to the CORESET.

- Within the search space set configuration associated with the CORESET, each of the one or more monitoring locations in the frequency domain corresponds to (and is confined within) an LBT bandwidth and has a frequency domain resource allocation pattern that is replicated from the pattern configured in the CORESET.

- CORESET parameters other than frequency domain resource allocation pattern are identical for each of the one or more monitoring locations in the frequency domain.

- Include this and the prior agreement on this issue in an LS to RAN2

Agreement:

For a search space set configuration associated with multiple monitoring locations in the frequency domain (as per the previous agreement defining such a search space set associated with a CORESET confined within an LBT bandwidth):

- PRBs allocated by frequencyDomainResources in the CORESET configuration are confined within one of LBT bandwidths within the BWP corresponding to the CORESET.

- Within the search space set configuration associated with the CORESET, each of the one or more monitoring locations in the frequency domain corresponds to (and is confined within) an LBT bandwidth and has a frequency domain resource allocation pattern that is replicated from the pattern configured in the CORESET.

- CORESET parameters other than frequency domain resource allocation pattern are identical for each of the one or more monitoring locations in the frequency domain.

- Include this and the prior agreement on this issue in an LS to RAN2

In RAN1#98bis meeting, the following agreement is made:

However, this is not implemented in TS 38.213 regarding PDCCH candidate calculation. Therefore, the following proposal is made to capture this agreement.

Proposal 2: RAN1 adopts the following TP2 for SS set configuration with freqMonitorLocations-r16.

------------------------------------------ Start TP2 for Section 10.1 of TS 38.213 ----------------------------------------

For a search space set  associated with CORESET  and configured with freqMonitorLocations-r16, it comprises of K sub search space set where K is the number of RB sets indicated as ‘1’ by freqMonitorLocations-r16. For each sub search space set, CCE is indexed according to CORESET p configuration in each RB set respectively. For a search space set s associated with CORESET p without configuration of freqMonitorLocations-r16, or a sub search space set as defined above, the CCE indexes for aggregation level  corresponding to PDCCH candidate  of the search space set in slot  for an active DL BWP of a serving cell corresponding to carrier indicator field value  are given by

where

for any CSS, ;

for a USS, , ,  for ,  for ,  for , and ;

;

is the number of CCEs, numbered from 0 to , in CORESET ;

is the carrier indicator field value if the UE is configured with a carrier indicator field by CrossCarrierSchedulingConfig for the serving cell on which PDCCH is monitored; otherwise, including for any CSS, ;

, where  is the number of PDCCH candidates the UE is configured to monitor for aggregation level  of a search space set  for a serving cell corresponding to ;

for any CSS, ;

for a USS,  is the maximum of  over all configured  values for a CCE aggregation level  of search space set  ;

the RNTI value used for  is the C-RNTI.

------------------------------------------ End TP2 for Section 10.1 of TS 38.213 ----------------------------------------

  - Spec alignment

The higher layer parameter name for multiple frequency domain locations in SS set configuration is not aligned between TS 38.213 and current running CR for TS 38.331. Thus, the following alignment TP is proposed:

------------------------------------------ Start TP3 for Section 10.1 of TS 38.213 ----------------------------------------

For each CORESET in a DL BWP of a serving cell, a respective frequencyDomainResources provides a bitmap.

-	if a CORESET is not associated with any search space set configured with freqMonitorLocations-r16, the bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of  NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index 6⋅NBWPstart/6 if rb-offset is not provided, or the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset where NRBoffset is provided by rb-offset.

-		if a CORESET is associated with at least one search space set configured with freqMonitorLocations-r16, the first NRBG,set0size  bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of  NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset. NRBG,set0size=(NRB,set0size-NRBoffset)/6, NRB,set0size is a number of available PRBs in the RB set 0 for the DL BWP, and NRBoffset is provided by rb-offset or NRBoffset=0 if rb-offset is not provided.

… …

- a bitmap by freqMonitorLocations-r16, if provided, to indicate one or more RB sets for the search space set s, where the MSB k in the bitmap corresponds to RB set k-1 in the DL BWP. For RB set k indicated in the bitmap, the first PRB of the frequency domain monitoring location confined within the RB set is given by NRB,  set kstart+NRBoffset, where NRB,set kstart is the index of first PRB of the RB set k, and NRBoffset is provided by rb-offset or NRBoffset=0 if rb-offset is not provided. The frequency domain resource allocation pattern for each monitoring location is determined based on the first NRBG, set 0size bits in frequencyDomainResources provided by the associated CORESET configuration.

------------------------------------------ End TP3 for Section 10.1 of TS 38.213 ----------------------------------------

- Conclusion

In this contribution, we focus on the remaining issues of wideband operation in NRU, and have the following proposals:

Proposal 1: RAN1 adopts the following TP1 for RB set definition.

Proposal 2: RAN1 adopts the following TP2 for SS set configuration with freqMonitorLocations-r16.

- References

- R2-2001920, Introduction of NR operation with Shared Spectrum Access in RRC, RAN2#109e, 24 February – 6 March, 2020.

- RAN1 chairman notes, RAN1 #99, Reno, USA, November 18th – 22nd, 2019.