3GPP TSG RAN WG1 Meeting #100 bis                            R1-2001853

e-Meeting, April 20th – April 24th, 2020

Source:           ZTE

Title:              Remaining issues on scheduling enhancement for NB-IoT

Agenda item:      6.2.1.3

Document for:     Discussion and Decision

Introduction

In this document, some corrections for scheduling enhancement are discussed.

Corrections for multi-TBs scheduling

According to the specification description [1], multi-TB-Unicast-config and twoHARQ-ProcessesConfig are configured separately. e.g., multi-TB-Unicast-config is configured, but it is possible twoHARQ-ProcessesConfig is not configured. In this situation, the NPDCCH monitoring mechanism for multi-TB scheduling UE would reuse the legacy method in R13, which will lead to efficiency loss. For this reason, the UE behavior that depends on multi-TB-Unicast-config should not be skipped. Note in the current spec, this has been taken care of in multiple occasions. However, the specification below still needs correction.

TP for 36.213 section 16.6:

<Unchanged parts are omitted>

If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig or multi-TB-Unicast-config

-	and if the UE has a NPUSCH transmission ending in subframe n,

-	the UE is not required to receive transmissions in the Type B half-duplex guard periods as specified in [3]for FDD ; and

-	the UE is not expected to receive an NPDCCH with DCI format N0/N1 for the same HARQ process ID as the NPUSCH transmission in any subframe starting from subframe n+1 to subframe n+3;

<Unchanged parts are omitted>

Similarly, another one need to be modified.

<Unchanged parts are omitted>

If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig or multi-TB-Unicast-config

-	the UE is not required to monitor an NPDCCH candidate of an NPDCCH search space if the candidate ends in subframe n, and if the UE is configured to monitor NPDCCH candidates of another NPDCCH search space having starting subframe k0 before subframe n+5

otherwise

-	the UE is not required to monitor NPDCCH candidates of an NPDCCH search space if an NPDCCH candidate of the NPDCCH search space ends in subframe n, and if the UE is configured to monitor NPDCCH candidates of another NPDCCH search space having starting subframe k0 before subframe n+5.

<Unchanged parts are omitted>

Proposal 1: Adopt the text proposals for TS 36.213.

Conclusion

In this contribution, we have discussed some corrections for scheduling enhancement for NB-IoT. We make the following observations and proposals:

Proposal 1: Adopt the text proposals for TS 36.213.

- References

- R2-2001782, Introduction of additional enhancements for NB-IoT in TS 36.331, 3GPP TSG-RAN WG2 Meeting #109-e,Online, 24th Feb - 6th Mar, 2020, Huawei.