3GPP TSG RAN WG1 #100bis-e		R1-2001897

e-Meeting, April 20th – 30th, 2020

Title: 	Remaining issues of in-device coexistence

Source: 	ZTE, Sanechips

Agenda item:	7.2.4.4

Document for:	Discussion and decision

# Introduction

At RAN1 #98bis meeting, the following agreements were made [1]:

Agreements:

  - For sidelink synchronization signal/channel (including S-SSB and LTE SLSS/PSBCH) priority for a UE is (pre)-configured per UE

    - The (pre)-configured priority is used in the same way as the priority for other channel/signals w.r.t. prioritization for handling in-device co-existence

    - Note: it is understood that the same priority (pre)-configuration is intended for all the related UEs

  - The priority of PSFCH is set as the priority of the corresponding PSSCH.

In this contribution, we present text proposal for the specification about in-device coexistence between NR V2X and LTE V2X in 38.213.  .

# Discussions

## Priority determination for PSFCH

In section 16.2.4.1 in 38.213, the mechanism is described for the overlap between simultaneous transmissions for a UE using LTE sidelink and NR sidelink, or between transmission or reception for a UE using LTE sidelink and NR sidelink respectively.

16.2.4.1 Simultaneous NR and E-UTRA transmission/reception If a UE - would transmit a first channel/signal using E-UTRA radio access and a second channel/signal using NR radio access, and - a transmission of the first channel/signal would overlap in time with a transmission of the second channel/signal, and - the priorities of the two channels/signals are known to the UE T msec prior to the start of the earlier of the two transmissions the UE transmits only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers.If a UE - would respectively transmit or receive a first channel/signal using E-UTRA radio access and receive or transmit a second channel/signal using NR radio access, and - a transmission or reception of the first channel/signal would respectively overlap in time with a reception or transmission of the second channel/signal, and - the priorities of the two channels/signals are known to the UE T msec prior to the start of the later transmission or reception the UE transmits or receives only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers.

In the text above, it is not described how to determine the priority of PSFCH when the channel/signal using NR radio access is PSFCH.

According to the agreements in RAN1 #98bis meeting, the priority of PSFCH is set as the priority of the corresponding PSSCH. So we propose the change for the text in section 16.2.4.1 in 38.213 as following:

-------------------------------------------< Start of text proposal for TS38.213 >------------------------------------------------ 16.2.4.1 Simultaneous NR and E-UTRA transmission/reception If a UE - would transmit a first channel/signal using E-UTRA radio access and a second channel/signal using NR radio access, and - a transmission of the first channel/signal would overlap in time with a transmission of the second channel/signal, and - the priorities of the two channels/signals are known to the UE T msec prior to the start of the earlier of the two transmissions the UE transmits only the channel/signal with the higher priority where the priority of a channel/signal - is determined by the SCI formats scheduling the PSCCH/PSSCH transmissions or, - is determined by the SCI format associating with the corresponding PSFCH transmission or, - is indicated by higher layers in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers. If a UE - would respectively transmit or receive a first channel/signal using E-UTRA radio access and receive or transmit a second channel/signal using NR radio access, and - a transmission or reception of the first channel/signal would respectively overlap in time with a reception or transmission of the second channel/signal, and - the priorities of the two channels/signals are known to the UE T msec prior to the start of the later transmission or reception the UE transmits or receives only the channel/signal with the higher priority where the priority of a channel/signal - is determined by the SCI formats scheduling the PSCCH/PSSCH transmissions or, - is determined by the SCI format associating with the corresponding PSFCH transmission or, - is indicated by higher layers in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers. -------------------------------------------< End of text proposal for TS38.213 >------------------------------------------------

# Conclusion

This contribution concludes with the following proposal:

Text change should be made in section 16.2.4.1 in 38.213 as following:

-------------------------------------------< Start of text proposal for TS38.213 >------------------------------------------------ 16.2.4.1 Simultaneous NR and E-UTRA transmission/reception If a UE - would transmit a first channel/signal using E-UTRA radio access and a second channel/signal using NR radio access, and - a transmission of the first channel/signal would overlap in time with a transmission of the second channel/signal, and - the priorities of the two channels/signals are known to the UE T msec prior to the start of the earlier of the two transmissions the UE transmits only the channel/signal with the higher priority where the priority of a channel/signal - is determined by the SCI formats scheduling the PSCCH/PSSCH transmissions or, - is determined by the SCI format associating with the corresponding PSFCH transmission or, - is indicated by higher layers in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers. If a UE - would respectively transmit or receive a first channel/signal using E-UTRA radio access and receive or transmit a second channel/signal using NR radio access, and - a transmission or reception of the first channel/signal would respectively overlap in time with a reception or transmission of the second channel/signal, and - the priorities of the two channels/signals are known to the UE T msec prior to the start of the later transmission or reception the UE transmits or receives only the channel/signal with the higher priority where the priority of a channel/signal - is determined by the SCI formats scheduling the PSCCH/PSSCH transmissions or, - is determined by the SCI format associating with the corresponding PSFCH transmission or, - is indicated by higher layers in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers. -------------------------------------------< End of text proposal for TS38.213 >------------------------------------------------

# References

RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #98bis, Chongqing, China, October 14th–20th, 2019.