3GPP TSG RAN WG1 Meeting #101bis-e	R1-2002199

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.8.2

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	Discussion on staggered SRS for NR Positioning

Document for:	Discussion & Decision

# Introduction

For positioning, staggered transmission of sounding reference signal (SRS) has been agreed in Rel. 16 to achieve better coverage and performance in low SNR scenarios. It was already discussed in previous meetings that there are mainly two shortcomings if SRS is generated/transmitted according to current specification:

- phase offset resulting due to the transmission comb offset and

- range of cyclic shift a UE can be configured is limited to 1/KTC of the OFDM symbol duration.

To address these issues, the following options were identified by several companies [1] as candidate solutions:

- Option 1:

- ru,v(αi,δ,l')n= ejαKTCn+ fl´maxKTC/NsymbSRS,1ru,vn  if configured

- ru,vα,δn=ejαnru,vn,otherwise

- Option 2:  			         ru,vαi,δ,l'n=ejαinru,vn ejαi'fl´KTC

αi'=αi+2πnSRSeffnSRScs,max

αi=2πnSRScs,inSRScs,max  with  nSRScs,i=nSRSeff+nSRScs,max(pi-1000)NAPmod nSRScs,max

nSRSeff∈{0,…,KTCnSRScs,max-1} is configured (range for cyclic shift is extended).

Note: the value of nSRScs,max is as in Release 15.

Further details on options 1 and 2 can be found in [2] and [3] respectively. A third method was proposed in [4] to solve the phase offset issue using the following mapping function (referred to as Option 3).

Option 3:

anKTC+k0(pi),l=βSRSejαnru,vnejγl'n=0,1,…,Msc,bRS-1l=0,1,...,NsymbSRS-10otherwise
where 
γl'=αimodk0pi,KTCKTc
The k0pi is the frequency-domain starting position of symbol l'.

# Comparison of the Options 1 - 3

The current specification limits the range of the cyclic shift to FFTlength/KTC, where KTC is the transmission comb number. Depending on the transmission comb factor, a de-staggering operation combines KTC OFDM symbols to a single symbol. This is equivalent to combining the cyclic correlations of each OFDM symbol to a common correlation function, which may be further evaluated by the time-of-arrival estimator.

Figure 1: Examples of correlation functions at OFDM symbol level and combined (“de-staggered”) correlation. Each of UE1, 2 and 3 using different cyclic shifts: no frequency offset (left) with frequency offset (right)

Figure 1 illustrates the correlation of SRS assuming no multiplex (the SRS of each UE is transmitted in different OFDM symbols or with different comb offset). The three colors represent different UEs with different cyclic shift settings. The figure shows the cyclic correlation of the signal received at the TRP with the (not cyclic shifted) reference sequence. The first four subplots visualize the correlation within one OFDM symbol. According to the comb factor four correlation peaks for each UE result. For the fifth subplot the four correlation functions are combined to obtain correlation of the “de-staggered” SRS. From Figure 1, we observe that the correlation on a single OFDM symbol includes KTC (KTC = 4 is selected for the example) equidistant and cyclically repeating replica of the ‘true’ correlation function. To achieve the full de-staggering gain, the following is required:

- a phase coherent addition of the desired peak must be applied, and

- KTC-1 replicas of the correlation function must be cancelled out.

For the set of plots on the left, no frequency offset is considered and for the set of plots on the right, the effect of frequency offset is also taken into account. The right figure shows that the “aliasing components” are no longer fully cancelled due to phase offsets resulting from (small) frequency offset. In the left figure some remaining very small peaks can be also observed caused by Doppler shift frequency according the movement during the sequential transmission of the OFDM symbols. For the example the Doppler shift frequency is very low and is equivalent to a speed of 3km/h.

It is possible that several UEs (or signals transmitted over different antenna ports) share the same resource elements. Figure 2 depicts the composite signal received at the TRP for three UEs sharing the same OFDM symbols and using the same comb offset. The correlation is done on the received signal which is the sum of signals from the three UEs. As with Figure 1, the first four correlation depicts the correlation on single OFDM symbols and the fifth plot shows the correlation of de-staggered SRS, which depicts the correlation peaks from three different UEs.

UE1

UE3

UE2

Figure 2: Correlation of the sum signal (3 UEs share the same REs): no frequency offset (left), with frequency offset (right)

With the above requirements which need to be fulfilled to achieve a full de-staggering gain, the performance of the three options mentioned in Section ‎1 can be  summarized by:

- All of the three options introduce a constant phase offset, thereby ensuring a coherent addition of the intended correlation peak. This applies at least to the cases where frequency offsets and/or Doppler frequency shifts (e.g. due to UE movement) do not introduce an additional phase offset.

- The number of cyclic shifts that can be configured by each of the 3 options and their resulting effects are different.

Considering the phase correction proposed in Option 2 the correction factor ejαi'fl´KTC can be split into two parts, as follows:

| ejαi'fl´KTC=ejαifl´KTC⋅ej2πnSRSeffnSRScs,maxfl´KTC. | (1) |
| --- | --- |

The first part ejαifl´KTC ensures the coherent combining of correlations from each of the OFDM symbols. The second part ej2πnSRSeffnSRScs,maxfl´KTC selects the UE-specific peak by applying a phase correction which is an integer multiple of  2πKTC . At the same time, the remaining aliasing replicas of the ‘true’ correlation peak are cancelled out. This second part can be therefore considered as “aliasing component selector”.

When we compare Option 2 to Option 1, it can be observed that:

- Option 1 multiples the nSRScs,i provided by the higher layer with the factor KTC. Having in mind that the correlation per OFDM symbol includes KTC peaks with a distance of FFTlength/KTC for all values of nSRScs,i≥KTC the correlation peaks per OFDM symbol are at the same position as for SRS using a cyclic shift of modnSRScs,i,KTC. This implies that only nSRScs,i/KTC can be separated before de-staggering.

- Compared to this, Option 2 maintains the cyclic shift stepsize of Rel.15 defined by nSRScs, max and increases the range of the cyclic shift by increasing the allowed range for the parameter provided by the higher layer. To distinguish this from the original nSRScs the parameter is called nSRSeff in this document. The nSRScs,i used for the calculation of the cyclic shifted sequence itself and the phase correction factor can be derived from nSRSeff.

If we compare Option 2 to Option 3, it is observed:

- Coherent combining and alias cancellation is achieved by applying phase correction to the cyclically shifted sequence. This is true if the sequence to which the above correction is applied is identical to Rel. 15 sequence.

- Option 3, however, does not increase the effective range of cyclic shifts for the de-staggered SRS. The range of cyclic shift a UE can be configured is limited to 1/KTC of the OFDM symbol duration

Table 1: Overview on the three options and current Rel-16 characteristics.

|  | Rel-16 SRS Pos | Option1 | Option2 | Option3 |
| --- | --- | --- | --- | --- |
| Phase correction for de-staggering | No | Yes | Yes | Yes |
| Single OFDM symbol containing SRS sequence identical to Rel- 15 | Yes | No | Yes (Only common phase is added) | Yes (Only common phase is added) |
| Cyclic shift range in samples | fftlength/KTC | fftlength | fftlength | fftlength/KTC |
| Steps KTC =2 KTC =4 KTC =8 | nSRScs,max 8 12 6 | nSRScs,max 8 12 6 | nSRScs,maxKTC 16 48 48 | nSRScs,max 8 12 6 |

From the analysis above, we can make the following observations:

Observation 1: All three options can achieve a coherent combining and can cancel the additional correlation peaks for an SRS Comb-N after de-staggering.

Observation 2: Options 2 and 3 maintain the same granularity for the cyclic shifts as Rel. 15. In addition, formulas applicable to SRS using several antenna ports can be kept as well.

As discussed earlier, the Option 1 reduces the effective number of available steps per OFDM symbol. This reduces flexibility and may cause additional interference between UEs that are sharing the same resource elements using different cyclic shifts in case of non-ideal frequency offset estimation or Doppler frequency shifts. This effect is further evaluated in Section ‎3.

Observation 3: Option 1 reduces the effective number of available steps per OFDM symbol. This reduces the flexibility and may cause false detection of ToA due to the additional interference between UEs sharing the same resource elements in case of non-ideal frequency offset estimation or Doppler frequency shifts.

# Impact of Doppler shift/frequency offset

After de-staggering the properties of the received signal is similar to an SRS transmission using a transmission comb factor of 1. When SRS is transmitted over several symbols, the sensitivity to Doppler frequency shifts and/or frequency offsets is higher compared to the case when SRS is transmitted in a single symbol. This could limit the performance of ToA estimation. There are two effects that need to be taken into account:

- Without correction of the phase offset resulting from Doppler frequency shift and/or frequency offset before combining, the full de-staggering gain will not be achieved.

- A Doppler shift/frequency offset causes a non-ideal “aliasing cancelation” resulting in additional interference between signals from different UEs. If different cyclic shifts are used are used and the same REs are shared by the UEs, a separation of the signals at the network may be no longer possible.

## Analyzing the aliasing effect for different cyclic shift configurations

In the case of high or moderate SINR and if the UEs are transmitting on distinct REs, the frequency offset may be compensated at the network before combining (de-staggering) the OFDM symbols. The Doppler frequency shift/frequency offset is typically different for each UE. Therefore, it is likely for some scenarios that correlation peaks from UEs assigned with adjacent cyclic shift values overlap. In such a case, the signals from those UEs are no longer distinguishable from each other which may result in a high ToA estimation error.

To illustrate this effect, we consider the shift of the correlation function (in samples), as a function of configured cyclic shift nSRScs,i,

csSamples(i)=-FFTlengthKTC ∙nSRScs,inSRScs,max.	(2)

Given a transmission comb factor KTC, the correlation function has cyclically shifted replicas of the peak at FFTlengthKTC samples Form (2), it can be observed that two UEs interfere with each other if the difference between csSamples(i) for UE i and csSamples(j) for UE j is a multiple of  FFTlengthKTC. Consider an example with three UEs, FFTlength=2048 and KTC=4 for both Option 1 and Option 2. Note that Option 3 is not considered because the cyclic shift range follows the Rel-15 behavior.

For Option 2, an extended cyclic shift nSRS,effcs,i, whose range is allowed to go beyond nSRScs,max can be configured. For nSRS,effcs,i ≥nmaxcs, the second phase correction factor (see equation (1)) is not zero. This ensures that the cyclic shift can be configured within the entire range of the OFDM symbol. Although the replicas still occur KTC times (including the original one), due to the extended range of nSRS,effcs,i over nSRScs,i, the replicas can be placed in such a way that they do not overlap with each other. The different configuration options using Comb-4 using Option 1 and Option 2 are summarized in Table 2 . It can be observed that Option 1 only supports a part of the cyclic shift configuration. For example, a cyclic configuration resulting in csSamples=-640 samples is not supported by Option 1.

Table 2:  csSamples(i) for different cyclic shift values for Option 1 and Option 2 for Comb-4 where cyclic shift values of the same color cause aliasing.

| Option 1 csSamples=-FFTtLength ∙nSRScs,inSRScs,max |  | Option 2 csSamples=-FFTLengthKTC ∙nSRS,effcs,inSRScs,max |  |
| --- | --- | --- | --- |
| nSRScs,i | csSamples(i) | nSRS,effcs,i | csSamples(i) |
| 0 | 0 | 0 ⁝ | 0 |
| 1 | -170.67 | 4 ⁝ |  |
| 2 | -341.33 | 8 ⁝ |  |
| 3 | -512 | 12 ⁝ |  |
| not supported | -640 | 15 ⁝ | -640 |
| 4 | -682.667 | 16 ⁝ | -682.667 |
| 5 | -853.33 | 20 ⁝ |  |
| 6 | -1024 | 24 |  |
| 7 | -1194.667 | 28 ⁝ | -1194.667 |
| not supported | - | 30 ⁝ | -1280 |
| 8 | -1365.33 | 32 ⁝ |  |
| 9 | -1536 | 36 ⁝ |  |
| 10 | -1706.67 | 40 ⁝ |  |
| 11 | -1877.33 | 44 ⁝ |  |
|  |  | 47 |  |

The interference was analyzed for the case when multiple UEs sharing the same OFDM symbols by simulation. Details on the simulation assumptions are given in the Annex.

The difference in the cyclic shift between UE1 and UE2 is 512 samples. The time difference of the correlation peaks before de-staggering depends on the timing advance (TA) and the distance of the UE from the TRP. Hence, if the UEs are at similar distance from the serving cell TRP, the timing advances are similar. In this case, the correlation peaks of two UEs may overlap. An example is depicted in Figure 3 with additional information to illustrate the source of error in the scenario from Figure 2. The different elements of the plot defined by the legend are:

Corr @ fs:	cyclic correlation function at the sampling frequency of 61.44MHz

Corr, upspl	resampling (to higher sampling frequency) of the correlation function to better show the characteristics

No intf (upspl)	correlation function (resampled) without interference from other UE

intf (upspl)	signal components resulting from interference

LOS delay	expected true ToA according to the distance and the time of transmit

estToa	estimated ToA using a ToA estimator detecting the first arrival path.

The figure shows that the aliasing component caused by a non-ideal de-staggering for UE1 (distance is approximately 19 m lower, and the same ToT (time of transmit) setting is assumed) arrives before the first path of UE2 and may be detect as first arrival path of UE2.

- UE2: 	Expected ToA @ 853 + distance and TA offset

- UE1: 	Expected ToA @ 1365 + distance and TA offset
	Aliasing peaks @ 341, 853, 1877  + distance offset   alias component close to UE2

error

UE2

„aliasing“
component of UE1

Figure 3:  False ToA detection due to aliasing component resulting from UE1 on UE2.

Clearly, if the expected peak of a UE lies at the same place as an aliasing replica from another UE, this can cause false detections if the aliasing replica cannot be fully cancelled out due to impairing factors such as a Doppler frequency shift or a frequency offset. A more flexible cyclic shift configuration can avoid this issue. The values used for the cyclic shifts may be selected according to the deployment (TRP positions and size of the localization area). As example, the setting for UE1 is changed to UE1B (nSRS,effcs,i=15 is used instead of nSRS,effcs,i=16), which results to:

- UE2: 	Expected ToA @ 853 + distance and TA offset (same as above)

- UE1B: 	Expected ToA @ 1408 + distance and TA offset (shifted by one CS stepsize of option 2) 
Aliasing peaks @ 384, 896, 1920  + distance offset (Aliasing components of UE1B are separated from UE2 peak positions by at least by 42.67samples (equivalent to a distance difference of 208m for SCS=30kHz)

The resulting correlation functions are depicted in Figure 4. The aliasing component of UE1B is now at a different place. The aliases and ‘true’ peaks can be better identified when the configuration allows the cyclic shifts for the UEs to be placed such that the replicas do not lie within the expected region of ‘true’ correlation peaks.

In summary, Option 2 provides a large flexibility to configure the cyclic shifts so that the alias of a first UE does not lie near to the expected location of a second UE.

„aliasing“
component of UE1B

UE2

Figure 4- No aliasing effect from UE1 to UE2 when selecting Option 2.

Observation 4: In case of frequency offset or Doppler shift, the “aliasing correlation peaks” arising due from the comb transmission are not fully cancelled and may interfere with the expected correlation peak.

Observation 5: Using a small stepsize for the cyclic shifts as supported by Option 2, the network has the flexibility to:

- assign more UEs on the same REs, and

- reduce the aliasing correlation peaks.

Proposal 1: For Rel-16 support at least:

- a phase correction for the staggered SRS, and

- maintain the cyclic shift step size of Rel-15.

Proposal 2: Extend the range of the cyclic shift by applying Option 2.

Proposal 3: Endorse the text proposal in Annex for inclusion in TS 38.211.

# Conclusions

Based on the discussion in this document, we propose the following:

Proposal 1: For Rel-16 support at least:

- a phase correction for the staggered SRS

- maintain the cyclic shift step size of Rel-15

Proposal 2: Extend the range of the cyclic shift by applying Option 2.

Proposal 3: Endorse the text proposal in Annex for inclusion in TS 38.211.

The proposals are made based on the following observations:

Observation 1: All three options can achieve a coherent combining and can cancel the additional correlation peaks for an SRS Comb-N after de-staggering.

Observation 2: Options 2 and 3 maintain the same granularity for the cyclic shifts as Rel. 15. In addition, formulas applicable to SRS using several antenna ports can be kept as well.

Observation 3: Option 1 reduces the effective number of available steps per OFDM symbol. This reduces the flexibility and may cause false detection of ToA due to the additional interference between UEs sharing the same resource elements in case of non-ideal frequency offset estimation or Doppler frequency shifts.

Observation 4: In case of frequency offset or Doppler shift, the “aliasing correlation peaks” arising due from the comb transmission are not fully cancelled and may interfere with the expected correlation peak.

Observation 5: Using a small stepsize for the cyclic shifts as supported by Option 2, the network has the flexibility to:

- assign more UEs on the same REs, and

- reduce the aliasing correlation peaks.

# References

| [1] | 3GPP R1-1913507, "Session notes #2 on offline discussion 7.2.10.2 UL Reference Signals for NR Positioning," Reno, USA, 2019-11. |
| --- | --- |
| [2] | 3GPP R1-1913136, " UL Reference Signals for NR Positioning," Reno, USA, 2019-11. |
| [3] | 3GPP R1-1913101, " Remaining details on SRS enhancements for positioning," Reno, USA, 2019-11. |
| [4] | 3GPP R1-2000540, "Remaining issues on UL SRS for NR Positioning," eMeeting, 2020-02. |

# Annex A

Text Proposal :

Text proposal (38.211):

-----unchanged text omitted ------ ##### 6.4.1.4.2 Sequence generation The sounding reference signal sequence for an SRS resource shall be generated according to r(pi)n,l'=ru,v(αi,δ)n if configured by SRS-Config IE r(pi)n,l'=ejαikoffsetl'KTC⋅ej2πnSRScsnSRScs,maxkoffsetl'KTCru,v(αi,δ)n if configured by [SRS-for-positioning] IE 0≤n≤Msc,bSRS-1 l'∈0,1,…,NsymbSRS-1 Where Msc,bSRS is given by clause 6.4.1.4.3, ru,vα,δ(n) is given by clause 5.2.2, with δ=log2KTC and the transmission comb number KTC∈2,4,8 is contained in the higher-layer parameter transmissionComb. The cyclic shift for antenna port is given as , The factor nSRScs∈0,…,KTCnSRScs,max-1 if configured according to [SRS-for-positioning] IE and nSRScs∈0,…,nSRScs,max-1 if configured according to SRS-Config IE is contained in the higher layer transmissionComb.

# Annex B

Simulation Assumptions

| Parameter |  |
| --- | --- |
| Deployment scenario | InF, Scenarios: InF-LOS, InF-NLOS-DH and InF-NLOS-SH |
| UE dropping | Random inside the hall |
| Channel model | InF according to TR38.901 |
| Carrier frequency | 4 GHz |
| Subcarrier spacing | 30 kHz for 50 MHz |
| Reference signal | SRS, 1 antenna port, Comb 4 |
| Uplink power control | Positioning Rel-16 power configuration |
| Interference | For UEs sharing the same REs |
| Doppler | 3 km/h |
| Frequency offset | [0,0.025, 0.05, 0.1] ppm |

Table 3- The CS configuration used for the examples. The CS values for UE1 and UE2 are supported by both Options, UE1B and UE3 is supported by option2 only.

|  | Option1 nSRScs,i | Option2 nSRS,effcs,i | csSamples(i) |
| --- | --- | --- | --- |
| UE1B (Used for Figure 4) | not supported | 15 | -640 (1408) Note1 |
| UE1 (Used other Figures 1,2 and 3) | 4 | 16 | -682.667 (1365.3) |
| UE2 (Used for all other figures) | 7 | 28 | -1194.667 (853.33) |
| UE3 (Used other Figures 1 and 2) | not supported | 30 | -1280 (768) |

Note1: csSamples(i) assuming window start as reference.

# Annex C

Impact of Doppler and frequency offsets to staggered SRS

An example of the effects is given in Figure 5. The correlation for each symbol shows 4 correlation peaks according to the COMB factor KTC=4. The last subplot shows the “de-staggered” signal. The left figure shows a nearly perfect cancellation of 3 of 4 correlation peaks. The remaining small peaks (app. 51dB below main peak) results from a minor Doppler (5kmh, carrier frequency 4GHz). The right figure shows the signal with a frequency offset of 200Hz (0.05ppm @ 4GHz) and 30kHz SCS.

Sidelobes according BW („SINC“ function)

„aliasing“ term only partly cancelled

Impact of minor „Doppler“

Figure 5: Impact of frequency offsets to de-staggering, Left: no frequency offset and 3kmh Doppler, right: with frequency offset