3GPP TSG-RAN WG1 Meeting #100bis-e 	R1-2002230

E-meeting, April 20 – April 30, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | - | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on rate-matching for PDSCH/PDSCH with SPS in TS38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - For PDSCH scheduled by DCI format 1_0 or PDSCHs with SPS activated by DCI format 1_0, the REs in the configured RB-symbol level resource-sets in rateMatchPatternGroup1 or rateMatchPatternGroup2 are not available for PDSCH. However, it is not clear whether the REs in the configured RB-symbol level resource-sets that are not included in either of the two groups are available for PDSCH. The same problem exists for PDSCHs with SPS activated in DCI format 1_1. - For PDSCH scheduled by DCI format 1_1, whether the REs in the configured RB-symbol level resource-sets in rateMatchPatternGroup1 or rateMatchPatternGroup2 is available for PDSCH depends on whether the rate matching indicator field in the corresponding DCI is equal to 1. However, the rate matching behavior for PDSCHs with SPS activated by DCI format 1_1 is not clear according to the current spec. - Similar to the case in bullet 2, for PDSCHs with SPS activated by DCI format 1_1, if the UE is configured with pdsch-AggregationFactor, the rate matching behavior for PDSCH with respect to the REs in the configured RB-symbol level resource-sets in rateMatchPatternGroup1 or rateMatchPatternGroup2 is not clear according to the current spec. |  |  |  |  |  |  |  |  |
| T |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that the configured RB-symbol level resource-sets that are not included in either rateMatchPatternGroup1 or rateMatchPatternGroup2 are not available for PDSCH scheduled by DCI format 1_0, PDSCHs with SPS activated by DCI format 1_0, or PDSCHs with SPS activated by DCI format 1_1. - Clarify that the configured RB-symbol level resource-sets that are in either rateMatchPatternGroup1 or rateMatchPatternGroup2 are not available for the first PDSCH of PDSCHs with SPS activated by DCI format 1_1 if a corresponding bit of the rate matching indicator field in DCI format 1_1 scheduling the PDSCH is equal to 1. - Clarify that when the UE is configured with pdsch-AggregationFactor, the REs corresponding to the resources in either rateMatchPatternGroup1 or rateMatchPatternGroup2 are not available for PDSCHs with SPS in the first pdsch-AggregationFactor consecutive slots if a corresponding bit of the rate matching indicator field in DCI format 1_1 scheduling the PDSCH is equal to 1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The PDSCH resource mapping with respect to the REs in the configured RB-symbol level resource-sets that are not included in either rateMatchPatternGroup1 or rateMatchPatternGroup2 is not clear for PDSCH scheduled by DCI format 1_0 or PDSCHs with SPS. - The PDSCH resource mapping with respect to the REs in the configured RB-level resource-sets in either rateMatchPatternGroup1 or rateMatchPatternGroup2 are not clear for PDSCH with SPS activated by DCI format 1_1. - The PDSCH resource mapping with respect to the REs in the configured RB-level resource-sets in either rateMatchPatternGroup1 or rateMatchPatternGroup2 are not clear for PDSCH with SPS activated by DCI format 1_1 when the UE is configured with slot aggregation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR has isolated impact. This CR impacts the rate matching of the configured RB-level resources for PDSCH. If the UE is implemented in accordance to this CR and the gNB is not, or the gNB is implemented in accordance to this CR and the UE is not, there will be different understanding between UE and gNB on PDSCH resource mapping and PDSCH detection failure occurs. |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

5.1.4.1	PDSCH resource mapping with RB symbol level granularity

A UE may be configured with any of the following higher layer parameters indicating REs declared as not available for PDSCH:

-	rateMatchPatternToAddModList given by PDSCH-Config, by ServingCellConfig or by ServingCellConfigCommon and configuring up to 4 RateMatchPattern(s) per BWP and up to 4 RateMatchPattern(s) per serving-cell. A RateMatchPattern may contain:

-	within a BWP, when provided by PDSCH-Config or within a serving cell when provided by ServingCellConfig or ServingCellConfigCommon, a pair of reserved resources with numerology provided by higher layer parameter subcarrierSpacing given by RateMatchPattern when configured per serving cell or by numerology of associated BWP when configured per BWP .The pair of reserved resources are respectively indicated by an RB level bitmap (higher layer parameter resourceBlocks given by RateMatchPattern ) with 1RB granularity and a symbol level bitmap spanning one or two slots (higher layer parameters symbolsInResourceBlock given by RateMatchPattern ) for which the reserved RBs apply. A bit value equal to 1 in the RB and symbol level bitmaps indicates that the corresponding resource is not available for PDSCH. For each pair of RB and symbol level bitmaps, a UE may be configured with a time-domain pattern (higher layer parameter periodicityAndPattern given by RateMatchPattern ), where each bit of periodicityAndPattern corresponds to a unit equal to a duration of the symbol level bitmap, and a bit value equal to 1 indicates that the pair is present in the unit. The periodicityAndPattern can be {1, 2, 4, 5, 8, 10, 20 or 40} units long, but maximum of 40ms. The first symbol of periodicityAndPattern every 40ms/P periods is a first symbol in frame nf mod 4 = 0, where P is the duration of periodicityAndPattern in units of ms. When periodicityAndPattern is not configured for a pair, for a symbol level bitmap spanning two slots, the bits of the first and second slots correspond respectively to even and odd slots of a radio frame, and for a symbol level bitmap spanning one slot, the bits of the slot correspond to every slot of a radio frame. The pair can be included in one or two groups of resource sets (higher layer parameters rateMatchPatternGroup1and rateMatchPatternGroup2). The rateMatchPatternToAddModList given by ServingCellConfig or ServingCellConfigCommon configuration in numerology µ applies only to PDSCH of the same numerology µ.

-	within a BWP, a frequency domain resource of a CORESET configured by ControlResourceSet with controlResourceSetId or ControlResourceSetZero and time domain resource determined by the higher layer parameters monitoringSlotPeriodicityAndOffset, duration and monitoringSymbolsWithinSlot of all search-space-sets configured by SearchSpace and time domain resource of search-space-set zero configured by searchSpaceZero associated with the CORESET as well as CORESET duration configured by ControlResourceSet with controlResourceSetId or ControlResourceSetZero. This resource not available for PDSCH can be included in one or two groups of resource sets (higher layer parameters rateMatchPatternGroup1 and rateMatchPatternGroup2).

A configured group rateMatchPatternGroup1 or rateMatchPatternGroup2 contains a list of RB and symbol level resource set indices forming a union of resource-sets not available for PDSCH dynamically if a corresponding bit of the Rate matching indicator field of DCI format 1_1 scheduling the PDSCH is equal to 1. The REs corresponding to the union of configured RB-symbol level resource-sets that are not included in either of the two groups are not available for PDSCH scheduled by DCI format 1_0, PDSCH scheduled by DCI format 1_1 or PDSCHs with SPS. When receiving PDSCH scheduled by DCI format 1_0 or PDSCHs with SPS activated by DCI format 1_0, the REs corresponding to configured resources in rateMatchPatternGroup1 or rateMatchPatternGroup2 are not available for PDSCH. When receiving PDSCH with SPS activated by DCI format 1_1, the REs corresponding to configured resources in rateMatchPatternGroup1 or rateMatchPatternGroup2 are not available for the first activated PDSCH with SPS if a corresponding bit of the rate matching indicator field in DCI format 1_1 activating the PDSCH with SPS is equal to 1. When receiving PDSCH with SPS activated by DCI format 1_1 and if the UE is configured with pdsch-AggregationFactor, the REs corresponding to configured resources in rateMatchPatternGroup1 or rateMatchPatternGroup2 are not available for the PDSCHs in the first activated PDSCH with SPS across pdsch-AggregationFactor consecutive slots if a corresponding bit of the rate matching indicator field in DCI format 1_1 activating the PDSCH with SPS is equal to 1.

For a bitmap pair included in one or two groups of resource sets, the dynamic indication of availability for PDSCH applies to a set of slot(s) where the rateMatchPatternToAddModList is present among the slots of scheduled PDSCH.

If a UE monitors PDCCH candidates of aggregation levels 8 and 16 with the same starting CCE index in non-interleaved CORESET spanning one OFDM symbol and if a detected PDCCH scheduling the PDSCH has aggregation level 8, the resources corresponding to the aggregation level 16 PDCCH candidate are not available for the PDSCH.

If a PDSCH scheduled by a PDCCH would overlap with resources in the CORESET containing the PDCCH, the resources corresponding to a union of the detected PDCCH that scheduled the PDSCH and associated PDCCH DM-RS are not available for the PDSCH. When precoderGranularity configured in a CORESET where the PDCCH was detected is equal to allContiguousRBs, the associated PDCCH DM-RS are DM-RS in all REGs of the CORESET. Otherwise, the associated DM-RS are the DM-RS in REGs of the PDCCH.

< Unchanged parts are omitted >