3GPP TSG RAN WG1 #100bis 			R1-2002297

e-Meeting, April 20th – 30th, 2020

Agenda item:		7.2.6.5

Source:	Nokia, Nokia Shanghai Bell

Title:	Maintenance on Rel-16 Low PAPR RS

Document for:		Discussion and Decision

# Introduction

This contribution is providing some missing parts of NR Rel-16 specification regarding to the introduction of “Low PAPR RS” even though we have already good implementation with the excellent effort from the specification editors.

The proposed updates are listed as below.

- Clarification of the condition for applying different identity for the nIDRS.

- Clarification of the number of ports for PUCCH format 3/4 when new DMRS is applied.

# TS 38.211

Though we have agreed on the clarification of the text for nIDRS at the RAN1 #100e meeting, the condition is not clearly captured.

The correct formulation is,

- nIDRS=NIDnSCID is applied if the higher-layer parameter dmrs-UplinkTransformPrecoding-r16 is configured, π/2-BPSK modulation is used for PUSCH

- nIDRS=nIDPUSCH if nIDPUSCH is configured by the higher-layer parameter nPUSCH-Identity in the DMRS-UplinkConfig IE, the higher-layer parameter dmrsUplinkTransformPrecoding-r16 is not configured or, the higher-layer parameter dmrsUplinkTransformPrecoding-r16 is configured but π/2-BPSK modulation is not used for PUSCH , the PUSCH transmission is not a msg3 transmission, and the transmission is not scheduled using DCI format 0_0 in a common search space;

===========================================================================

6.4.1.1.1.2	Sequence generation when transform precoding is enabled

<unchanged text omitted>

The sequence group u=fgh+nIDRS mod 30, where nIDRS is given by

-	nIDRS=nIDPUSCH if nIDPUSCH is configured by the higher-layer parameter nPUSCH-Identity in the DMRS-UplinkConfig IE and,

- the higher-layer parameter dmrsUplinkTransformPrecoding-r16 is not configured or the higher-layer parameter dmrsUplinkTransformPrecoding-r16 is configured and π/2-BPSK modulation is not used for PUSCH, and

- the PUSCH is neither scheduled by RAR UL grant nor scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI according to clause 8.3 in [5, TS 38.213];

-	nIDRS=NIDnSCID if the higher-layer parameter dmrs-UplinkTransformPrecoding-r16 is configured, π/2-BPSK modulation is used for PUSCH, the PUSCH transmission is not a msg3 transmission, and the transmission is not scheduled using DCI format 0_0 in a common search space;

-	nIDRS=NIDcell otherwise

<unchanged text omitted>

===========================================================================

There is one agreement for the multiplexing capability for PUCCH format 3/4 when new sequence is used.

Agreement The PUCCH multiplexing capacity when Rel-16 DMRS is configured for pi/2 BPSK PUCCH is a single port

This implies that only single PUCCH port is available when new DM-RS is applied.

In order to avoid misunderstanding, it is proposed that

- NSFPUCCH,3=1, wn=1 if two interlaces are configured or pi2BPSK is enabled and dmrsUplinkTransformPrecodingPUCCH-r16 is configured;

- Orthogonal sequence index” n shall be 0 if pi2BPSK is enabled and dmrsUplinkTransformPrecodingPUCCH-r16 is configured;

- cyclic shift hopping doesn’t work, so it is good to delete the related text to improve the specification text. (Since the text is only applicable for new DMRS, cyclic shift alpha doesn’t work)

#### 6.3.2.6	PUCCH formats 3 and 4

<unchanged text omitted>

##### 6.3.2.6.3	Block-wise spreading

<unchanged text omitted>

For PUCCH format 3 with interlaced mapping and PUCCH format 4, block-wise spreading shall be applied according to

ylMscPUCCH,s+k=wnkNSFPUCCH,sMscPUCCH,sdlMscPUCCH,sNSFPUCCH,s+k mod MscPUCCH,sNSFPUCCH,s
k=0,1,…, MscPUCCH,s-1
l=0,1,…,NSFPUCCH,sMsymbMscPUCCH,s-1

where

-	for PUCCH format 3 with interlaced mapping, NSFPUCCH,3∈1,2,4 if a single interlace is configured and NSFPUCCH,3=1, wn=1 if two interlaces are configured or pi2BPSK is enabled and dmrsUplinkTransformPrecodingPUCCH-r16 is configured;

-	for PUCCH format 4,MRBPUCCH,4=1, NSFPUCCH,4∈2,4;

and  is given by Tables 6.3.2.6.3-1 and 6.3.2.6.3-2 for NSFPUCCH,s>1 where n is the index of the orthogonal sequence to use according to clause 9.2.1 of [5, TS 38.213].

<unchanged text omitted>

##### 6.4.1.3.3	Demodulation reference signal for PUCCH formats 3 and 4

###### 6.4.1.3.3.1	Sequence generation

The reference-signal sequence rlm shall be generated according to

where MscPUCCH,s is given by clause 6.3.2.6.3 and ru,vα,δ(m) depends on the configuration:

-	if the higher-layer parameter dmrsUplinkTransformPrecodingPUCCH-r16 is configured and pi2BPSK is enabled, ru,vα,δ(m) is given by clause 5.2.3 with cinit given by clause 6.4.1.3.2.1. The sequence group u and the sequence number v depend on the sequence hopping in clause 6.3.2.2.1 and α orthogonal sequence index n=0.

-	otherwise, ru,vα,δ(m) is given by clause 6.3.2.2.

The cyclic shift α varies with the symbol number and slot number according to clause 6.3.2.2.2 with  for PUCCH format 3 without interlaced mapping and obtained from Table 6.4.1.3.3.1-1 with the orthogonal sequence index n given by clause 6.3.2.6.3 for PUCCH format 3 with interlaced mapping and PUCCH format 4.

αn

Table 6.4.1.3.3.1-1: Cyclic shift index m0 for PUCCH format 3 with interlaced mapping and PUCCH format 4.

| Orthogonal sequence index n | Cyclic shift index |  |  |
| --- | --- | --- | --- |
|  | NSFPUCCH,s=1 | NSFPUCCH,s=2 | NSFPUCCH,s=4 |
| 0 | 0 | 0 | 0 |
| 1 | - | 6 | 6 |
| 2 | - | - | 3 |
| 3 | - | - | 9 |

<unchanged text omitted>

====================================================================================

# Conclusion

In this contribution, we have provided the following CR for the latest updated specification on “Low PAPR RS”

The proposed updates are listed as below.

- Clarification of the condition for applying different identity for the nIDRS.

- Clarification of the number of ports for PUCCH format 3/4 when new DMRS is applied.

# References

- TS38.211 v16.1.0

- Chairman’s Report for RAN1 #96bis