3GPP TSG-RAN WG1 #100bis-e		R1-2002426

e-Meeting, April 20th – 30th, 2020

Source:	Ericsson

Title:	Discussion on UE features for MR-DC

Agenda Item:	7.2.11.10

Document for:	Discussion and Decision

# Introduction

In this document, we provide some comments on Rel16 UE features for MR-DC

# Discussion

Capabilities related to NR-DC UL Power Control

- Proposed FGs [18-1], [18-1a], [18-1b],

  - SFN sync between MCG and SCG is related to Rel15 parameter sfn-SyncNRDC. Since discussion related to this issue is ongoing in RAN (i.e., as per [2]), we propose to treat any related Rel16 discussion in RAN plenary instead of RAN1.

  - If specifying a separate UE capability for synchronous NR-DC and asynchronous NR-DC, we propose to define ‘synchronous’ and ‘asynchronous’ according to requirements in 38.133 (i.e., NR-DC can have similar requirements as that of sync and async EN-DC currently captured in 38.133).

Capabilities related to “Enhancements to Single Tx Switched Uplink Solution for EN-DC”

- FGs 18-2,18-2a

  - Regarding component 5,

    - We propose to confirm addition of component 5. If shorter description is desirable it can be reworded as follows – “dropping of NR (SCG) UL transmission when an overlapping LTE (MCG) UL transmission is present (for Type 1 UEs that indicate lack of support of simultaneous UL transmissions using singleUL-Transmission)”.

Capabilities related to “Cross-carrier scheduling with mixed numerologies”

- Propose to add new FG 18-5b for UL CA with mixed numerologies

  - The feature for cross-carrier scheduling with mixed numerology for uplink carrier aggregation is missing since 18-5 describes only DL CA. Introduce a new feature 18-5b  for supporting UL CA, mirroring 18-5 with following changes:

      - Change DL CA to UL CA in component 1)

      - Delete component 3)

- FG 18-5

  - Regarding component 2

    - We propose to confirm the text in square brackets around component 2. For improved scheduling flexibility (e.g. contiguous scheduling) and efficient operation, especially in case of low SCS scheduling high SCS, it is desirable to allow increasing number of DCIs within a span.

  - Regarding component 3

    - Propose to confirm the text in square brackets as default beam for different SCS case does not need separate capability.

- FG 18-5a

  - Prefer to define this capability only for same SCS as different SCS can be handled by 18-5.

# Conclusions

We propose that the input in section 2 be considered for Rel16 MR-DC UE features discussion.

# References

[1] R1-2001484 – “RAN1 UE features list for Rel-16 NR after RAN1#100-E”, AT&T, NTT DOCOMO, RAN1#100-e-Meeting, Feb 2020.

[2] RP-192345 – “Discussion outcome on synchronous NR-NR DC”, Qualcomm, RAN#85, September 2019.

# Annex A

Below is the list of MR-DC related features proposed in R1-2001484

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18. MR-DC/CA enhancement | 18-1 | Basic UL power sharing for DC | Semi-static power sharing mode1 between MCG and SCG cells of same FR for NR dual connectivity. |  | Yes | N/A | Intra-frequency range DC is not supported by the UE | Per band combination | N/A | N/A |  | Absence means intra-FR DC is not supported. | Optional with capability signalling |
|  | 18-1a | Semi-static UL power sharing mode 2 for DC | Semi-static power sharing mode 2 between MCG and SCG cells of same FR for NR dual connectivity. | 18-1 |  |  |  | Per band combination | N/A | N/A |  |  | Optional with capability signalling |
|  | 18-1b | Dynamic UL power sharing for DC | Dynamic power sharing between MCG and SCG cells of same FR for NR dual connectivity. - Supported scenario for dynamic power sharing - T_offset | 18-1 |  |  |  | Per band combination | N/A | N/A |  | 1) {Synch DC only, Sync and Async DC} 2) {short, long} | Optional with capability signalling |
|  | [18-1] | Synchronous NR-DC operation | NR-DC operation with synchronization between MCG and SCG Power-sharing mode within the frequency range - semi-static power-sharing mode 1 - semi-static power-sharing mode 2 - dynamic power-sharing and the value of T_offset |  |  |  |  | Per band combination | N/A | N/A |  | Absence means synchronous NR-DC operation for the given band combination is not supported. 1) {Supported} 2) {not supported, supported} 3) {not supported, short, long} | Optional with capability signalling |
|  | [18-1a] | Non-SFN synchronous NR-DC operation | Operation with non-zero slot offset between MCG and SCG Power-sharing mode within the frequency range - semi-static power-sharing mode 1 - semi-static power-sharing mode 2 - dynamic power-sharing and the value of T_offset | 18-1 |  |  |  | Per band combination | N/A | N/A |  | Absence means non-SFN synchronous NR-DC operation for the given band combination is not supported. 1) {Supported} 2) {not supported, supported} 3) {not supported, short, long} | Optional with capability signalling |
|  | [18-1b] | Asynchronous NR-DC operation | Operation with no slot alignment between MCG and SCG Power-sharing mode within the frequency range - semi-static power-sharing mode 1 - dynamic power-sharing and the value of T_offset | 18-1 |  |  |  | Per band combination | N/A | N/A |  | Absence means asynchronous NR-DC operation for the given band combination is not supported. 1) {Supported} 2) {not supported, supported} 3) {not supported, short, long} | Optional with capability signalling |
|  | 18-2 | Single UL TX operation for TDD PCell in intra-band EN-DC | TDM restriction to LTE TDD PCell in EN-DC for singleUL-Transmission associated functionality when tdm-patternConfig-r16 is configured 1) TDD UL/DL configuration#2, #4, #5 configured as DL-reference UL/DL configuration 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) HARQ subframe offset 4) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern [5) if UE indicates that it does not support simultaneous UL transmissions as defined in TS 38.101-3 [4] using singleUL-Transmission, NR (SCG) UL transmission is dropped when an overlapping LTE (MCG) UL transmission is present (for type 1 UE).”] | EN-DC |  | N/A |  | Per band combination | Applicable to TDD-TDD EN-DC only | Applicable to FR1 only |  | Extension of the R15 capability tdm-Pattern to TDD PCell 3) {not supported, supported} | Optional with capability signalling |
|  | 18-2a | Enhanced single UL TX operation for FDD Pcell EN-DC | TDM restriction to LTE FDD Pcell in EN-DC for singleUL-Transmission associated functionality when tdm-patternConfig-r16 is configured 1) DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) HARQ sub-frame offset 4) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern (for type 1 UE) [5) if UE indicates that it does not support simultaneous UL transmissions as defined in TS 38.101-3 [4] using singleUL-Transmission, NR (SCG) UL transmission is dropped when an overlapping LTE (MCG) UL transmission is present (for type 1 UE).”] | 6-13 |  | N/A |  | Per band combination | Applicable to in FDD-LTE TDD-NR EN-DC | Applicable to FR1 only |  | Enhancement to the R15 capability tdm-Pattern | Optional with capability signalling |
|  | 18-3 | Dual Tx transmission for EN-DC with FDD PCell(TDM pattern for dual Tx UE) | TDM restriction to LTE FDD PCell in EN-DC for dual UL Tx operation when tdm-patternConfig-r16 is configured 1) DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) HARQ subframe offset 4) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern | EN-DC |  | N/A |  | Per band combination | Applicable to EN-DC with LTE FDD PCell only | Applicable to FR1 only |  | Extension of the R15 capability tdm-Pattern to a 2Tx UE | Optional with capability signalling |
|  | 18-3a | Semi-statically configured LTE UL transmissions in all UL subframes not limited to tdm-pattern | UE configured with tdm-patternConfig-r16 can be semi-statically configured with LTE UL transmissions in all UL subframes not limited to the reference tdm-pattern | 18-2, 18-2a |  |  |  | Per UE |  |  |  |  | Optional with capability signaling |
|  | 18-4 | SCell dormancy within active time | Support for SCell dormancy indication sent within the active time on PCell with DCI format 0_1/1_1 |  |  | N/A |  | Per UE | No | No |  |  | Optional with capability signalling |
|  | 18-4a | SCell dormancy outside active time | Support for SCell dormancy indication sent outside the active time on PCell with DCI format 2_6 | [19-1] |  |  |  | Per UE | No | No |  |  | Optional with capability signalling |
|  | 18-5 | Cross-carrier scheduling with different SCS | 1) The UE supports cross carrier scheduling for the different numerologies with carrier indicator field (CIF) in DL carrier aggregation where numerologies for the scheduling cell and scheduled cell are different [2) Processing up to X unicast DCI scheduling (DL and UL) per scheduled CC ] [3 Default QCL assumption for cross-carrier scheduling with different SCS] |  |  | N/A |  | Per band combination | No | No |  | crossCarrierScheduling-OtherSCS 1) {Scheduling cell of lower SCS and scheduled cell of higher SCS, Scheduling cell of higher SCS and scheduled cell of lower SCS, both} [2) ] X is based on pair of (scheduling CC SCS, scheduled CC SCS): [4] for (15,120), (15,60), (30,120), [2] for (15,30), (30,60), (60,120 kHz), Note: This applies also to the case where there is a single span in the slot for the scheduling CC. In case UE supports 3-5b, the limits apply for each span for FDD scheduling cell and TDD scheduling cell. | Optional with capability signalling |
|  | [18-5a] | Default QCL assumption for cross-carrier scheduling | Indicates whether the UE can be configured with enabledDefaultBeamForCCS for default QCL assumption for cross-carrier scheduling with same SCS. |  |  |  |  | Per band combination |  |  |  | FFS if this is needed or if it should cover also component 3 of 18-5 | Optional with capability signalling |
|  | 18-6 | Cross-carrier A-CSI RS triggering with different SCS | Cross-carrier A-CSI RS triggering with different SCS | 2-33 |  | N/A |  | Per band combination | No | No |  | 1) {PDCCH cell of lower SCS and A-CSI RS cell of higher SCS, PDCCH cell of higher SCS and A-CSI-RS of lower SCS, both} . | Optional with capability signalling |
|  | [18-6a] | Default QCL assumption for cross-carrier A-CSI-RS triggering | Indicates whether the UE can be configured with enabledDefaultBeamForCCS for default QCL assumption for cross-carrier A-CSI-RS triggering. |  |  |  |  | Per band combination |  |  |  | FFS if this is needed | Optional with capability signalling |
|  | 18-7 | CA with non-aligned frame boundaries | CA with non-aligned frame boundaries for inter-band CA |  |  | N/A |  | Per band combination | No | No |  | Defines whether the UE supports carrier aggregation operation where the frame boundaries of the PCell and the SCell are not aligned, while the slot boundaries are. | Optional with capability signalling |
|  | 18-8 | HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group | HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group | 6-7 |  | N/A |  | Per UE | No | No |  | Support HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group. Rel-15 had this per cell group | [Optional with capability signalling or Mandatory conditioned to support for multiple PUCCH groups 6-7] |

FG18-2/2a/3

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | 18-2 and 18-2a: - Add component 5 as following to reflect agreed Rel-16 UE behavior for single Tx case “ 5) if UE indicates that it does not support simultaneous UL transmissions as defined in TS 38.101-3 [4] using singleUL-Transmission , NR (SCG) UL transmission is dropped when an overlapping LTE (MCG) UL transmission is present (for type 1 UE).” Align terminology with RAN2 specs by replacing “for single UL Tx operation” with “for single UL-transmission associated functionality, as specified in TS 36.331” Rapporteur: This sounds like functional specification through UE capabilities. I added the component for now in […], but this would seem like text to 38.214. I think the “single UL Tx Operation” was lifted from the Rel-15 8-2, but indeed it is not exactly the best practice precedence. |

FG18-5

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | - Add component “Default beam for cross-carrier scheduled PDSCH when scheduling cell and scheduled cell have different numerologies” - Add below details for component (2) - X is based on pair of (scheduling CC SCS, scheduled CC SCS): - [4] for (15 kHz, 120 kHz), (15 kHz, 60 kHz), (30 kHz, 120 kHz), - [2] for (15 kHz, 30 kHz), (30 kHz, 60 kHz), (60 kHz, 120 kHz), - Note : This applies also to the case where there is a single span in the slot for the scheduling CC. - In case UE supports 3-5b, the limits apply for each span for FDD scheduling cell and TDD scheduling cell. (New) 18-5a: Add new feature for “Default beam for cross-carrier scheduling with same numerology” - Component : Configuration of default beam selection for cross-carrier scheduled PDSCH when scheduled cell and scheduling cell have same numerology - Per-UE, optional with capability signalling |