3GPP TSG RAN WG1 #100bis-e		                                                 R1-2002476

e-Meeting, April 20th – 30th, 2020

Agenda item: 	7.2.11.6

Source: 	Nokia, Nokia Shanghai Bell

Title:	On UE features for eMIMO

Document for:	Discussion and Decision

# Introduction

The Rel-16 UE feature e-mail discussions have gone through two phases in RAN1 e-mail reflector, with the current proposals available in [1]. In this contribution we present our further views regarding the UE features for eMIMO.

# Discussion

As a general comment, it is not clear the reason behind all the FFS points in the feature list. Some of those seem to relate to details in a component definition, but they could also mean that the existence of the component is questioned. This makes it harder to provide detailed feedback. In this contribution we proceed under the assumption that all FFS points will be discussed as part of RAN1#100-bis-e, and hence we focus on selected points only. Another general comment is that remaining specification details of the feature itself should be left for the group responsible for designing the features themselves, e.g. under eMIMO maintenance AI.

The detailed comments are provided below following the numbering of features in [1], and which can be found in the Annex for reference:

- 16-1a, component 2: confirm “Support of group-based reporting for L1-SINR”

- 16-1b, components 4 and 5: details to be finalized by corresponding eMIMO maintenance work

- 16-1e, component 3: this is a RAN4 issue, not related to UE features

- 16-1f:

  - component 5: this is an essential component for SCell BFR

  - component 7: this is a design issue that should be discussed under eMIMO maintenance first

- 16-2a:

  - Component 7 and 8: clarify if they are per CORESETPoolIndex  or per UE.

  - Optional component 2: Should be part of basic component list. Otherwise, basic component 3 does not work and requires specifying  a new UE behaviour when this is not supported.

- 16-2b: component 1 is part of basic operation, no need for FFS.

- 16-2b-1: component 1 is part of basic operation of SDM scheme, no need FFS.

- 16-3a:

  - Component 1: reuse same numbers as in R15.

  - Component 2: Only the first 6 combinations are mandatory. The last 2 are optional and can be moved to the optional components with 1-bit capability (L=6).

  - Component 3: add the following FFS: and, for N3≤19, with value R=2

  - Optional component 2: Support of PMI sub-bands with R=2 [FFS: and N3>19]

  - Optional component 3: distinction is needed between support of “amplitude subset restriction” and “no amplitude subset restriction”

- 16-3b:

  - Component 1: reuse same numbers as in R15.

  - Component 3: add the following FFS: and, for N3≤19, with value R=2

  - Optional component 1: Support of PMI sub-bands with R=2 [FFS: and N3>19]

- 16-5a: not a new feature, to be removed.

- 16-5c: Add that UE indicating support to this feature needs to support 16-5b as well, otherwise it becomes very difficult to utilize the feature in the network in practice.

# Summary

In this contribution we presented our proposals on UE features for eMIMO.

# References

[1] R1-2001484, “RAN1 UE features list for Rel-16 NR after RAN1#100-E”, AT&T, NTT DOCOMO, April 2020

# Annex

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16. NR_eMIMO | 16-1 | Multi-beam operation | - The maximum number of L1-SINR based beam measurement and reporting based on ZP IMR and/or NZP IMR (FFS whether to be a separate UE feature, e.g. 16-1a) - FFS: Support of group-based reporting for L1-SINR - Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH (FFS whether to be a separate UE feature, e.g. 16-1b) - Simultaneous spatial relation update across multiple CCs: AP-SRS, SP-SRS (FFS whether to be a separate UE feature, e.g. 16-1c) - The maximum number of PUCCH resources per BWP for simultaneous spatial relation update (FFS whether to be a separate UE feature, e.g. 16-1c) - Support of default spatial relation pathloss reference RS for dedicated-PUCCH/SRS and PUSCH scheduled by DCI format 0_0 - [Support of / The maximum number of] spatial relation update for AP-SRS via MAC CE (FFS whether to be a separate UE feature, e.g. 16-1c) - The maximum number of configured pathloss reference RS update for PUSCH/SRS/PUCCH via MAC CE (FFS whether to be a separate UE feature, e.g. 16-1c) - FFS: The maximum number of activated pathloss reference RS update for PUSCH/SRS/PUCCH [across CCs, within a slot across all CCs] (FFS whether to be a separate UE feature, e.g. 16-1c) - The maximum number of SCells for SCell beam failure recovery (FFS whether to be a separate UE feature, e.g. 16-1d) - FFS: Support of PUCCH-BFR (FFS whether to be a separate UE feature, e.g. 16-1d) - FFS: The maximum number of CSI-RS and/or SSB resources for new beam identification of SCell BFR [across all CCs, within a slot across all CCs] (FFS whether to be a separate UE feature, e.g. 16-1d) - FFS: Densigy of CSI-RS for SCell BFR The components in FG 16-1 are separated into individual FGs from 16-1a to 16-1g as follows. | TBD for #1 2-1, 2-4 for #2 2-53, 2-59 for #3-#6 8-2 for #7 2-31 for #8 |  | N/A |  | TBD [Per band for #1, #2, #5, #6 Per UE for , #7, #8 Per BC for #3, #4] | N | N for #1, #2, #7, #8 Y for #3 - #6 |  |  | Optional |
|  | 16-1a | L1-SINR reporting | - The maximum number of L1-SINR based beam measurement and reporting based on ZP IMR and/or NZP IMR (FFS details on the sub-components, e.g., FG 2-24) - FFS: Support of group-based reporting for L1-SINR | TBD |  | N/A |  | TBD [Per band] | N | N |  |  | TBD |
|  | 16-1b | TCI state activation and spatial relation update | - [Support of / maximum number of lists for] Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH (FFS whether to be a separate UE feature, e.g. 16-1b) - [Support of / maximum number of lists for] Simultaneous spatial relation update across multiple CCs: AP-SRS, SP-SRS - [Support of / The maximum number of] PUCCH resource groups per BWP for simultaneous spatial relation update - FFS: details on whether/how to indicate band pairs which can share the same DL TCI state - FFS: details on whether/how to indicate band pairs which can share the same UL spatial relation info | Component 1: 2-1, 2-4 Component 2: 2-59, 2-60 Component 3: 2-53, 2-59, 4-24 |  | N/A |  | TBD [Per BC or per band] | N | Y |  |  | TBD |
|  | 16-1c | Default spatial relation | Support of default spatial relation and pathloss reference RS for dedicated-PUCCH/SRS and PUSCH scheduled by DCI format 0_0 | 2-53, 2-59 |  | N/A |  | TBD [Per band] | N | Y |  |  | TBD |
|  | 16-1d | MAC CE spatial relation update for AP-SRS | [Support of / The maximum number of] spatial relation update for AP-SRS via MAC CE (FFS whether to be a separate UE feature, e.g. 16-1c) | 2-53, 2-59 |  | N/A |  | TBD [Per UE] | N | N |  |  | TBD |
|  | 16-1e | Pathloss reference RS activation via MAC CE | - The maximum number of configured pathloss reference RSs for PUSCH/SRS/PUCCH via MAC CE by RRC for MAC-CE based pathloss reference RS update (FFS whether to be a separate UE feature, e.g. 16-1c) - FFS: The maximum number of activated pathloss reference RS update for PUSCH/SRS/PUCCH [across CCs / within a slot across all CCs / per CC] (FFS whether to be a separate UE feature, e.g. 16-1c) - FFS: Number of measurement samples N to apply newly activated pathloss reference RS | 8-2, 8-3 |  | N/A |  | TBD [Per UE] | N | N |  |  | TBD |
|  | 16-1f | SCell beam failure recovery | - The maximum number of SCells for SCell beam failure recovery (FFS whether to be a separate UE feature, e.g. 16-1d) - FFS: Support of PUCCH-BFR (FFS whether to be a separate UE feature, e.g. 16-1d) - FFS: The maximum number of CSI-RS and/or SSB resources for new beam identification of SCell BFR [across all CCs / within a slot across all CCs / per CC] (FFS to replace this component to 14) - FFS: Densigy Density of CSI-RS for new beam identification for SCell BFR | 2-31 |  | N/A |  | TBD | N |  |  |  | TBD |
|  | 16-1g | FFS: Resources for beam management, pathloss measurement, and BFR | - FFS: The maximum number of SSB/CSI-RS resources across all CCs for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, and new beam identification. - FFS: The maximum number of SSB/CSI-RS resources across all CCs for pathloss measurement - FFS: The maximum number of SSB/CSI-RS resources across all CCs for BFD - FFS: The maximum number of SSB/CSI-RS resources across all CCs for new beam identification |  |  | N/A |  | TBD | N |  |  |  | TBD |
|  | 16-2 | Multi-TRP operation | Multi-DCI-based: (FFS whether to be a separate UE feature, e.g. 16-2a) - The maximum number of CORESETs configured per “PDCCH-Config” - The maximum number of CORESETs configured per CORESETPoolIndex (if configured) per “PDCCH-Config” - The value of R=[1,2] for BD/CCE - Whether the UE shall rate match around configured CRS patterns which is associated with CORESETPoolIndex (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex - The maximum number of PUCCH transmissions within a slot - Support of fully/partially time/frequency overlapped PDSCH reception (PDSCHs overlapping types in time and frequency domain) and out of HARQ process. - Support of two PDSCH scrambling sequences per serving cell - Support of default QCL assumption per CORESETPoolIndex - The maximum number of activated TCI states - FFS: The maximum number of MIMO layers of scheduled PDSCHs - Support of separate HARQ-ACK and/or joint HARQ-ACK Single-DCI-based: (FFS whether to be a separate UE feature, e.g. 16-2b) - Whether supporting two PTRS ports - Support of new DMRS entries - Support of default QCL assumption with two TCI states - FFS: Support of URLLC scheme 1a - Support of FDMSchemeA - Support of FDMSchemeB - Support of TDMSchemeA - Support of RepNumR16 in PDSCH-TimeDomainResourceAllocation and the maximum value of RepNumR16 - For FDMSchemeB, whether the UE can support CW soft combining - Supported maximum TBS size for TDMSchemeA - Supported maximum TBS size according to RepNumR16 in PDSCH-TimeDomainResourceAllocation - FFS: TCI state mapping to PDSCH transmission occasions (Cyclical mapping or Sequential mapping) The previous components in FG 16-2 are separated into individual FGs from 16-2a to 16-2b-5 as follows. | TBD |  | N/A |  | TBC [Per band] | N | TBD |  |  | TBD |
|  | 16-2a | Multi-DCI based multi-TRP | Basic components: - The maximum number of CORESETs configured per “PDCCH-Config” - The maximum number of CORESETs configured per CORESETPoolIndex ( if CORESETPoolIndex is not configured, it is assumed CORESETPoolIndex = 0) per “PDCCH-Config” - The value of R=[1,2] for BD/CCESupport of fully/partially time/frequency overlapped PDSCH reception (PDSCHs overlapping types in time and frequency domain) - Support of out-of-order operation for PDCCH to PDSCH (FFS whether to be a basic component) - Support of out-of-order operation for PDSCH to HARQ-ACK (FFS whether to be a basic component) - Support of out-of-order operation for PDCCH to PUSCH (FFS whether to be a basic component) - FFS: The maximum number of activated TCI states - FFS: The maximum number of MIMO layers of scheduled PDSCHs - FFS: the maximum number of CCs supporting multi-DCI based multi-TRP Optional components: - Whether the UE shall rate match around configured CRS patterns which is associated with CORESETPoolIndex (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex - FFS: Support of two PDSCH scrambling sequences per serving cell - Support of default QCL assumption per CORESETPoolIndex - Support of separate HARQ-ACK - Support of joint HARQ-ACK - Support of two TDMed long PUCCHs in a slot | TBD |  | N/A |  | TBD [per band / per FSPC] | N | TBD |  |  | TBD |
|  | 16-2b | Single-DCI based multi-TRP | Basic component(s): - FFS: Support of MAC CE to activate multiple TCI states for a TCI codepoint - FFS: Number of CCs supporting single-DCI based multi-TRP operation Optional components: - Support of default QCL assumption with two TCI states | TBD |  | N/A |  |  | N | TBD |  |  | TBD |
|  | 16-2b-1 | Single-DCI based SDM scheme | - FFS: Support of DCI indication of of 2 TCI states by a codepoint and DMRS ports within two CDM groups - Whether supporting two PTRS ports - FFS Support of DMRS entry {0, 2, 3} | 16-2b, TBD |  | N/A |  |  | N | TBD |  |  | TBD |
|  | 16-2b-2 | Single-DCI based FDMSchemeA | Support of FDMSchemeA | 16-2b, TBD |  | N/A |  |  | N | TBD |  |  | TBD |
|  | 16-2b-3 | Single-DCI based FDMSchemeB | - Support of FDMSchemeB - For FDMSchemeB, whether the UE can support CW soft combining | 16-2b, TBD |  | N/A |  | TBD [per FSPC] | N | TBD |  |  | TBD |
|  | 16-2b-4 | Single-DCI based TDMSchemeA | - Support of TDMSchemeA - Supported maximum TBS size for TDMSchemeA | 16-2b, TBD |  | N/A |  |  | N | TBD |  |  | TBD |
|  | 16-2b-5 | Single-DCI based inter-slot TDM | - Support of RepNumR16 in PDSCH-TimeDomainResourceAllocation and the maximum value of RepNumR16 - Supported maximum TBS size according to RepNumR16 in PDSCH-TimeDomainResourceAllocation - FFS: TCI state mapping to PDSCH transmission occasions (Cyclical mapping or Sequential mapping) | 16-2b, TBD |  | N/A |  |  | N | TBD |  |  | TBD |
|  | 16-3 | eType-II codebook | Regular eType-II (FFS whether to be a separate UE feature, e.g. 16-3a) - FFS: {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II - 8 parameter combinations (FFS: Value of L per the number of antenna ports) - Number of PMI sub-bands (R=1 is mandatory, FFS: R=2 is mandatory or optional) - Rank 1 to 4 - CBSR - Rank restriction - FFS: UCI omission - FFS: The maximum number of configured aperiodic CSI Report Settings - FFS: Support of mixed codebook types Port selection eType-II (FFS whether to be a separate UE feature, e.g. 16-3b) - FFS: {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II - 6 parameter combinations (combos with L=6 don’t apply) (FFS: Value of L per the number of antenna ports) - Number of PMI sub-bands (R=1 is mandatory, FFS: R=2 is mandatory or optional) - Rank 1 to 4 - Rank restriction - FFS: UCI omission - FFS: The maximum number of configured aperiodic CSI Report Settings - FFS: Support of mixed codebook types The components in FG 16-3 are separated into individual FGs 16-3a and 16-3b as follows. | TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
|  | 16-3a | Regular eType-II | Basic components: - FFS: {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II - 8 parameter combinations (FFS: Value of L per the number of antenna ports) - Number of PMI sub-bands (R=1 is mandatory, FFS: R=2 is mandatory or optional) Support of PMI sub-bands with value R=1 - Rank restriction - FFS: UCI omission Optional components - Number of PMI sub-bands (R=1 is mandatory, FFS: R=2 is mandatory or optional) Support of PMI sub-bands with R=2 - Rank 1 to 4 Support of rank 3,4 - CBSR - FFS: The maximum number of configured aperiodic CSI Report Settings - FFS: Support of mixed codebook types | TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
|  | 16-3b | Port selection eType-II | Basic components: - FFS: {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II - 6 parameter combinations (combos with L=6 don’t apply) (FFS: Value of L per the number of antenna ports) - Number of PMI sub-bands (R=1 is mandatory, FFS: R=2 is mandatory or optional) Support of PMI sub-bands with value R=1 - Rank restriction - FFS: UCI omission Optional components: - Number of PMI sub-bands (R=1 is mandatory, FFS: R=2 is mandatory or optional) Support of PMI sub-bands with R=2 - Rank 1 to 4 Support of rank 3,4 - FFS: The maximum number of configured aperiodic CSI Report Settings - FFS: Support of mixed codebook types | TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
|  | 16-4 | Low PAPR DMRS for DL | Low PAPR DMRS for PDSCH | TBD |  | N/A |  | Per UE | N | N |  |  | [Optional] |
|  | 16-5 | Power-efficient UL transmission | UL full power transmission (mode [0], 1, 2) - Supported UL full power transmission mode ([mode 0], mode 1, mode 2, [mode1AndMode2]) - FFS: New UL codebook set(s) per supported mode - The maximum number of SRS resources in set with different number of ports [for usage set to ‘codebook’] - FFS: Number of ports per SRS resource - FFS: Maximum number of different spatial relationship info for all SRS resources for usage set to ‘codebook’ in a resource set - TPMI group which delivers full power Note: UE indicating mode 2 shall support full power transmission for 1 antenna port The components in FG 16-5 are separated into individual FGs from 16-5a to 16-5c as follows. | [2-13, 2-14] for #1-#3 |  | N/A |  | Per UE | N | N |  |  | TBD |
|  | 16-5a | UL full power transmission mode 0 | - Supported UL full power transmission [mode 0] - Number of Tx to support mode 0: {2Tx, 4Tx, 2Tx_4Tx} | 2-13, 2-14 |  | N/A |  | FFS: Per FS or Per band or Per band per BC | N | N |  |  | TBD |
|  | 16-5b | UL full power transmission mode 1 | - Supported UL full power transmission mode 1 - Number of Tx to support mode 1: {2Tx, 4Tx, 2Tx_4Tx} - FFS: New UL codebook set(s) per supported mode per supported Tx | 2-13, 2-14 |  | N/A |  | FFS: Per FS or Per band or Per band per BC | N | N |  |  | TBD |
|  | 16-5c | UL full power transmission mode 2 | - Supported UL full power transmission mode 2 - Number of Tx to support mode 2: {2Tx, 4Tx, 2Tx_4Tx} - The maximum number of SRS resources in set with different number of ports [for usage set to ‘codebook’]. FFS on details for supported number of Tx. - FFS: Number of ports per SRS resource - FFS: Maximum number of different spatial relation info for all SRS resources for usage set to ‘codebook’ in a resource set - TPMI group which delivers full power. FFS on details for supported number of Tx. Note: UE indicating mode 2 shall support full power transmission for 1 antenna port | 2-13, 2-14 |  | N/A |  | FFS: Per FS or Per band or Per band per BC | N | N |  |  | TBD |
|  | 16-6 | Low PAPR DMRS for UL | - For PUSCH without transform precoding - For PUSCH with transform precoding and with pi/2 BPSK modulation - For PUCCH format 3 and/or PUCCH format 4 with transform precoding and with pi/2 BPSK modulation The components in FG 16-6 are separated into individual FGs 16-6a and 16-6b as follows. | TBD for #1-#3 |  | N/A |  | Per UE | N | N |  |  | TBD |
|  | 16-6a | Low PAPR DMRS for PUSCH | - For PUSCH without transform precoding - For PUSCH with transform precoding and with pi/2 BPSK modulation | TBD |  | N/A |  | FFS: Per band | N | N |  |  | FFS: Optional with capability signalling |
|  | 16-6b | Low PAPR DMRS for PUCCH | For PUCCH format 3 and/or PUCCH format 4 with transform precoding and with pi/2 BPSK modulation | TBD |  | N/A |  | FFS: Per band | N | N |  |  | FFS: Optional with capability signalling |