3GPP TSG-RAN WG1 Meeting #100bis-e	R1-2002491

# Online, April 20th – 30th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | rev |  | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on CSI-RS resource set configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current text limits the number of NZP-CSI-RS resources for channel measurements to 64. However, this limitation only applies to when the reportQuantity is set to ‘cri-RI-CQI’. When the report quantity is set to ‘cri-RSRP’, the UE can be configured with up to 128 NZP-CSI-RS resources, according to 38.214, subclause 5.2.1.4.3. ‘ssb-Index-RSRP’ cannot be used with NZP-CSI-RS resources. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the reference to ‘none’, ‘cri-RSRP’ and ‘ssb-Index-RSRP’ in the paragraph |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclarity about the configuration of NZP CSI-RS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: This CR is intended for clarifying the ambiguity in the existing text. No functional change. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.2.1.4.1	Resource Setting configuration

For aperiodic CSI, each trigger state configured using the higher layer parameter CSI-AperiodicTriggerState is associated with one or multiple CSI-ReportConfig where each CSI-ReportConfig is linked to periodic, or semi-persistent, or aperiodic resource setting(s):

-	When one Resource Setting is configured, the Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel measurement for L1-RSRP computation.

-	When two Resource Settings are configured, the first one Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel measurement and the second one (given by either higher layer parameter csi-IM-ResourcesForInterference or higher layer parameter nzp-CSI-RS-ResourcesForInterference) is for interference measurement performed on CSI-IM or on NZP CSI-RS.

-	When three Resource Settings are configured, the first Resource Setting (higher layer parameter resourcesForChannelMeasurement) is for channel measurement, the second one (given by higher layer parameter csi-IM-ResourcesForInterference) is for CSI-IM based interference measurement and the third one (given by higher layer parameter nzp-CSI-RS-ResourcesForInterference) is for NZP CSI-RS based interference measurement.

For semi-persistent or periodic CSI, each CSI-ReportConfig is linked to periodic or semi-persistent Resource Setting(s):

-	When one Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is configured, the Resource Setting is for channel measurement for L1-RSRP computation.

-	When two Resource Settings are configured, the first Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel measurement and the second Resource Setting (given by higher layer parameter csi-IM-ResourcesForInterference) is used for interference measurement performed on CSI-IM.

A UE is not expected to be configured with more than one CSI-RS resource in resource set for channel measurement for a CSI-ReportConfig with the higher layer parameter codebookType set to 'typeII' or to 'typeII-PortSelection'. A UE is not expected to be configured with more than 64 NZP CSI-RS resources in resource setting for channel measurement for a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-CQI'. If interference measurement is performed on CSI-IM, each CSI-RS resource for channel measurement is resource-wise associated with a CSI-IM resource by the ordering of the CSI-RS resource and CSI-IM resource in the corresponding resource sets. The number of CSI-RS resources for channel measurement equals to the number of CSI-IM resources.

If interference measurement is performed on NZP CSI-RS, a UE does not expect to be configured with more than one NZP CSI-RS resource in the associated resource set within the resource setting for channel measurement. The UE configured with the higher layer parameter nzp-CSI-RS-ResourcesForInterference may expect no more than 18 NZP CSI-RS ports configured in a NZP CSI-RS resource set.

For CSI measurement(s), a UE assumes:

-	each NZP CSI-RS port configured for interference measurement corresponds to an interference transmission layer.

-	all interference transmission layers on NZP CSI-RS ports for interference measurement take into account the associated EPRE ratios configured in 5.2.2.3.1;

-	other interference signal on REs of NZP CSI-RS resource for channel measurement, NZP CSI-RS resource for interference measurement, or CSI-IM resource for interference measurement.