| 3GPP TSG-RAN WG1 Meeting #100bis e-Meeting, April 20th – 30th, 2020 | R1-2002558 |
| --- | --- |

Agenda item:	7.2.9.1

Source: 	Qualcomm Incorporated

Title: 	Maintenance for NR mobility enhancements

Document for:	Discussion/Decision

# Introduction

The contribution discusses clarification on uplink power control to support Rel-16 NR DAPS handover.

# Uplink Power Control

RAN1#99 made the following agreements on uplink power control to source and target cells:

Agreement: - If a UE is configured with DAPS HO operation, the UE performs transmission power control based on Section 7.6.2 of 38.213 replacing the MCG with target MCG and SCG with source MCG. - Note: This assumes the latest draft CR in R1-1913214 as the baseline - Note: The specific terminology used for “target MCG” and “source MCG” may be different in 38.213 to align with other specifications - Note: Existing agreement on UL signal/channel dropping in UL transmission collisions should apply on top of the updated agreement above.

The agreement was implemented in Clause 15 of TS 38.213 [1] as:

If the UE indicates UplinkPowerSharingDAPS-HO = Semistatic-mode1 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode1, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HO = Semi-static-mode1 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE indicates UplinkPowerSharingDAPS-HO = Semistatic-mode2 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode2, the UE determines a transmission power for the target MCG or for the source SCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HO = Semi-static-mode2 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE indicates UplinkPowerSharingDAPS-HO = Dynamic and is provided UplinkPowerSharingDAPS-HO-mode = Dynamic, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HO = Dynamic by considering the target MCG as the MCG and the source MCG as the SCG.

To properly capture the agreement, we should change UplinkPowerSharingDAPS-HO in “… as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HO …” to NR-DC-PC-mode. Furthermore, we should align the terminology for Semi-static mode i.e., changing “Semistatic-mode” to “Semi-static-mode”.

Proposal: Adopt the following TP to Clause 15 of TS38.213:

15 Dual active protocol stack based handover <unchanged text omitted> If the UE indicates UplinkPowerSharingDAPS-HO = Semi-static-mode1 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode1, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for NR-DC-PC-mode = Semi-static-mode1 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE indicates UplinkPowerSharingDAPS-HO = Semi-static-mode2 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode2, the UE determines a transmission power for the target MCG or for the source SCG as described in Clause 7.6.2 for NR-DC-PC-mode = Semi-static-mode2 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE indicates UplinkPowerSharingDAPS-HO = Dynamic and is provided UplinkPowerSharingDAPS-HO-mode = Dynamic, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for NR-DC-PC-mode = Dynamic by considering the target MCG as the MCG and the source MCG as the SCG. <unchanged text omitted>

# Conclusion

The contribution provides our clarification on physical layer aspects to support DAPS handover. More specifically, we have made the following proposals:

Proposal: Adopt the following TP to Clause 15 of TS38.213:

15 Dual active protocol stack based handover <unchanged text omitted> If the UE indicates UplinkPowerSharingDAPS-HO = Semi-static-mode1 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode1, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for NR-DC-PC-mode = Semi-static-mode1 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE indicates UplinkPowerSharingDAPS-HO = Semi-static-mode2 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode2, the UE determines a transmission power for the target MCG or for the source SCG as described in Clause 7.6.2 for NR-DC-PC-mode = Semi-static-mode2 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE indicates UplinkPowerSharingDAPS-HO = Dynamic and is provided UplinkPowerSharingDAPS-HO-mode = Dynamic, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for NR-DC-PC-mode = Dynamic by considering the target MCG as the MCG and the source MCG as the SCG. <unchanged text omitted>

# Reference

- TS 38.213-g00