3GPP TSG-RAN WG1 Meeting #100bis-e	R1-2002650

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.3.3

Source:	Ericsson

Title:	Definition of IAB half-duplex operation in paired spectrum

Document for:	Discussion, Decision

# Introduction

In RAN1#99, the following working assumptions on IAB resource multiplexing in paired spectrum were reached [1] :

Working assumption: For paired spectrum, the H/S/NA DU resource configuration framework is extended with the following: - Two separate H/S/NA per-cell DU semi-static configurations are provided (with slot level resolution) for downlink and uplink FFS: whether the definition of half-duplex operation has to be modified for IAB nodes operating in paired spectrum

In this contribution, we provide some additional views on this issue.

# Discussion

## Half-duplex operation for paired spectrum

In IAB Rel-16 context, half-duplexing refers to

- No simultaneous DU transmission (downlink) and MT reception (downlink), or

- No simultaneous DU reception (uplink) and MT transmission (uplink)

which is different from the classic half-duplex on link level, i.e. no simultaneous Tx and Rx on the same link.

In Figure 1, two examples are given to illustrate the IAB half-duplex and full-duplex operation in paired spectrum, where fDL is the DL carrier and fUL is the UL carrier, respectively. In Figure 1A, IAB-MT and IAB-DU are TDM multiplexed, subjected to the half-duplex constraint. In IAB full duplex operation, as depicted in Figure 1B, simultaneous DU DL Tx and MT DL Rx operation during time slots t1, and/or simultaneous DU UL Rx and MT UL Tx operation during time slots t2 could take place. We note that IAB operation in paired spectrum would require that intra-node interference on the same carrier can be sufficiently suppressed, e.g. the transmission of the DU on a DL carrier must not leak into the MT reception on the same carrier, if used simultaneously. This is no different issue as has led to the restriction to IAB half-duplexing in Rel-16 IAB.

IAB operation in paired spectrum faces the same intra-node interference challenges as operation in unpaired spectrum.

Figure 1: Examples of IAB half-duplex and IAB full-duplex operation for paired spectrum.

The definition of IAB half-duplex operation in unpaired spectrum applies for IAB nodes operating in paired spectrum.

Confirm the working assumption from RAN1 #99 on DU resource configuration for IAB nodes operating in paired spectrum, taking into account Proposal 1.

# Conclusion

In this paper we made the following observations:

Observation 1	IAB operation in paired spectrum faces the same intra-node interference challenges as operation in unpaired spectrum.

Based on the discussion above we propose the following:

Proposal 1	The definition of IAB half-duplex operation in unpaired spectrum applies for IAB nodes operating in paired spectrum.

Proposal 2	Confirm the working assumption from RAN1 #99 on DU resource configuration for IAB nodes operating in paired spectrum, taking into account Proposal 1.

# References

RAN1, Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #99, November 2019.