3GPP TSG RAN WG1 Meeting #100bis-e	R1-2002714

E-meeting, April 20 – April 30, 2020

Agenda Item:	6.2.2.2

Source:	Moderator (Huawei)

Title:	Feature lead summary #2 on transmission in pre-configured UL resources for NB-IoT

Document for:	Discussion and Decision

# Introduction

This contribution summarizes NB-IoT Rel-16 PUR related discussions and proposals. According to the guidance of the chairman below, the intention of this contribution is to provide a list of issues/proposals and identify the set of email threads.

- April 13-17: preparation phase

  - April 13th – 14th: FLs to prepare summary

  - April 15th – 17th: FLs to lead the discussion identifying the set of email threads

Section 2 lists all the issues discussed in the contributions, and FL’s view on the priority of each issue is summarized in Section 3.

# Issues

## Issue#1: Discussion on “LS on open PUR issues for NB-IoT/eMTC”

Description: RAN2 has sent an LS [6] to RAN1 to ask for confirmation/clarification on some issues related to L1 adjustment on the (N)PUSCH repetition number. Several companies present analysis on the LS in AI 6.2.2.2 [1][2][3] and AI 5 [7][8][9]. [2][3] have provided TP for Section 16.5.1.1 of TS 36.213.

In the RAN2 LS [6], the following questions are asked to RAN1:

- Q1: Confirm whether the L1 adjustment on the (N)PUSCH repetition number is not intended to update the higher layer (i.e. RRC) configuration but to be used instead of the configuration provided by higher layers.

- Q2: Clarify whether the L1 adjustment on the (N)PUSCH repetition number is intended to apply only to the next upcoming PUR UL transmission or for all future PUR UL transmissions, and whether PHY layer would store the adjustment.

- Q3: Clarify how the L1 adjustment on the (N)PUSCH repetition number is updated/released and whether RRC layer can update or release the adjustment after the L1 adjustment has been previously received.

Company views in RAN1#100bis-e are summarized as follows:

- Regarding Q1:

  - Alt1 (is not intended to update): Qualcomm, Ericsson

  - Alt2 (is intended to update): Huawei/HiSilicon, ZTE

- Regarding Q2:

  - Q2-1: Apply only to the next PUR or all future PUR?

    - Apply only to the next PUR: Ericsson

    - Apply to all future PUR: Huawei/HiSilicon, Qualcomm, ZTE

  - Q2-2: PHY layer would store the adjustment?

    - Yes: Qualcomm, Ericsson

    - No: Huawei/HiSilicon, ZTE

- Regarding Q3:

  - (N)PUSCH repetition number can be further updated by dedicated PUR ACK DCI or PUR re-configuration RRC signaling: Huawei/HiSilicon, Qualcomm, Ericsson, ZTE

FL’s comment: RAN1 needs to discuss this issue, and send LS response to RAN2. Based on the response, RAN1 may need to work out the TP for TS 36.213.

TP provided by Qualcomm [3] is:

<TP1, TS 36.213, Subclause 16.5.1.1> The resource allocation information in uplink DCI format N0 for NPUSCH transmission or configured by higher layers for NPUSCH transmission using preconfigured uplink resource indicates to a scheduled UE - a set of contiguously allocated subcarriers () of a resource unit determined by the Subcarrier indication field, - a number of resource units () determined by the resource assignment field according to Table 16.5.1.1-2, - a repetition number () determined by the repetition number field according to Table 16.5.1.1-3. For a NPUSCH transmission using preconfigured uplink resource, the UE shall use the repetition number determined by the NPUSCH repetition adjustment field according to Table 16.5.1.1-3 from the most recent NPDCCH DCI format N0 with CRC scrambled by PUR C-RNTI with the value of "modulation and coding scheme" field () set to '14' if detected, or from the most recent reception of PUR-Config-NB containing pur-PhysicalConfig, whichever was received later configured by higher layers otherwise. </TP1>

TP provided by ZTE [2] is:

TS36.213 16.5.1.1 Resource allocation <Unchanged parts are omitted> The resource allocation information in uplink DCI format N0 for NPUSCH transmission or configured by higher layers for NPUSCH transmission using preconfigured uplink resource indicates to a scheduled UE - a set of contiguously allocated subcarriers () of a resource unit determined by the Subcarrier indication field or by high layers for a NPUSCH transmission using preconfigured uplink resource, - a number of resource units () determined by the resource assignment field according to Table 16.5.1.1-2 or by high layers for a NPUSCH transmission using preconfigured uplink resource, - a repetition number () determined by the repetition number field according to Table 16.5.1.1-3. For a NPUSCH transmission using preconfigured uplink resource, the UE shall use the repetition number determined by high layers. If the UE detects the NPUSCH repetition adjustment field according to Table 16.5.1.1-3 from NPDCCH DCI format N0 with CRC scrambled by PUR C-RNTI with the value of "modulation and coding scheme" field () set to '14', the UE shall apply the NPUSCH repetition adjustment and forward it to higher layers. The subcarrier spacing of NPUSCH transmission is determined by the uplink subcarrier spacing field in the Narrowband Random Access Response Grant according to Subclause 16.3.3. For NPUSCH transmission with subcarrier spacing, where is the subcarrier indication field. is reserved. <Unchanged parts are omitted>

## Issue#2: Collision handling between PUR transmission/ PUR SS monitoring and Paging CSS/ WUS

Description: During RAN1#100-e discussion, it was mentioned that RAN2 already agreed PUR transmission is prioritized when overlapping with Paging CSS, and RAN1#100-e agreed on the related TP for 36.213. Several companies point out the following collision cases also need to be discussed:

- Case1: What’s the UE behavior when PUR transmission overlaps with WUS?

  - Prioritize PUR transmission: ZTE

- Case2: What’s the UE behavior when PUR SS monitoring overlaps with Paging CSS?

  - Prioritize PUR SS monitoring: Huawei/HiSilicon

- Case3: What’s the UE behavior when PUR SS monitoring overlaps with WUS?

  - Prioritize PUR SS monitoring: Huawei/HiSilicon

  - Up to UE implementation: ZTE

TP provide by Huawei/HiSilicon [1] is:

------------------------------------- Start of Text Proposal for TS 36.213------------------------------- ------------------------------------------- Unchanged parts omitted -------------------------------------- ## 16.6 Narrowband physical downlink control channel related procedures … If the UE has initiated a NPUSCH transmission using preconfigured uplink resource ending in subframe n, the UE shall monitor the NPDCCH UE-specific search space in a search space window starting in n+4 subframe with duration given by higher layer parameter pur-SS-window-duration. Upon detection of a NPDCCH with DCI format N0 with CRC scrambled by PUR C-RNTI intended for the UE within the search space window and the value of "modulation and coding scheme" field () in the corresponding DCI is set to '14', the UE is not required to monitor the NPDCCH UE-specific search space for the remaining search space window duration. The UE is not required to monitor Type1-NPDCCH common search space or NWUS if it overlaps with the NPDCCH UE-specific search space within the search space window. -------------------------------------------- Unchanged parts omitted ------------------------------------- ---------------------------------------------- End of Text Proposal ----------------------------------------

## Issue#3: Clarification on “after the UE has initiated a NPUSCH transmission using preconfigured uplink resource” in TS 36.213

Description: Ericsson [4] points out “after the UE has initiated a NPUSCH transmission using preconfigured uplink resource” is not very clear due to the following reasons:

- It does not clarify how long this “after” lasts.

- It does not restrict whatever comes next to a particular search space or RNTI

- The legacy text (which is of the highest interest to most readers) have been made less clear or even obscure

So Ericsson [4] proposes to replace the formulation with “in case of NPDCCH transmission associated with PUR C-RNTI using NPDCCH UE-specific search space” and rearrange some of the sentences.

TP provided by Ericsson [4] is:

------------------------------------- Start of Text Proposal for TS 36.213-------------------------------- -------------------------------------------- Unchanged parts omitted -------------------------------------- # 16.6 Narrowband physical downlink control channel related procedures … For NPDCCH UE-specific search space, the aggregation and repetition levels defining the search spaces and the corresponding NPDCCH candidates are listed in Table 16.6-1 by substituting the value of with the higher layer configured parameter npdcch-NumRepetitions, except in case of NPDCCH transmission associated with PUR C-RNTI using NPDCCH UE-specific search space in which case it is given by higher layer parameter npdcch-NumRepetitions-PUR. For Type1-NPDCCH common search space and Type1A-NPDCCH common search space, the aggregation and repetition levels defining the search spaces are listed in Table 16.6-2 by substituting the value of … The locations of starting subframe are given by where is the th consecutive NB-IoT DL subframe from subframe , excluding subframes used for transmission of SI messages, and , and , and where - subframe is a subframe satisfying the condition , where , T≥4. - for NPDCCH UE-specific search space, - is given by the higher layer parameter npdcch-StartSF-USS, except in case of NPDCCH transmission associated with PUR C-RNTI using NPDCCH UE-specific search space in which case it is given by higher layer parameter npdcch-StartSF-USS-PUR, - is given by the higher layer parameter npdcch-Offset-USS, except in case of NPDCCH transmission associated with PUR C-RNTI using NPDCCH UE-specific search space in which case it is given by higher layer parameter npdcch-Offset-USS-PUR, - for NPDCCH Type2-NPDCCH common search space, - is given by the higher layer parameter npdcch-StartSF-CSS-RA, - is given by the higher layer parameter npdcch-Offset-RA, -------------------------------------------- Unchanged parts omitted -------------------------------------- ---------------------------------------------- End of Text Proposal ----------------------------------------

## Issue#4: Clarification on NPUSCH repetition adjustment for PUR

Description: Ericsson and Nokia have provided TP for Section 16.5.1.1 for TS 36.213 to make the spec clearer:

- Ericsson [4]: The actual parameter NRUPUR that the UE uses to determine the repetition number when not indicated via DCI has been added.

- Nokia [5]: the adjustment in the number of repetitions can occur regardless of whether the DCI indicates ACK or fallback, or an UL-Grant. The current text in the specification excludes a repetition adjustment given by an UL-Grant and the initial PUR transmission.

TP provided by Ericsson [4]:

------------------------------------- Start of Text Proposal for TS 36.213-------------------------------- -------------------------------------------- Unchanged parts omitted -------------------------------------- #### 16.5.1.1 Resource allocation The resource allocation information in uplink DCI format N0 for NPUSCH transmission or configured by higher layers for NPUSCH transmission using preconfigured uplink resource indicates to a scheduled UE - a set of contiguously allocated subcarriers () of a resource unit determined by the Subcarrier indication field, - a number of resource units () determined by the resource assignment field according to Table 16.5.1.1-2, - a repetition number () determined by the repetition number field according to Table 16.5.1.1-3. For a NPUSCH transmission using preconfigured uplink resource, the UE shall use the repetition number determined by the NPUSCH repetition adjustment field according to Table 16.5.1.1-3 from the most recent NPDCCH DCI format N0 with CRC scrambled by PUR C-RNTI with the value of "modulation and coding scheme" field () set to '14' if detected, otherwise as configured by the higher layer parameter NRUPUR. -------------------------------------------- Unchanged parts omitted -------------------------------------- ---------------------------------------------- End of Text Proposal ----------------------------------------

TP provided by Nokia [5]:

------------------------------------- Start of Text Proposal for TS 36.213-------------------------------- -------------------------------------------- Unchanged parts omitted -------------------------------------- #### 16.5.1.1 Resource allocation The resource allocation information in uplink DCI format N0 for NPUSCH transmission or configured by higher layers for NPUSCH transmission using preconfigured uplink resource indicates to a scheduled UE - a set of contiguously allocated subcarriers () of a resource unit determined by the Subcarrier indication field, - a number of resource units () determined by the resource assignment field according to Table 16.5.1.1-2, - a repetition number () determined by the repetition number field according to Table 16.5.1.1-3. For a NPUSCH (re)transmission using preconfigured uplink resource, the UE shall use the repetition number determined by - the higher layer parameter in case of an initial NPUSCH transmission. - the NPUSCH repetition number field in case of an NPUSCH retransmission, - the NPUSCH repetition adjustment field in case of an ACK/fallback indication according to Table 16.5.1.1-3, from the most recent NPDCCH DCI format N0 with CRC scrambled by PUR C-RNTI with the value of "modulation and coding scheme" field () set to '14' if detected, configured by higher layers. otherwise. -------------------------------------------- Unchanged parts omitted -------------------------------------- ---------------------------------------------- End of Text Proposal ----------------------------------------

## Issue#5: Support of NPDCCH order for PUR

Description: Support of NPDCCH order for PUR was discussed in previous meetings, but there was no consensus. ZTE [2] proposes to support NPDCCH order for PUR considering the benefits of contention-free, less signaling overhead, larger max TBS size than EDT, etc.

TP provided by ZTE [2] is:

TS36.212 6.4.3.2 DCI Format N1 <Unchanged parts are omitted> Format N1 is used for random access procedure initiated by a NPDCCH order only if NPDCCH order indicator is set to '1', format N1 CRC is scrambled with C-RNTI or PUR-RNTI, and all the remaining fields are set as follows: - Preamble format indicator – 1 bit, where value 0 indicates preamble format 0/1 and value 1 indicates preamble format 2. This field is only present if SystemInformationBlockType2-NB or SystemInformationBlockType23-NB is configured and the UE indicates the nprach-Format2 capability. - Starting number of NPRACH repetitions – 2 bits as defined in subclause 16.3.2 of [3] - Subcarrier indication of NPRACH – 6 or 8 bits, this field is 8 bits only if Preamble format indicator is present and set to 1, as defined in subclause 16.3.2 of [3] - Carrier indication of NPRACH – 4 bits as defined in subclause 16.3.2 of [3]. This field is only present if ul-ConfigList is configured and the UE indicates the multiCarrier-NPRACH capability. - All the remaining bits in format N1 are set to one <Unchanged parts are omitted>

## Issue#6: Support of UE sends ACK, i.e., NPUSCH format 2, upon receiving PUR ACK DCI to solve the misalignment issue between eNB and UE

Description: As pointed out by ZTE [2], since RAN2 reconfirms the RAN2#107 agreement “Fallback after D-PUR transmission is not successful is not specified i.e. it is up to UE implementation to initiate legacy RA, MO-EDT or wait for next D-PUR occasion”, it is possible that eNB and UE may have some misalignment in case PUR ACK DCI carrying NPUSCH repetition adjustment and Timing advance adjustment has been sent by eNB but is not successfully received by PUR UE. In order to solve the misalignment issue between eNB and UE, ZTE [2] proposes that UE sends ACK, i.e., NPUSCH format 2, upon receiving PUR ACK DCI.

## Issue#7: Clarification on PUR power control

Description: Ericsson [4] points out when PONPUSCH,  c(3), its relation to the HL parameter “pur-NPUSCH-power-control-P0” has not been added. In addition, replacing “” by “PO_NPUSCH,cj”, since the character “c” overlaps the character “H” in the following equation that today appears in specs.

TP provided by Ericsson is:

------------------------------------- Start of Text Proposal for TS 36.213-------------------------------- -------------------------------------------- Unchanged parts omitted -------------------------------------- ##### 16.2.1.1.1 UE behaviour … - PO_NPUSCH,cj, is a parameter composed of the sum of a component provided from higher layers and a component provided by higher layers for j=1 and for serving cell where . For NPUSCH (re)transmissions corresponding to a dynamic scheduled grant or a semi-persistent grant then j=1, for NPUSCH (re)transmissions corresponding to the random access response grant then j=2 and for NPUSCH transmission using preconfigured uplink resource then j=3. PO_UE_NPUSCH,c2=0 and PO_UE_NPUSCH,c3= pur-NPUSCH-power-control-P0. If enhanced random access power control is not applied, PO_NOMINAL_NPUSCH,c(2)=PO_PRE+∆PREAMBLE_Msg3, where the parameter preambleInitialReceivedTargetPower [8] () and ∆PREAMBLE_Msg3 are signalled from higher layers for serving cell . If enhanced random access power control is applied, PO_NOMINAL_NPUSCH,c2=MSG3_RECEIVED_TARGET_POWER +ΔPREAMBLE_Msg3 - For j=1, for NPUSCH format 2, =1; for NPUSCH format 1, is provided by higher layers for serving cell . For j=2, For j=3, is the parameter pur-NPUSCH-power-control-alpha provided by higher layers for serving cell . -------------------------------------------- Unchanged parts omitted -------------------------------------- ---------------------------------------------- End of Text Proposal ----------------------------------------

## Issue#8: Clarification on PUR SS window

Description: Regarding the start of PUR SS window, Ericsson [4] points out the order in which the word “subframe” appears in the text is not in line with how similar statements have been captured in the specifications.

TP provided by Ericsson is:

------------------------------------- Start of Text Proposal for TS 36.213-------------------------------- -------------------------------------------- Unchanged parts omitted -------------------------------------- ## 16.6 Narrowband physical downlink control channel related procedures … If the UE has initiated a NPUSCH transmission using preconfigured uplink resource ending in subframe n, the UE shall monitor the NPDCCH UE-specific search space in a search space window starting in subframe n+4 with duration given by higher layer parameter pur-SS-window-duration. Upon detection of a NPDCCH with DCI format N0 with CRC scrambled by PUR C-RNTI intended for the UE within the search space window and the value of "modulation and coding scheme" field () in the corresponding DCI is set to '14', the UE is not required to monitor the NPDCCH UE-specific search space for the remaining search space window duration. -------------------------------------------- Unchanged parts omitted -------------------------------------- ---------------------------------------------- End of Text Proposal ----------------------------------------

# Summary of preparation phase (13th – 17th April)

During the preparation phase, companies provided their views on each issue, which are collected in Annex A. Based on these, FL’s view on the priority of each issue in given in Table 1 below.

The following two email discussions are suggested:

- Email discussion#1: Collision handling between PUR transmission/ PUR SS monitoring and Paging CSS/ WUS

  - Refer to Issue#2

- Email discussion#2: Clarification on PUR power control

  - Refer to Issue#7

Besides, the following are also suggested:

- Email discussion: Discussion on “LS (R1-2001517) on open PUR issues for NB-IoT/eMTC” and related TP

  - Refer to Issue#1 and Issue#4

  - Note: how to manage the discussion on LS is up to Chairman’s decision. And as per Chairman’s guideline, the discussion on LS does not consume the email thread budget for the respective WI

- Editorial issue: Replacing “n+4 subframe” by “subframe n+4” in Section 16.6 of TS 36.213

  - Refer to Issue#8

  - It is consensus that this is an editorial issue, so the FL will request the spec editor to handle this

Table 1 FL’s view on the priority of each issue

| Issue | FL’s view on the priority | Summary of company view |
| --- | --- | --- |
| Issue#1: Discussion on “LS on open PUR issues for NB-IoT/eMTC” | High | High: 7 |
| Issue#2: Collision handling between PUR transmission/ PUR SS monitoring and Paging CSS/ WUS | High | High: 4 Low: 2 |
| Issue#3: Clarification on “after the UE has initiated a NPUSCH transmission using preconfigured uplink resource” in TS 36.213 | Low | High: 1 Medium: 1 Low: 5 |
| Issue#4: Clarification on NPUSCH repetition adjustment for PUR | High (jointly discussed with #1) | High: 4 Low: 1 |
| Issue#5: Support of NPDCCH order for PUR | Low | Medium: 1 Low: 6 |
| Issue#6: Support of UE sends ACK, i.e., NPUSCH format 2, upon receiving PUR ACK DCI to solve the misalignment issue between eNB and UE | Low | Medium: 1 Low: 6 |
| Issue#7: Clarification on PUR power control | Medium | High: 1 Medium: 2 Low: 3 Editorial: 1 |
| Issue#8: Clarification on PUR SS window | Editorial | Editorial: 7 |

# Reference

- R1-2001570	Corrections on transmission in preconfigured UL resources	Huawei, HiSilicon

- R1-2001851	Remaining issues for transmission in preconfigured UL resources for NB-IoT	ZTE

- R1-2002176	Support for transmission in preconfigured UL resources	Qualcomm Incorporated

- R1-2002507	Corrections for Preconfigured UL resources for NB-IoT	Ericsson

- R1-2002645	Remaining issues for preconfigured UL resources	Nokia, Nokia Shanghai Bell

- R1-2001517	LS on open PUR issues for NB-IoT/eMTC	RAN2, Ericsson

- R1-2001849	Discussion on RAN2 LS on open PUR issues	ZTE

- R1-2002501	On the LS on open PUR issues for NB-IoT/eMTC	Ericsson

- R1-2002603	Draft reply LS on open PUR issues for NB-IoT/eMTC	Huawei, HiSilicon

# Annex A. Company’s view during preparation phase (13th – 17th April)

| Issue | FL’s initial view on priority | Company’s view on priority and comments |
| --- | --- | --- |
| Issue#1: Discussion on “LS on open PUR issues for NB-IoT/eMTC” | High | [Lenovo&MotoM] High [Ericsson] High ZTE/Sanechips: High. But we think the discussion of LS should not take the allocated time/emal thread of the original agenda item. [Sierra] High [Qualcomm:] High priority [Huawei/HiSilicon] High [LG Electronics] High |
| Issue#2: Collision handling between PUR transmission/ PUR SS monitoring and Paging CSS/ WUS | High | [Lenovo&MotoM] High [Ericsson] Low. RAN2 has previously decided on prioritizations of paging and PUR, in a similar way they should reach agreements on issue # 2. Low in case a LS were needed. ZTE/Sanechips: High. Trigger by ran2 LS and left over from last meeting. The collision issue need to be solved in anyway , even if the answer is implied by ran2 LS it is still need to be discussed and agreed in ran1. [Sierra] Low. [Huawei/HiSilicon] High. Such collision issues were raised in RAN1 during Rel-16, but has not been closed due to the limited time. Since RAN1 has already addressed one collision case in RAN1#100-e, it is straightforward to continue addressing the remaining collision cases in RAN1. And also the spec impact will be in RAN1. So we think RAN1 can discuss and decide on these issues. [LG Electronics] High |
| Issue#3: Clarification on “after the UE has initiated a NPUSCH transmission using preconfigured uplink resource” in TS 36.213 | Medium | [Lenovo&MotoM] Low [Ericsson] High. The legacy text (which is of the highest interest to most readers) has been made less clear or even obscure, hence the correction is important. ZTE/Sanechips: low. effort of editorial clarification. specs is clear .. [Sierra] Low [Qualcomm:] Low priority. Still, the intent of Ericsson’s wording seems better than current wording. [Huawei/HiSilicon] Low. The spec is clear considering the context. [LG Electronics] Med. Need discussion for better wording. |
| Issue#4: Clarification on NPUSCH repetition adjustment for PUR | Low | [Lenovo&MotoM] Low [Ericsson] High. The clause under discussion touches upon Issue 1 and that is why we gave the same classification. Issues #1 and #4 can be discussed together. ZTE/Sanechips: High. Need to be clarifed for PUR to work. Many companies submitted proposal. [Sierra] High – can be treated as part of LS response to RAN2 [Qualcomm:] Needs to be jointly discussed with LS (may depend on what we tell RAN2) [Huawei/HiSilicon] Since Issue#1 may result in TP for the same section as Issue#4, i.e., Section 16.5.1.1 of TS 36.213, we suggest Issue#4, if necessary, can be jointly discussed with Issue#1. [LG Electronics] High. Can be discussed as part of Issue#1. |
| Issue#5: Support of NPDCCH order for PUR | Low | [Lenovo&MotoM] Low [Ericsson] Low. As pointed out in the FLS, it “was discussed in previous meetings, but there was no consensus”. ZTE/Sanechips: Mid. Since both DCI format N0/N1 are supported, and PDCCH order support also exsits with N0/N1, so it needs to be clarified to be support . [Sierra] Low – spec not broken [Qualcomm:] Low priority. No agreements in this regard. [Huawei/HiSilicon] Low. Not essential. [LG Electronics] Low. Not essential. |
| Issue#6: Support of UE sends ACK, i.e., NPUSCH format 2, upon receiving PUR ACK DCI to solve the misalignment issue between eNB and UE | Low | [Lenovo&MotoM] Low [Ericsson] Low. This proposal makes the L1-signaling one step closer to the L2/L3-signaling. We do not see Issue #6 as correction, but rather as a proposal. ZTE/Sanechips: Mid. In RAN2 LS （R1-1913505）there is the following statement "if nothing is received by the UE in response to the UL data transmission on PUR, it is left up to UE implementation to initiate legacy RA, MO-EDT or wait for next D-PUR occasion” This implies there could be a misalignmentissue between eNB and UE , if the PUR ack DCI has TA or repetition adjustment but UE fails to detect and UE decides to wait for next PUR to transmit. [Sierra] Low [Qualcomm:] Low priority. No agreements in this regard. [Huawei/HiSilicon]: Low. As discussed in previous meetings, this can be solved by implementation, nothing is wrong with the specification. [LG Electronics] Low |
| Issue#7: Clarification on PUR power control | Editorial | [Lenovo&MotoM] Low [Ericsson] High. Issue #7 has one editorial aspect, and one technical issue which are unrelated. The technical issue is that forPONPUSCH, c(3), its relation to the HL parameter “pur-NPUSCH-power-control-P0” has not been established. ZTE/Sanechips: editorial [Sierra] Medium – spec is unclear. [Qualcomm:] Low priority. Clear from context. [Huawei/HiSilicon] Low. The spec is clear. Replacing “” by “PO_NPUSCH,cj” is editorial. [LG Electronics] Med. Need discussion for better wording/clarification. |
| Issue#8: Clarification on PUR SS window | Editorial | [Lenovo&MotoM] Low，Editorialissue [Ericsson] Medium. Editorial issue that Editor of TS 36.213 can correct, as it was done with similar editorial issues in the previous meeting. ZTE/Sanechips:editorial. [Sierra] – Editor can correct. [Qualcomm:] Editorial. [Huawei/HiSilicon] Editorial. [LG Electronics] Editorial |