3GPP TSG RAN WG1 #100b	   	 	  	                                         R1-2002844

e-Meeting, April 20th – 30th, 2020

Agenda item: 7.2.6.3.

Source: Moderator (LG Electronics)

Title: TP for capturing outcome of email thread [100b-e-NR-eMIMO-MB1-02]

Document for: Discussion and Decision

- Analysis

| # Reasons for change | # When the number of RS resources configured by RRC for pathloss estimation for PUCCH, PUSCH and SRS is greater than 4, it is unclear on which RS resources UE is required to maintain for pathloss estimation. |
| --- | --- |
| # Summary of changes | # Add description with regards to the RS resources UE is required to maintain for pathloss estimation when the number of RS resources configured by RRC for pathloss estimation for PUCCH, PUSCH and SRS is greater than 4 |
| # Specs/Sections impacted | # TS 38.213 V16.1.0, clause 7 |
| # Consequences if not approved | # Specification is ambiguous on which RS resources UE is required to maintain for pathloss estimation |

- Text Proposal

TP for clause 7 of TS 38.213 V16.1.0

# 7 Uplink Power control Uplink power control determines a power for PUSCH, PUCCH, SRS, and PRACH transmissions. A UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions as described in Clauses 7.1.1, 7.2.1, and 7.3.1, except for SRS transmissions configured by IE SRS-Positioning-Config as described in Clause 7.3.1. If the number of RS resources configured by RRC for pathloss estimation for PUCCH, PUSCH and SRS is greater than 4, UE is only required to maintain the RS resources which are used as qd for pathloss estimation in 7.1.1, 7.2.1 and 7.3.1 for any uplink channels or signals. --------------- Unchanged parts omitted -------------