3GPP TSG RAN WG1 #100bis-e			R1-2002886

e-Meeting, April 20th – 30th, 2020

Source:	NTT DOCOMO, INC.

Title:	Summary on Email discussion [100b-e-NR-UEFeatures-MRDCCA-04]

Agenda Item:	7.2.11.10

Document for: 	Discussion and Decision

- Introduction

This contribution summarizes the following email discussion in AI 7.2.11.10 regarding UE features for MR-DC/CA.

[100b-e-NR-UEFeatures-MRDCCA-04]  Email discussion/approval on issues with capability signaling impacts for Enhancements to Single Tx Switched Uplink Solution for EN-DC (dates TBD) – Hiroki (DCM)

- Discuss followings on FG18-2/2a/3/3a

  - Whether 18-3a is per band combination or per UE

  - Whether the component 5 “if UE indicates that it does not support simultaneous UL transmissions as defined in TS 38.101-3 [4] using single UL-Transmission, NR (SCG) UL transmission is dropped when an overlapping LTE (MCG) UL transmission is present (for type 1 UE)” for FG18-2/2a is kept (i.e., remove bracket) or removed

  - Whether/how the FG18-2a/3a are modified so that the capabilities generally allow LTE UL transmissions outside the HARQ-ACK designated subframes for single-Tx operation

  - Whether/how to clarify some details for FG18-2/2a/3/3a as below

    - Whether or not to clarify that EN-DC single-Tx operation is for synchronous EN-DC

    - For 18-2, whether or not “intra-band” in the name of FG should be removed

    - For 18-2a, whether “TDD” should be removed from “Applicable to in FDD-LTE TDD-NR EN-DC” or not

    - What is prerequisite for 18-3a

Following agreements were made in the email discussions [100b-e-NR-UEFeatures-MRDCCA-03].

Agreements:

- A new FG (18-2b) for support of HARQ-offset for SUO case1 in EN-DC with LTE TDD PCell for type 1 UE is introduced

Agreements:

- Keep FG18-2/2a/3/3a and discuss details including following proposals

Reformulate FG18-2/2a/3/3a as below

- For EN-DC single-Tx with FDD-PCell (FG18-2a):

  - Any types of LTE UL transmissions are NOT limited to tdm-pattern.

    - Including semi-static LTE UL

- For EN-DC dual-Tx with FDD-PCell (FG18-3):

  - Any types of LTE UL transmissions are NOT limited to tdm-pattern.

    - Including semi-static LTE UL

- For EN-DC single-Tx with TDD-PCell (FG18-2):

  - Any types of LTE UL transmissions are limited to tdm-pattern.

    - Including semi-static LTE UL

- For advanced EN-DC single-Tx with TDD-PCell (FG18-3a):

  - Any types of LTE UL transmissions are NOT limited to tdm-pattern.

    - Including semi-static LTE UL

Alternative proposal for updating FG18-2/2a/3/3a

- Keep FG18-2 for Single UL TX operation for TDD PCell in intra-band EN-DC

  - Component 4 should be (for type1 UE)

  - Component 5 should be deleted

- Keep FG18-2a for Enhanced single UL TX operation for FDD Pcell EN-DC

  - Component 5 should be deleted

- Keep FG18-3 for Dual Tx transmission for EN-DC with FDD PCell(TDM pattern for dual Tx UE)

- Keep FG18-3a for Semi-statically configured LTE UL transmissions in all UL subframes not limited to tdm-pattern

  - Should be per BC

  - Should be only for type 1 UE (as per RAN1#99 agreement)

- FG18-2/2a/3/3a based on [1]

Based on [1] and agreements made in [100b-e-NR-UEFeatures-MRDCCA-03], FG18-2/2a/3/3a can be defined as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18. MR-DC/CA enhancement | 18-2 | Single UL TX operation for TDD PCell in EN-DC | TDM restriction to LTE TDD PCell in EN-DC for single UL-Transmission associated functionality when tdm-patternConfig-r16 is configured 1) TDD UL/DL configuration#2, #4, #5 configured as DL-reference UL/DL configuration 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) HARQ subframe offset 4) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern (for type 1 UE) [5) if UE indicates that it does not support simultaneous UL transmissions as defined in TS 38.101-3 [4] using singleUL-Transmission, NR (SCG) UL transmission is dropped when an overlapping LTE (MCG) UL transmission is present (for type 1 UE).”] | EN-DC | Yes | N/A |  | Per band combination | Applicable to TDD-TDD EN-DC only | Applicable to FR1 only |  | Extension of the R15 capability tdm-Pattern to TDD PCell 3) {not supported, supported} | Optional with capability signalling |
|  | 18-2a | Enhanced single UL TX operation for FDD Pcell EN-DC | TDM restriction to LTE FDD Pcell in EN-DC for single UL-Transmission associated functionality when tdm-patternConfig-r16 is configured 1) DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) HARQ sub-frame offset 4) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern (for type 1 UE) [5) if UE indicates that it does not support simultaneous UL transmissions as defined in TS 38.101-3 [4] using singleUL-Transmission, NR (SCG) UL transmission is dropped when an overlapping LTE (MCG) UL transmission is present (for type 1 UE).”] | 6-13 | Yes | N/A |  | Per band combination | Applicable to in FDD-LTE -NR EN-DC | Applicable to FR1 only |  | Enhancement to the R15 capability tdm-Pattern | Optional with capability signalling |
|  | 18-3 | Dual Tx transmission for EN-DC with FDD PCell(TDM pattern for dual Tx UE) | TDM restriction to LTE FDD PCell in EN-DC for dual UL Tx operation when tdm-patternConfig-r16 is configured 1) DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) HARQ subframe offset 4) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern | EN-DC | Yes | N/A |  | Per band combination | Applicable to EN-DC with LTE FDD PCell only | Applicable to FR1 only |  | Extension of the R15 capability tdm-Pattern to a 2Tx UE | Optional with capability signalling |
|  | 18-3a | Semi-statically configured LTE UL transmissions in all UL subframes not limited to tdm-pattern | UE configured with tdm-patternConfig-r16 can be semi-statically configured with LTE UL transmissions in all UL subframes not limited to the reference tdm-pattern (only for type 1 UE) | 18-2, 18-2a | Yes | N/A |  | Per UE Per band combination | Applicable to EN-DC only | Applicable to FR1 only |  |  | Optional with capability signaling |

Companies are encouraged to provide feedbacks focusing on signaling design aspects (e.g., components with candidate values for reporting, Type, Need of xDD/FRx differentiation).

| Company | Comment |
| --- | --- |
| Ericsson | - For component 5 in 18-2 and 18-2a, suggest replacing the text in square brackets with “dropping NR transmission when LTE and NR transmissions collide for Type 1 UE”. It reflects the NR side impact of these FGs for Type1 UE (all other components only reflect LTE side functionality) and captures the NR dropping for collision case when all uplink subframes are allowed to be scheduled for LTE (i.e. per component 4) Agreements: For the single-Tx case, for FDD LTE Pcell, - All uplink subframes can be scheduled for LTE for type 1 UEs - In which case, NR transmission is dropped for when the LTE and NR transmissions collide - Note: there is no change of UL scheduling timing for LTE Agreements: For the single-Tx case, for TDD LTE Pcell, - All uplink subframes can be scheduled for LTE for type 1 UEs - In which case, NR transmission is dropped for when the LTE and NR transmissions collide - Prefer FG 18-3a to be per UE. Whether Case 1 HARQ timing is supported or not is indicated by per BC capability signaling already. Then additionally allowing reporting of separate variants of semi-static UL transmission handling for different BCs seems not necessary. |

Following views are provided in contributions for the RAN1#100bis-e meeting.

Based on the feedbacks, at least following points should be discussed.

- Whether 18-3a is per band combination or per UE

- Whether the component 5 “if UE indicates that it does not support simultaneous UL transmissions as defined in TS 38.101-3 [4] using single UL-Transmission, NR (SCG) UL transmission is dropped when an overlapping LTE (MCG) UL transmission is present (for type 1 UE)” for FG18-2/2a is kept (i.e., remove bracket) or removed

- Whether/how the FG18-2a/3a are modified so that the capabilities generally allow LTE UL transmissions outside the HARQ-ACK designated subframes for single-Tx operation

- Whether/how to clarify some details for FG18-2/2a/3/3a as below

  - Whether or not to clarify that EN-DC single-Tx operation is for synchronous EN-DC

  - For 18-2, whether or not “intra-band” in the name of FG should be removed

  - For 18-2a, whether “TDD” should be removed from “Applicable to in FDD-LTE TDD-NR EN-DC” or not

  - What is prerequisite for 18-3a

| [2] | ZTE Corporation | For Component 4 of FG-2, FG-2a and FG-3, the description ‘LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern’ is not very clear. ‘Any UL subframe of the TDM pattern’ seems to refer to the UL subframes designated as UL by TDM. However, FG-2a says this is only for type1 UE, which make this description confusing. We propose to update the Component 4 of FG-2, FG-2a and FG-3 as below. 4) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern (for type1 UE). |
| --- | --- | --- |
| [4] | Intel Corporation | HARQ subframe offset is agreed as optional feature for EN-DC with LTE TDD PCell in RAN1#98, which is not clearly reflected in FG 18-2 “3) HARQ subframe offset”. Since FG 18-3a was created for semi-static configured LTE UL transmission, component 4) in FG 18-2/2a/3 could be revised to apply to dynamic LTE UL transmission only. “3) Dynamic LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern (for type 1 UE)”. Proposal 2: for single TX - FG 18-2, component 3, to clarify ARQ subframe offset is optional for EN-DC with LTE TDD PCell - FG 18-3a, component 3, to clarify it is for dynamic LTE UL transmissions |
| [5] | Ericsson | - FGs 18-2,18-2a - Regarding component 5, - We propose to confirm addition of component 5. If shorter description is desirable it can be reworded as follows – “dropping of NR (SCG) UL transmission when an overlapping LTE (MCG) UL transmission is present (for Type 1 UEs that indicate lack of support of simultaneous UL transmissions using singleUL-Transmission)”. |
| [7] | Qualcomm Incorporated | We propose to update the latest feature groups as following. - Some changes on top of Rapporteur’s updated list are suggested. - We propose to clarify that EN-DC single-Tx operation is for synchronous EN-DC. - FG18-2a and FG18-3a can be UE capabilities that generally allows LTE UL transmissions outside the HARQ-ACK designated subframes for single-Tx operation. - Some editorial changes have been made. Component [5] added in the Rapproteur’s version is not necessary, as it is the UE behavior, not the component of the UE feature group. 18-2; Single UL TX operation for TDD Pcell in intra-band EN-DC; TDM restriction to LTE TDD Pcell in EN-DC for singleUL-Transmission associated functionality when tdm-patternConfig-r16 is configured 1) TDD UL/DL configuration#2, #4, #5 configured as DL-reference UL/DL configuration 3) HARQ subframe offset ]; EN-DC; N/A; Per band combination; Applicable to synchronous EN-DC with TDD-Pcell only; Applicable to FR1 only; Extension of the R15 capability tdm-Pattern to TDD Pcell 3) {not supported, supported}; Optional with capability signalling. 18-2a; Enhanced single UL TX operation for FDD Pcell EN-DC; TDM restriction to LTE FDD Pcell in EN-DC for singleUL-Transmission associated functionality when tdm-patternConfig-r16 is configured 1) DL-reference UL/DL configuration defined for LTE-FDD-Scell in LTE-TDD-FDD CA with LTE-TDD-Pcell 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) HARQ sub-frame offset 4) LTE UL transmissions scheduled/triggered by a DCI in any UL subframes (for type 1 UE) 5) semi-statically configured LTE UL transmissions in any UL subframes (for type 1 UE) ]; 6-13; N/A; Per band combination; Applicable to synchronous EN-DC with FDD-Pcell only; Applicable to FR1 only; Enhancement to the R15 capability tdm-Pattern; Optional with capability signalling. 18-3; Dual Tx transmission for EN-DC with FDD Pcell(TDM pattern for dual Tx UE); TDM restriction to LTE FDD Pcell in EN-DC for dual UL Tx operation when tdm-patternConfig-r16 is configured 1) DL-reference UL/DL configuration defined for LTE-FDD-Scell in LTE-TDD-FDD CA with LTE-TDD-Pcell 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) HARQ subframe offset 4) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern 5) semi-statically configured LTE UL transmissions in any UL subframes (for type 1 UE); EN-DC; N/A; Per band combination; Applicable to EN-DC with LTE FDD PCell only; Applicable to FR1 only; Extension of the R15 capability tdm-Pattern to a 2Tx UE; Optional with capability signalling. 18-3a; LTE UL transmissions in all UL subframes for FG18-2; For a UE configured with tdm-patternConfig-r16 for EN-DC single-Tx operation with TDD-Pcell, LTE UL transmissions in all UL subframes, not limite to tdm-pattern, is allowed 1) PRACH transmission in non-designated UL subframes given by the DL-reference configuration (for type 1 UE) 2) LTE UL transmissions scheduled/triggered by a DCI in any UL subframes (for type 1 UE) 3) semi-statically configured LTE UL transmissions in any UL subframes (for type 1 UE); 18-2; Per band combination; Applicable to synchronous EN-DC with TDD-Pcell only; Optional with capability signaling. |
| [8] | Huawei, HiSilicon | - FG 18-2 The “intra-band” in the name of FG 18-2 should be removed, because it is not aligned with WID nor agreements. In the WID or agreements of RAN1 for single TX enhancement, there is no limitations that the MCG and the SCG should be in the same band. “3) HARQ subframe offset” should be modified to “HARQ subframe offset (Optional feature with Rel-15 FG 8-1 as prerequisite feature)”. This is exactly shown in the agreement, since it should be a separate capability signaling for HARQ subframe offset. One alternative is to take this component out and create a new FG for it.” There should be a space between “single” and “UL” as captured in TS 38.306. Also its Italic format should be removed. This comment is also applied to FG 18-2a. “Component 5” about association with 38.101-3 should be removed because it causes big confusion by overlapping with component 4, and not sustained by any agreement. This comment is also applied to FG 18-2a. Proposal 2: For FG 18-2 the following should be accepted. - The “intra-band” in the name of FG 18-2 should be removed - “3) HARQ subframe offset” should be modified to “HARQ subframe offset (Optional feature with Rel-15 FG 8-1 as prerequisite feature)” - There should be a space between “single” and “UL” - “Component 5” about association with 38.101-3 should be removed, as well as in FG 18-2a - FG 18-2a “TDD” should be removed from “Applicable to in FDD-LTE TDD-NR EN-DC”, which is not aligned with agreements. Proposal 3: Remove “TDD” from “Applicable to in FDD-LTE TDD-NR EN-DC” in FG 18-2a. - FG 18-3a Add 18-3 as prerequisite according to the RAN1#99 agreement Proposal 4: Add 18-3 as prerequisite in FG 18-3a according to the RAN1#99 agreement. 18-2; Single UL TX operation for TDD PCell in EN-DC; TDM restriction to LTE TDD PCell in EN-DC for singleUL-Transmission associated functionality when tdm-patternConfig-r16 is configured 1) TDD UL/DL configuration#2, #4, #5 configured as DL-reference UL/DL configuration 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) HARQ subframe offset (Optional feature with Rel-15 FG 8-1 as prerequisite feature) 4) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern; Per band combination; Applicable to TDD-TDD EN-DC only; Applicable to FR1 only; Extension of the R15 capability tdm-Pattern to TDD PCell 3) {not supported, supported}; Optional with capability signalling. 18-2a; Enhanced single UL TX operation for FDD Pcell EN-DC; TDM restriction to LTE FDD Pcell in EN-DC for singleUL-Transmission associated functionality when tdm-patternConfig-r16 is configured 1) DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) HARQ sub-frame offset 4) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern (for type 1 UE); Per band combination; Applicable to in FDD-LTE NR EN-DC; Applicable to FR1 only; Enhancement to the R15 capability tdm-Pattern; Optional with capability signalling. 18-3a; Semi-statically configured LTE UL transmissions in all UL subframes not limited to tdm-pattern; UE configured with tdm-patternConfig-r16 can be semi-statically configured with LTE UL transmissions in all UL subframes not limited to the reference tdm-pattern; 18-2, 18-2a, 18-3; Per UE; Optional with capability signaling. |

- Reformulated version of FG18-2/2a/3/3a based on [9]

Based on agreements made in [100b-e-NR-UEFeatures-MRDCCA-03] and proposal in [9], reformulated version of FG18-2/2a/3/3a can be defined as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18. MR-DC/CA enhancement | 18-2 | Single UL TX operation for TDD Pcell in intra-band EN-DC | TDM restriction to LTE TDD Pcell in EN-DC for singleUL-Transmission associated functionality when tdm-patternConfig-r16 is configured - TDD UL/DL configuration#2, #4, #5 configured as DL-reference UL/DL configuration 3) HARQ sub-frame offset | EN-DC | Yes | N/A |  | Per band combination | Applicable to synchronous EN-DC with TDD-Pcell only | Applicable to FR1 only |  | Extension of the R15 capability tdm-Pattern to TDD Pcell 3) {not supported, supported} | Optional with capability signalling |
|  | 18-2a | Enhanced single UL TX operation for FDD Pcell EN-DC | TDM restriction to LTE FDD Pcell in EN-DC for singleUL-Transmission associated functionality when tdm-patternConfig-r16 is configured 1) DL-reference UL/DL configuration defined for LTE-FDD-Scell in LTE-TDD-FDD CA with LTE-TDD-Pcell 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) HARQ sub-frame offset 4) LTE UL transmissions scheduled/triggered by a DCI in any UL subframes (for type 1 UE) 5) semi-statically configured LTE UL transmissions in any UL subframes (for type 1 UE) ] | 6-13 | Yes | N/A |  | Per band combination | Applicable to synchronous EN-DC with FDD-Pcell only | Applicable to FR1 only |  | Enhancement to the R15 capability tdm-Pattern | Optional with capability signalling |
|  | 18-3 | Dual Tx transmission for EN-DC with FDD Pcell(TDM pattern for dual Tx UE) | TDM restriction to LTE FDD Pcell in EN-DC for dual UL Tx operation when tdm-patternConfig-r16 is configured 1) DL-reference UL/DL configuration defined for LTE-FDD-Scell in LTE-TDD-FDD CA with LTE-TDD-Pcell 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) HARQ subframe offset 4) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern 5) semi-statically configured LTE UL transmissions in any UL subframes (for type 1 UE) | EN-DC | Yes | N/A |  | Per band combination | Applicable to EN-DC with LTE FDD PCell only | Applicable to FR1 only |  | Extension of the R15 capability tdm-Pattern to a 2Tx UE | Optional with capability signalling |
|  | 18-3a | LTE UL transmissions in all UL subframes for FG18-2 | For a UE configured with tdm-patternConfig-r16 for EN-DC single-Tx operation with TDD-Pcell, LTE UL transmissions in all UL subframes, not limite to tdm-pattern, is allowed 1) PRACH transmission in non-designated UL subframes given by the DL-reference configuration (for type 1 UE) 2) LTE UL transmissions scheduled/triggered by a DCI in any UL subframes (for type 1 UE) 3) semi-statically configured LTE UL transmissions in any UL subframes (for type 1 UE) | 18-2 | Yes | N/A |  | Per band combination | Applicable to synchronous EN-DC with TDD-Pcell only | Applicable to FR1 only |  |  | Optional with capability signaling |

Companies are encouraged to provide feedbacks focusing on signaling design aspects (e.g., components with candidate values for reporting, Type, Need of xDD/FRx differentiation) as well as whether we.

| Company | Comment |
| --- | --- |
| Ericsson | Prefer the original version compared to reformulated version. Reformulated version is proposing to allow reporting of two variants of Case1 HARQ timing with TDD PCell for Rel16 (with/without transmissions outside the ‘UL’ subframes from reference pattern). Prefer to have only one variant. |

- FG18-2b: Support of HARQ-offset for SUO case1 in EN-DC with LTE TDD PCell for type 1 UE

Based on [1] and the agreements, FG18-8 can be defined as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type ( 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18. MR-DC/CA enhancement | 18-2b | Support of HARQ-offset for SUO case1 in EN-DC with LTE TDD PCell for type 1 UE | Support of HARQ-offset for SUO case1 in EN-DC with LTE TDD PCell for type 1 UE | TBD | Yes | N/A |  | Per band combination | N/A | N/A | N/A |  | Optional with capability signaling |

Companies are encouraged to provide feedbacks focusing on signaling design aspects (e.g., components with candidate values for reporting, Type, Need of xDD/FRx differentiation).

| Company | Comment |
| --- | --- |

Following feedbacks are provided in contributions for the RAN1#100bis-e meeting.

- Conclusion

Following is FL proposal based on the conclusion made in the conference call:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18. MR-DC/CA enhancement | 18-2 | Single UL TX operation for TDD PCell in EN-DC | TDM restriction to LTE TDD PCell in EN-DC for single UL-Transmission associated functionality when tdm-patternConfig-r16 is configured 1) TDD UL/DL configuration#2, #4, #5 configured as DL-reference UL/DL configuration 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern (for type 1 UE) [4) if UE indicates that it does not support simultaneous UL transmissions as defined in TS 38.101-3 [4] using singleUL-Transmission, NR (SCG) UL transmission is dropped when an overlapping LTE (MCG) UL transmission is present (for type 1 UE).”] | EN-DC (TBD) | Yes | N/A |  | Per band combination | Applicable to TDD-TDD EN-DC only | Applicable to FR1 only |  | Extension of the R15 capability tdm-Pattern to TDD PCell | Optional with capability signalling |
|  | 18-2a | Enhanced single UL TX operation for FDD Pcell EN-DC | TDM restriction to LTE FDD Pcell in EN-DC for single UL-Transmission associated functionality when tdm-patternConfig-r16 is configured 1) DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern (for type 1 UE) [4) if UE indicates that it does not support simultaneous UL transmissions as defined in TS 38.101-3 [4] using singleUL-Transmission, NR (SCG) UL transmission is dropped when an overlapping LTE (MCG) UL transmission is present (for type 1 UE).”] | 6-13 (TBD) | Yes | N/A |  | Per band combination | Applicable to in FDD-LTE -NR EN-DC | Applicable to FR1 only |  | Enhancement to the R15 capability tdm-Pattern | Optional with capability signalling |
|  | 18-2b | Support of HARQ-offset for SUO case1 in EN-DC with LTE TDD PCell for type 1 UE | Support of HARQ-offset for SUO case1 in EN-DC with LTE TDD PCell for type 1 UE | TBD | Yes | N/A |  | Per band combination | N/A | N/A | N/A |  | Optional with capability signaling |
|  | 18-3 | Dual Tx transmission for EN-DC with FDD PCell(TDM pattern for dual Tx UE) | TDM restriction to LTE FDD PCell in EN-DC for dual UL Tx operation when tdm-patternConfig-r16 is configured 1) DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern | EN-DC (TBD) | Yes | N/A |  | Per band combination | Applicable to EN-DC with LTE FDD PCell only | Applicable to FR1 only |  | Extension of the R15 capability tdm-Pattern to a 2Tx UE | Optional with capability signalling |
|  | 18-3a | Semi-statically configured LTE UL transmissions in all UL subframes not limited to tdm-pattern | UE configured with tdm-patternConfig-r16 can be semi-statically configured with LTE UL transmissions in all UL subframes not limited to the reference tdm-pattern (only for type 1 UE) | 18-2, 18-2a (TBD) | Yes | N/A |  | [Per band combination] | Applicable to EN-DC only | Applicable to FR1 only |  |  | Optional with capability signaling |

References

[1]	R1-2001484	RAN1 UE features list for Rel-16 NR after RAN1#100-E	Moderator (AT&T, NTT DOCOMO, INC.)

[2]	R1-2001631	Discussion on UE feature for MR-DC CA	ZTE

[3]	R1-2001833	Views on Rel-16 UE features for MR-DC/CA	MediaTek Inc.

[4]	R1-2002024	UE feature for MR-DC	Intel Corporation

[5]	R1-2002426	Discussion on UE features for MR-DC	Ericsson

[6]	R1-2002477	On UE features for MR-DC/CA	Nokia, Nokia Shanghai Bell

[7]	R1-2002571	Discussion on UE features for MR-DC/CA	Qualcomm Incorporated

[8]	R1-2002595	Rel-16 UE features for MR-DC/CA	Huawei, HiSilicon

[9]	R1-2002885	Summary on Email discussion [100b-e-NR-UEFeatures-MRDCCA-03]	Moderator (NTT DOCOMO, INC.)