3GPP TSG RAN WG1 Meeting #100bis-e	R1-2002926

E-Meeting, April 20 – 30, 2020

Agenda Item:	6.2.2.4

Source:	Moderator (Huawei)

Title:	Feature lead summary #1 on 100b-e-LTE-NB_IoTenh3-Coex-NR-02

Document for:	Discussion and Decision

Introduction

This document provides the proposals and summary of discussions of the following second phase email discussion for identified issues of NB-IoT co-existence with NR [10].

[100b-e-LTE-NB_IoTenh3-Coex-NR-02] Transmission of DMRS in resource reservation and corresponding TP (if any) by 4/30 – Yubo (Huawei)

- Issues #2 in R1-2002700

Discussion

## Issue #2: UL DMRS

For the spec “In a slot that is fully reserved, the demodulation reference signal transmission is dropped.” [2] points out that it may be misunderstood to drop DMRS in a subframe that is fully reserved, but actually NPUSCH in this subframe is postponed. For the spec “In a SC-FDMA symbol that is reserved, the demodulation reference signal transmission is dropped.”, [2] [5] propose to consider 3.75kHz case as it is not clear whether the whole DMRS or only the overlapped part of DMRS is dropped. And [5] proposes to correct some typo.

The corresponding TP proposed by [2] is as below.

-----------------------------------------------------Start of Text Proposal---------------------------------------------

#### 10.1.4.2	Mapping to physical resources

Unchanged part is omitted

If higher layer parameter valid-subframe-config-UL or slot-reserved-resource-config-UL is configured, then in case of NPUSCH transmission format 1 associated with C-RNTI or SPS C-RNTI using UE-specific NPDCCH search space and the Resource reservation field in the DCI is set to 1, or in case of NPUSCH format 2 transmission associated with C-RNTI using UE-specific NPDCCH search space,

-	In a subframe that is partially reserved, the demodulation reference signal transmission in a SC-FDMA symbol overlapping with the reserved symbols is dropped.

-----------------------------------------------------End of Text Proposal---------------------------------------------

The corresponding TP proposed by [5] is as below.

------------------------------------------------------------- Start -------------------------------------------------------------

If higher layer parameter valid-subframe-config-UL or slot-reserved-resource-config-UL is configured, then in case of NPUSCH format 1 transmission associated with C-RNTI or SPS C-RNTI using UE-specific NPDCCH search space and the Resource reservation field in the DCI is set to 1, or in case of NPUSCH format 2 transmission associated with C-RNTI using UE-specific NPDCCH search space,

-	In a slot that is fully reserved, the demodulation reference signal transmission is dropped.

-	In a SC-FDMA symbol that is reserved, the demodulation reference signal transmission is dropped.

-------------------------------------------------------------- End -------------------------------------------------------------

| Companies | Comments |
| --- | --- |
| ZTE | We are fine with 2nd TP proposed by [5]. For the 1st TP proposed by [2], NPUSCH transmission have 3.75 KHz and 15 kHz subcarrier spacing and NPUSCH transmission in overlapped slots is postponed until the next slots not overlapping. The postponement for 3.75 kHz SCS case would be done in unit of slots instead of subframe. |
| Nokia, NSB | We are fine with both TPs |
| Ericsson | We propose the following alternative 36.211 TP for the UL DMRS related clause 10.1.4.2, aligning it with the formulation in NPUSCH clause 10.1.3.6. <Unchanged parts are omitted> If higher layer parameter valid-subframe-config-UL or slot-reserved-resource-config-UL is configured, then in case of NPUSCH format 1 transmission associated with C-RNTI or SPS C-RNTI using UE-specific NPDCCH search space and the Resource reservation field in the DCI is set to 1, or in case of NPUSCH format 2 transmission associated with C-RNTI using UE-specific NPDCCH search space, - In a subframe that is fully reserved, - for , the demodulation reference signal transmission is postponed until the next NB-IoT uplink subframe that is not fully reserved. - for , the demodulation reference signal transmission in the slot is postponed until the next slot spanning over two contiguous uplink subframes not overlapping with any uplink subframe that is fully reserved. - In a subframe that is partially reserved, the demodulation reference signal transmission in a SC-FDMA symbol that is reserved is dropped. <Unchanged parts are omitted> |
| Qualcomm | We are fine with the 2nd TP proposed by [5]. For the above alternative 36.211 TP proposed by Ericsson, we would like to make a small change to the last bullet since the resource reservation is configured for the SC-FDMA symbol of 15kHz SCS instead of 3.75kHz SCS. - In a subframe that is partially reserved, the demodulation reference signal transmission in SC-FDMA symbols overlapping with reserved symbols is dropped. |
| Huawei, HiSilicon | We support both TP. For the first TP, the problem is that if the whole subframe is reserved, the DMRS should be postponed instead of dropped. We can accept the alternative changes from Ericsson and QC combined as below, although we think the postpone part is not needed as in legacy the spec does not capture explicitly the DMRS postponing either. In addition, I guess some brackets are needed for the last bullet considering the number of DMRS and reserved symbols. <Unchanged parts are omitted> If higher layer parameter valid-subframe-config-UL or slot-reserved-resource-config-UL is configured, then in case of NPUSCH format 1 transmission associated with C-RNTI or SPS C-RNTI using UE-specific NPDCCH search space and the Resource reservation field in the DCI is set to 1, or in case of NPUSCH format 2 transmission associated with C-RNTI using UE-specific NPDCCH search space, - In a subframe that is fully reserved, - for , the demodulation reference signal transmission is postponed until the next NB-IoT uplink subframe that is not fully reserved. - for , the demodulation reference signal transmission in the slot is postponed until the next slot spanning over two contiguous uplink subframes not overlapping with any uplink subframe that is fully reserved. - In a subframe that is partially reserved, the demodulation reference signal transmission in SC-FDMA symbol(s) overlapping with reserved symbol(s) is dropped. <Unchanged parts are omitted> |
| ZTE | A subframe has two slots for 15 kHz SCS while a slot includes two subframes for 3.75 kHz SCS. We think it would be better to separately describe 15 kHz and 3.75 kHz case. <Unchanged parts are omitted> If higher layer parameter valid-subframe-config-UL or slot-reserved-resource-config-UL is configured, then in case of NPUSCH format 1 transmission associated with C-RNTI or SPS C-RNTI using UE-specific NPDCCH search space and the Resource reservation field in the DCI is set to 1, or in case of NPUSCH format 2 transmission associated with C-RNTI using UE-specific NPDCCH search space, - for , - in a subframe that is fully reserved, the demodulation reference signal transmission is postponed until the next NB-IoT uplink subframe that is not fully reserved. - in a subframe that is partially reserved, the demodulation reference signal transmission in SC-FDMA symbols overlapping with reserved symbols is dropped. - for , - In a slot that is overlapping with any uplink subframe that is fully reserved, the demodulation reference signal transmission is postponed until the next slot spanning over two contiguous uplink subframes not overlapping with any uplink subframe that is fully reserved. - In a slot that is not overlapping with any uplink subframe that is fully reserved, the demodulation reference signal transmission in SC-FDMA symbol(s) overlapping with reserved symbol(s) is dropped. <Unchanged parts are omitted> |
| Ericsson 2 | If we go with ZTE’s TP, perhaps we should update the corresponding text in the PUSCH clause (10.1.3.6) as well so that they are aligned. |
| Huawei, HiSilicon 2 | Regarding ZTE’s TP, if we can make it clear, we prefer to have a more concise description, and in such way we can avoid to change the PUSCH wording as mentioned by Ericsson. As to my understanding, the following wording seems to cover both 3.75kHz and 15kHz cases. So it is appreciated that ZTE can elaborate more about the problem of following wording so that I can understand the proposed TP by ZTE. - In a subframe that is partially reserved, the demodulation reference signal transmission in SC-FDMA symbol(s) overlapping with reserved symbol(s) is dropped. |
| ZTE | According to the spec in 10.1.3.6 of 36.211: “In a subframe that is fully reserved, for , the NPUSCH transmission in the slot is postponed until the next slot spanning over two contiguous uplink subframes not overlapping with any uplink subframe that is fully reserved.”, if subframe #0 is fully reserved and subframe #1 is partially reserved (In above figure, symbol #2, #3, #4 in subframe #1 are reserved for 15 kHz SCS that is overlapping with DMRS symbol for 3.75 kHz SCS), then NPUSCH format 1 transmission in NB-slot #0 would be postponed. However, based on Huawei’s TP, DMRS transmission in NB-slot #0 would be dropped instead of postponement. |
| Huawei, HiSilicon | Thanks ZTE for the explanation, we understand the issue now. We would also agree that the NPUSCH may also need similar update. As parts for both NPUSCH and DMRS need changes, so maybe a more concise update is more preferrable. Please check the following update, which can have less update and more concise description. For DMRS <Unchanged parts are omitted> If higher layer parameter valid-subframe-config-UL or slot-reserved-resource-config-UL is configured, then in case of NPUSCH format 1 transmission associated with C-RNTI or SPS C-RNTI using UE-specific NPDCCH search space and the Resource reservation field in the DCI is set to 1, or in case of NPUSCH format 2 transmission associated with C-RNTI using UE-specific NPDCCH search space, - In a subframe for or a slot for that is overlapping with any fully reserved uplink subframe, - for , the demodulation reference signal transmission is postponed until the next NB-IoT uplink subframe that is not fully reserved. - for , the demodulation reference signal transmission in the slot is postponed until the next slot spanning over two contiguous uplink subframes not overlapping with any uplink subframe that is fully reserved. - In a subframe for or a slot for that is not overlapping with any fully reserved uplink subframe, the demodulation reference signal transmission in SC-FDMA symbol(s) overlapping with reserved symbol(s) is dropped. <Unchanged parts are omitted> For NPUSCH: <Unchanged parts are omitted> If higher layer parameter valid-subframe-config-UL or slot-reserved-resource-config-UL is configured, then in case of NPUSCH format 1 transmission associated with C-RNTI or SPS C-RNTI using UE-specific NPDCCH search space with the Resource reservation field in the DCI set to 1, or in case of NPUSCH format 2 transmission associated with C-RNTI using UE-specific NPDCCH search space, - In a subframe for or a slot for that is overlapping with any fully reserved uplink subframe, - for , the NPUSCH transmission is postponed until the next NB-IoT uplink subframe that is not fully reserved. - for , the NPUSCH transmission in the slot is postponed until the next slot spanning over two contiguous uplink subframes not overlapping with any uplink subframe that is fully reserved. - In a subframe for or a slot for that is not overlapping with any fully reserved uplink subframe, the SC-FDMA symbols overlapping with reserved symbols shall be counted in the NPUSCH mapping but not used for transmission of the NPUSCH. <Unchanged parts are omitted> |

Summary

- References

- R1-1913595, “RAN1 agreements for Rel-16 Additional Enhancements for NB-IoT”, Futurewei, Reno, USA, November 2019.

- R1-2001572	Corrections on coexistence of NB-IoT with NR	Huawei, HiSilicon

- R1-2001856	Remaining issues on NB-IoT resource reservation	ZTE

- R1-2002177	Coexistence of NB-IoT with NR	Qualcomm Incorporated

- R1-2002509	Corrections for NR coexistence performance improvements for NB-IoT	Ericsson

- R1-2002647	Remaining issues for co-existence of NB-IoT with NR	Nokia, Nokia Shanghai Bell

- R1-2001518  LS on NR coexistence, From: RAN2, To: RAN1, e-Meeting, April 20th-30th, 2020.

- R1-2001848	Discussion on RAN2 LS on NR coexistence	ZTE

- R1-2002502	On the LS on NR coexistence for NB-IoT/eMTC	Ericsson

- R1-2002700	Feature lead summary #1 on coexistence of NB-IoT with NR	Moderator(Huawei)